CL-GD546X 
Display Drivers and Utilities

User Guide

Product: GDS546X-X-D120P

Software Revision: 1.20

Release Date: October 4, 1996

Notice

CIRRUS LOGIC, Inc. believes the information contained in this document is accurate and reliable. However, CIRRUS LOGIC, Inc. reserves the right to make corrections, improvements, or changes to this document at any time and without notice. No responsibility is assumed by CIRRUS LOGIC, Inc. for its use; nor for infringements of patents or other rights of third parties. This document implies no license under patents or copyrights, other than those explicitly stated.

This document is copyrighted. This document may not, in whole or part, be copied; photocopied, reproduced, translated, or reduced to any electronic medium or machine readable form without prior consent, in writing, from CIRRUS LOGIC, Inc.

© Copyright 1996, CIRRUS LOGIC, Inc.
All rights reserved.

Trademarks

Windows, MS-DOS, and MS Word are trademarks of Microsoft Corporation. AutoCAD, AutoShade, ADI and Autodesk are registered trademarks of Autodesk, Inc. Display List Driver-VGA, TurboDLDClassic, panaStation Classic and CustomColors are trademarks of Spacetec IMC Corporation. Panacea is a registered trademark of Spacetec IMC Corporation. PC-DOS, OS/2 and Presentation Manager are trademarks of IBM Corporation. MicroStation is a trademark of Intergraph Corporation and Bentley Systems. RenderMan is a registered trademark of Pixar. VESA, VBE/PM, and DPMS are trademarks of Video Electronics Standards Association.  All other brand and product names are trademarks or registered trademarks of their respective companies.

FCC Information

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

This device complies with Part 15 of FCC Rules. Operation is subject to the following two conditions: (1) This device may not cause harmful interference and, (2) this device must accept any interference received, including interference that may cause undesired operation.

Notice to user: Changes or modifications to this product not approved by the party responsible for FCC compliance could void your authority to operate this equipment.

In order for an installation of this product to maintain compliance with the limits for a Class B device, shielded cables must be used for the connection of any devices external to this product.

Contents

Introduction

Before You Begin

Conventions Used

Package Contents

Technical Support

Software Upgrades

Installation

Preparation

Setup

Opening Your Computer

Installing the Adapter

Configuring Your Computer

Utility Software

TSRFONT Utility

Installing the CLMode and TSRFONT Utilities

Using the CLMode Utility

Viewing the Current VGA Controller Status

Selecting and Configuring the Monitor Type

Previewing the Video Modes

Getting Help and Information

Exiting the CLMode Utility

CLMode Command Line Options

WinMode Utility

Display Drivers

Before You Begin

Microsoft Windows 3.1 Driver

Windows 3.1 Installation

Using the WinMode Utility

Colors

Resolution

Monitor Brand and Model

Monitor Refresh Rates

Operating System

Font Size

About

Options

Help

Cancel

OK

The Energy Star Program

VESA VBE/PM

Microsoft Windows NT Drivers

Windows NT 3.5x Installation

Selecting Resolution and Color Depth

Windows NT 4.0 Installation

Configuring the Windows NT 4.0 Driver

Microsoft Windows 95 Driver

Windows 95 Installation

Configuring the Windows 95 Driver

OS/2  3.0 (Warp) Driver

OS/2  3.0 Installation

Configuring the OS/2 Driver

Autodesk ADI 4.2 Driver

Features of the TurboDLDClassic

Before You Begin

System Requirements

What TurboDLDClassic Does

What TurboDLDClassic Uses

TurboDLDClassic Installation

Reconfiguring AutoCAD

Configuring for AutoCAD  Release 12 and 
   13 (AVE Render)

Configuring for AutoCAD Release 11

Configuring for AutoShade 2.0 with RenderMan

Configuring for 3D Studio

Configuring the TurboDLDClassic Driver

TurboDLDClassic Configuration Options

Browsing the TurboDLDClassic

Saving Configuration Information

Verifying Your TurboDLDClassic Installation

Reconfiguring TurboDLDClassic

AutoCAD Release 12 and 13

AutoCAD Release 11

Changing Colors

TurboDLDClassic Commands

Memory Usage and Lists

Memory Needed

Keeping the Display List Small

Tips and Tricks

Switching Color Modes

Zoom Dynamic

Zooming and Snap

Upgrading from Older Panacea DLD Drivers

Using AutoCAD Commands

Static BP in Panning Mode

ZOOM $1

Command Summary

Troubleshooting

Sticky Cursors

Third-Party Software

Memory Management

TurboDLDClassic and Paper Space

Slow Scrolling

Corrupt TurboDLDClassic Data Files

Slow Mouse/Digitizer Response

MicroStation Driver

Features of the panaStation Classic

Before You Begin

panaStation Classic Installation

Reconfiguring MicroStation 4.x

Reconfiguring MicroStation 5.x

Configuring panaStation Classic

Selecting Graphics Board and Resolution

Adjust Aspect Ratio Menu

Saving Configuration Information

Reconfiguring panaStation Classic

This page has been left blank intentionally.

# Introduction

This manual provides operating instructions for user utilities and installation instructions for the display drivers supplied with your [PRODUCT NAME].

The [PRODUCT NAME] graphics system is VGA compatible. The utilities and drivers supplied with your [PRODUCT NAME] provide support for additional features of your [PRODUCT NAME], such as greater speed, higher resolution, and more available colors.

## Before You Begin

The following instructions assume that the user is familiar with DOS and certain DOS commands. Please review the associated DOS commands before performing the installation.

## Conventions Used

Throughout this manual, the term 'DOS' refers to both MS-DOS and PC-DOS, except when noting features that are unique to one or the other. Table 1 shows the typographic conventions that are used throughout this manual.

Table 1:  Conventions Used

| Type Style | Description |
| --- | --- |
| User Input | This text must be typed exactly as it appears. |
| [F10] | Text within brackets indicates certain keyboard keys (such as [Enter], [F1], and so on). |
| Bold | System output. Any message that is displayed by the computer. |
| ALL CAPITALS | Directory names, files and acronyms. |

## Package Contents

Before installing your [PRODUCT NAME], check to see that your package contains the following items:

[Insert package contents here.]

Contact your dealer if you find anything missing or damaged, or you find that you need another size diskette for your [PRODUCT NAME].

## Technical Support

[Insert technical support information here.]

## Software Upgrades

[Insert software upgrade information here.]

# Installation

Follow the detailed procedures described in this section for easy installation of the [PRODUCT NAME] into your computer system.

## Preparation

The [PRODUCT NAME] adapter may or may not be shipped with the zero-wait-state jumper installed. If so, the zero-wait-state jumper will be set to not assert the zero-wait-state line. You may achieve better performance in some systems by installing this jumper. If any problems are encountered with zero-wait-state enabled, remove the jumper to disable this feature.

Before you begin the actual installation of your [PRODUCT NAME], you may need some or all of the following tools:

- Medium size flat blade screwdriver.

- Medium size Phillips head screwdriver.

- A 3/16 inch nut driver or wrench.

## Setup

The 386, 486, and Pentium class computers use a program typically called Setup to tell the computer what equipment is installed in your system. The Setup program is usually part of the package of programs provided with your PC compatible computer. The 'Guide To Operations' (or equivalent manual) will explain the use of the Setup program. After the adapter is installed, Setup will need to be run to configure the computer for proper operation with the [PRODUCT NAME] card.

NOTE:	When using a memory manager, be aware that using the memory ranges B000 - B7FF will conflict with monochrome modes. The conflict could occur even if you are not using a monochrome monitor but an application (such as CLMode, Microsoft Windows compatibility tests, HIMEM, EMM386 and QEMM) which goes into monochrome mode causing your system to hang.

## Opening Your Computer

To begin installing your adapter into the computer, follow the procedure listed below.

- You must first open your computer system. Before opening the system, be sure that the power to the computer system unit and all devices connected to it is turned off.

- Disconnect the cables from the back of the system in order to give more room to work. Note how all cables are connected prior to disconnection.

- In a typical PC computer all cover mounting screws are located on the back of the computer. Remove these screws and save them.

- Carefully remove the cover. On most computers the cover will slide forward or backward, or tilt off.

- Choose an expansion slot for the [PRODUCT NAME] adapter card.

- Your adapter must be plugged into a slot that exactly matches its connector.  These connectors can include ISA, VLB, and PCI.  You must be certain to choose the correct type of slot.

## Installing the Adapter

After opening your computer system, you are now ready to install your adapter.

- Check that the power is turned off.

- If necessary, remove a system expansion slot cover by removing its retaining screw and lifting it out. Save the screw.

WARNING! Your [PRODUCT NAME] is static sensitive. Before you install the [PRODUCT NAME], touch a metal surface of your computer, while it is plugged in, to ground yourself and dissipate any static electricity.

- Hold the [PRODUCT NAME] adapter by its top corners and slide it into the system unit. Make sure that it is correctly seated in the expansion slot.

- Secure the card with the screw that you removed in the step above.

- Are the computer system's switches and jumpers set correctly? Check the user's manual for your computer for any additional configuration instructions.

- Replace and secure the system cover.

- Plug a VGA compatible or multi-frequency display into the 15 pin video connector at the back of the [PRODUCT NAME] adapter.

IMPORTANT NOTE: If you are using a multi-frequency monitor, you may need a special 15 pin PS/2 compatible adapter cable, and you will need to make sure that your multi-frequency monitor is configured properly for a PS/2 compatible analog VGA signal. Some multi-frequency displays have a switch to select 'TTL' or 'analog' operation; set this switch for 'analog'. Check the manual that came with your monitor for additional information on configuring it.

## Configuring Your Computer

After the card is installed, use the Setup program supplied with the system to tell the computer about the equipment installed. When using Setup programs with the [PRODUCT NAME] adapter, please follow the steps listed below.

- Run Setup to configure the computer. Sometimes this utility comes on a disk, sometimes it is built in to your system's ROM BIOS.

- When you arrive at the part of the program where video support is specified, the program may ask you if the monitor you are looking at will be the primary monitor, answer Yes.

- If you are presented with a list of video options, select the option for VGA. If no option for VGA is listed, select support for EGA. The [PRODUCT NAME] adapter will look to many systems like an EGA card. If you do not see an option for EGA, try the option for NO MONITOR, RESERVED or SPECIAL. These options will usually work. If not, see your equipment dealer.

- At the conclusion of the Setup program, the computer will restart as if you had just turned on the power.

# Utility Software

There are three utility programs that are supplied with your [PRODUCT NAME]: CLMode, TSRFONT and WinMode. CLMode is a DOS utility supplied with your [PRODUCT NAME] to configure it's various options, allowing the user to define the type of monitor attached, set the video modes supported by the [PRODUCT NAME] and preview the different video modes. The TSRFONT program is used to load the 8x14 font that is not included in the VGA BIOS. WinMode lets the user change the operating resolution and the number of colors from within Windows 3.x.  A brief description of the WinMode utility is included in this section.  However, the WinMode utility is described in detail in the Microsoft Windows 3.1 Driver section where it is installed and used.

## TSRFONT Utility

The TSRFONT program is used to load the 8x14 font that is not included in the VGA BIOS image. To installed the TSRFONT program, follow the instructions in the Installing the CLMode and TSRFONT Utilities section below. After installation, TSRFONT can be executed from the DOS command line or from AUTOEXEC.BAT at boot time.

## Installing the CLMode and TSRFONT Utilities

CLMode and TSRFONT use the same install program described below.

- Insert the Utilities diskette, Driver Disk 1,  into Drive A: (or B:). If you want to install the Japanese or Chinese version of the [PRODUCT NAME] utilities, insert the Japanese Utilities diskette, Driver Disk 2, into Drive A: (or B:).

- Run the installation program (INSTALL.EXE) by typing 
	A:\INSTALL
at the DOS prompt.

- After the Introduction screen, select a destination drive and directory or use the C:\CLUTIL default and press [Enter].

- Select which utility you want to install by using the up and down arrow keys and tagging your selection using the space bar on or off. After you have made your selection(s), press [Enter].

- All files will automatically be copied onto your designated drive. Follow the on-screen prompts to complete the installation.

If you chose to install CLMode, it will begin automatically after installation is complete.

To use CLMode again after exiting, type CLMODE where you installed the [PRODUCT NAME] utilities or at the default C:\CLUTIL directory.

## Using the CLMode Utility

CLMode is a DOS utility and should only be run from the DOS prompt and not under an application shell such as Microsoft Windows 95 or Windows 3.1. To start and configure the CLMode utility, type the following at the DOS prompt where the CLMode directory is located (default is CLUTIL):

CLMODE [Enter]

The main pop-up dialog box (Screen Refresh Setup) consists of a number of buttons.  Each button represents a different command, option or list.  The underlined letter of a button name along with the [Alt] key specifies the ‘hot key’ combination for that item.  Hot key combinations are key combinations used to change from one session or function on a workstation to another. For example, pressing the [Alt] and the underlined letter keys simultaneously selects an option.  Note that to use a mouse, a mouse driver should be installed prior to running the CLMode utility in order to use the mouse pointer for button selection.

The various options you can configure in CLMode are described below.

IMPORTANT NOTE:  The Options button, used with specific chips and configurations, is not available for use.

### Viewing the Current VGA Controller Status

The information in the main CLMode dialog box displays the BIOS version number, the amount of video memory present and whether or not EEPROM is present.

### Selecting and Configuring the Monitor Type

Selecting the proper monitor type  allows the highest quality output that your [PRODUCT NAME] is capable of with the attached monitor.  The monitor type determines what video modes are available to your system.  It also determines the vertical refresh rates available.  Generally, the higher the refresh, the better the screen will look.

To quickly search for your monitor in the Screen Refresh Setup dialog box, press the first letter of your monitor name and use the down arrow key to scroll through the alphabetical listing. If you cannot find your monitor in the list presented, scroll to the bottom of the list and select Other.

Once you have chosen a monitor brand, select a monitor model by clicking on the Monitor Model scroll button to make your selection. Depending on the monitor brand and model you selected, the monitor refresh rates change accordingly. CLMode automatically determines the highest refresh rate for your monitor and selects it. If you want to select different refresh rates, click on the Monitor Refresh Rates scroll buttons to choose from the other refresh rates available for your monitor and model selection.

### Previewing the Video Modes

Before saving the new monitor type, you may want to select the Preview button to view each of the new video timings that you have selected to verify that they are compatible with your monitor.  Select the Preview button in the Screen Refresh Setup dialog box.  The Modes Preview window displays all the modes supported according to the monitor type attached and the amount of video memory present.  This list of video modes will tell you which are available in your current configuration for use with extended resolution drivers.

To see what different video modes look like on your monitor, select the Show button.  After each test screen is displayed, press [Enter] to view the next video mode screen.  If you do not want to see any more video modes, press [ESC] to return to the Modes Preview window. Press the Quit button to go back to the Screen Refresh Setup dialog box.

If there is a display problem when viewing your test screen, press [ESC] to return to the Screen Refresh Setup dialog box.  Try a slower setting for the resolution that did not work and try verifying your choices again.

NOTE:	When using a memory manager, be aware that using the memory ranges B000 - B7FF will conflict with monochrome modes. The conflict could occur even if you are not using a monochrome monitor but an application (such as CLMode, Microsoft Windows compatibility tests, HIMEM, EMM386 and QEMM) which goes into monochrome mode causing your system to hang.

### Getting Help and Information

CLMode provides on-line help for selecting the correct video modes by clicking on the Help button in the Screen Refresh Setup dialog box. The Video Modes help window defines the information displayed in the dialog box. If you want to know what version of CLMode you are using, clicking the About button displays a window with the CIRRUS LOGIC copyright message and the CLMode version number.

### Exiting the CLMode Utility

If you want to discard any changes before you exit and revert back to the last saved monitor setting, press the Undo button. You can exit CLMode at any time by pressing the [Alt] and [F4] keys simultaneously, clicking the top left corner mouse button of the main window, or selecting the Exit button. If you have made setting changes in CLMode, upon exiting CLMode an AUTOEXEC.BAT Update window appears. Choose Yes if you want to save your new settings and update your AUTOEXEC.BAT file. Choose No if you just want to exit CLMode and not save or update anything.

## CLMode Command Line Options

When command line options for CLMODE.EXE are given at the DOS prompt, the menu-driven windows will not be displayed.  Instead, video mode and refresh rate will be set at the DOS prompt.  The command line options for CLMODE.EXE are listed below:

CLMODE {modenum{+|*|-}} {refresh} [Enter]

Where:

{modenum{+|*|-}} = video mode number

and	+	selects 400 scan lines (default)
			*	selects 350 scan lines
			-	selects 200 scan lines

{refresh} = monitor refresh rates

t640=x(Hz)	640x480 @ (x=60, 72, 75, 85)

t800=x(Hz)	800x600 @ (x=56, 60, 72, 75, 
				85, 0=unavailable)

t1024=x(Hz)	1024x768 @ (x=87, 60, 70, 
				75, 85, 0)

t1280=x(Hz)	1280x1024 @ (x=87, 60, 72, 
				75, 85, 0)

t1600=x(Hz)	1600x1200 @ (x=48, 60, 65, 
				70, 75, 80, 85, 0)

For example, to set video mode 3 type:

CLMODE 3 [Enter]

For example, to select custom monitor timings with 640x480 at 60 Hz and 800x600 at 72 Hz refresh type:

CLMODE t640=60 t800=72 [Enter]

Typing an invalid option will display the command line help text.  Typing [S] as a command line option will display current CLMode information and settings.

## WinMode Utility

The WinMode utility configures your graphics system for Windows 3.1 in either DOS or OS/2 3.x. It allows the user to change the monitor brand and model, the monitor refresh rates, number of available colors, resolution, and large or normal size fonts for Windows 3.x.

After new options have been selected, the user can either immediately restart Windows, in which case the new resolution will take effect immediately, or continue working in the current resolution, in which case the new resolution will take effect the next time Windows is started. In some configurations the AUTOEXEC.BAT file needs to be modified to make the changes permanent.

WinMode assumes that the Windows driver has been correctly installed and configured using the installation utility provided on the ‘Windows 3.1 Driver’ Diskette. For installation instructions and a complete description of the WinMode options, see the section on Microsoft Windows 3.1 Driver in this manual.

# Display Drivers

The video controller is VGA compatible.  The display drivers described in this manual are supplied to improve the resolution for each supported software application package. It can support improved text resolution, providing greater readability when using the supplied drivers in text-based word processing programs.  It also supports higher graphics resolutions, providing greater detail when using the supplied drivers in graphics-based programs.

In each application section, there is a brief introduction describing the application and the revision level of the application supported by the supplied display drivers.

The installation instructions for each display driver will follow the introduction section.  Follow the instructions carefully to be sure that each display driver is correctly installed.  All of the installation instructions assume that the Driver Diskette is located in drive A:.  If drive B: is used, the instructions should be changed appropriately.

The installation utility (INSTALL.EXE) should be used to copy display drivers directly to the appropriate application directories where they may be configured by the application software.

## Before You Begin

It is assumed that the user is familiar with DOS and certain DOS commands.  Please review the installation instructions and the associated DOS commands before attempting the actual installation.

For the OS/2 3.0 (Warp) driver, it is assumed that the user is familiar with OS/2 and certain OS/2 commands. Please review the installation instructions and the associated OS/2 commands before attempting the actual installation.

# Microsoft Windows 3.1 Driver

The Windows 3.1 driver installation utility copies all of the graphics driver and utility files to your hard disk. It also allows you to configure your graphics system for Windows 3.1 in either DOS or OS/2 v3.x. You can change the monitor refresh rates, resolution, number of available colors, large or normal size fonts and font cache size.

After new options have been selected, you can either immediately restart Windows, in which case the new configuration will take effect immediately, or you can continue working in the current resolution, in which case the new resolution will take effect the next time Windows is started. In some configurations the AUTOEXEC.BAT file needs to be modified to make the changes permanent.

The display driver supports both Windows versions 3.1 and 3.11 and also Windows for Workgroups version 3.11. To load the display driver with Windows 3.11 or Windows for Workgroups 3.11, you must install the VGA driver first before installing the display driver.  The display driver will only run in enhanced mode and not in standard mode.

Before starting, make a backup copy of Windows 3.1 Driver disk using the DOS DISKCOPY command, and put it in a safe place.

## Windows 3.1 Installation

To run the installation program follow the installation procedure listed below.

- Start Windows 3.1 and insert the diskette labeled 'Windows 3.1 Driver’ into your disk drive.

- From the Windows Program Manager select Run from the File menu.

- Type the letter of the disk drive that the driver diskette is in, followed by the word Install. For instance, if the driver disk is in drive A:, type 
	A:\INSTALL 
and click on the OK button.

- In the first dialog box that is displayed, you can set the path where you would like the utility programs to be installed. If you want to use the default directory, click on Continue. To change to another directory, you can either type the path name or you can click on the down arrow and select a directory for installation. The drop down box works just like the Directory field in a file open dialog box. After you select the directory, press the Continue button.

- After the driver has been copied to your hard disk, the install program will automatically execute the WinMode utility. See Using the WinMode Utility section below for a complete description of all the WinMode features.

You may reconfigure the Windows driver at any time by running the WinMode utility. Under Windows, select the WinMode icon to reconfigure your driver.

## Using the WinMode Utility

WinMode is run by selecting it's icon.  The icon will be in the group that you specified during the install process. When WinMode is started, the screen may blank for up to two seconds. This happens while WinMode attempts to discover the capabilities of the monitor. If this is successful, you will see only one choice besides the other brand and model choices in the Monitor Brand and Monitor Model drop down combo boxes. The various options you can configure in WinMode are described below.

IMPORTANT NOTE:  The Font Cache Size option in the WinMode dialog box is not available at this time.

### Colors

These buttons let you choose the number of colors that will be available to Windows 3.1 after Windows is restarted. Some of these choices may be unavailable. This is because of the amount of video memory in your system and the resolution that you have selected. If you need more colors, there will be some slowdown in graphics performance. If you chose OS/2-Windows in the Operating System field, then the only possible choice is 256 colors.

### Resolution

These buttons let you choose the resolution that WinMode will use to run Windows 3.1 after Windows is restarted. Some of these buttons may be unavailable because of the amount of video memory in your system or if some of the monitor refresh rates have been set to Unavailable.

### Monitor Brand and Model

The Monitor Brand and Monitor Model drop down list boxes reflect which monitor setup you are currently using and the vertical refresh rates available.  Generally, the higher the refresh, the better the screen will look.

To change your monitor selection, click the scroll button and use the arrow keys to scroll through the alphabetical listing. Once you have chosen a new monitor brand, select a monitor model by clicking on the Monitor Model scroll button to make your selection. Depending on the monitor brand and model you selected, the monitor refresh rates change accordingly.

### Monitor Refresh Rates

The drop down list boxes let you select the monitor refresh rates for each resolution for your monitor selection. If you select None for any screen resolution, any higher resolutions will be unavailable. This will also turn off the corresponding choices in the Resolution box.

In general, the higher the refresh rate, the better the display quality and the lower the performance. This is because the graphics system can only do a fixed number of operations per second. The more time it spends redrawing the screen, the less time it has available to perform other operations. Consult the manual that came with your monitor to determine the proper settings for these fields.

### Operating System

The Operating System buttons let you set the operating system that you are running Windows in. This is very important because the configuration and capabilities of the driver are very different in OS/2 v3.x and DOS. The most obvious difference is that the driver will only work in 16 and 256 color modes in OS/2. There are a number of other differences in the configuration of the driver. Note that prior to installing OS/2 for Windows, you will need to run WinMode and select the OS/2 v3.x option.

### Font Size

The small fonts are intended for lower resolutions and higher resolutions on large monitors. With smaller monitors, the large fonts are more readable at higher resolutions.

### About

The About button will display a dialog box with the version number of WinMode and a copyright notice.

### Options

The Options button in the WinMode dialog box brings up the Display Options window to select Device Bitmaps and Virtual Screen. The Help button provides information about the options. Selecting the Cancel button takes you back to the WinMode dialog box. The About button displays the Option files and a copyright notice. Choosing the OK button closes the Display Options window and accepts any changes you have made.

### Help

For help on WinMode, select the Help button. The main help screen contains an image of the WinMode screen. Place the cursor over the field(s) that you need help with. When the mouse pointer changes to a hand, click to display the help text for that field. Click again to make the pop-up text disappear.

### Cancel

The Cancel button closes the dialog box and exits the program without making any changes. Selecting Close from the control menu or double clicking on the control menu box will have the same effect.

### OK

The OK button closes the dialog box and accepts the choices that you have made.

After clicking OK, your computer will be reconfigured to use the choices that you have made. These changes may need to be added to your AUTOEXEC.BAT file for them to be permanent. If this is necessary, you will be prompted by the program.

If WinMode detects that you changed your Windows 3.1 configuration, you will be asked if Windows should be restarted. If you answer Yes, Windows will restart immediately,. If you answer No, then you will have to exit Windows and restart it manually for the changes to take effect.

### The Energy Star Program

The Energy Star is a program created by the US Environmental Protection Agency (EPA) to promote energy efficiency. The goal of this program is to lower electricity usage by making computers and related hardware more energy efficient.

Computers currently use an estimated 5% of commercial electricity consumption. If no actions are taken, this could rise to 10% by the year 2000. Ironically, much of this electricity is wasted. Research shows that the vast majority of time personal computers are on, they are not actively in use. Additionally 30% - 40% are left running at night and on weekends.

Electricity generation accounts for 35% of all US emissions of carbon dioxide - the most prevalent greenhouse gas. It also accounts for 75% and 38% of all US emissions of sulfur dioxide and nitrogen oxides respectively - the two pollutants most responsible for acid rain.

By using more energy-efficient equipment in our homes, offices and factories, we can reduce this pollution - while saving money.

The Energy Star logo is used to mark computers, peripherals and software that have adopted the EPA’s power saving guidelines.

For more information on the Energy Star program contact:

Energy Star Computers
US EPA (6202J)
Washington, DC  20460, USA
Phone:  (USA) 202-233-9230    FAX:  (USA) 202-233-9578

### VESA VBE/PM

The VESA VBE/PM is a software interface to the Video Electronics Standards Association (VESA) Display Power Management Signaling (DPMS) standard. For more information on VESA and these specifications contact:

VESA
2150 North First Street
San Jose, CA  95131-2029, USA
Phone:  (USA) 408-435-0333    FAX:  (USA) 408-435-8225

# Microsoft Windows NT Drivers

The Microsoft Windows NT display drivers work with Microsoft Windows NT version 3.5x and version 4.0. Before starting, make a backup copy of the Windows NT Driver diskettes using the DOS DISKCOPY command, and put them in a safe place.

## Windows NT 3.5x Installation

Once Windows NT 3.5x has been installed on your system, follow the procedure listed below to install the Microsoft Windows NT 3.5x display driver.

- Select Control Panel from the Main group.

- Select the Display icon.

- Select the Change Display Type button in the Display Settings dialog box.

- Select the Change button from the Display Type dialog box.

- Select the Other button at the Select Device dialog box.

- Place the driver diskette labeled ‘Windows NT Drivers’ into Drive A: and type
	A:\NT35
then click OK. In the Models window the Cirrus Logic driver, Cirrus Logic VisualMediaä Accelerator is displayed.

- Next, select the Install button and click Yes to begin the driver installation.

- When the Windows NT Setup dialog box appears again, select Drive A:\NT35 and click Continue.

- When the Installing Driver dialog box appears click OK and click OK again at the next dialog box..

- You must now restart Windows NT 3.5x to use the Windows NT 3.5x display driver. If you select the Don’t Restart Now option, your Windows NT driver will not be installed until after you shut down your system.

- If, after installation, the Invalid Display Settings dialog box appears, click OK and verify or change the information viewed in the Display Settings dialog box. See the Selecting Resolution and Color Depth section below for additional information on display settings.

### Selecting Resolution and Color Depth

To select resolution and color depth for the Windows NT version 3.5x display driver, follow the procedure listed below.

- Select Control Panel from the Main group.

- Select the Display icon.

- Select Color Palette to change between 256 colors, 65,536 colors, 16,777,216 colors and True Color.

- NOTE:  Not all colors are available for all resolutions listed below.

- To select desktop resolution size, go to the Desktop area and use the slide bar to change resolution from 640x480, 800x600, 1024x768, 1280x1024 and 1600x1200.

- Select Test to test the resolution.

- If the display test screen was good then select Yes when the Test Mode dialog box appears. Windows NT 3.5x will prompt you to restart Windows NT 3.5x.

- If the display test screen was bad then select No.  Windows NT will give you an error message.

## Windows NT 4.0 Installation

Once Windows NT 4.0 has been installed on your system, follow the procedure listed below to install the Microsoft Windows NT 4.0 display driver.

- Right click on the Windows NT desktop (the Windows background where no windows or icons are displayed) and select Properties.

- At the top of the dialog box, click on the Settings tab.

- At the bottom of the Settings display screen, click on the Display Type button.

- Click on the Change button adjacent to the Adapter Type box.

- In the Change Display screen, click on the Have Disk button.

- Insert the driver diskette labeled ‘Windows NT Drivers’ in drive A: and follow the on-screen instructions.

- When Windows NT asks where the driver is located, type
	A:\NT40
The Windows NT 4.0 driver will be installed in the Windows NT System directory.

To configure the Windows NT 4.0 driver, follow the procedure listed in the next section, Configuring the Windows NT 4.0 Driver.

### Configuring the Windows NT 4.0 Driver

To configure your Windows NT 4.0 driver once it is installed, follow the procedure listed below.

- Right click on the Windows NT desktop (the Windows background where no windows or icons are displayed) and select Properties.

- From the Display Properties dialog box, you can change your driver and monitor configurations. Click on the Settings tab located at the top of the dialog box and adjust the settings.

- Follow the on-screen prompts to complete the Windows NT 4.0 driver or monitor configuration.

# Microsoft Windows 95 Driver

The Microsoft Windows 95 display driver works with Microsoft Windows 95 and supports 3D DirectDrawä. Before starting, make a backup copy of the Windows 95 Driver diskettes using the DOS DISKCOPY command, and put them in a safe place.

## Windows 95 Installation

Once Windows 95 has been installed on your system, follow the procedure listed below to install the Microsoft Windows 95 display driver.

- Right click on the Windows 95 desktop (the Windows background where no windows or icons are displayed) and select Properties.

- At the top of the dialog box, click on the Settings tab.

- At the bottom of the Settings display screen, click on the Change Display Type button.

- Click on the Change button adjacent to the Adapter Type box.

- In the Select Device display screen, click on the Have Disk button.

- Insert the driver diskettes labeled ‘Windows 95 Driver, 1 of 2’ in drive A: and follow the on-screen instructions. The Windows 95 driver will be installed in the Windows 95 System directory.

To configure the Windows 95 driver, follow the procedure listed in the next section, Configuring the Windows 95 Driver.

## Configuring the Windows 95 Driver

To configure your Windows 95 driver once it is installed, follow the procedure listed below.

- Right click on the Windows 95 desktop (the Windows background where no windows or icons are displayed) and select Properties.

- From the Display Properties dialog box, you can change your driver and monitor configurations.

- To change your Windows 95 driver configuration, click on the Refresh tab button located at the top of the dialog box and adjust the settings.

- To change your Windows 95 monitor configuration, click on the Settings tab located at the top of the dialog box and adjust the settings.

- Follow the on-screen prompts to complete the Windows 95 driver or monitor configuration.

# OS/2  3.0 (Warp) Driver

The OS/2  3.0 (Warp) driver is a driver to enhance the Graphical User Interface (GUI) of IBM operating systems. The OS/2 driver only works with IBM OS/2 version 3.0.

## OS/2  3.0 Installation

Follow the procedure listed below to install the OS/2  3.0 display driver. If you are installing OS/2 for the first time, install OS/2 for VGA first and reconfigure the driver later.

- Open an OS/2 Window.

- Place the driver diskette labeled ‘OS/2  3.0 (Warp) Driver’ into Drive A: and click OK.

- Change to the A: drive.

- At the A: prompt type
	SETUP C
where ‘C’ signifies your boot drive, then press [Enter].

- From the Display Driver Install dialogue box, check Primary Display and click OK.

- Next, verify the correct Source Directory and click Install to begin the installation process.

- From the Display Driver Install information box, click OK to proceed.

- The Setup Program will take you back to the OS/2 Window once the driver has been copied to your bootable drive. Type
	Exit
at the prompt and click OK.

- Remove the driver diskette from drive A:.

- Shutdown to restart OS/2 with your newly installed  OS/2 3.0 driver.

## Configuring the OS/2 Driver

To configure your OS/2 driver once it is installed, follow the procedure listed below.

- Click on the System icon in the System Setup folder.

- Select the Screen tab to display resolutions and screen refresh rates on page 1 of the System Settings dialog box.

- Select the resolution and refresh rate you want and follow the on-screen prompts.

- Shutdown and restart your system for these changes to take effect.

# Autodesk ADI 4.2 Driver

The ADI 4.2 driver provided with your [PRODUCT NAME] is the TurboDLDClassic display list driver from Panacea. It has two purposes:

- To speed up AutoCAD REDRAWs, PANs, and ZOOMs.

- To provide a more productive, user-friendly interface to AutoCAD (via the features described later in this manual).

The driver is memory-resident and inserts itself between AutoCAD and the graphics board. It has no other effect on AutoCAD’s operation besides increasing the speed of the program. It runs with AutoShade 2 with RenderMan and all versions of 3D Studio, but does not affect the speed of these programs, since they do not support Display Lists.

Installing TurboDLDClassic does not change any of the AutoCAD program files or alter any of the drawing files stored on disk. TurboDLDClassic was designed to be an easy-to-use, feature-filled product that makes using AutoCAD faster and more productive.

There are two things the TurboDLDClassic driver does to speed up AutoCAD:

- AutoCAD stores drawings in a hierarchical structure, with simple elements intermixed with complex ones. Every time the screen is updated, AutoCAD must decode this structure. TurboDLDClassic works differently. While you are working, it translates the normal hierarchical structure into a Display List, a series of vectors or polygon fills. When you pan or zoom, TurboDLDClassic uses the Display List, then writes the resulting vectors to the video board hardware. Since the hierarchical structure does not have to be decoded, drawing proceeds very quickly.

- TurboDLDClassic gives you numerous new user-interface features, such as the bird's eye view (via the features described later in this driver section).

The TurboDLDClassic driver is a display list driver. PANs and ZOOMs, aided by only the Display List, run from two to twelve times faster than a non-display list driver. The TurboDLDClassic’s internal display list processing can speed things up to the point that REDRAWs can be up to twenty-five times faster than the graphics drivers shipped with AutoCAD.

## Features of the TurboDLDClassic

The features provided by the TurboDLDClassic driver include:

- Bird's eye view and a graphics-mode text window.

- Accelerated redraws, pans, and zooms.

- Easy to use - no new commands or special menus to learn.

- Protected-mode ADI 4.2 driver - completely compatible with AutoCAD Release 13, 12, Release 11/386, 3D Studio and AutoShade 2 with RenderMan.

- No memory conflicts - works with AutoCAD's built-in Virtual Memory Manager

- Includes CustomColors™, allowing you to interactively customize your logical and physical colors from within AutoCAD.

- Completely compatible with all Autodesk ADI 4.2 compatible third party software.

- Supports higher resolutions on all popular graphics boards.

- Supports all AutoCAD Release 12 features, including rendering to viewports and 31-bit REGEN space.

## Before You Begin

Before starting, make a backup copy of the TurboDLDClassic driver disk using the DOS DISKCOPY command, and put it in a safe place. When installing the TurboDLDClassic driver, you should be familiar with the following abbreviated name references used throughout these driver instructions:

BP	=	Big Picture, TurboDLD's birds eye view 
R13	=	AutoCAD Release 13 
R12	=	AutoCAD Release 12 
R11	=	AutoCAD Release 11

Every AutoCAD installation is different. However each one requires the configuration of the same basic parameters in order to function. The examples used in this driver section assume that you have installed AutoCAD on drive C using the installation defaults. This means that your main AutoCAD directory is C:\ACAD and all other directories (such as, SUPPORT, FONTS, DRV, and so on) fall under it.

It is also assumed that you are using the AutoCAD created ACADR12.BAT file to start AutoCAD Release 12.

If you have not configured AutoCAD using defaults, you can still easily install TurboDLDClassic. All you have to do is substitute your specific sub-directory names with those in the examples. If you are using a menu system or another AutoCAD start-up file, use that when the ACADR12.BAT file is referenced.

The references to the location of TurboDLD files assume that you have chosen to install TurboDLDClassic into the C:\ACAD\DRV sub-directory for AutoCAD Release 12, or in a sub-directory named C:\TURBODLD for AutoCAD Release 11.

### System Requirements

TurboDLDClassic requires a '386, '486 or Pentium based PC which supports AutoCAD Release 13, 12, 11/386, AutoShade 2 with RenderMan, or 3D Studio v3. Additional extended memory is recommended for optimal performance.

## What TurboDLDClassic Does

TurboDLDClassic 's main function is to speed up AutoCAD pans, zooms and redraws. The driver accomplishes this by creating and maintaining a Display List - a fast-displaying object list of the current drawing - dramatically increasing performance on pans and zooms.

In addition to offering display list speed enhancement, TurboDLDClassic provides add-on  productivity features: a bird's-eye view and a graphics mode text window to further speed up the use of AutoCAD.

## What TurboDLDClassic Uses

The only resource used by TurboDLDClassic is memory. The driver actively uses about 300 Kbytes of RAM from extended memory for its operation. This memory is drawn from AutoCAD's memory pool and therefore does not affect normal DOS operation. The driver is loaded by AutoCAD at AutoCAD load time and is unloaded when AutoCAD is exited.

Additionally, the Display List size can vary from one tenth to three times the size of the current drawing file. It is recommend that you use 8 Mbytes of RAM with AutoCAD Release 11, 12 Mbytes for AutoCAD Release 12, and 16 Mbytes for AutoCAD Release 13.

You may want to purchase and install additional RAM before installing TurboDLDClassic, since it shares memory with AutoCAD. If AutoCAD is using a lot of memory, TurboDLDClassic may not have enough. If there is significant hard disk activity while you are using TurboDLDClassic, it may be an indication that you should add more memory to your system.

## TurboDLDClassic Installation

To install the TurboDLDClassic ADI 4.2 driver, follow the procedure listed below. You can exit the installation program any time by pressing the [ESC] key.

- Insert the driver diskette labeled ‘CAD Drivers’ into Drive A: (or B:).

- Run the installation program (INSTALL.EXE) by typing

- A: 
at the DOS prompt.

- At the A: prompt type
	cd \ADI42

- Next type 
	Install
at the ADI42 subdirectory.

- After viewing the first two information screens, select the drive where you want the TurboDLDClassic files to reside.

- Select the disk drive where the TurboDLDClassic files will be copied to.

- Select the version of AutoCAD you are running.

- Next, you will be asked for the name of the directory you want to install TurboDLDClassic into. In the space provided, enter a complete sub-directory path name (such as C:\ACAD\DRV).

- Once the path name has been entered, verify that the information is correct and press [Enter]. The TurboDLDClassic files are then copied from the distribution diskette to your hard drive.

Since the start-up is a little bit different for AutoCAD Release 12 and 13 than it is for older versions, please follow the instructions, presented later on in these instructions, for AutoCAD Release 11.

## Reconfiguring AutoCAD

Once all of the program files have been successfully transferred, it is necessary to reconfigure AutoCAD to use TurboDLDClassic. Follow the steps below for use with either AutoCAD Release 12 and 13, or Release 11, and then proceed to the Driver Configuration sections below, to complete the installation.

### Configuring for AutoCAD Release 12 and 13

To configure the TurboDLDClassic driver for AutoCAD Release 12 and 13, follow the procedure listed below.

- Begin AutoCAD with the reconfigure switch by typing
	ACADR12 -R [Enter]	Release 12
	ACADR13 -R [Enter]	Release 13

- Choose option 3, Configure Video Display from the AutoCAD configuration menu.

- Type Y at the Do you want to select... message to display the available video options for AutoCAD.

- Select TurboDLD Classic v2.1 by Panacea Inc. from the list of display options.

Alternately, to get to the AutoCAD Configuration menu described above, start AutoCAD as you normally would, and then type CONFIG at the Command prompt.

NOTE:	If you chose to install TurboDLDClassic into a sub-directory other than ACAD\DRV, be sure to modify the ACADDRV environment variable to include that sub-directory. Otherwise, the TurboDLDClassic selection will not appear in the list of available drivers.

NOTE:	TurboDLDClassic usually appears as option 11 in most installations of AutoCAD Release 12. If TurboDLD does not appear in the list of available video drivers, be sure that the AutoCAD environment variables in your AutoCAD start-up file, ACADDRV in particular, have been properly set before typing ACAD -R.

Configuring AVE Render

AutoCAD 12 and 13's AVE Render uses TurboDLDClassic's rendering capabilities to render objects and drawings. If AVE Render has never been configured, you will be forced to run through the configuration process when you first select the AutoCAD render command. You will be prompted as follows.

- To Select a Rendering Display Device, type
	rconfig
at the command line.

- Next, press [Enter] and then select the number 2 option.

- The P386 ADI Combined Display/Rendering Driver will be referenced on your display. Choose No when asked to select a different driver.

- To configure the rendering graphics board and resolution, press the [Enter] key when the TurboDLDClassic configuration program appears on the screen to continue past the help screen and display the Render Device Configuration menu.

- Choose Select Graphics Board/Resolution to display graphics board and resolution menu selections.

- First, choose Select Render Graphics Board to select the graphics board to be used for renderings. Then choose Cirrus CL-GD546X.

- Next, choose Select Render Resolution to select the desired rendering resolution from the list of available choices.

- Select Return to Previous Menu and then select Save and Exit.

- Next, select a render mode for TurboDLDClassic.

- Select either Render to Display Viewport or Render to Rendering Screen, depending on how you want to view your renderings. Note that in order to render to a viewport you must be using a display resolution with at least 256 colors. Otherwise, AVE Render will not allow a render to viewport selection.

- If you are using a render hard copy device, select your device type from the list of choices. If you are not using a hard copy device, accept the default of NULL.

After configuring AVE Render, you will return to the drawing to render the current drawing. To reconfigure AVE Render, reference the next section.

### Configuring for AutoCAD Release 11

To configure the TurboDLDClassic driver for AutoCAD Release 11/386, follow the procedure listed below.

- Run the FASTACAD.BAT file that was copied to the TurboDLDClassic sub-directory by typing
	C:\TURBODLD\FASTACAD[Enter]
The above example shows that TurboDLDClassic was installed on drive C: in the TURBODLD sub-directory.

- NOTE:  FASTACAD sets up the necessary AutoCAD environment variables for use with TurboDLDClassic. For more information on environment variable, reference the next section.

- Start AutoCAD and reconfigure it to use TurboDLDClassic by selecting option 5, Configure AutoCAD from the AutoCAD main menu.

- From the next menu, select option 3, Configure Video Display.

- Then choose Item 1, P386 ADI 4.0/4.1 (R11), as your display device. (For more information about configuring for ADI drivers, see your AutoCAD 'Installation and Performance Guide.')

Loading an AutoCAD 11 Environment

During the TurboDLDClassic installation process, a FASTACAD.BAT file is created and placed into the TURBODLD sub-directory. FASTACAD.BAT contains four lines that set four separate environment variables: DLDCFG - used by TurboDLDClassic to find all of its configuration files; and DSPADI, RCPADI, RDPADI - used by AutoCAD, 3D Studio and AutoShade 2 with RenderMan, respectively, to find the driver file. FASTACAD.BAT must be run prior to starting AutoCAD R11 and only needs to be run once per system boot. For automatic loading of the environment variables, FASTACAD.BAT may be added to the AUTOEXEC.BAT file or an AutoCAD start-up batch file.

To add FASTACAD.BAT to your AUTOEXEC.BAT file, insert the line

CALL D:\TURBODLD\FASTACAD

anywhere in the file. The above example assumes that FASTACAD.BAT resides in a sub-directory on drive D called \TURBODLD.

If you do not want to put FASTACAD in your AUTOEXEC.BAT file, you may put it in a batch file which also starts AutoCAD, or simply remember to run the file before starting AutoCAD. (You should also stop using any other ADI display driver that may have come with your graphics board, since TurboDLDClassic is a superset of that type of driver.)

If you get the message Out of Environment Space when you run FASTACAD.BAT, you will need to enlarge your system's environment. This is accomplished by adding the following line,

SHELL=C:\COMMAND.COM /P /E:768

to your CONFIG.SYS file. The /E:768 specifies an environment size of 768 bytes. Change this number as appropriate. You will need to reboot after modifying your CONFIG.SYS file in order for the changes to take effect.

### Configuring for AutoShade 2.0 with RenderMan

Follow the instructions below to configure AutoShade v2.0 to use the TurboDLDClassic driver.

- First run the FASTACAD.BAT file from your TurboDLDClassic sub-directory to set the AutoShade environment variables.

- Next, start AutoShade with SHADE /R, which will allow you to reconfigure AutoShade.

- For the display device, select P386 Autodesk Device Interface display driver, and for the rendering display select the P386 Autodesk Device Interface rendering driver.

- If you are running the display and rendering screen on the same monitor (such as a single monitor), make sure to tell AutoShade this. A single monitor approach will require a redraw of the display screen after a rendering screen.

- Once the P386 ADI devices have been selected, the TurboDLDClassic configuration menu will be displayed. Follow the same installation steps that were used to select the graphics board and display and rendering resolutions for AutoCAD.

### Configuring for 3D Studio

Configuring 3D Studio requires three steps.

First, you need to set the environment variables for 3D Studio by running the PANA3DS.BAT file that was copied into your TurboDLDClassic sub-directory.

Second, for 3D Studio v3.x and 4.x, use the Autodesk supplied utility to edit the 3DS.SET file. From your 3DS sub-directory, type

	3DS VIBCFG

to display the configuration screen. Use the [Tab] key to move the cursor among the available parameters. The [Enter] key allows you to select an available driver. Change the MAIN DISPLAY , MATERIAL DISPLAY and RENDER-DISPLAY fields to use RCPADI and then press [Tab] to move to OK. Press [Enter] and save your changes.

Because RCPADI device drivers, by definition, are combined Display and Rendering devices, during 3D Studio reconfiguration, you may be brought to the TurboDLDClassic Configuration menu more than one time each. It is not necessary to select display and/or rendering resolutions a second time. Simply press the [Enter] key to remove the help screen and then highlight Save and Exit to continue to the next question. Also, if FASTACAD.BAT is used instead of PANA3DS.BAT to define the AutoCAD operating environment, the TurboDLDClassic Configuration Menu will also appear for RDPADI and DSPADI devices if they are present.

A limitation of the 3D Studio TurboDLDClassic driver is that it only provides still rendering support for 3D Studio. This driver will not play back rendered FLI or FLC files at resolutions above the 320x200 default of 3D Studio. If you have a VESA compatible graphics board you may be able to use the RCPVESA.EXP driver that comes with 3D Studio to display animations at high resolution. Refer to your 3D Studio Installation and Performance Guide for specific information.

The final steps in the configuration process to configure TurboDLDClassic for use with 3D Studio are listed below.

- Delete (or rename) the 3DADI.CFG file from your 3D Studio sub-directory by typing

		DEL  3DADI.CFG 
Deleting this file will cause 3D Studio to start in its reconfiguration mode. 3DADI.CFG will be recreated after 3D Studio has been reconfigured.

- Start the 3D Studio reconfiguration by typing

		3DS

- During the 3D Studio start-up, you are prompted with a series of configuration questions. After answering each question, TurboDLDClassic will display either a Display Screen or a Rendering Screen Configuration menu, depending on whether the 3D Studio question references a display or a rendering option.

- To configure the 3D Studio Main Display from the Display Configuration menu, select a display graphics board and then a display resolution.

- To configure 3D Studio for the Rendering and/or Materials screens, select a rendering graphics board (usually the same as the Display board) and a rendering resolution. The first two questions are linked. Please note that the second question overrides any settings answered in the first question and will change settings accordingly.

CAUTION!  The MATERIAL-DISPLAY should only be configured for RCPADI when both the Display and Rendering Resolutions are configured for 256 color resolutions. An unpredictable Materials Editor screen appears if anything other than 256 colors is selected. If you are unsure about the function of the Materials Editor screen, use the Materials Editor as VGA; you should not have any problems. Please note too that the use of the Materials Editor will not give you any more colors to choose from, just higher resolution.

## Configuring the TurboDLDClassic Driver

After selecting the proper display device, the TurboDLDClassic  Welcome screen will appear. This screen is an informational screen about what the TurboDLDClassic setup program does. Press any key to continue once you have read the overview. As you continue past the Welcome screen, the TurboDLDClassic Driver Configuration menu will appear.

Set-up of TurboDLDClassic requires the configuration of its various operating parameters. These parameters have been logically grouped into menus based on their interaction with AutoCAD. A quick trip through each menu will complete the configuration process.

Context sensitive help can be obtained by typing [?] at any menu option. Please note that the DLDSETUP.HLP file must be present in order for help to appear. If you get an error message, be sure that the file DLDSETUP.HLP is in the ACAD\DRV sub-directory (R12/13), or in the TURBODLD sub-directory (R11). Also, be sure that there are no references to older DLD drivers which may cause TurboDLDClassic to look elsewhere for DLDSETUP.HLP.

If at any point during the configuration process you want to change an item on a previous screen, just press the [ESC] key to back up. Selecting NO SAVE, EXIT at the main configuration menu will return you to the AutoCAD configuration menu without making any changes.

NOTE:	At a minimum, a graphics board and screen resolution must be chosen from the Select Graphics Board/Resolution menu. If no display options are configured, the driver will be automatically configured for Generic VGA, 640 x 480 resolution at 16 colors.

The first menu, Select Graphics Board/Resolution, configures TurboDLDClassic for the graphics board, and display and rendering resolutions to be used

- Select Graphics Board - Selects the graphics platform being used

- Select Display Resolution - Selects AutoCAD, 3D Studio and AutoShade main display screen resolutions

- Select Rendering Resolution - Selects AVE Render, 3D Studio and AutoShade rendering resolutions

NOTE:	Rendering menu options only appear when configuring the rendering screens of AVE Render, 3D Studio and AutoShade2 with RenderMan. You will not see both menu options at the same time.

### TurboDLDClassic Configuration Options

There are three types of TurboDLDClassic configuration options: Basic, User Interface and Expert. All three type of options are listed below with a brief description of each option.

Basic Configuration Options

Basic Configuration options set AutoCAD screen characteristics, number of text lines in the command-line area, font size and screen configuration mode.

- AutoCAD Text Lines  - Selects number of lines in AutoCAD command prompt area

- Screen Configuration - Provides a choice of single screen, dual screen or single screen with a graphics mode text window.

- Font Selection - Allows selection of TurboDLDClassic supplied fonts or Autodesk supplied .BDF fonts menus and pull-downs.

User Interface Options

User Interface options include the parameters for maneuvering within AutoCAD.

- Double Click - Sets mouse/digitizer double click detect time.

- BP Button - Select which mouse or tablet button to use for bird's eye pop-up.

- BP Highlight Mode - Controls how The Big Picture is displayed.

- BP Refresh - Controls the refresh mode of The Big Picture.

Expert Configuration Options

The Expert Configuration options set the function of TurboDLDClassic itself. This menu allows for customization of the driver for speed or to adjust for memory constraints.

- Display List - Enables or disables the display list feature of TurboDLDClassic.

- AutoCAD Logical Drawing Space - Sets AutoCAD's logical space to 15 bits or 31 bits (R12 or higher only).

- Internal Command Echo - Enables or disables the echo of TurboDLDClassic internal commands at the AutoCAD Command line when such commands are executed.

- Big Picture Zoom Mode - Sets Big Picture zoom area display options.

- Display List in PaperSpace - Disables or enables display list while in PaperSpace.

After all options have been set, use the arrow keys to scroll down to the Save and Exit option and then press the [Enter] key to continue.

Next, configure the AutoCAD screen display characteristics and then type [Y] to accept the changes.

Exit to the AutoCAD drawing editor to begin using TurboDLDClassic.

The last step in setting-up TurboDLDClassic is to configure the colors for AutoCAD. From the AutoCAD Command Line, type DLDCOLOR[Enter] to start CustomColors; TurboDLDClassic's color configuration utility. Make any desired color changes and then Save the new color palette and Exit to return to the drawing editor.

### Browsing the TurboDLDClassic

A quick trip through each menu will complete the TurboDLDClassic configuration process.

Select Graphics Board and Resolution Menu

The Select Graphics Board/Resolution menu configures TurboDLDClassic for video parameters. From this menu, graphics board/type and display and rendering resolutions for TurboDLDClassic will be determined.

Select Graphics Board allows you to manually select your computer's graphics board. A list of video cards is given, based on the findings of TurboDLDClassic's built-in graphics platform detection utility. Use the Up and Down arrow keys, and the Page Up and Page Down keys to scroll through the list and highlight your graphics board. Please note that graphics boards are listed by both platform type and manufacturer's name, alphabetically, therefore you may need to scroll to the bottom of the list to find your particular board.

If your graphics board is not listed, you may choose the generic board option for your video chip type.

Press [Enter] to accept your choice and return to the Select Graphics Board/Resolution menu.

Select Display Resolution configures the resolution of the AutoCAD drawing screen. Display screen options are based on the findings of TurboDLDClassic's graphics platform detection utility.  Use the Up and Down arrow keys, and the Page Up and Page Down keys to scroll through and highlight the resolution and color option you want to use.

Press the [Enter] key to accept your resolution choice and return to the Select Graphics Board/Resolution menu.

NOTE:	When configuring TurboDLDClassic's video options, the Rendering Board/Resolution options described below will automatically appear ONLY when configuring the rendering modes of AVE Render, 3D Studio or AutoShade 2 with RenderMan. Otherwise, the Display Board/Resolution options will be the only menu choices available.

Similar to the Select Display Resolution menu, Select Rendering Resolution configures the resolution of the AutoCAD rendering screen. Rendering screen options are also based on the findings of TurboDLDClassic's detection utility. Use the Up and Down arrow keys, and the Page Up and Page Down keys to scroll through and highlight the resolution and color option you want to use.

Press ENTER to accept your choice and return to the Select Graphics Board/Resolution menu.

NOTE: 	TurboDLDClassic's graphics platform detection utility queries the graphics card for graphics platform type and installed memory. Based on the detector's information, TurboDLDClassic will list a set of Display and Rendering resolutions. Due to the variety of graphics boards and their specific implementations of platform chip sets, the detector is general purpose and cannot always be 100% accurate. Therefore, when selecting a Display or Rendering resolution, be sure to use only resolutions that you know your graphics board, and monitor, will support. If you are unsure which resolutions are supported by your graphics board and monitor, refer to your owner's manual(s).

Highlight Return To Previous Menu and press the RETURN key to return to the Main Configuration Menu.

Basic Configuration Menu

The Basic Configuration menu configures TurboDLDClassic's AutoCAD screen characteristics; text lines; screen mode; and font size.

The first Basic option is the number of text lines you want to use for the AutoCAD command line. The default is 3 lines, but values from 1 to 10 can be specified. A number larger than 3 might be useful if you are using this driver in a high resolution mode with small fonts, eliminating the need to frequently swap back and forth between the text and graphics screens.

If you do not want any lines of text at the bottom of the display, use AutoCAD to disable the command area (see the AutoCAD "Interface, Installation and Performance Guide" for more information).

The Screen Configuration section is used to tell TurboDLDClassic under which of three available modes you want to use AutoCAD -- dual screen, one monitor for graphics and a second for text; single screen with combined text and graphics; or single screen with combined text and graphics using a graphics mode text window.

Use the arrow keys to highlight a choice based on the following explanations of each mode. Select Dual to run in dual screen mode.

NOTE:	Do not select Dual if you don't have a second video board in your system, or unpredictable results will occur.

Choose Single to run AutoCAD in the standard single screen mode, using a video mode switch to standard VGA text mode during a flip screen. To eliminate the video mode switch to text mode and back into high resolution graphics, choose Text Window on Single to configure the single screen to use a graphics mode text window. Using graphics mode to display text allows an instantaneous return from a flip screen. The graphics text window is especially useful when using large monitors which tend to be slow when switching modes. A SHELL command will switch to text mode regardless of the setting.

TurboDLDClassic offers you a selection of fonts to use for AutoCAD menus, pull-downs, and dialog boxes. Provided by Panacea are 6x8, 8x8, 8x14, 8x16, 12x20 and 12x24 font options. Also available for use are any of the .BDF fonts, included with AutoCAD Release 12, you may have on your system. For resolutions over 800x600 we recommend the 12x20 or 12x24 fonts.

To make a font selection, highlight Select Font From List to display a list of available fonts. Use the Up and Down arrow keys, and the Page Up and Page Down keys to scroll through the list and highlight the font you want to use. Press the ENTER key to return to the Basic Configuration menu. To limit the available font choices, choose the Select Font Filter menu selection to choose a .BDF font type from those listed. When you return to the Select Font From List option again, only those files matching the filter name will be available for use.

NOTE:	If you do not see any .BDF font selections listed in the Basic Configuration menu, you will need to add the following line to your AutoCAD start-up batch file. 

	SET DLDFONT = C:\ACAD\DRV

Highlight Return To Previous Menu and press the RETURN key to return to the Main Configuration Menu.

User Interface Menu

The User Interface options configure the productivity features of TurboDLDClassic. These features change the way you maneuver around AutoCAD and execute commands.

Double Click Timing

The first timing option sets the Double Click delay. This time represents the largest amount of time that can elapse between the release of the digitizer button and the subsequent press of the same button in order to detect a double click. Setting this number too low may result in the computer seeing two separate clicks, while setting the time too high may slow down response time for single button presses.

Values between 10 and 30 are recommended.

Double Click time is measured in hundredths of seconds. PC systems have a timing resolution of 5/100ths of a second, therefore, the entered time is rounded internally by the driver to most closely approximate the nearest multiple of 5. This means that a time setting of 23 is the same as a setting of 27, both of which get rounded to 25 (or one-quarter of a second). In simplest terms, when adjusting these values, it is best to use multiples of 5.

Big Picture Options

The Big Picture, Panacea's bird's-eye view, can be configured to suit your needs or allow for better differentiation within a complex drawing.

At the BP Button menu option, enter the number of the button on your mouse or tablet that you want to use as your bird's eye "hot key". A Double Click on that button will cause TurboDLDClassic's bird's eye to pop-up on -screen.

BP Highlight controls how the current viewport reference in the Big Picture appears on the screen. Patt Line displays the viewport reference using dotted lines. XOR Rect uses a contrasting rectangle to display the viewport reference. The Both option will use a combination of Patt Line and XOR Rect to display the viewport reference.

In its fastest mode, TurboDLDClassic only refreshes the Big Picture when objects are added to a drawing, until a DLDREFRESH command is issued.

Moved or erased objects will remain in the Big Picture. Depending on a drawing's complexity, if BP Refresh is Disabled, the Big Picture could become confusing, displaying vectors that are actually erased, and no longer part of the drawing. To configure the Big Picture to be updated as objects are drawn AND erased, ENABLE the BP Refresh. If speed is a major concern, DISABLE it.

Highlight Return To Previous Menu and press the RETURN key to return to the Main Configuration Menu.

Expert Configuration Menu

The Expert Configuration menu sets the basic internal functions of TurboDLDClassic. Options in this menu allow for customization of the driver for speed or to adjust for memory constraints.

Display List enables or disables the display list feature of TurboDLDClassic', the core of the driver's AutoCAD performance boost.

This option should always be set to ENABLE, as disabling the Display List will cause TurboDLDClassic to run as an ordinary non-display list driver.

Use AutoCAD 31-bit Space, configures TurboDLDClassic for use with AutoCAD R12's 31-bit logical drawing space. When set to Yes, the driver will use AutoCAD's extended 31-bit drawing space. Selecting No, will use a 15-bit drawing space, similar to that of AutoCAD R11.

The 31-bit logical space allows you to extend your Regen-less zooming ability by a factor of several million, at the cost of more memory. Additional memory is used by AutoCAD for the drawing space, and by TurboDLDClassic for the Display List. 31-bit zooming and panning is about 10%-20% slower than for 15-bit logical space.

The Use AutoCAD 31-bit Space setting is ignored for AutoCAD R11 installations as these versions only support a 15-bit logical drawing space.

The echo of TurboDLDClassic internal commands can be enabled or disabled with the Internal Command Echo option. If you would like to see TurboDLDClassic's internal commands display at the AutoCAD command line as they are executed, enable this option.

BP Zoom Mode sets the Big Picture (BP) zoom definition area display options. In a zoomed view of the static BP, as the current viewport is zoomed or panned, FLOAT mode causes the image in the BP to move around within the bird's-eye window, keeping the zoomed viewport area fixed in the center of the bird's-eye. FREEZE mode will lock the current BP contents into place to provide a better overall frame of reference.

Disable DL in PSpace allows you to turn off the Display List while in PaperSpace only. This function is useful for PSpace users who find the display list size grows excessively for their drawings.

After all Expert options have been set, highlight Return To Previous Menu and press the RETURN key to return to the Main Configuration Menu.

### Saving Configuration Information

Once all of the TurboDLDClassic parameters have been configured, select SAVE, EXIT to save the configuration information and create DLDSETUP.DAT in either the ACAD\DRV sub-directory (R12/13) or the TURBODLD (R11) sub-directory. DLDSETUP.DAT is TurboDLDClassic's configuration.

After answering AutoCAD's standard driver configuration questions, you will return back to the AutoCAD configuration menu. To begin your accelerated AutoCAD, for AutoCAD R12/13, select option 0, Exit to Drawing Editor; for AutoCAD R11, choose option 1 or 2, Open New/Existing Drawing.

## Verifying Your TurboDLDClassic Installation

To verify that TurboDLDClassic is running and installed correctly, follow one of these two simple tests.

- If you have the AutoCAD side menu enabled, look for the Panacea Logo in the lower right-hand corner.

- If you are running AutoCAD without a side menu, type DLDVER[Enter] at the AutoCAD command prompt. If TurboDLDClassic is loaded and running, this command should return your current version and serial number. If TurboDLDClassic was not correctly installed, you will receive an AutoCAD error when the DLDVER command is entered.

## Reconfiguring TurboDLDClassic

To reconfigure TurboDLDClassic, follow the instructions below for your version of AutoCAD.

To completely reconfigure TurboDLDClassic using Panacea’s defaults, delete DLDSETUP.DAT from either the \ACAD\DRV sub-directory (R13/12) or the TURBODLD sub-directory (R11) and follow the Driver Configuration instructions earlier in this driver section.

### AutoCAD Release 12 and 13

If you need to make changes to your Release 12/13 TurboDLDClassic configuration, type CONFIG[Enter] at the AutoCAD command prompt or type ACADR13 -R[Enter] or ACADR12 -R [Enter] when starting the program. Select option 3, Configure Video Display. Answer No to the Select a new video driver ... prompt to start the TurboDLDClassic configuration program. Make the desired changes to the driver and then Save and Exit to continue to the AutoCAD drawing editor.

### AutoCAD Release 11

To change a Release 11 configuration, select option 5, Configure AutoCAD, from the AutoCAD main menu. Then from the configuration menu, select option 3, Configure Video Display. Answer No to the Select a new video driver ... prompt to start the TurboDLDClassic configuration program. Change the desired driver options. Save and Exit to return to the AutoCAD configuration menu. Open or begin in a new drawing.

## Changing Colors

Included with TurboDLDClassic is the CustomColors configuration utility. What is so unique about CustomColors is that it gives you the ability to modify all of your changeable AutoCAD colors, including menu colors, text colors, dialog box colors, and even drawing colors while running AutoCAD.

CustomColors simulates an AutoCAD screen, complete with all possible objects. At the AutoCAD drawing editor command line, type DLDCOLOR[Enter] to edit the color configuration. If you have configured TurboDLDClassic for 256 colors, you will be greeted with an informational warning message. The message appears to remind you that if you assign a drawing color to a physical color above color 15, you may experience unusual results from AVE Render.

Once you enter the utility, you are provided with the following configuration menu at the bottom of the screen:

(O)bject,

(D)rawing,

(P)hysical,

(A)DI reset,

(V)GA reset,

(L)oad,

(S)ave,

(E)xit

The menu items perform the following functions.

- Object - Allows you to change the color of any AutoCAD screen object, such as the graphics area background color, the menu area text color, or the border line color.

- Select the object whose color you want to change by moving the highlight box around with the left, right, up and down cursor control keys. Press [Enter] when you have highlighted the desired object. Note that a one line description of the object type is displayed at the top of the screen as you move the box around.

Once you have selected the object, another highlight box will appear around the color boxes nearest to the bottom of the screen. These are the physical colors that the video board supports. Use the left and right cursor keys to select the physical color to be used for the object you have selected and press [Enter]. The screen will quickly redraw with the new color selection for the object you have chosen, and you will be back at the start of object selection in case you want to change the color of another object.

Note that at any time during color configuration, you can press the [ESC] key once or twice and return to AutoCAD. Also, pressing the question mark [?] key will provide you with context sensitive help.

- Drawing - Lets you modify AutoCAD drawing colors in a fashion similar to the way you change object colors. 

When this option is selected, a highlight square appears in the drawing color area of the simulated AutoCAD display. The square can be manipulated using the four cursor keys. You may notice that the drawing color portion of the display is laid out just like the CHROMA drawing supplied with AutoCAD. When you press [Enter], the highlight moves down to the 16 physical colors, just as it does during the object color selection. Using the arrow keys, pick the physical color you want to represent the selected drawing color. Press [Enter] to have your selection take effect. When using 256 colors, you do not have a (D)rawing Color option. All 256 colors are available as drawing colors.

- Physical - Allows you to alter the red, green, and blue components of the physical colors. A highlight box will appear in the row of 16 physical colors at the bottom of the screen.

Use the arrow keys to select the color you want to edit and press [Enter]. Three sliders will appear near the bottom of the display, with the horizontal position of the slider for each of the color components (red, green, or blue - RGB) indicating the relative intensity of the component. You can use the left and right arrow keys to move the slider for the selected component, or type in a number from 0 to 255. 

Many graphics boards do not support 255 different intensities for each color component. Therefore, when you enter an intensity, CustomColors will round it to the nearest intensity which your graphics board supports. For example, VGAs support 64 intensities per RGB color component. This means intensities increase in multiples of 4 (256/64), therefore an intensity of 0 is the same as an intensity of 3.

The up and down arrows allow you to select which component you want to alter. As you manipulate the sliders, all objects on the display that are of the same color as the selected physical color will change in hue. This way you can visually determine the most appropriate setting for your display. Pressing [Enter] will set the RGB values you have selected for the physical color you were modifying.

- ADIreset - Constructs a default ADI color palette, as defined in the Autodesk Device Interface Driver Development Kit.

- VGAreset - Constructs a default VGA color palette in the first 16 color entries. The remaining palette colors remain identical to those used for ADI devices.

- Load - Reloads the color information from the file DLDCOLOR.DAT. This command is useful because it allows you to go back to your previously edited color configuration in case you have made some mistakes in configuring your colors. DLDCOLOR.DAT will be loaded from the directory pointed to by DLDCFG (AutoCAD R11), or from the AutoCAD R13/12 \DRV directory. If DLDCOLOR.DAT cannot be found, an error message will be displayed.

CAUTION!  Do not accidentally erase your changes! Selecting ADIreset, VGAreset, or Load will irrevocably wipe out any current color changes you may have made, unless you have just saved your new setting. Use them only when you really need to, such as when you have made so many color changes that you cannot seem to get back to a reasonable place and just want to start over.

- Save - Saves the current color palette to DLDCOLOR.DAT. It will be saved in the directory pointed to by the DLDCFG environment variable in FASTACAD, or to the \ACAD\DRV (R13/12) directory if FASTACAD is not used.

- Exit - Exits CustomColors. If you have made palette changes, but not saved them, you will be asked if you want to save your changes before exiting.

When running at more than 8 bits-per-pixel (256 colors), setting physical colors via DLDCOLOR will have no visible effect until a SAVE and EXIT occurs.

## TurboDLDClassic Commands

TurboDLDClassic offers AutoCAD users many features and productivity options. This section lists the basic commands found in TurboDLDClassic and provides the correct syntax for their usage. For a brief summary of TurboDLDClassic commands, type DLDHELP at the AutoCAD command prompt.

The most important feature of TurboDLDClassic is the support of a world view called The Big PictureTM or BP for short. Bringing up the BP allows you to view where in the current drawing you are currently located in your active viewport. It also gives you the ability to dynamically move to another part of the drawing without having to exit the function you are currently executing.

NOTE:	The commands with an asterisk '*' following them, when issued within AutoCAD, will override the selections made during TurboDLDClassic configuration for the current drawing session only. Exiting AutoCAD and subsequently restarting will cause all feature settings to revert back to those selected in the TurboDLDClassic configuration menu. If you want to make the current changes permanent, reconfigure TurboDLDClassic.

To use the TurboDLDClassic commands type the name of a command (listed below) and zd to view The Big Picture. To toggle your current command on or off, type the command again. You can also set up a series of commands similar to the AutoCAD’s Auto Lisp feature.

NOTE:	You can create shortcuts through AutoCAD. Reference your AutoCAD manuals for details.

The TurboDLDClassic commands are listed below in alphabetical order.

CD

Returns any unused DLM memory to the VMM.

DLDBIGPIC

Calls up The Big PictureTM (BP) at the AutoCAD command line. Type DLDBIGPIC[Enter], or double click the right mouse button or button 2 on your digitizer puck (if you have not redefined your BP Button double click) to bring up the BP.

You will see cross-hairs and a highlighted section indicating which part of the whole drawing you are currently viewing. The size of the image will be determined by the AutoCAD logical drawing space being used, 15-bit or 31-bit. The selection or pick box will have an 'X' through it. Move the mouse/digitizer around to locate your pick box. If you can not see a small box moving, click the left mouse button (the pick button) to size down the pick area. In size mode, the pick box will have a right pointing arrow in it ('-->'). If the contents of the BP are too small to work with, you may increase the size of the image in the window by pressing the [+]key on your numeric keypad. This performs an incremental zoom within the BP. Likewise, pressing the [-] key will reduce the size of the contents of the BP.

Pressing the [Home] and [End] keys will put the BP into its smallest and largest size, respectively. The smallest BP size is defined as being exactly the same view as that in the viewport referenced by the BP. The largest BP size is defined as the largest image possible without causing a REGEN.

The BP pick box is always proportioned to the proper aspect ratio for the current viewport. Clicking the pick button again will put you back into the pick box move mode ('X'). This operation is very similar to using the Zoom Dynamic feature of AutoCAD. Once you have positioned the pick box on the area you would like displayed in the active viewport, click any button, other than the pick button, to initiate the zoom or press the [Enter] key. If you want to abort the operation, just hold any button down, other than the pick button, until the BP disappears. You can also hit any key on the keyboard, other than [Enter], to cancel the BP. Note that the BP is updated with new drawing commands, but that moves or erases will not update the image unless BPREFRESH is enabled. A DLDREFRESH or a REGEN will manually update the BP with the current drawing changes.

DLDBPDIM

Resizes and repositions the BP by typing this command at the AutoCAD prompt. DLDBPDIM allows you to tell the driver where you want the BP placed and how large you want it to be. The size is limited to being between one-quarter and one-half the width of the display. This command functions like the pan/zoom selection box in the BP, as far as positioning and dimensioning goes. To abort, hold the right mouse button, or any other digitizer button, down until the moving box disappears.

DLDBPFREEZE

Sets the Big Picture (BP) zoom definition area display options. DLDBPFREEZE toggles the Big Picture Zoom Mode parameter in TurboDLDClassic's Expert Configuration Menu.

In a zoomed view of the static BP, as the current viewport is zoomed or panned, Float mode causes the image in the BP to move around within the bird's-eye window, keeping the zoomed viewport area fixed in the center of the bird's-eye. Freeze mode will lock the current BP contents into place to provide a better overall frame of reference.

DLDBPHILIGHT*

Changes the highlight used to display the Big Picture among three modes; Patt Line, XOR Rect, and Both. Patt Lines will use dotted lines to outline the Big Picture pick box. XOR Rect will use a contrasting rectangle to highlight the pick box, and Both will use a combination of Patt Lines and XOR Rect.

DLDBPMODE

Toggles the Big Picture between Panning and Zooming modes. In panning mode, the AutoCAD drawing will dynamically display the contents of the Big Picture pick box as you size and move it. In Zoom mode, the AutoCAD drawing remains stationary until the area in the Big Picture pick box is accepted. At which time it changes to the chosen zoom level.

DLDBPREFRSH*

Toggles the Big Picture update mode. If disabled, the BP is updated manually by issuing a DLDREFRESH command. When enabled, DLDBPREFRSH will cause the BP to refresh automatically when an object is drawn or erased. Therefore, the BP operations will be slower with DLDBPREFRSH enabled.

NOTE:	This command is used in conjunction with the DLDBPSTATIC command. DLDBPSTATIC should be used first before using this command.

DLDBPSTATIC

Toggles the BP to stay on the screen at all times. Use this command if you want the BP on the screen in order to conveniently move into it whenever you want to zoom or pan. Typing DLDBPSTATIC will put up the BP at the position you set with DLDBPDIM. Note that the static BP will disappear when other menus pop-up on the screen, and it will also disappear if you draw any objects which may overwrite the BP. With these exceptions, the BP will remain on-screen until you either exit the drawing screen or you type DLDBPSTATIC again.

DLDBPSTATIC is also used in conjunction with the DLDBPREFRSH and DLDREFRESH commands.

DLDCOLOR

Invokes CustomColorsTM, Panacea's color configuration program.

DLDCOMPACT

Forces a manual garbage collect of Display List memory thereby returning unused display list memory back to the AutoCAD memory pool.

DLDDLIST*

Toggles the TurboDLDClassic display list function on and off.

NOTE:	If the display list is turned off, you will be running AutoCAD as though you were using a standard non display list driver - pans, zooms and redraws will be MUCH slower with DLDDLIST disabled.

DLDECHO*

Toggles internal TurboDLDClassic command echoing at the AutoCAD command line. When TurboDLDClassic commands are executed via the digitizer or pop-up menus, they generate internal commands which will be displayed at the AutoCAD command line if DLDECHO is enabled. Disable DLDECHO to simplify the command line.

DLDHELP

Provides a list of TurboDLDClassic commands with a one-line description of each within AutoCAD. It is recommended that you flip to the text screen by pressing the [F1] key to view the output.

DLDREFRESH

Refreshes the Big Picture to reflect the most current changes to a drawing. If the static BP is up, it will be updated. If not, the next time the BP is brought up, it will contain a current representation of your drawing provided no changes were made between the DLDREFRESH command and the DLDBIGPIC command. For Automatic update of the Big Picture, see the DLDBPREFRSH command above.

NOTE:	This command is used in conjunction with the DLDBPSTATIC command. DLDBPSTATIC should be used first before using this command.

DLDSTAT

Displays the current TurboDLDClassic status. A listing of the current TurboDLDClassic parameters will be displayed at the AutoCAD command line.  A flip to the graphics screen is recommended for this command.

DLDUSAGE

Breaks down, to-the-byte, how memory is being used, specifically for display list processing. DLDUSAGE returns information regarding the memory each viewport is occupying. Since AutoCAD supports multiple viewports, it is possible to have multiple display lists. A new or empty drawing will have a display list of 4 Kbytes.

DLDVER

Displays the TurboDLDClassic version, serial number and registered user's name at the AutoCAD command line.

PD, ZD, ZT

Included in TurboDLDClassic for backwards compatibility with MACH-12. These commands will invoke the Big Picture.

ZI, ZO

ZI - Zoom In and ZO - Zoom Out will zoom the current viewport in or out incrementally.

ZP, ZA

ZP - Zoom Previous and ZA- Zoom All are passed directly to AutoCAD.

ZW

ZW - Zoom Window. Invokes an AutoCAD Zoom Window command.

## Memory Usage and Lists

TurboDLDClassic shares extended memory with AutoCAD via the PharLap Virtual Memory Manager. This means that TurboDLDClassic automatically pages to disk if it uses up all the RAM that AutoCAD has left for its use. See the AutoCAD 'Installation and Performance Guide' for more information on Virtual Memory Management.

If you start seeing excessive hard disk accesses during PANs, REDRAWs, and ZOOMs while using AutoCAD with TurboDLDClassic, try using the DLDCOMPACT command. If this does not affect the amount of disk access, it is probably time to add more memory to your system. Contact your AutoCAD dealer for assistance in upgrading your memory.

Regarding display list memory, it is important to realize that TurboDLDClassic speeds up AutoCAD operations by creating a Display List in memory, and sending that list to the graphics board for pans, zooms and redraws. But a display list takes up memory and, therefore, needs to be discussed in greater detail in the next section.

### Memory Needed

For production use, we recommend that at least 1 Mbyte is available for TurboDLDClassic. To determine how much memory AutoCAD is using, use the status command while in AutoCAD (refer to the AutoCAD 'Installation and Performance Guide' for more information). The Display List for a simple drawing like the shuttle Columbia might only require 20 Kbytes for the Display List. Complex drawings may require several Mbytes.

Display Lists for a drawing range from one-tenth the size of the drawing file to three times the size; in general, the Display List averages about one and one-half times the size of the DWG file size. This means that you should reserve approximately two and one-half times your average DWG file size in RAM for the drawing and the Display List.

Complex objects such as circles and text “expand” when translated into display list format, so a drawing with a lot of complex objects and text will have a larger Display List than a simpler drawing. If there is significant hard disk activity while you are using TurboDLDClassic, it may be an indication that you should add more memory to your system.

### Keeping the Display List Small

Text takes up a disproportionate amount of space in the Display List. To keep the list small, put text in its own layer of the drawing. Then do not display the text layer when editing the rest of the figure. This will keep memory consumption down and speed up PANs, REDRAWs, and ZOOMs.

NOTE:	The AutoCAD manuals have an excellent section on performance, concentrating on memory usage. To get the most performance out of the program, read the appropriate sections of those manuals as well as this one.

## Tips and Tricks

The section highlights some tips and tricks to help gain performance from AutoCAD itself. These tips and tricks are a compilation of the most commonly asked Technical Support questions. Therefore, if you are having problems, please review this section. You may be able to quickly answer your own question.

### Switching Color Modes

When switching color modes, (for example, from 16 to 256 or vice versa) the DLDCOLOR command should be run in order to reconfigure your color palette for the number of colors selected.

If you would like to switch from TurboDLDClassic's default black background to a white or light background, type

DLDCOLOR

at the Command prompt to start the CustomColors utility. Once there, press [O] for (O)bject and then move the highlight to the Background item and press [Enter]. Move the color highlight to the desired new background color and press [Enter]. Next, move the highlight to the Cursor  item and press [Enter].  Move the color highlight to the desired new Cursor  color and press [Enter].  Press the [ESC] key and then S to (S)ave and E to (E)xit. You should now have a light background with a dark cross hair.

### Zoom Dynamic

Using any TurboDLDClassic features while in the Zoom Dynamic mode of AutoCAD is not recommended. Because TurboDLDClassic cannot determine when Zoom Dynamic has been initiated, unpredictable results will occur. Use the Big Picture feature of TurboDLDClassic to accomplish the same task as Zoom Dynamic, in a much more efficient fashion.

### Zooming and Snap

When zooming into an image, be aware of your AutoCAD grid snap setting. If you are zoomed extremely far into a drawing and you are having trouble moving your digitizer cursor, you may be snapping to a point which is not part of the zoomed viewport. If the cursor only moves to a single point or is not on screen at all, Snap might be turned on. Check the status bar to be sure. If you see Snap listed, press the [F8] key to turn it off.

### Upgrading from Older Panacea DLD Drivers

Be sure to remove any reference to previous DLD driver commands (FASTACAD calls or SET DLDCFG, RCPADI, RDPADI and DSPADI parameters) or sub-directories that may be in your AUTOEXEC.BAT file or in AutoCAD start-up batch files. Such references could cause AutoCAD and TurboDLDClassic to look in the wrong place for setup information.

If you have been using a previous Panacea DLD driver and want to use the color palette you customized for it with TurboDLDClassic, you may copy the DLDCOLOR.DAT file from your older DLD's sub-directory to the sub-directory you specified during the TurboDLDClassic installation process.

### Using AutoCAD Commands

Since TurboDLDClassic is totally transparent to users with respect to using normal AutoCAD commands to REDRAW, PAN, and ZOOM, you still have to suffer from some of AutoCAD’s nuances. One of these nuances is that ZOOM ALL and ZOOM EXTENTS both force a REGEN, because AutoCAD does not keep track of various boundaries necessary to avoid the REGENs. And REGENs are rather time-consuming and do not use any display list processing to speed themselves up. One way around this problem is to use another of AutoCAD’s built-in features, namely the VIEW command.

When you first load your image and see the whole drawing on the screen at once, just type VIEW Save ALL[Enter], which will save the display position you see under a view named 'All'. Then, after you have done some detailed editing and want to return back to the big picture, type VIEW Restore ALL[Enter] instead of ZOOM All [Enter] or ZOOM Extents[Enter], and the full drawing will be restored to the display at display list speeds, without a REGEN.

Another nuance of AutoCAD’s is that if you zoom in too far or pan over too far, you may inadvertently cause a REGEN. AutoCAD again provides a very simple solution: the REGENAUTO command. Just type REGENAUTO Off[Enter] at the AutoCAD command prompt, and automatic REGENs will be disabled. The REGENAUTO setting is also saved as part of your drawing file, so you only need to execute it once per drawing. You may even want to set REGENAUTO off in your ACAD.DWG drawing template so that all your drawings are created with REGENAUTO set off.

### Static BP in Panning Mode

When using the static Big Picture in panning mode, the drawing screen will continue to pan until you move the cursor out of the Big Picture viewport. The drawing screen will display the selected view correctly.

### ZOOM $1

An undocumented AutoCAD command can help speed up performance in some situations, ZOOM $1. ZOOM $1 will allow you to set the extents of AutoCAD Release 12's logical drawing space. by issuing the following command sequence:

ZOOM 
	$1 
	CTRL-C 
	CTRL-C 
	CTRL-C 
	CTRL-C

The catch is that the command is only in effect until a REGEN is issued.

Be sure to zoom to the view of your drawing you want to use as the extents before issuing the above. If you find this command to be useful, you may also want to try configuring for a 15-bit drawing space.

## Command Summary

This section lists the basic commands found in TurboDLDClassic. Commands have been listed alphabetically.

CD 	Returns any unused DLM memory to the VMM.

DLDBIGPIC	Calls up the Big Picture - use digitizer clicks to define an area and zoom into it.

DLDBPDIM	Allows resizing and repositioning of the Big Picture.

DLDBPFREEZE	Toggles the Big Picture Zoom Mode between Float and Fixed modes.

DLDBPHILIGHT	Selects the highlight mode used to display the Big Picture.

DLDBPMODE	Toggles Big Picture between Zoom and Pan modes.

DLDBPREFRSH	Enables and disables the Big Picture automatic update mode.

DLDBPSTATIC	Toggles the Big Picture to stay on the screen at all times.

DLDCOLOR	Invokes CustomColorsTM, Panacea's color configuration program.

DLDCOMPACT	Forces a manual clean-up of Display List memory, returning any unused memory back to the AutoCAD memory pool.

DLDDLIST	Toggles the display list function of TurboDLDClassic on and off.

DLDECHO	Toggles internal TurboDLDClassic command echoing at the AutoCAD command line.

DLDHELP	Provides a list of TurboDLDClassic commands with a one-line description of each, within AutoCAD. It is recommended that you flip to the text screen to view the output.

DLDREFRESH	Refreshes the Big Picture to reflect the most current changes to a drawing.

DLDSTAT	Displays a listing of the current TurboDLDClassic parameters. A flip to the graphics screen is recommended for this command.

DLDUSAGE	Gives a to-the-byte breakdown of how memory is being used for display list processing.

DLDVER	Displays the TurboDLDClassic version, serial number and registered user's name at the AutoCAD command line.

PD, ZD, ZT	Included in TurboDLDClassic for backwards compatibility with MACH-12. These commands will invoke the Big Picture.

ZI, ZO	ZI - Zoom In and ZO - Zoom Out will zoom the current  viewport in or out incrementally.

ZA, ZP	ZA - Zoom All and ZP-Zoom Previous are passed directly to AutoCAD.

ZW	Invokes an AutoCAD Zoom Window command.

## Troubleshooting

This section outlines known configuration and usage problems of the TurboDLDClassic and of various graphics platforms. The list of troubleshooting ideas below are a compilation of the most commonly asked Technical Support questions. Therefore, if you are having problems, please review this section. You may be able to quickly answer your own question.

### Sticky Cursors

Sometimes, when using the Big Picture, the digitizer cursor will appear to stick to the edge of the bird's-eye. This is normal and is the result of the digitizer puck responding to the whole screen area while the Big Picture only occupies a small portion of the screen. Because of the difference in resolutions between the screen and the digitizer when the screen cursor enters the Big Picture area, the digitizer puck has more drawing area to cover in order to get to the same location as the screen cursor.

### Third-Party Software

If you are having trouble with TurboDLDClassic and third-party AutoCAD applications, be sure that the third-party application supports ADI 4.2. In order to use the ADI 4.2 specification, third-party applications require new T-Drivers and therefore must be revised. If an application does not specifically say that it is ADI 4.2 compatible, it probably is not. Check with the manufacturer to be sure.

If your third-party application is ADI 4.2 compatible and you are having trouble using TurboDLDClassic, try running AutoCAD without the third-party application to try to isolate the problem. Also, try the third-party application with the SVADI driver shipped with AutoCAD to isolate the source of the problem.

Also note that any third-party TSR that needs to access the display may not work properly when using AutoCAD with any advanced ADI display driver, especially if the TSR switches graphics modes. Most TSRs will not support the same graphics platforms as TurboDLDClassic and, therefore, will not be able to accommodate mode switching back and forth.

### Memory Management

System lock-ups, random re-boots or missing displays, indicate you may have a memory conflict on your system. Different video platforms access different, specific, portions of upper memory blocks. If these memory addresses are in use by TSRs or other programs or memory managers, the above symptoms will occur. To avoid these problems it is necessary to use an expanded memory manager to allocate the specific address range to the graphics board. The address to exclude usually falls within the range of A000 to C7FF. XGAs and some TIGAs go beyond C7FF up to DFFF.

Some common expanded memory managers are QEMM, 386MAX and EMM386 (which comes with DOS 5.0 and later and Windows).

QEMM and EMM386 use similar commands to perform the exclusion.

X=B000-B7FF

is added to the QEMM386.SYS or EMM386.EXE command line in the CONFIG.SYS file.

386MAX uses a video memory parameter which looks like this

VIDMEM=B000-B7FF

Note that it may be necessary to run an optimization utility with some memory managers in order for the exclusion to take effect. Refer to your memory manager's documentation for specific details.

If the range of A000-C7FF seems like a large area to exclude, you may decrease the range by changing the ending address to determine the exact range that you need to exclude. Typically B000-B7FF, as shown in the examples above, will be sufficient. You may try other ranges, by altering the beginning and/or ending address, to isolate the exact region the graphics board requires. Remember to re-boot after changing the CONFIG.SYS file.

CAUTION!  Due to the manner in which they re-map video BIOS memory regions in RAM, memory optimization utilities, such as QEMM's Stealth and EMM386's HIGHSCAN, should be avoided.

### TurboDLDClassic and Paper Space

TurboDLDClassic features do not work in AutoCAD's Paper Space. The execution of any TurboDLDClassic command in Paper Space will result in an error message at the command prompt.

### Slow Scrolling

If you have configured for the graphics mode text window and are experiencing slow script file execution, it might be your graphics board. Some graphics boards do not handle graphics mode text scrolling very quickly. To test this, try configuring for the SINGLE screen option in the Basic Configuration sub-menu of the TurboDLDClassic Configuration menu.

### Corrupt TurboDLDClassic Data Files

If you are having problems configuring TurboDLDClassic, be sure that you are using either the original distribution diskettes or a faithful copy made with the DOS DISKCOPY command. We have at times received Tech calls where we could not trace a specific recurring problem which later turned out to be a bad duplicate disk.

If everything appears to have installed correctly and you still can't seem to figure out what's happening, try deleting the DLDSETUP.DAT, from your TurboDLDClassic sub-directory. You will then have to reconfigure the driver from scratch.

### Slow Mouse/Digitizer Response

The following solutions have been used to fix slow mouse/digitizer response problems when using the TurboDLD driver.

- Make sure that you are not using a real mode driver for your digitizer. Real-mode drivers do not interact properly with protected-mode drivers. AutoCAD Releases 11/386 and 12 are all protected-mode. It is recommended that you switch to either the AutoCAD supplied driver or the manufacturer's latest ADI 4.2 driver for the digitizer.

- Make sure that no digitizer key re-assigning TSRs are being used. Some digitizers, specifically the Kurta IS/One, use TSR's to allow digitizer programming; these will not work with TurboDLD.

- Try deleting all files with a .DAT extension from your TurboDLD sub-directory and re-configuring TurboDLD from scratch. Occasionally, these .DAT files can be corrupted or updated improperly during installation or reconfiguration.

- Try changing the double-click interval time to 5 or 10. This can be adjusted from the User Interface Menu of TurboDLD.

- If a specific digitizer key is causing a problem, use the Delete key to remove any AutoCAD commands from that button. Button programming commands are in the User Interface menu within the Button Configuration Menu. Be sure to use the Delete key, not Spaces to remove existing commands.

- Depending on the digitizer you are using and how it is configured, try adjusting CFIG386's -VSCAN parameter. Check the AutoCAD "Interface, Installation and Performance Guide"  for specific instructions on how to use CFIG386.

- Try using the CFIG386 -DEMANDLOAD ON setting for AutoCAD.

- If there is one available, try to duplicate the problem with a second mouse or digitizer, this will help to isolate the cause of the problem.

- If your digitizer is capable of it, try configuring for SummaSketch emulation and use the driver included with AutoCAD.

- If there is a choice in your digitizer's configuration, try configuring it for Polled vs. Interrupt driven, or vice versa.

# MicroStation Driver

The MicroStation driver provided with your [PRODUCT NAME] is the panaStation Classic driver from Panacea. It is a protected-mode driver and completely compatible with MicroStation v4.0 and v5.0.

## Features of the panaStation Classic

The following are the features of the panaStation Classic driver:

- Easy to use - no new commands or special menus to learn.

- No memory conflicts. Works with MicroStation's built-in Virtual Memory Manager

- Will support higher resolutions on many popular graphics boards.

- Supports all MicroStation features, including rendering, page frame swapping, texture mapping and 3D animation.

## Before You Begin

In order to run the [PRODUCT NAME] panaStation Classic driver efficiently and effectively, make your system is a '386, '486 or Pentium based PC which supports MicroStation PC v4.0 or later.  Before starting, make a backup copy of the MicroStation Driver disk using the DOS DISKCOPY command, and put it in a safe place.

## panaStation Classic Installation

The panaStation Classic installation procedure below is exact, and describes what to do at each step.

- Insert the panaStation Classic driver disk labeled ‘CAD Drivers’ into drive A: (or B: if you are using drive B) and type
	A: 
at the DOS prompt.

- At the A: prompt type
	cd \MSTN

- Next type 
		Install
at the MSTN subdirectory.

The first time the installation program is run, you will be prompted for registration information. This information must be given. If you choose to ignore this step, panaStation Classic will not be installed.

- panaStation Classic will prompt you for information about your MicroStation installation. Verify that the given information is correct and then press [Enter].  panaStation Classic files are then copied to the hard drive sub-directory you specified during installation.

- After all of the program files have been successfully transferred to your system, it is necessary to reconfigure MicroStation to use the panaStation Classic driver. Reference the next two sections on reconfiguring your driver for MicroStation 4.x or 5.x..

## Reconfiguring MicroStation 4.x

After the panaStation driver is installed, MicroStation 4.x must be reconfigured. To reconfigure MicroStation 4.x, follow the procedure listed below.

- Start the MicroStation configuration program by typing

- USCONFIG
from your \USTATION sub-directory.

- From the MicroStation Main Configuration menu, use your arrow keys to highlight the DISPLAY ADAPTERS option.

- Press [Enter] to display the Right Screen Graphics Adapter menu.

- From the list of available display drivers, highlight Vendor Supplied Driver and press[Enter].

- Next, highlight panaStation Classic, by Panacea, Inc. from the list of vendor supplied drivers. Press [Enter] to continue to the Left Screen Graphics Adapter menu.

- If you are configuring a single monitor system, highlight No Second Adapter and press [Enter] to display the Virtual Swap Screen dialog box.

- To use virtual page swapping, answer "N" to the "Do you want to disable the virtual graphics page?" question. To disable virtual page swapping answer "Y". When using panaStation Classic, MicroStation's Virtual Swap Screen feature is available to all graphics boards. On graphics boards using dual page memory switching, the page swap is done using the graphics board hardware. On all other graphics boards, the page swap is simulated by panaStation Classic. Once the Swap Screen question has been answered you will return to the Main Configuration menu.

- If you are configuring a dual monitor system, select which display you want the Cirrus Logic accelerator to use (left or right). Highlight Vendor Supplied Driver from the list of available display drivers. Then highlight panaStation Classic, by Panacea, Inc. from the list of vendor supplied drivers. Press [Enter] to return to the main configuration menu.

- NOTE:  When using two monitors, set your other display card up as indicated in the display card manual and configure for that display (left or right) and monitor.

- At the main menu, highlight Exit and Save to exit the USCONFIG program and return to the DOS prompt. Skip to Configuring panaStation Classic.

## Reconfiguring MicroStation 5.x

After the panaStation driver is installed, MicroStation 5.x must be reconfigured. To reconfigure MicroStation 5.x, follow the procedure listed below.

- Start the MicroStation configuration program by typing

- USCONFIG
from your \USTATION sub-directory.

- At the menu Generate For, choose DOS.

- From the MicroStation Main Configuration menu, use your arrow keys to highlight the DISPLAY ADAPTERS option.

- Press the [Enter] key to display the Right Screen Graphics Adapter menu.

- From the list of available display drivers, highlight Panacea panaStation Right Screen, by Panacea, Inc. and press [Enter].

- IMPORTANT NOTE:  It does not matter if you choose the right of left screen display driver for a single monitor system. However, for a dual monitor system, select the monitor offering the highest resolution to use with the Cirrus Logic VisualMediaä accelerator..

- Answer Y to the prompt "Do you wish to reconfigure panaStation Classic?"

- Press the [Enter] key to display the Left Screen Graphics Adapter menu.

- If you are configuring a single monitor system, highlight No Second Adapter and press [Enter] to display the Virtual Swap Screen dialog box.

- To use virtual page swapping, answer "N" to the "Do you want to disable the virtual graphics page?" question. To disable virtual page swapping answer "Y". When using panaStation Classic, MicroStation's Virtual Swap Screen feature is available to all graphics boards. On graphics boards using dual page memory switching, the page swap is done using the graphics board hardware. On all other graphics boards, the page swap is simulated by panaStation Classic. Once the Swap Screen question has been answered you will return to the Main Configuration menu.

- At the main menu, highlight Exit and Save to exit the USCONFIG program and return to the DOS prompt.

## Configuring panaStation Classic

In order for the new panaStation Classic driver to take effect, it is necessary to re-boot your computer. Turn off the power to your system or press the RESET button if you have one. To configure your panaStation Classic driver, follow the procedures listed below.

NOTE:	Help during panaStation Classic setup is only a keystroke away. At any time, you may press the question mark ("?") key to get help information pertaining to the current prompt.

Please also note that the PSTATION.HLP file MUST be present in order for help information to appear. If you get an error message, be sure that the file PSTATION.HLP is in the MicroStation sub-directory given during installation.

- After re-booting change to your \USTATION directory and type
		USTATION
to start MicroStation.

- Upon start up of MicroStation. you will be greeted with the panaStation Classic welcome screen. If you do not see the welcome screen, be sure you have properly selected panaStation Classic from the list of vendor supplied drivers and that you have re-booted your computer.

- Once you have read through the panaStation Classic welcome message, press any key to continue to the Left Screen Main Configuration menu.

- You will need to configure panaStation Classic for use with your graphics board type and the resolution under which you want to run MicroStation. Use your arrow keys to highlight Select Graphics Board and then press [Enter] to display the Select Graphics Board/Resolution menu

Below are the panaStation menu descriptions used to continue configuring your panaStation Classic driver.

### Selecting Graphics Board and Resolution

The Select Graphics Board/Resolution menu configures panaStation Classic settings for graphics board/type and display and rendering resolutions. Follow the procedure listed below to continue configuring your panaStation Classic driver.

- Choose Select Graphics Board.

- Select the Cirrus Logic CL-GD546X.  Use the Up and Down arrow keys, and the Page Up and Page Down keys to scroll through the list and highlight your selection. Please note that graphics boards are listed alphabetically, therefore you may need to scroll through the list in order to find your particular board or graphics platform.

- panaStation Classic includes a graphics platform detection utility that queries the graphics card for type, Hi-Color support and installed memory. Based on the graphics platform detector's information, panaStation Classic will list a set of Display/Rendering resolutions. 

Due to the variety of graphics boards and their specific implementations of platform chip sets, the detector is not always 100% accurate. Therefore, when selecting a Display or Rendering resolution, be sure to use only resolutions that your graphics board will support. If you are unsure which resolutions are supported by your graphics board, refer to your owner's manual.

- Press [Enter] to accept your choice and return to the Select Graphics Board/Resolution menu.

- Choose Select Display Resolution to configure the display/rendering resolution of your MicroStation screen. Display/rendering options are also based on the findings of panaStation Classic's graphics platform detection utility. Use the Up and Down arrow keys, and the Page Up and Page Down keys to scroll through and highlight the resolution and color option you want to use.

- Press [Enter] to accept your resolution choice and return to the Select Graphics Board/Resolution menu.

### Adjust Aspect Ratio Menu

panaStation Classic provides you the option of adjusting the aspect ratio of your MicroStation screen. Follow the procedure listed below to continue configuring your panaStation Classic driver.

- To change the default aspect ratio, select the Adjust Aspect Ratio menu and measure, in millimeters, the height and width of the Aspect Ratio dialog box.

- Enter the measured values in the corresponding height and width data boxes.

- Highlight Return to Previous Menu and press [Enter] to return to the Main Configuration menu.

Dual Monitor Text Screen Selection

When configuring a dual screen MicroStation configuration, panaStation Classic allows you to select which monitor will display text. Follow the procedure listed below to continue configuring your panaStation Classic driver.

- To designate a monitor for text, select the Dual Screen Configuration menu and simply choose Left if you want to have the text appear on the monitor configured as MicroStation's Left screen. Or, choose Right if you would like text to appear on the monitor configured as the Right screen.

- Highlight Return to Previous Menu and press [Enter] to return to the Main Configuration menu.

- When configuring a dual monitor system a Right Screen Main Configuration menu will be displayed. Configure for the second screen as you did for the Left Screen above.

## Saving Configuration Information

Once all of the panaStation Classic parameters have been configured, select SAVE, EXIT to save the configuration information and create PANACFG1.DAT and, for dual screen configurations, PANACFG2.DAT in the \USTATION sub-directory. After the PANACFG files have been created, MicroStation will start up with the newly selected panaStation Classic resolution.

NOTE:	If you start MicroStation from a sub-directory other than the \USTATION sub-directory given during installation, PANACFG files will be created in that directory.

## Reconfiguring panaStation Classic

If panaStation Classic has already been configured, run USCONFIG and reselect your display options. Answer "Y" to go to the reconfigure panaStation dialog and restart MicroStation.

This page has been left blank intentionally.

Index

—A—

About

in WinMode utility, 20

Adapter installation, 4

ADI 4.2 driver, 30

AutoCAD commands, 68

basic configuration 
options, 43, 47

before you begin, 32

Big Picture, 49

changing colors, 54, 66

command descriptions, 58

command summary, 70

configuration options, 43

configuring 3D Studio, 40

configuring AutoCAD 
Release 11, 37

configuring AutoCAD 
Release 12 and 13, 35

configuring AutoShade 2.0 with RenderMan, 39

configuring AVE Render, 36

configuring the driver, 42

display list, 50, 65

drawing space, 50

expert configuration 
options, 44, 50

features, 30, 31

font selection, 43, 48

installation, 34

installation verification, 52

internal command echo, 51

memory management, 33, 64, 73

memory needed to run, 64

menus, 45

overview, 33

paper space, 51

reconfiguring AutoCAD, 35

reconfiguring the driver, 53

saving your configuration, 52

screen configuration, 43, 47

system requirements, 32

text lines, 43

tips and tricks, 66

troubleshooting, 72

user interface options, 44, 49

ADIreset

in ADI 4.2 driver, 56

Aspect ratio

in MicroStation driver, 84

AutoCAD

commands, 68

environment size, 38

environment variable, 35, 38

reconfiguring, 35

AutoCAD Release 11

ADI 4.2 driver. See

AutoCAD Release 12 and 13

ADI 4.2 driver. See

AUTOEXEC.BAT

in ADI 4.2 driver, 38

in WinMode utility, 14, 21

modifying for Windows 3.1 driver, 16

updating CLMode utility, 12

upgrading older DLD drivers, 67

using with TSR utility, 7

AVE Render, 36

—B—

Basic configuration options

in ADI 4.2 driver, 43, 47

Before you begin, 1

loading ADI 4.2 driver, 32

loading display drivers, 15

loading MicroStation driver, 77

Big Picture (BP), 49

BP button, 44, 49

BP highlight mode, 44, 50

BP in panning mode, 69

BP refresh, 44, 50

BP zoom mode, 44, 51

BIOS

version number, 9

Bird's eye in AutoCAD

Big Picture (BP). See

—C—

Cancel

in WinMode utility, 21

CD command, 58

CLMode utility, 8

command line options, 13

exiting, 12

help, getting, 12

installation, 8

selecting monitor type, 10

using the utility, 9

VGA controller status, 9

video modes, 11

Colors

in ADI 4.2 driver, 54, 66

in Windows NT drivers, 24, 25

in WinMode utility, 18

Command line options

in CLMode utility, 13

Commands

in ADI 4.2 driver, 58, 70

Computer

configuration, 6

installing adapter, 4

opening, 4

preparation, 3

setup, 3

Configuration

computer, 6

monitor type and selection, 10

options in ADI 4.2 driver, 43

Configuring

3D Studio, 40

ADI 4.2 driver, 42

AutoCAD Release 11, 37

AutoCAD Release 12 and 13, 35

AutoShade 2.0 with 
RenderMan, 39

AVE Render, 36

MicroStation driver, 82

OS/2 3.0 (Warp) driver, 29

Windows 3.1 driver, 18

Windows 95 driver, 27

Windows NT drivers, 24, 25

Conventions used, 1

CustomColors utility

in ADI 4.2 driver, 54

—D—

Data file corruption

in ADI 4.2 driver, 74

Display drivers, 15

ADI 4.2 driver. See

before you begin, 15

MicroStation driver. See

OS/2 3.0 (Warp) driver. See

Windows 3.1 driver. See

Windows 95 driver. See

Windows NT drivers. See

Display list

in ADI 4.2 driver, 44, 50, 65

DLDBIGPIC command, 59

DLDBPDIM command, 60

DLDBPFREEZE command, 60

DLDBPHILIGHT command, 60

DLDBPMODE command, 60

DLDBPREFRSH command, 61

DLDBPSTATIC command, 61

DLDCOLOR command, 61

DLDCOMPACT command, 61

DLDDLIST command, 62

DLDECHO command, 62

DLDHELP command, 62

DLDREFRESH command, 62

DLDSTAT command, 63

DLDUSAGE command, 63

DLDVER command, 63

DOS usage

before you begin, 1

CLMode utility, 8, 9

Double click option

in ADI 4.2 driver, 44, 49

Drawing

in ADI 4.2 driver, 55

space in ADI 4.2 driver, 44, 50

Drivers

display drivers. See

Dual monitor text screen

in ADI 4.2 driver, 43, 47

in MicroStation driver, 84

—E—

EEPROM, 9

EGA, 6

Energy Star program, 22

Environment variables

for 3D Studio, 40

for AutoCAD Release 11, 38

for AutoCAD Release 
12 and 13, 35

for AutoShade 2.0 with RenderMan, 39

Exit

in ADI 4.2 driver, 57

in AutoCAD, 42

in CLMode utility, 12

Expert configuration options

in ADI 4.2 driver, 44, 50

—F—

Features

of ADI 4.2 driver, 31

of MicroStation driver, 77

Font selection

in ADI 4.2 driver, 43, 48

Font size

in TSR utility, 7

in WinMode utility, 20

—G—

Graphics board selection

in ADI 4.2 driver, 42, 45

in MicroStation driver, 83

—H—

Help, getting

from technical support, 2

in ADI 4.2 driver, 42, 55, 82

in CLMode utility, 12

in WinMode utility, 21

—I—

Installation

adapter installation, 4

ADI 4.2 driver, 34

before you begin, 1, 15

computer preparation, 3

computer setup, 3

conventions used, 1

MicroStation driver, 78

opening computer, 4

OS/2 3.0 (Warp) driver, 28

Windows 3.1 driver, 17

Windows 95 driver, 26

Windows NT 3.5x driver, 23

Windows NT 4.0 driver, 25

Internal command echo

in ADI 4.2 driver, 44, 51

—J—

Jumper setting, 3

—L—

Load

in ADI 4.2 driver, 57

—M—

Memory management

in ADI 4.2 driver, 33, 64, 73

Memory manager conflict, 3, 11

MicroStation driver, 77

adjusting aspect ratio, 84

before you begin, 77

configuring the driver, 82

dual monitor text screen, 84

installation, 78

reconfiguring MicroStation version 4.x, 79

reconfiguring MicroStation version 5.x, 80

reconfiguring the driver, 85

saving your configuration, 85

selecting graphics board and resolution, 83

Monitor

dual-monitor setup in MicroStation, 80, 84

multi-frequency, 5

Monitor selection and configuration

in CLMode utility, 10

in WinMode utility, 19

Mouse

response in ADI 4.2 driver, 75

Multi-frequency monitor, 5

—O—

Object

in ADI 4.2 driver, 54

OK

in WinMode utility, 21

Operating system

in WinMode utility, 20

Options

in WinMode utility, 18

in CLMode utility, 9

OS/2 3.0 (Warp) driver, 28

configuring the driver, 29

installation, 28

—P—

Package contents, 2

panaStation Classic driver

MicroStation driver. See

Paper space

in ADI 4.2 driver, 44, 51, 74

PD command, 63

Physical

in ADI 4.2 driver, 56

—R—

Reconfiguring

ADI 4.2 driver, 53

AutoCAD, 35

MicroStation driver, 85

Windows 95 driver, 26

Windows NT drivers, 25

REGEN in AutoCAD, 68

Rendering resolution

for 3D Studio and AutoShade, 43

Resolution

in ADI 4.2 driver, 43, 45

in CLMode utility, 11

in MicroStation driver, 83

in Windows NT drivers, 24, 25

in WinMode utility, 18

—S—

Save

in ADI 4.2 driver, 57

Screen configuration

in ADI 4.2 driver, 43, 47

Scrolling

in ADI 4.2 driver, 74

Setup program, 3

Single monitor text screen

in ADI 4.2 driver, 47

Software

display drivers, 15

third-party in ADI 4.2 driver, 72

upgrade information, 2

utilities, 7

Speeding up AutoCAD, 30

Static sensitivity, 4

Sticky cursors

in ADI 4.2 driver, 72

System requirements

in ADI 4.2 driver, 32

—T—

Technical support, 2

Text lines

in ADI 4.2 driver, 43, 47

Third-party software with 
ADI 4.2 driver, 72

Tips and tricks for 
ADI 4.2 driver, 66

Troubleshooting

in ADI 4.2 driver, 72

TSR software in ADI 4.2 driver, 73

TSRFONT utility, 7

installation, 8

TurboDLDClassic driver

ADI 4.2 driver. See

—U—

Upgrading

older DLD Panacea drivers, 67

package software, 2

User interface options

in ADI 4.2 driver, 44, 49

Utilities

CLMode, 7

CustomColors, 54

TSRFONT, 7

WinMode, 7, 18

—V—

VESA VBE/PM, 22

VGA, 6

viewing controller status, 9

VGAreset

in ADI 4.2 driver, 56

Video

connector, 5

controller, 15

modes in CLMode utility, 11

—W—

Warning!

static sensitivity, 4

Windows 3.1 driver, 16

configuring the driver, 18

Energy Star program, 22

installation, 17

reconfiguring the driver, 17

using the WinMode utility, 18

Windows 95 driver, 26

configuring the driver, 27

installation, 26

Windows NT drivers, 23

3.5x installation, 23

4.0 installation, 25

selecting color depth, 24, 25

selecting resolution, 24, 25

WinMode utility, 14

about WinMode, 20

canceling WinMode, 21

changing colors, 18

changing operating systems, 20

changing the font size, 20

help, getting, 21

OKing selections, 21

selecting a resolution, 18

selecting monitor brand and model, 19

selecting monitor refresh rates, 19

selecting options, 20

using the utility, 18

—Z—

ZA command, 63

ZD command, 63

ZI command, 63

ZO command, 63

Zoom $1 command, 69

Zoom Dynamic

in ADI 4.2 driver, 67

Zooming and snap

in ADI 4.2 driver, 67

ZP command, 63

ZT command, 63

ZW command, 63