Disclaimer

10/100VG

(EISA/ ISA)

Selectable Network Adapters

User’s  Manual

Version  1.4E

Manual Number:  M-0018-V1.4E

Manual Release Date:  October 1995

The manufacturer provides this manual without waranty of any kind, either expressed or implied, including but not limited to the implied warranties of merchantability and fitness for a particular purpose.  The manufacturer may make improvements and/or changes to the product and/or specifications of the product described in this manual, without notice.

The manufacturer will not be liable for any technical inaccuracies or typographical errors found in this manual.  Changes are periodically made to the information contained herein and will be incorporated into revised versions of the manual.  The information contained in this document is subject to change without prior notice.

Trademark  Information

All brand and product names are the trademarks or registered trademarks of their respective owners.

Notice

Copyright  ã1995  All rights reserved.  Reproduction, adaptation, or translation without prior written permission of the manufacturer is prohibited, except as allowed under copyright laws.

Important  Information

All the latest updates and changes after the release of this manual can be found in the  RELEASE.TXT  file in the latest 10/100VG release disk.

FCC Notice

This device has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules.  These limits are designed to provide reasonable protection against harmful interference in a residential installation.  This device generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications.  However, there is no guarantee that interference will not occur in a particular installation.  If this device does cause harmful interference to radio or television reception, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the computer and receiver.

- Connect the computer into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

- Caution:  Any changes or modifications not expressly approved by the grantee of this device could void the user's authority to operate the equipment.

FCC Compliance Statement

This device complies with Part 15 of the FCC Rules.  Operation is subject to the following two conditions:

- This device may not cause harmful interference,  and

- This device must not accept any interference received, including interference that may cause undesired operation

Contents

Chapter 1:       Introduction   Seite 3

Chapter 2:       Installation Overview   Seite 6

Chapter 3:       EISA configuration utility   Seite 8

Chapter 4:       Configuring the Adapter   Seite 9

Chapter 5:       Installing the Drivers  Seite 12

Appendix A:  Technical Information   Seite 13

Chapter  ~ 1 ~

Introduction

The 10/100VG Series are network adapters based on the new high-speed LAN technology ¾ 100VG-AnyLAN.  These adapters operate at 100-Mbit/sec in a 100VG-AnyLAN network, or at 10-Mbit/sec in an IEEE 802.3 Ethernet network.

The adapters are an ideal low-cost solution for those who presently have an IEEE 802.3 Ethernet network but plan to upgrade to a 100-Mbit/sec 100VG-AnyLAN network.

In this chapter, you will find:

- An overview of the 100VG-AnyLAN technology

- Diagrams depicting the different adapter models

- 10/100VG adapter features

100VG-AnyLAN: Overview

The 100VG-AnyLAN is a new network technology, defined as the IEEE 802.12 standard.  Pioneered by Hewlett-Packard, AT&T, and others, this standard was developed primarily in response to the need for low-cost 100-Mbit/sec networks to support increasingly exacting, bandwidth-hungry and time-critical applications.

100VG-AnyLAN delivers 100-Mbit/sec performance through the use of two complimentary and related technologies: Quartet Signaling  and  Demand Priority  access method.

Quartet Signaling  makes use of all four pairs of wire in a standard 10Base-T 4-pair UTP cable to transmit or receive information.  In contrast, 10Base-T uses just two pairs (one for transmitting and another for receiving).

Demand Priority  centralizes the function of managing and allocating access to the network within the hub.  Nodes wishing to transmit a packet over the network must first make a request to the hub.  These requests will be serviced in turn and also according to whether it is a normal or high priority request.  High priority requests are serviced first regardless of their position in the queue.

Together, the two technologies give 100VG-AnyLAN these features:

- Low-cost and easy migration: Message-frame compatibility with 802.3 Ethernet ensure compatibility with existing 10Base-T cabling infrastructure, topology, bridging and routing hardware, network operating systems, network management solutions, and user software applications. In addition, 100VG-AnyLAN networks can connect to existing Ethernet networks via a simple bridge so upgrading can occur in portions.

- Greater efficiency:  By avoiding packet collisions, Demand Priority lowers network latency and minimizes network delay. Network operations are simplified and are more efficient.

- Improved Network security:  Packets are directly transmitted from originating node to hub to destination address, unlike in standard 10Base-T networks where a packet is broadcast to the entire network. This helps prevent eavesdroppers from monitoring transmissions.

- Supports time-sensitive applications:  By prioritizing requests from the nodes, the network can guarantee that time-sensitive applications will get the access time they need.  Such applications would probably be those that transmit voice, video and large graphical data, for example, multimedia applications like interactive video conferencing.  Not widely used now, these applications could be quotidian in future.  And with current network technologies not able to support them, they will need a high-speed 100VG-AnyLAN network which provides bounded latency and guaranteed bandwidth through the Demand Priority approach.

Adapter models

The  10/100VG Series  is made up of two adapter models.

- 10/100VG/EISA ¾ an EISA adapter for use in an EISA computer.

- 10/100VG/ISA ¾ an ISA adapter for use in both ISA and EISA computers

The name 10/100VG adapter is used in this manual to refer to all adapters. In situations where the instructions differ, the two adapters will be referred to as the EISA adapter and the ISA adapter.

Standard Features

- Conforms with both IEEE 802.3 10Base-T Ethernet standard, and the proposed IEEE 802.12 standard

- .

- Large on-board memory for packet buffering: 128 KB on the EISA adapter and 64 KB on the ISA adapter.

- Uses the same driver for EISA, and ISA adapters.

- Link LED indicators for each RJ-45 port to confirm good twisted-pair connection.

- 2-pin header for connection to an external LED indicator. This LED indicator will light up to show the level of network activity at the node. The 2-pin header is located at the top right corner of the adapter.

- Boot PROM socket to facilitate remote booting.

- Fully software configurable.

- Nine Interrupt Request Channels: 3,  4,  5,  7, 9,  10,  11,  12 and  15.

Advanced Features and Benefits

- Supports a variety of data transfer modes. ISA adapter support I/O Map and Memory Map modes; while EISA adapter supports I/O Map, Memory Map and Bus Master modes. This ensures compatibility with a wide range of computers.

- Connectors for 10-Mbit/sec and 100-Mbit/sec networks. The 10/100VG adapters provide high performance 10-Mbit operation in a 10Base-T (twisted-pair); or high performance 100-Mbit operation in a 100VG-AnyLAN network.

- Easy, low-cost upgrading. Because the adapters function in both 10-Mbit/sec or 100-Mbit/sec networks, they allow you to easily upgrade from a 10-Mbit/sec network to a 100-Mbit/sec network. The upgrade need not incur extra cost or investment because the same hardware and drivers are used for 10-Mbit and 100-Mbit transmission. 100VG-AnyLAN networks can also connect to existing 10Base-T Ethernet networks via a simple bridge. In this way, you can just upgrade portions of your network.

- Auto-sensing capability. The adapter automatically senses the connector used and transmits data at the appropriate data rate.

- Advanced controller chip. The 10/100VG adapters are equipped with the sophisticated ATT2MD01 chip, designed by AT&T. This chip incorporates the most advanced in application specific integrate circuits (ASICs) and surface-mount technology. The result is fewer components and a better MTBF rating which translates into an inexpensive and more reliable adapter.

- Excellent technical support. All our products are backed by our commitment to provide the best possible technical support. You can contact your vendor for more details.

Package Contents

The 10/100VG Series  network adapter package contains the following:

- One network adapter card

¾  10/100VG/ EISA  or

¾  10/100VG/ ISA

One 3.5-inch 10/100VG Diskette

- User's Manual

- Warranty Registration Card

Chapter  ~ 2 ~

Installation  Overview

This chapter guides you through the process of installing the 10/100VG network adapter in your computer. You will find instructions for:

- Installing the following 10/100VG adapters

- ¾  EISA

¾  ISA

- Remote Booting (optional) ¾ describes the installation procedure to enable remote booting on your computer.

Installing the 10/100VG Adapter

The summary shows the step-by-step sequence you need to follow in order to install the 10/100VG adapter in a computer.

Procedure:

1.	Power OFF the computer and any attached peripherals.

2.	Disconnect the cables connected to the computer.

3.	Move the keyboard, etc. to clear the area.

4.	Remove the computer's cover casing according to the manufacturer's instructions.

5.	Install the adapter in the computer.  Choose the correct expansion slot on the computer's 	motherboard as follows:

¾  EISA  adapter: choose any available EISA 32-bit slot.

¾  ISA  adapter: choose any available ISA 16-bit slot.

6.	Unscrew and remove the dummy slot cover.

7.	Align and insert the adapter into the slot. Make sure that the adapter fits securely and the 	connection is good.

8.	Fasten the screw on the mounting bracket. Replace the cover casing.

9.	Reconnect the cables to the computer.

10. 	For  EISA  adapter only:  Run the EISA configuration utility 	(ECU) supplied with the adapter.

11. 	Connect a network cable. Connect the adapter to the network via a twisted-pair cable connected to 	the appropriate RJ-45 port. For 100-Mbits, connect to any 100VG AnyLAN Hub. For 10-Mbits, 	connect to any 10Base-T Hub.

13.  	Configure the adapter. If you want to change the factory default settings of the adapter (see Chapter 	4), run the VGConfig configuration program which resides in the 10/100VG Disk.

14. 	Install the network drivers. Depending on your Network Operating System, install the proper 	network driver provided in the 10/100VG Disk.

Remote Booting (optional)

Use this section if you plan to remote boot your computer from the server.  You must first obtain a Boot PROM for the 10/100VG adapter.  The Boot PROM is sold as an optional product.

The steps below summarizes the procedures you need to follow.

1. 	Create a boot image file at the server. The procedure to create a boot image file varies between 	different Network Operating Systems (NOS). Refer to your NOS documentation for instructions.

2. 	Install the Boot PROM on the adapter.

3. 	Insert the 10/100VG Disk into the floppy drive and run the VGconfig program.

4.  	Select option "PROM Setting". Then select the PROM Address (e.g. D0000) and PROM size

(e.g. 32KB). Press <F2> to save the changes.

5.  	To exit from the VGconfig program, press <Esc> twice and select  <Yes>  to exit.

6. 	The card is now ready to boot from the server.

Installing the Boot PROM

Procedure:

1.	Locate the empty 28-pin IC socket on the network adapter card.

2.	Note that there is a notch:

¨  On one end of the Boot PROM.

¨  On one end of the 28-pin IC socket on the card.

3.	Position the Boot PROM over the IC socket with the notch on the Boot PROM aligned with the notch 	on the IC socket.

4.	Place the Boot PROM on the IC socket. Make sure that the pins are correctly aligned with the pin 	holes of the IC socket.

5.	Gently push the Boot PROM down securely on the IC socket.  Do not bend the pins.

Chapter  ~ 3 ~

EISA  configuration  utility

This chapter guides you through the process of using the EISA configuration utility (ECU).

Running the EISA configuration utility

This section is only for the EISA adapter.

EISA computers come with an automatic configuration program that allocates the system's resources to each device in the computer. The EISA configuration utility (ECU) must be executed every time you physically add, remove, or move the EISA adapter in your computer.

The ECU selects the following resources for the EISA adapter installed.

- Interrupt Request Channel (IRQ)

- Boot ROM support

- Operating Mode

EISA configuration file

For the ECU to work, it needs to read an EISA configuration file(or CFG file) for each EISA board in the system (including the system board). The CFG file describes the adapter characteristics and the system resources that the adapter requires. CFG files come with the EISA adapter and have filenames that start with an exclamation point (!) and end with the extension  “.CFG”

The CFG file for 10/100VG/EISA adapter is "!CPX0301.CFG" and it resides in the root directory of the 10/100VG Disk. The ECU will prompt you to insert the 10/100VG Disk.

Using the EISA configuration utility (ECU)

1. 	Run the EISA configuration utility. You may run the program from a diskette or you may copy the 	program to the hard disk and run it from there.

2. 	Follow the instructions on the screen.

3. 	When prompted for the vendor diskette, insert the 10/100VG Disk.

Note: 	If the ECU did not prompt for a vendor diskette, exit the utility and copy the EISA configuration file, 	!CPX0301.CFG, to the same directory as the ECU program. Then, start again from Step 1.

4.  	Follow the instructions in the ECU manual to change the resources.

5.	 Save the configuration and exit the program.

Chapter  ~ 4 ~

Configuring  the  Adapter

This chapter explains how to configure the hardware options on the 10/100VG adapter.

The hardware options and factory-default settings for EISA and ISA adapters are:

Operating Mode			[I/O Map]

I/O Base Address			[300h]

Interrupt channel (IRQ)	 	[5]

Cable type 			[AUTO]

PROM setting			[Disable]

Hardware options

The following is a brief description of each hardware option for the 10/100VG adapter.  Table 4-1 lists the available settings for each option.

Operating Mode

Set the Operating Mode to I/O Map mode, Memory Map mode, or Bus Master mode (possible for EISA adapters only).

For EISA and ISA, the adapter is set by default, to I/O Map mode, which uses the computer's standard I/O channels for communication.

For EISA and ISA, Memory Map provides slightly better performance than I/O Map, but it uses 8 KB of your system's upper memory space (the memory between 640 KB and 1 MB) for communication with the adapter.

Bus Master mode is only available for the EISA adapter. In this mode, the controller on the adapter has direct memory access to the computer system memory. This means the adapter has complete control over the systems' address and data control lines. So Bus Master mode gives higher performance and lower CPU utilization, compared to I/O Map or Memory Map.

I/O Base Address

The I/O Base Address marks the beginning of the portion of the computer's I/O address space that is used by the adapter for communication between the adapter and the computer. The adapter uses 32 consecutive bytes of I/O space. No other device in the computer should use an I/O address in the same range.

IRQ Level

The Interrupt Request Level (IRQ) is used by the adapter for communication with the interrupt handler of the adapter driver. This allows the adapter to send an interrupt request signal to the computer when it needs attention. No other device installed in the computer should use the same IRQ level.

Cable type

Depending on the connector used, you may set to TPI (VG) for 100-Mbit/sec operation, or TPI for 10-Mbit/sec 10Base-T operation. AUTO will detect which port is with cable.

PROM setting

A remote Boot PROM allows the computer to start DOS operations through the network without a local startup disk. The Disabled setting indicates that no Boot PROM is installed or you do not want it active.

If you are using a Boot PROM, set the PROM Base Address and the PROM size. The PROM Base Address marks the beginning of the portion of the computer's Memory address space that is assigned to the boot PROM. The adapter supports 8, 16, and 32 KB Boot PROMs.

Available settings

The table below lists the hardware options and their available settings.  Default settings are marked with an asterisk (*).

Table 4-1: Option settings

| Hardware option | Available settings |
| --- | --- |
| Operating Mode | I/O Map*, Memory Map, Bus Master@@ |
| I/O Base Address | 200h, 240h, 280h, 2C0h, 300h*, 340h, 380h, 3C0h |
| Interrupt Channel (IRQ) | 3, 4, 5*, 7, 9, 10, 11, 12, 15 |
| Cable type | TPI (VG), TPI, AUTO* |
| PROM Base Address | Disable*, B000h, C000h, D000h, E000h |
| PROM size | 8KB, 16KB, 32KB* |

@@ ¾  For EISA adapter only

Running VGconfig

VGconfig is an easy-to-use, menu-based configuration program for the 10/100VG adapters. You can find VGconfig in the \UTILITY directory of the 10/100VG Disk.

To configure the adapter:

1.	Insert the 10/100VG Disk into the floppy drive on your computer.

2.	Switch to the floppy drive and at the DOS prompt, type VGCONFIG  <Enter>

The program automatically scans for the adapters in the computer (maximum four) and displays the adapters detected on the screen. Note:  For faster operation, you can copy the VGconfig program to the computer's hard disk and run the program from there.

3.	For more than one adapter in the system, use the arrow keys to choose the adapter you want to 	configure, and then press  <Enter>.

4.	From the "Change" box, select the option to change and press <Enter>.  The menu of that option 	appears.

5.	Select a setting and press <Enter>.

6.	Repeat steps 4 and 5 to change the other hardware options, if  necessary.

7.	When you have finished configuring the adapter, press <F2> to save the new configuration.

8.	Repeat steps 3 to 7 to change the configuration of the other adapters in the computer.

9.	Press <Esc> to exit the program.

Chapter  ~ 5 ~

Installing  the  Drivers

The 10/100VG Disk contains network drivers that enable the adapter to work in your Network Operating System (NOS).

To install a particular network software, follow the step-by-step instruction (found in the document text files).  The detailed procedures can be found in the particular directory for that network software program.

The procedure for installing a network driver varies from one NOS to another, and sometimes from one version of a NOS to another. Because these procedures can change at any time, it's a good idea to follow the procedures described in the latest  10/100VG LAN Adapter Disk.

It is always safer to make a backup copy of your original diskette. Use DOS DISKCOPY to make a copy of the 10/100VG adapter diskette before beginning installation.  Use the backup copy diskette for installation.  Store the original diskette in a safe place.

NOTE: New drivers may be added to the floppy diskette or the present drivers modified or improved.  To keep track of changes, read the README.TXT  file in the diskette's root directory.

Running the Diagnostic program

The driver diskette contains a Diagnostic program which allows you to test 10/100VG adapters.

- Important:  Ensure that no network software or drivers are loaded and a proper network cable is connected to the card before you run the Diagnostic program.

To run the Diagnostic program:

1. 	Insert diskette into the floppy drive and make drive 	current.

2.  	At DOS prompt, type VGDIAG and press  <Enter>.  The main menu screen appears. If you have 	more than one adapter, the screen will show the number of adapters connected.

3. 	Use the arrow keys to highlight the adapter option and press  <Enter>.  A screen display appears to 	indicate the adapter configuration.

4. 	Use the arrow keys to highlight  Yes  and press  <Enter>. This starts the Diagnostics tests on the 	adapter/s.  When 	the tests are over, the test results will be displayed.

5. 	Note the results and press any key to  exit  the Diagnostics program.

Appendix  ~ A ~

Technical  Information

This appendix provides technical information about the 10/100VG network adapter.

- 10/100VG/EISA specifications

Physical  Dimensions

Height  (without bracket):	113.0 mm (4.45 inch)

Length  (without bracket):	167.6 mm (6.60 inch)

Power  Requirements

0.50 A @ 5V typical; 1.0 A @ 5V maximum

Memory  on  the  adapter

128 KB of RAM

Environmental  Operating  Range

Operating temperature: 	0° C to 50° C (ambient) 32° F to 122° F

Relative humidity: 	15% to 95% at  40° C (104° F),

non-condensing

Cable  Interfaces

The 100-Mbit/sec RJ-45 twisted-pair port can use twisted-pair cables that comply with the proposed IEEE 802.12 standard.

The 10-Mbit/sec RJ-45 twisted-pair port can use twisted-pair cables that comply with the IEEE 802.3 10Base-T standard.

Communication  Standards

IEEE 802.3 industry standard 10Base-T (10-Mbits/sec)

IEEE 802.12 proposed standard (100-Mbits/sec)

Electromagnetic

FCC part 15 Class A*/ Class B* (in US, Canada, and Latin America)  ¾ (*as applicable)

- 10/100VG/ISA specifications

Physical  Dimensions

Height  (without bracket):	106.7 mm (4.20 inch)

Length  (without bracket):	167.6 mm (6.60 inch)

Power  Requirements

0.50 A @ 5V typical; 1.0 A @ 5V maximum

Memory  on  the  adapter

64 KB of RAM

Environmental  Operating  Range

Operating temperature: 	0° C to 50° C (ambient) 32° F to 122° F

Relative humidity: 	15% to 95% at  40° C (104° F),

non-condensing

Cable  Interfaces

The 100-Mbit/sec RJ-45 twisted-pair port can use twisted-pair cables that comply with the proposed IEEE 802.12 standard.

The 10-Mbit/sec RJ-45 twisted-pair port can use twisted-pair cables that comply with the IEEE 802.3 10Base-T standard.

Communication  Standards

IEEE 802.3 industry standard 10Base-T (10-Mbits/sec)

IEEE 802.12 proposed standard (100-Mbits/sec)

Electromagnetic

FCC part 15 Class A*/ Class B* (in US, Canada, and Latin America)  ¾ (*as applicable)