DPT

BSDi 3.X Patch and

DPT Storage Manager

Build Document

1. BSDi 3.X Patch Instructions ………………………………..……. 3

A. Getting the Source from the DPT FTP site ….…………. 3

B.  Installing  the Eata30 patch   .……………………………. 3

C. Make the new kernel  ….…………………………………… 4

2. Installing Storage Manager for BSDi 3.X ………………………. 5

# 1. BSDi 3.X Patch Instructions

Install either the source or binary version of BSDi (Internet Server Version 3.X, or Version 3.X SOURCE CODE).

We hope this document provides enough detailed instructions such that those of us who are not “Unix Gurus” or “vi experts” could go through the process.

### A. Getting Source from the DPT FTP site

- Download Eata30.zip  from ftp site under the directory for your adapter type “ftp.dpt.com/pub/techsup/S???/software/drivers/unix/bsdi21/”

- This is the DPT driver patch for BSDi 3.X and this  BSDi30 Build document. You will need to unzip this file to a DOS floppy.

Note: the ??? in the ftp path should be replaced with the proper designation for your type of DPT adapter.

Example: SCIII for SmartCache III, or SRIV for SmartRAID IV

### B. Installing the Eata30 patch

| Action | Command |
| --- | --- |
| Copy Eata30.tgz from the floppy to / | (mcopy a:Eata30.tgz /) |
| From the root directory, execute the command: | gunzip Eata30.tgz |
| Execute: | tar -xvf Eata30.tar |

### C. Make the new kernel

| Action | Command |
| --- | --- |
| Change directory to the /sys/i386/conf directory | cd /sys/i386/conf |
| Copy the GENERIC or your current kernel configuration file to DPT | cp GENERIC DPT |
| Generate the /sys/compile/DPT directory. | config DPT |
| Change directories to the DPT compile directory | cd /sys/compile/DPT |
| Generate the dependencies. | make depend |
| Make the kernel | make |
| Make a copy of the current kernel | cp /bsd /bsd.orig.sav |
| Move the new kernel to root | cp bsd /bsd |
| Reboot to use the new kernel. | ctrl alt del |

2.  Installing Storage Manager for BSDi 3.X

- Getting the BSDI 3.X Storage Manager from the DPT FTP site

- Download the BSDiSM30.zip file from the beta directory on the ftp site.

- Example: ‘ftp.dpt.com/pub/techsup/beta/’

B.  Installing the Files

| Action | Command |
| --- | --- |
| To install Storage Manager for BSDI insert the Storage Manager diskette into A: and execute the following: | installfloppy |
| Create the correct Inode entry for the DPT | cd /dev rm dptr0 mknod dptr0 c 38 0 0 |
| At the prompt, execute: | startx |
| Change directory to /usr/dpt and execute the following *Note - To use the motif Storage Manager you must first configure a mouse. | Cd /usr/dpt ./dptmgr motif |