�������������������������������������������������������������������ͻ

� Generic CDROM Device Driver � SDLP SCSI Controllers � Version 1.0 �

�������������������������������������������������������������������Ķ

� Copyright (c) 1991 by James C. Davis � 11-Nov-91 � (312) 539-5506 �

�������������������������������������������������������������������ͼ

The enclosed driver, GCDROM.SYS, supports limited (data only) access to all

SCSI-based CDROM drives, when used in conjunction with an SDLP-based

SCSI controller. What this means is, if you own an SDLP SCSI

controller (a list of such controllers follows), you can add a bare

CDROM drive to your system without using up another card slot for

another controller, using this "freeware" device driver.

If all of these acronyms are too much for you, let me

break them down with some introductory information (skip to the end

for GCDROM.SYS usage info, if you already know this driver is for you):

SCSI	= "Small Computer System Interface"

Most PC compatibles are now configured with IDE drives. ESDI

drive/controllers are also popular with "power users" (sorry about

adding further acronyms), but the hottest "new" standard is SCSI.

SCSI is already THE standard for the non-compatible micro world.

The Macintosh, Sun, Apollo, Commodore Amiga, NeXt, and

various other micro and workstation vendors all use SCSI drives and

controllers. In our PC-compatible universe, SCSI is less known, but

growing in popularity. Certainly, in the future, all CDROM drives will

be SCSI drives, because it makes economic sense for the CDROM OEM (I

can't help it- can you write a computer document without alphabet soup?).

Why make two versions of a drive, one specific to the PC-bus, another

for all those other markets? A SCSI CDROM drive can be unplugged from

a Mac and plugged into a NeXt or an Amiga or a UNameIt, with no

worries, mate. So, expect to see more and more CDROM vendors turning

exclusively to SCSI. But since most PCs don't have SCSI controllers,

many vendors will still bundle a minimalist SCSI controller with

their CDROM drives.

That bundled controller becomes a burden for those of us who

use SCSI as our main drive system. My PC is SCSI-based. It came with

one of those multi-function IDE cards, but the FD/HD controller portion

is disabled-- I'm only using the card for its parallel and serial ports.

I know there are others like me, who configured their systems with

SCSI at the heart. Byte Magazine's Jerry Pournelle has often expounded on

the virtues of DPT and PSI controllers (alright-- all acronyms will be

spelled out at the end of this doc!), which, in their SCSI versions

are powerful engines for moving data through a PC. SCSI is a growing

part of the PC world, and SCSI is expandable, capable of supporting a

wide variety of devices (not just hard drives). I have 4 drives

connected to my caching controller, and when I wanted to add a CDROM

unit, I bought a "bare-drive" version. It just didn't make sense for

me to use up another card slot, another IRQ or DMA or IO space, spend

another $200-- all to connect the CDROM to an inferior controller!

I've got a SCSI controller already-- why do I need another? In my

previous experience (in the Mac and Amiga universe), a SCSI drive was

just "plug-and-play", so why couldn't I just plug my bare CDROM drive

into the bus and use it?

Well, it wasn't (and isn't) quite that easy or simple. In addition to the

hardware issues, you need two pieces of software to use a CDROM on a PC.

MSCDEX and CDROM Device Drivers

First, there's MSCDEX.EXE, the Microsoft CD Extensions. Created as

part of Microsoft's big push for CDROM a few years ago, MSCDEX

translates and interpretes CDROM data into something recognizable by DOS.

Second, there's the controller's device driver, which is (until now)

specific to the CDROM controller. The two work together: DOS calls

MSCDEX, and MSCDEX calls the device-specific driver, and your

application profits from all that work with data from the CD.

This means that, if you were to add three CDROM drives from three

different vendors to your system, not only would you use up three

separate card slots (and IO space), but you'd have three more drivers

to squeeze into high memory (you'd need only one copy of MSCDEX loaded).

Some vendors bundle cards which can support more than one drive (SCSI

itself can support 8 devices, each of which can add 8 LUNs ("sub-devices")

for a potential 64 total), and with those you could add bare drives,

but for the most part, CDROM vendors want you to use their

(recommended or bundled) controller with their drive. This also makes

economic sense, since a vendor doesn't want to write drivers for

dozens of different SCSI controllers, and controller vendors don't

want to write drivers for dozens of different CDROM drives. So you'll

see various controller vendors "married" to various drive vendors.

So, why don't controller vendors agree on a standard? Then, anybody's

driver would work with anybody's card, and anybody's SCSI drive would

be accessible. Towards that dream, someone proposed the SDLP standard.

SDLP	= "Standard Device Level Protocol".

The SDLP "standard" is still growing, and was proposed by Columbia

Data Products, a Florida company which specializes in SCSI software.

Several SCSI controller manufacturers have adopted the standard,

releasing controllers which use the protocol in one manner or

another. Here's a list of hardware which, according to Columbia Data,

currently supports or plans to support SDLP:

QUOTE...

The SDLP 1.6 Protocol is built into Columbia's SST Software package,

version 3.36.2 and higher.  The SST Software, and therefore the SDLP

1.6 DOS API, is currently supported on the following SCSI host adapters:

Western Digital   7000-FASST

Western Digital   ATXT-FASST

Fast Technology   TNT-4000

Unisys            SCSI/VGA Combo Controller

Texas Instruments Travelmate SCSI Adapter

With the 4.0 release of the SST Software package, the SDLP 1.6 protocol

will also be supported.  This release, due in the Second Quarter of 1991,

will also support these host adapters:

Western Digital   7000-EX

Western Digital   7000-SX

Adaptec           1540a,1540b,1542a,1542b

Adaptec           1640

Bustek            ISA, EISA, and MicroChannel Adapters

IBM               MicroChannel Boards

Mylex             EISA SCSI Caching Controller

DPT               EATA Adapter Family

By the end of 1991, Columbia Data Products intends to support

72 different SCSI Host Adapters.  In addition, Western Digital,

UNISYS, Texas Instruments, AT&T, Fast Technologies, and others

have licensed SDLP for use with host adapters and machines that

they ship and sell.

...ENDQUOTE

Add to this list (published by Columbia in their 1.6 spec), the

Corporate Systems Center's FastCache 32 SCSI controller, which I can

vouch for as a fine product. It also supports SDLP (at least in its

current ROM version 2.04).

I join Columbia Data in urging manufacturers to adopt and support the SDLP

standard. My contribution to the efforts of standardization is my

"generic" CDROM driver.

If you own (or plan to buy) one of the "73" controllers listed above, you

can possibly use my generic driver to access one or more CDROM drives.

One of the benefits for you: you'll free up a card slot on your PC.

I personally use "generic" to access my Pioneer DRM600 via the

CSC FC-32 caching controller. I know I can now add further CDROM

drives, if I wish, with that desired "plug-and-play" simplicity.

GCDROM.SYS	-Usage Notes

First, you'll still need a copy of MSCDEX.EXE. I can't distribute it,

because I don't have a license from Microsoft to do so... only drive

vendors can bundle it with their drives. Chances are, your drive came

with a floppy with MSCDEX included. Mine did.

MSCDEX.EXE is (usually) invoked in your autoexec.bat file.

Second, there's my GCDROM.SYS, which is added to your config.sys file.

Usage is simple:

device=GCDROM.SYS

or

device=GCDROM.SYS /D:MSCD001

A device driver which services requests from MSCDEX must name itself,

and MSCDEX must be told the driver's name. The /D: parameter tells

both the driver and MSCDEX what name to use. For my generic driver,

the name parameter is not absolutely necessary. By default, my driver

will call itself "MSCD000 ", and your invocation of MSCDEX need only

mention this, thusly:

MSCDEX /D:MSCD000

But, if you have other drivers accessing other CDROMs (why would you?),

you can name my driver with the /D: command. Just make sure MSCDEX

knows my driver's name. I've enclosed a portion of Microsoft's

documentation for MSCDEX in the file "MSCDEX.DOC".

Other IMPORTANT caveats...

1) My driver currently supports data access only. If you want to play

audio CDs-- use your stereo, go back to the OEM driver/card, or wait

until I release the next version of this driver (which will include

audio support, if all those drive vendors start answering my calls).

2) This driver will only work on SDLP-based controllers. Most of the

existing SDLP controllers use a RAM-based driver for their BIOS,

supplied by Columbia Data, usually with a name like "SSTBIOS.SYS". If

you use one of those drives, my driver MUST follow theirs in the

config.sys file. In other words, if your config.sys already has a

line in it like:

device=C:\path\SSTBIOS.SYS

...you must make sure that you put my driver AFTER that line.

3) This driver has only been tested with one CDROM drive on one SCSI

controller: the previously mentioned Pioneer 6-pack CDROM and the CSC

FastCache 32. The FastCache uses a ROM-based SDLP Bios, and the

people at CSC wrote the SDLP software themselves (they didn't license

it from Columbia Data), so it may be slightly different in its

implementation, and thus... this driver may not work for you, even if

you have a SDLP-based controller. If so, puh-lease! -call me or EMAIL

me and let me know.

4) This driver has been tested and is known to work with Microsoft's

MSCDEX versions 2.1 and 2.2 (the latest). It is 3866 bytes long, and

will work fine when laoded high, using only 3K where-ever loaded.

5) In making this driver "generic", many drive- and controller-specific

features were ignored. Audio CDs, as mentioned, cannot be played.

The most important absence, however, is the limited ability to change CDs.

My driver doesn't completely support a media change, which doesn't

mean you can't change CDs, but problems may arise:

MSCDEX caches various data about the CD, and when it suspects that

the media has changed, it asks the driver, did the media change?

If my driver says, yeah, sure, then MSCDEX reads all directory info

off the CD again (even if the media didn't really change).

If my driver says, duh, I don't know, then MSCDEX hits the disk for

the directory info anyway, to make sure.

Finally, if my driver says no, definitely not, then MSCDEX operates

on its previously cached info about the CD, which may or may not be

accurate.

The current version of GCDROM.SYS always tells MSCDEX no, the media

hasn't changed. There are several reasons why I chose to do this:

a. I use a caching controller, which has 4 megs on it. When

MSCDEX reads the CD, my controller caches the data requested. If I

then change the CD media, it doesn't matter what my driver or MSCDEX do,

because the next time I read the same numbered blocks from the CD,

my controller is going to give me the old, cached data (from the removed

CD), which is of course invalid.

b. My driver currently has no easy way of knowing if the media has

changed. I have played around with reading and caching specific

blocks at boot time, then re-reading and comparing those blocks when

queried by MSCDEX, but that means I would have to "hit the disk"

every time MSCDEX asked if the media has changed.

c. Another possible solution would have my driver reply "I

don't know" when MSCDEX asked if the media had changed. But that

means that MSCDEX would have to "hit the disk" everytime. This is

Microsoft's prescribed course of action, but I chose, for performance

reasons, to ignore their prescription, because...

d. I own a Pioneer 6-pack, and find little need to be

changing CDs in normal usage. I wrote this "generic" driver for my

own needs, after all, and am releasing it as freeware to see if

anyone else can use it. If it just sits in the ether of the BBS

universe, gathering dust under some Utility sub-menu, I'll probably

never release an improved version.

If, however, YOU can use this driver, drop me a line and I will add

enhancements for you. I intend to add audio support, as well as solve

the disk-change dilemma with the next release, but there will only be

a next release if someone asks for it. I'll be adding and improving

this driver for my own use, and it would only take a few hours of my

time to prepare it for public distribution-- but why distribute it if

nobody wants it? Why clutter up the BBS libraries with it?

BTW- if you're a fellow programmer and want to look at some working

driver code, give me a holler. I'll give the source code to those who

ask for it.

About Me...

I am Jim Davis, a 35-year-old independent programmer specializing in

C, C++, PostScript and assembly language on a variety of platforms,

including but not limited to MS-DOS, Windows, the Mac, and the Amiga.

I enjoy programming everything from device drivers to GUI applications.

I am a published author and photographer, and like most programmers

I'm into science fiction and science fact.

I have been a professional programmer for 7 lucky years (lucky to get

paid for doing what I'd do for free anyway).

And I'm looking for work (as always), so if you have a contract, give

me a call!

(Now that's enough sentences that begin with I)

References & Contacts

Contact me (for all your contract programming needs) via

CIS as 76616,2763

BIX as jamescdavis

Voice:	(312) 539-5506

US Mail:

James C. Davis

3818 N. Kimball Ave.

Chicago, IL 60618-4420

For more info on SDLP, contact

Columbia Data Products Incorporated

1070B Rainer Drive

Altamonte Springs, FLA 32751

Voice (407)-869-6700 -- Fax (407)-862-4725

BBS (407)-862-4724 (FidoNet 1:363/111)

MCIMAIL Account #394_5007.

(The complete SDLP Spec is available from their BBS).

For more info on SCSI try NCR's SCSI BBS:	(316) 636-8700 (data)

Pioneer DRM600 (6-pack) CDROM Drive:		(800) 527-3766

CSC FastCache Controller:			(408) 737-7312

DPT Controllers:				(407) 830-5522

Microsoft Info Center:				(800) 426-9400

Copyrights & Trademarks

This file and the driver "GCDROM.SYS" is Copyrighted (c) 1991 by

James C. Davis. All rights reserved, though you can use the driver

on your computer without any further permissions from me. You can't

redistribute the driver as your own.

The file "MSCDEX.DOC" is a portion of another file

Copyright (c) 1989 by Microsoft, and bears their copyright notice.

A portion of this file was quoted from the SDLP Specification 1.6,

which is copyrighted by Columbia Data Products (though the spec itself

is openly published and freely distributable, it's not public domain).

The various product names listed herein may be registered trademarks

belonging to their respective owners.

As promised, here's a list of the (30+!) acronyms used in this doc:

API	Application Programmer's Interface

BBS	Bulletin Board System

BIOS	Basic Input Output System

BIX	Byte Information Exchange

BTW	By The Way...

CD	Compact Disk (or is it Disc?)

CDROM	CD-Read Only Memory

CIS	CompuServe Information Service

CSC	Corporate Systems Center

DMA	Direct Memory Access

DOS	Disk Operating System

DPT	Distributed Processing Technology

EDSI	Enhanced Small Device Interface

EISA	Enhanced Industry Standard Architecture

FD/HD	Floppy Drive/Hard Drive

GUI	Graphical User Interface

IDE	Integrated Drive Electronics

IO	Input Output

IRQ	Intelligent Reader's Quotient (if you said, huh?, you passed)

ISA	Industry Standard Architecture

LUN	Logical Unit Number

MS-DOS	MicroSoft Disk Operating System

OEM	Original Equipment Manufacturer

PC	Personal Computer

PSI	Perceptive Solutions, Inc.

RAM	Random Access Memory

ROM	Read-Only Memory

SCSI	Small Computer System Interface

SDLP	Standard Device Level Protocol

SST 	You Got Me

VGA	Video Graphics Array