NDIS2 DRIVER FOR KTI 100VG ADAPTERS

===================================

KVGND2.DOS is the NDIS driver that complies with NDIS 2.01 specifications. It

supports the KTI 100VG ISA, EISA and PCI LAN adapters using I/O-mapped

operating mode.

DRIVER FILE:  kvgnd2.dos

DRIVER NAME:  kvgnd2$

DRIVER KEYWORDS:  the driver defines several keywords for specifying certain

adapter configuration parameters. The keywords can be specified in the system

PROTOCOL.INI file. Normally, if no keyword is specified, the driver will

search the adapter automatically and use the EEPROM settings for the adapter

operation.

As the keyword is specified, the driver will use it for the adapter and ignore

the associated setting in EEPROM dynamically.

The keywords are defined as below:

Keyword         Description

--------------- --------------------------------------------------------------

IOADDRESS       Base I/O address : specify the base I/O address for the ISA

adapter. If the keyword is not specified, the driver will

search the adapter automatically. For the EISA and PCI

adapters, the keyword is not required. Options are:

Not present : auto scan the adapter

Valid values: 0x200,0x240,0x280,0x2C0,0x300,0x340,0x380,0x3C0

------------------------------------------------------------------------------

SLOT            EISA Slot number : specify the number of the EISA slot in

which the EISA adapter is installed. If SLOT is not specified,

the driver will search the adapter automatically. Options are:

Not present : auto scan the adapter

Valid values: 1,2,3,4,5,6,7,8,9,10,11,12,13,14,15

------------------------------------------------------------------------------

BOARD		PCI board number : specify the board number of the PCI adapter

which the configuration will apply if multiple PCI adapters

are installed in one system.

Not present : the first one found

Valid values : 1, 2, 3, 4

------------------------------------------------------------------------------

INTERRUPT       Interrupt (IRQ) : specify the hardware interrupt for the

adapter. If the keyword is not present, the driver will

use the interrupt setting stored in adapter's EEPROM. Options

are:

Not present : use adapter's EEPROM setting

Valid values: 3, 4, 5, 6, 7, 9, 10, 11, 12, 15

------------------------------------------------------------------------------

MODE            Operating mode : specify the operating mode for the adapter.

This driver release only supports I/O-mapped mode. Options are:

Not present : I/O-mapped mode

Valid values: IO, MEM

IO -- I/O-mapped mode

MEM - Memory-mapped mode (* reserved in this release)

------------------------------------------------------------------------------

RAMADDRESS      Memory-mapped address : specify the memory-mapped address for

memory-mapped mode operation. But, reserved in this release.

Do not specify this keyword in this release.

------------------------------------------------------------------------------

PRIORITY        Priority level for transmission : specify the priority level

for packet transmission.

Not present : normal priority

Valid values: HIGH, NORMAL

HIGH -- set all packets transmitted as high priority packets

NORMAL- set all packets transmitted as normal priority packets

Before using HIGH for this option, contact your LAN administrator

to obtain authorization.

------------------------------------------------------------------------------