��������������������������������������������������������������Ŀ

�  HOW TO INSTALL YOUR LCS-8534 ON WINDOWS FOR WORKGROUPS 3.11 �

����������������������������������������������������������������

This subdirectory includes an OEMSETUP.INF file for WINDOWS FOR

WORKGROUPS 3.11. Using the OEMSETUP.INF file, the user can select

ShineNet LCS-8534 LAN card on Windows dialog menus.

The procedures are following:

1. Insert the Driver Diskette into the floppy drive.

For example, Drive A.

2. In the Windows for Workgroups 3.11 Program Manager, double click on

the Windows Setup icon from the MAIN window.

3. While in the Windows Setup dialog box, select the Change Network

Settings from the Options menu to install the LCS-8534 driver from

the Driver Diskette.

4. From the Network Setup dialog box, select the "Drivers..." button to

install LCS-8534 driver.

5. Then, in the Network Drivers dialog box, select Add Adapter button

to install the LCS-8534 drivers.

6. Now, in the Add Network Adapter window, choose the "Unlisted or

Updated Network Adapter" from the menu and then select OK.

7. The Install driver dialog box will appear. Enter the drive with the

LCS-8534 Driver Diskette and specify the WFW311 path to install the

LCS-8534 driver. For example, A:\WFW311. Then select OK.

8. The ShineNet LCS-8534 Adapter selection will appear under the

Network Adapters list. Simply select OK to continue.

9. Now, in the Setup dialog box, select the correct IObase and IRQ.

10.Then, select OK to exit. Follow the remaining Windows instructions,

to restart the computer to have the driver to take effect.

11.Now, If you select the ODI driver to connect to NetWare and WFW,

you have to copy the ODI driver LCS8534.COM to windows directory

and modify your AUTOEXEC.BAT file as follows:

set8534 		;or use the SystemSoft's Cardsoft

D:\WINDOWS\net start

lsl			;provided by NOVELL

d:\windows\lcs8534	;LCS-8534 ODI driver.

D:\WINDOWS\odihlp.exe

ipxodi			;provided by NOVELL, you can connect to

vlm			;NOVELL NetWare.