# Chapter 2 - Audio Services

The audio services available to  application developers for the purpose of developing Mwave compatible audio applications are identical to those described in the OS/2 publications: MMPM Application Programming Guide and MMPM Programming Reference, and the Windows publications: Microsoft Windows Software Development Kit Multimedia Programmer's Guide and the Microsoft Windows Software Development Kit Multimedia Programmer's Reference.

This chapter provides a brief overview of the Mwave Audio Architecture, the Mwave Audio Device Driver, the Mwave Windows Sound System 2.0 audio architecture and implementation description, and the facilities available for developing Mwave audio applications.

## Mwave Audio Architecture

Application developers can access the audio capabilities of Mwave compliant audio hardware through the high-level and low-level audio services provided in OS/2 MMPM/2 and  Microsoft Windows 3.1.

The host high-level and low-level audio services provide a device-independent software interface, which enables a multimedia application to take advantage of different levels of audio support on a wide range of audio hardware. The Mwave Audio device driver provides the link between the device-independent high-level and low-level audio services of the host PC and the Mwave system software and audio hardware.

## Windows Sound System 2.0 Implementation

The Windows Sound System 2.0 API is a standardized, low level, Microsoft developed mixer API that provides applications and other PC based code the ability to gather information, setup, and remain informed about the audio sources, destinations, and controls that exist on a particular hardware platform.  It provides developers with a central repository of information and an easy way to get that information in the form of a standardized Windows API.  It also allows developers to share a particular piece of hardware in that each registered user is informed of any changes made by any other registered user

The Mwave Sound System audio subsystem was architected to implement the Microsoft Windows Sound System  2.0 API and still retain the flexibility of Mwave, which can dynamically MAKE and BREAK connections, as well as mix digital and analog connections in a single stream to a specific destination.  The design point included the following requirements:

- Manage in a single functional module all audio sources that utilize common subsystem components.  These components consist of such things as SPEAKER (CDDAC), MICROPHONE (ADC), master and source volumes, etc.

- Allow ISV’s and ourselves to easily add controls and/or source/destinations to suit new hardware and application requirements.

- A desire to separate hardware specific code from non hardware specific code,  in order to minimize the amount of code that would have to be rewritten between hardware platforms

NOTE:  Not all Mwave subsystems that utilize audio are completely integrated with Sound System.  For example, TAM currently uses the SPEAKER and MICROPHONE connections, but is not tied in with the Sound System driver.  Therefore, when TAM changes volume, gains, etc., the only user that knows about those changes is TAM.  Future releases of Mwave software will include further audio interaction with Sound System.

The following diagram illustrates the relationship between the host PC high-level and low-level audio services, the Mwave Audio device driver, and the Mwave system software and audio hardware:

Figure 2-1:   Mwave Audio Architecture0

(Example for Microsoft Windows)

A multimedia application can access the audio capabilities of Mwave hardware in one of two ways:

Use the high-level audio services, the host PC's Media Control Interface (MCI) provides a high-level command interface to control the audio capabilities of Mwave hardware. As the diagram above illustrates, MCI uses the low-level audio functions to provide high-level audio services to a multimedia audio application.

## The Mwave Audio Device Drivers

The Mwave Audio Sound System architecture is architected as shown in the following diagram:

Figure 2-2:  Host Drivers Diagram

mwcmConnect, mwcmDisconnect,  wcmSetVolume, etc.

Application Messages:

SetControlDetails

Other registered drivers

Waveout Driver:

WAVE, WAVEOUTIN

Wavein Driver

WAVEIN

MIDI Driver

MIDI, MIDIOUTIN

GAMES Driver

GAMES, GAMESOUTIN

MWCM

Sound System

MIXER Driver

MWCM calls:  Peak meters,

Master  Volume, Effects

Connect and Disconnect Information:

MXDM_USER_CONNECT

MXDM_USER_DISCONNECT

MIXER_CALLBACK:

MM_MIXM_CONTROL_CHANGE,

MM_MIXM_LINE_CHANGE

MixerOpen

MIXER_CALLBACK to all registered drivers

### Component Description

The following paragraphs provide a brief description of each of the components in the audio architecture.

#### MWCM

MWCM is an acronym for the Mwave Connection Manager.   MWCM provides drivers with an API that allows drivers to connect sources to destinations at driver specified source and destination data rates.  MWCM determines, based on the driver specified data rates, what interpolators, decimators, and/or mixers are required for a particular audio connection.  If any of these connections already exist on the DSP for a different stream, it will hook into that connection, thereby saving DSP resources.  It will load and activate all required tasks, connect up all connections including the connections to the source and destination provided by the driver, and set all volumes on the stream initially to a maximum.

MWCM also provides volume set and get API’s for both individual streams (set on the input of the first mixer in the stream attached to the source) and master settings (set on the output of the LAST mixer in a stream before a specified destination).

MWCM provides API’s to insert, delete, and control effects in a stream based upon the destination specified.  Currently effects destined for the SPEAKER destination are inserted into the last mixer in the stream prior to connecting to the SPEAKER destination.  Effects destined for the RECORD destination are placed after the first mixer connected to the analog input source (the 44K mixer connected to MIC, LINE, and/or CD).

MWCM also provides an API to read the peak meters on all of its active streams.  This peak meter information is read from the input of the first mixer (the source mixer) attached to the specified source type.  There are no MASTER peak meters; a master peak is calculated in the mixer driver by reading the individual peaks and summing their result.

Note: This has the effect of showing a master peak even when the master output is MUTED, because the master peak meter is a sum of  all the input sources.

#### Multimedia Drivers

WAVE, MIDI, AUX, and WAVEIN are the standard Windows 3.1 drivers that interface to the Microsoft multimedia software layer called MMSYSTEM.  The GAMES driver is a VDD that emulates the SoundBlaster audio driver.

The following standard MCI drivers, provided in the OS/2 MMPM and Microsoft Windows 3.1, are used for processing high-level MCI calls to Mwave audio hardware:

| Device Type | Driver Filename | Description |
| --- | --- | --- |
| cdaudio | MCICDA.DRV | An MCI device driver for playing CDDA format files |
| sequencer | MCISEQ.DRV | An MCI device driver for playing standard MIDI and RIFF MIDI (RMID) files |
| waveaudio | MCIWAVE.DRV | An MCI device driver for playing and recording waveform audio files |
| Table 2-1: MCI Drivers |  |  |

These functions provide a device-independent interface which enable applications to communicate directly with the Mwave Audio device driver.

Low-level audio functions provide additional control over the multimedia device, and as a result, require more programming and are usually more complicated than using high-level services.

#### Mixer Device Driver

The Mixer Device Driver interfaces to MWCM, to the multimedia drivers, and to the Microsoft Mixer Manager.  The Mixer Manager provides applications with a low level interface to/from the mixer device driver.

### Mwave Audio Operations

The following paragraphs describe the interaction between the audio components and Sound System applications.

#### Connect and Disconnect

The interaction starts when a new audio stream is added to the system.  This occurs when an application starts to initiate a stream, say for example Media Player starts playing a WAVE file.  The app (Media Player) opens the WAVE driver, causing the WAVE driver to load its driver specific DSP code, and make a connection to MWCM.  When MWCM receives the connection, it loads and/or modifies existing DSP tasks in order to connect the supplied source GPC and data rate (specified by the driver in the MWCM connection) to the supplied destination GPC and data rate.  In the example provided the source would be the output of the WAVE PCM task, and the destination would be the CDDAC BIOS input GPC.    Once the connections are made (post connection) MWCM will inform the mixer driver that a new connection was made.

The mixer driver,  when it receives connection information, checks the connection information against its “active map”.  The active map is an internal data structure in the mixer driver used to describe which sources and destinations are currently ACTIVE (a signal is flowing through them), MUTED, or DISCONNECTED.  If a line changes state (in the above example the line would go from DISCONNECTED to ACTIVE) the mixer driver sends a callback to all registered devices indicating this change.  Then it returns control to MWCM, who returns control to the multimedia driver.  The connection has now been established.

The reverse occurs on a disconnect.  On a disconnect, the multimedia application tells the multimedia driver to close.  The multimedia driver sends a DISCONNECT message to MWCM.  MWCM, prior to unloading any DSP code, sends a DISCONNECT message to the mixer driver (predisconnect).  The mixer driver checks the line status of the disconnecting line against its “active map”, changes the status of the source to DISCONNECTED, and, if the destination has no active sources, changes the status of the destination to DISCONNECTED.  It then sends a callback to all registered devices indicating a change in line status, and returns control to MWCM, which unloads the stream, and returns control to the multimedia driver.  The disconnect sequence is now complete.

#### Handling Mixer Callbacks

Mwave has a distinct advantage over fixed, hardware only based audio platforms in that the audio streams are mixed digitally on the DSP.  It then becomes very useful to provide control over this mix, and allow multiple destinations to receive all possible sources.  With the advent of Sound System, control over these streams is relatively simple, and it just becomes necessary to manage stream mixing.  This management of stream mixing also uses the Sound System facilities and requires the drivers that are Sound System enabled to follow a few simple rules.  When the drivers implement these rules, play destinations can be easily connected to the record destination (and any other destination that the drivers choose), to provide digital mixing of analog (MIC, CD, LINE) and digital streams into the final output (record file).

The rules for drivers to hook into the record destination are as follows.

1.  Each driver has a new exported function entry point, in a FIXED code segment (Microsoft requirement) to handle the mixer callbacks.   This function must handle the two callbacks: line status changes, and control status changes.  It is up to each driver to determine what they will do when they receive these callbacks.

2.  Each driver must open the mixer, and register the callback function with the mixer driver on the mixerOpen.

3.  Once the mixerOpen has completed, drivers will then receive notice of all line and control status changes that occur.

#### Control (e.g. Volume Status Change)

For control status changes, drivers are primarily interested in changes to their volumes that occur as a result of some other mixer client changing volume.  An example of another client may be a mixer application where a user just adjusted the volume slider.  When the slider changes the mixer driver will callback the multimedia driver which can update its own volume parameters accordingly.  In addition, if an app sends a volume message to the driver, the driver should make a Sound System call (SetControlDetails) to set the volume on a stream so that other applications will also receive callback notice of the change and can update their own volume data.

#### Line status changes

For line status changes, drivers can use this information to hook into the record stream if they so choose.  For example, if WAVE play is in progress, and the RECORD destination becomes active as the result of someone starting a recording, then WAVE play can make a second MWCM connection to this new destination.  The WAVE play stream will now be digitally mixed into the RECORD stream.  In order to do this, the conditions described below must be properly handled by the driver:

1.  If WAVE is playing and RECORD starts,  when the callback occurs, WAVE calls MWCM to connect to the new RECORD destination.

2.  If RECORD is recording and WAVE starts, WAVE, as a part of its open sequence, calls MWCM to connect to both the PLAY and RECORD destinations.  All line status callbacks are ignored.

3.  If WAVE is playing and RECORD stops, when the callback occurs, WAVE disconnects the RECORD destination MWCM connection.

4.  If RECORD is recording and WAVE stops, as a part of the WAVE close sequence, WAVE disconnects both of its PLAY and RECORD destinations from MWCM.  All line status callbacks are ignored.

### Mixer Driver Description

The following discusses the Sound System mixer driver architecture and implementation.  This discussion is included to give an overview of both the generic architecture, which can be updated and modified to include new sources, destinations, and controls, and the specific implementation available today for a set of customers based off of the Mwave WHALE DSP reference design.

#### Architecture

The mixer driver is architected to handle the standard WSS 2.0 messages, and to interface into the existing Mwave multimedia drivers (MIDI, WAVE, AUX, GAMES).  It was partitioned to separate hardware specific functions from generic functions, and to allow modification of controls, sources and destinations.

Mixer Driver

Mixer Manager

Multimedia driver

MIDI, WAVE, AUX

RC Text file of Controls

MWAVE.INI

initializations of all controls

FIGURE 2-3:  Mixer Architecture

The controls, sources, and destinations are defined in an RC text file that gets loaded and parsed at Windows start.  The initial values of all controls are contained in the MWAVE.INI file, and each control is initialized at Windows start to the value contained in the INI file.  These values are written by the driver at Windows exit back into the INI file.

Special note: Because the MSMIXMGR driver loads after MMSYSTEM loads, the mixer controls are not available to the AUX, WAVE, and MIDI drivers at windows start because MMSYSTEM loads the AUX and WAVE drivers and the MSMIXMGR has not yet been initialized.  Therefore, the AUX

and  WAVE drivers must read their initial volumes out of the MWAVE.INI file, they cannot query them from the mixer driver at Windows start.  Once Windows is up however, they must get their control values from the mixer driver.

In order to manage controls, the mixer driver has a hardware specific piece of code, CONTROLS.C, which contains all the hardware specific functions necessary to set the controls defined in the RC text file.  A table exists in the mixer driver based off of control type that calls each hardware specific function (VOLUME, MUTE, etc.).

NOTE:  In the future, this control specific piece of code, along with the table lookup, may exist in a separate DLL in order to facilitate adding or changing  controls.

The RC text file is written to be human readable and changeable to allow users to change, add, or delete destinations, sources, and controls.  An INI file entry (SSRC=) in the PCMWAVE section of MWAVE.INI tells the mixer driver to read from an RC file or directly from the text file to get its setup information.  This means that a user can modify the text file, try it, change it, and when they are satisfied with the results they can recompile the text file in the RC file, and tell the driver to read from RC file again.  To read from the RC file, set SSRC equal to 1.  To read directly from the text file, set SSRC equal to 0.

It is envisioned that in the future, applications may exist that provide the user with a set of control objects (VOLUME, MUTE, REVERB, etc.), and a set of sources and destinations.  Users could design their own mixer driver by visually placing the objects on a template.  Once satisfied with the design, they could save the results, which would cause the application to write to the mixer text definition file, which would then be picked up by the mixer driver when Windows is restarted.

The following diagrams describe the current set of controls that have been designed to suit the Mwave hardware reference design for MDSP2780.

SPEAKER

Destination 0

ON/OFF

ID: 5 (5h)

BOOLEAN

Reverb/Chorus

Slider

ID:10 (A)

FADER

Bass Depth

Slider

ID:9 (9h)

FADER

Treble Depth

ON/OFF

ID:8 (8)

BOOLEAN

Bass/Treble

Slider

ID:7 (7)

FADER

Chorus Depth

Slider

ID:6 (6h)

FADER

Reverb Depth

Qsound

ID:4 (4h)

STEREOENH

ON/OFF

ID:38

BOOLEAN

ON/OFF

ID:43

BOOLEAN

ON/OFF

ID:48

BOOLEAN

PeakMeter

ID:29

PEAKMETER

PeakMeter

ID:33

PEAKMETER

PeakMeter

ID:25

PEAKMETER

PeakMeter

ID:37

PEAKMETER

PeakMeter

ID:42

PEAKMETER

PeakMeter

ID:47

PEAKMETER

PeakMeter

ID:3 (3h)

PEAKMETER

Mute

ID:24

MUTE

Mute

ID:28

MUTE

Mute

ID:32

MUTE

Mute

ID:36

MUTE

Mute

ID:41

MUTE

Mute

ID:46

MUTE

Mute

ID:2

MUTE

Balance

ID:23

BALANCE

Balance

ID:31

BALANCE

Balance

ID:27

BALANCE

Balance

ID:35

BALANCE

Balance

ID:40

BALANCE

Balance

ID:45

BALANCE

Balance

ID:1 (1h)

BALANCE

Volume

ID:26

VOLUME

Volume

ID:30

VOLUME

Volume

ID:22

VOLUME

Volume

ID:34

VOLUME

Volume

ID:39

VOLUME

Volume

ID:44

VOLUME

Volume

ID:0 (Oh)

VOLUME

MIDI

Source

0

MIC

Source

5

LINE

Source

4

CD

Source

3

WAVE

Source

1

GAMES

Source

2

FIGURE 2-4:  Play Summing Junction (Outbound Audio)

FIGURE 2-5:  Record Summing Junction (Inbound Audio)

RECORD

Destination 1

ON/OFF

ID: 16 (10h)

BOOLEAN

Reverb/Chorus

Slider

ID:21 (15h)

FADER

Bass Depth

Slider

ID:20 (14h)

FADER

Treble Depth

ON/OFF

ID:19 (13h)

BOOLEAN

Bass/Treble

Slider

ID:18 (12h)

FADER

Chorus Depth

Slider

ID:17 (11h)

FADER

Reverb Depth

Qsound

ID:15 (F)

STEREOENH

ON/OFF

ID:65

BOOLEAN

ON/OFF

ID:70

BOOLEAN

ON/OFF

ID:75

BOOLEAN

PeakMeter

ID:56

PEAKMETER

PeakMeter

ID:60

PEAKMETER

PeakMeter

ID:52

PEAKMETER

PeakMeter

ID:64

PEAKMETER

PeakMeter

ID:69

PEAKMETER

PeakMeter

ID:74

PEAKMETER

PeakMeter

ID:14 (E)

PEAKMETER

Mute

ID:51

MUTE

Mute

ID:55

MUTE

Mute

ID:59

MUTE

Mute

ID:63

MUTE

Mute

ID:68

MUTE

Mute

ID:73

MUTE

Mute

ID:13

MUTE

Balance

ID:50

BALANCE

Balance

ID:58

BALANCE

Balance

ID:54

BALANCE

Balance

ID:52

BALANCE

Balance

ID:67

BALANCE

Balance

ID:72

BALANCE

Balance

ID:12 (C)

BALANCE

Volume

ID:53

VOLUME

Volume

ID:57

VOLUME

Volume

ID:49

VOLUME

Volume

ID:61

VOLUME

Volume

ID:66

VOLUME

Volume

ID:71

VOLUME

Volume

ID:11 (B)

VOLUME

MIDI

Source

0

MIC

Source

5

LINE

Source

4

CD

Source

3

WAVE

Source

1

GAMES

Source

2

## Developing an Mwave Audio Application

The Mwave Audio device driver is 100% compliant with the low-level command interface of the OS/2 MMPM and Microsoft Windows 3.1 audio device driver specifications. As a result, any OS/2 MMPM or Microsoft Windows 3.1 application which calls functions provided in the high or low level audio services of these systems Windows will operate correctly on Mwave compliant audio hardware.

Mwave audio applications utilize the high-level and low-level audio services provided in OS/2 and Microsoft Windows 3.1.

The following manuals, provided in the Microsoft Windows 3.1 Software Development Kit, describe these services in detail, and also explain how to use these services to add multimedia audio capabilities to your Microsoft Windows 3.1 application:

·	The Microsoft Windows Software Development Kit Multimedia Programmer's Guide describes how to develop Multimedia applications for Microsoft Windows 3.1. Chapters 2-5 describe the programming interface and audio services provided in Microsoft Windows 3.1.

·	The Microsoft Windows Software Development Kit Multimedia Programmer's Reference provides a summary of the Microsoft Windows Multimedia API, including function and message descriptions, data types and structures, and Multimedia file formats.

The following manuals, provided in the Multimedia Presentation Manager Toolkit/2 , describe these services in detail, and also explain how to use these services to add multimedia audio capabilities to your OS/2 application:

·	The MMPM Application Programming Guide  describes how to develop Multimedia applications for OS/2. Chapters 2-5 describe the programming interface and audio services provided in OS/2.

·	The MMPM Programming Reference  provides a summary of the MMPM API, including function and message descriptions, data types and structures, and multimedia file formats.

All audio capabilities described in the above documentation are available to Mwave audio application developers.

### Audio Mixer API Reference

The following describes each of the Windows Sound System 2.0 messages and the Mwave Sound System driver’s implementation of each of the messages provided by the Sound System API.

#### MXDM_OPEN

The calling client is added to the mixer driver’s list of registered clients.  The calling client will be notified, via callback, of changes in controls or line status.

Note that Microsoft does not require a client to open the mixer driver in order to use the mixer driver.  A client can access the mixer driver independent of the open/close message.  The only service that open and close provides is the ability of the client to receive a callback on any change to control or line status.

#### MXDM_CLOSE

The calling client is removed from the mixer driver’s list of registered clients.  The calling client will no longer be notified, via callback, of changes in controls or line status.

#### MXDM_GETDEVCAPS

Returns the Mwave mixer device driver capabilities copied into the passed in MIXERCAPS structure.  Currently the returned values are as follows:

wMid: 		MM_MICROSOFT;

wPid:		MM_MSFT_WSS_MIXER;

vDriverVersion:	0x200

fdwSupport:	NULL

cDestinations:	2

szPname:	“Mwave Mixer Audio Driver”

#### MXDM_GETNUMDEVS

Returns 1, only 1 mixer device supported.

#### MXDM_GETLINEINFO

Returns information about a specific source or destination.  For queries of TARGETTYPE, valid targettypes are WAVEOUT, WAVEIN, and MIDIOUT.

#### MXDM_GETLINECONTROLS

Returns information about a specific set of controls.  This driver only supports the standard three queries: ALL, ONEBYID, and ONEBYTYPE.

#### MXDM_GETCONTROLDETAILS

Returns the current setting(s) of a specific control.

#### MXDM_SETCONTROLDETAILS

Sets the control to the specified state, updates the driver’s internal tables, and notifies all registered users (those that OPENED the mixer driver via MXD_OPEN) of the change in the control.  The mixer driver retains this information while Windows is running in its own internal data structures, and when Windows is shut down in INI file entries in the PCMWAVE section of the MWAVE.INI file.

#### MXDM_USER_CONNECT

Notification from MWCM (Mwave Connection Manager) of the connection of a line.  The following parameters are expected in this call:

dwUser:  	Specific instance data (none used)

dwParam1: 	MWCM Connection name

dwParam2: 	MWCM connection handle, type HMWCM.

#### MXDM_USER_DISCONNECT

Notification from MWCM (Mwave Connection Manager) of the disconnection of a line.  The following parameters are expected in this call:

dwUser:  	Specific instance data (none used)

dwParam1: 	MWCM Connection name

dwParam2: 	MWCM connection handle, type HMWCM.

### Mixer Callbacks API Reference

Callbacks are passed to a function or a window handle based on the type of callback specified in the clients call to mixerOpen.  Note that only those devices that have opened the mixer driver will receive callbacks.  The callback messages notify clients of changes in line status (ACTIVE, MUTED, DISCONNECTED), and changes in control values.  Source lines are ACTIVE when they have data flowing through them (which, on Mwave, is when they have a MWCM connection), and destination lines are ACTIVE when any source line is ACTIVE.

Callbacks are the mechanism used by Sound System enabled drivers (MIDI, WAVE, etc.) to implement Record what you play, which allows users to record both digital and analog input streams, and to update and modify global volume parameters.

#### MM_MIXM_LINE_CHANGE

This callback occurs when a line is connected or disconnected from MWCM, or changes state (MUTE, UNMUTE).

#### MM_MIXM_CONTROL_CHANGE

This callback occurs when a control changes state.