# Chapter 8 - Error Codes

This chapter contains descriptions of device specific error codes supported by the Mwave FAX and TAM drivers.  Other error codes that may be returned by the drivers are defined by the MMPM and Windows MCI support.

The Mwave FAX and TAM driver errors are returned by the mciSendCommand and mciSendString function when a failure occurs. The error code constants are defined in the MCIFTDD.H include file.  The application can issue mciGetErrorString to retrieve a textual description of the given error code.

The error codes are comprised of seven functional groups; each group is represented by a range of error codes and an error code prefix. The following table lists the error code ranges, the associated error code prefix, and the issuing drivers.

| Error Codes (Windows) | Error Codes (OS/2) | Prefix | Issuing Driver |
| --- | --- | --- | --- |
| 1-511 | 1-5255 | MCIERR_ | Errors defined by Windows or MMPM |
| 512-545 | 5256-5289 | MCIERR_FT_ | Errors common to FAX and TAM |
| 613-652 | 5357-5396 | MCIERR_FAX_ | Errors specific to FAX |
| 813-897 | 5557-5641 | MCIERR_TAM_ | Errors specific to TAM |
| 913-932 | 5657-5676 | MCIERR_DIS_ | Errors specific to the Call Discriminator |
| 1025-1213 | 5769-6334 | MCIERR_FAX_TIF_ | Errors specific to the FAX driver's BMP/TIFF conversions. |
| 1313 | 6569 | MCIERR_MEIO_ | Errors specific to MEIO |
| Table 8-1: Error Code Summary |  |  |  |

When using wave files in the TAM drivers, errors less than MCIERR_FT_DSP_NO_RESOURCES (which is  defined as MCIERR_CUSTOM_DRIVER_BASE) come directly from the wave device driver.  These are the codes below 512 (for Windows) and 5256 (for OS/2).

Error codes for the FAX and TAM specific errors are listed below in numeric order. Where appropriate, possible causes and solutions to the error are provided.

## FAX/TAM Driver Error Codes

The error codes described in this section are common to both the Mwave FAX and TAM drivers.

Win	OS/2

512	5256	MCIERR_FT_DSP_NO_RESOURCES

Insufficient resources in the DSP card.

513	5257	MCIERR_FT_DSP_FILE_NOT_FOUND

DSP file or module in the DSP file not found.

514	5258	MCIERR_FT_DSP_LABEL_NOT_FOUND

Label of resource in the DSP card not found.

515	5259	MCIERR_FT_DSP_INVALID_HANDLE

Invalid handle for DSP resource.

516	5260	MCIERR_FT_DSP_CALL_FAILED

Call to the Mwave Manager failed.

517	5261	MCIERR_FT_UNRECOGNIZED_COMMAND

Invalid or unknown command requested.

518	5262	MCIERR_FT_CMD_COMPLETE_NOT_RTN

Command complete status for FAX or TAM command not received.

519	5263	MCIERR_FT_UNRECOGNIZED_MODE

Invalid or unknown FAX or TAM mode.

520	5264	MCIERR_FT_POSTMESSAGE

Error in executing function, PostMessage.

521	5265	MCIERR_FT_MAKEPROCINSTANCE

Error in executing function, MakeProcInstance.

522	5266	MCIERR_FT_SETWINDOWSHOOKEX

Error in executing  function, tWindowsHookEx.

523	5267	MCIERR_FT_GLOBALALLOC

Error in executing function, GlobalAlloc.

524	5268	MCIERR_FT_GLOBALLOCK

Error in executing function, GlobalLock.

525	5269	MCIERR_FT_GLOBALPAGELOCK

Error in executing function, GlobalPageLock.

526	5270	MCIERR_FT_GLOBALUNLOCK

Error in executing function, GlobalUnlock.

527	5271	MCIERR_FT_GLOBALPAGEUNLOCK

Error in executing function, GlobalPageUnlock.

528	5272	MCIERR_FT_GLOBALFREE

Error in executing function, GlobalFree.

529	5273	MCIERR_FT_DSP_HARDWARE_IN_USE

Requested hardware already allocated.

530	5274	MCIERR_FT_DSP_HARDWARE_UNAVAILABLE

Requested hardware unavailable for allocation.

531	5275	MCIERR_FT_MEIO_MIC_S1_TO_CDADC_S1

Requested connection could not complete.  Check to assure audio is off.

532	5276	MCIERR_FT_MEIO_MIC_L1_TO_VOICEADC_1

Requested connection could not complete.

533	5277	MCIERR_FT_MEIO_HANDIN_1_TO_VOICEADC_1

Requested connection could not complete.

534	5278	MCIERR_FT_MEIO_HANDIN_1_TO_TELEOUT_1

Requested connection could not complete.

535	5279	MCIERR_FT_CDDAC_S1_TO_LINEOUT_1

Requested connection could not complete.

536	5280	MCIERR_FT_CDDAC_S1_TO_INTSPKROUT_L1

Requested connection could not complete.

537	5281	MCIERR_FT_MEIO_TELEDAC_1_TO_TELEOUT_1

Requested connection could not complete.

538	5282	MCIERR_FT_MEIO_VOICEDAC_1_TO_HANDOUT_1

Requested connection could not complete.

539	5283	MCIERR_FT_INSUFFICIENT_MIPS

Insufficient DSP MIPs available to satisfy the requested operation.

540	5284	MCIERR_FT_INVALID_ABS_SEG_START

Invalid Mwave absolute segment start address (0). This indicates that a DSP task was not loaded when I/O was requested.

541	5285	MCIERR_FT_INI_LABEL_NOT_FOUND

Unable to find label in an ini file.

542	5286	MCIERR_FT_CALLER_ID_NOT_VALID

Caller ID is no longer available.

543	5050	MCIERR_FT_INVALID BUFFER

The buffer length specified on MCI_INFO is not large enough to hold all of the information.  See MCI_INFO for further explanation.

544	5288	MCIERR_FT_POWERED_DOWN

System is in "power saving" mode.

545	5289	MCIERR_FT_CANT_CALL_NOW

The system is not permitted to call this phone number at the current time.  Some countries restrict automated calling machines from calling the same number too often.

546	5290	MCIERR_FT_UNKNOWN_CALLER_ID_FORMAT

The received caller ID is not in a format that the driver knows how to parse.

547	5291	MCIERR_FT_PREEMPTED_BY_HIGHER_PRTY

The driver is temporarily unavailable because higher priority work is using the DSP.

548	5292	MCIERR_FT_FUNCTION_NOT_ALLOWED_IN_COUNTRY

The requested function is not permitted based on the laws of the particular country.

549	5293	MCIERR_FT_LINE_NOT_IN_USE

The phone is not in use by any application.

550	5294	MCIERR_FT_WRONG_PHONE_COUPLER

FAX or TAM Error, the selected country does not match the external telephone coupler.

## FAX Driver Error Codes

The error codes in this section are specific to the Mwave FAX driver.

Win 	OS/2

613	5357	MCIERR_FAX_GLOBALALLOC

Error in executing function, GlobalAlloc.

614	5358	MCIERR_FAX_GLOBALLOCK

Error in executing function, GlobalLock.

615	5359	MCIERR_FAX_GLOBALUNLOCK

Error in executing function, GlobalUnlock.

616	5360	MCIERR_FAX_GLOBALFREE

Error in executing  function, GlobalFree.

617	5361	MCIERR_FAX_LCLOSE

Error in executing  function,  _lclose.

618	5362	MCIERR_FAX_LLSEEK

Error in executing function, _llseek.

619	5363	MCIERR_FAX_LREAD

Error in executing function, _lread.

620	5364	MCIERR_FAX_LSTRCPY

Error in executing function, lstrcpy.

621	5365	MCIERR_FAX_OPENFILE

Error in executing function, OpenFile.

622	5366	MCIERR_FAX_POSTMESSAGE

Error in executing function, PostMessage.

623	5367	MCIERR_FAX_NO_ELEMENT_ALLOWED

MCI_OPEN was called with a device element specified. No device element is allowed for simple devices.

624	5368	MCIERR_FAX_FLAGS_NOT_COMPATIBLE

Flags cannot be set together.

625	5369	MCIERR_FAX_UNRECOGNIZED_KEYWORD

Invalid or unknown keyword used in request.

626	5370	MCIERR_FAX_CANNOT_SET_HOOK

Phone hook cannot be set.

627	5371	MCIERR_FAX_UNRECOGNIZED_COMMAND

Invalid or unknown command requested.

628	5372	MCIERR_FAX_UNRECOGNIZED_FLAG

Invalid or unknown flag used in request.

629	5373	MCIERR_FAX_INVALID_DIAL_DIGIT

Invalid dial digit found in dial string.

630	5374	MCIERR_FAX_NULL_DIAL_STRING

Empty dial string.

631	5375	MCIERR_FAX_FILENAME_REQUIRED

Filename is required for the execution of command.

632	5376	MCIERR_FAX_UNSUPPORTED_FUNCTION

Requested function is not supported.

633	5377	MCIERR_FAX_MISSING_FLAG

Required flag not set.

634	5378	MCIERR_FAX_GLOBALREALLOC

Error in executing function, GlobalReAlloc.

635	5379	MCIERR_FAX_LSTRCAT

Error in executing function, lstrcat.

636	5380	MCIERR_FAX_WRONG_COMMAND

Command complete status received is not for outstanding command.

637	5381	MCIERR_FAX_COMMAND_REJECT

Undefined command was received.

638	5382	MCIERR_FAX_NO_FREE_STATUS_BLOCK

Insufficient buffers for status blocks.

639	5383	MCIERR_FAX_UNRECOGNIZED_STATUS

Invalid or unknown FAX status received.

640	5384	MCIERR_FAX_GLOBALPAGEUNLOCK

Error in executing GlobalPageUnlock

641	5385	Unused

642	5386	MCIERR_FAX_LWRITE

Error in executing  function, _lwrite.

643	5387	MCIERR_FAX_UNRECOGNIZED_STREAM_ID

Invalid or unknown stream identifier.

644	5388	MCIERR_FAX_INVALID_CONFIG

Configuration requested is invalid.

645	5389	MCIERR_FAX_FILTER_NOT_SET

MCI_RECEIVE issued when SET_CALL_FILTER is FALSE.

646	5390	MCIERR_FAX_MULTIPLE_OPEN

MCI_OPEN issued when another application had device open.

647	5391	MCIERR_FAX_TASK_NOT_FOUND

Driver tried to find an address in a non-existent task.

648	5392	MCIERR_FAX_INVALID_HANDLE

dwCallback specified an invalid HWND.

649	5393	MCIERR_FAX_CONFLICT_FLAGS

Specified flags conflict with one another.

650	5394	MCIERR_FAX_INVALID_STATE

Device is in incorrect state for option specified.

651	5395	MCIERR_FAX_INVALID_PARM

Invalid parameter was used in the request.

652	5396	MCIERR_FAX_HEADINGNOTSET

MCI_FAX_SET_HEADING was not performed before request to use heading (MCI_SEND_HEADING).

## TAM Driver Error Codes

The error codes in this section are specific to the Mwave TAM driver.

Win 	OS/2

813	5557	MCIERR_TAM_GLOBALALLOC

Error in executing function, GlobalAlloc.

814	5558	MCIERR_TAM_GLOBALLOCK

Error in executing function, GlobalLock.

815	5559	MCIERR_TAM_GLOBALUNLOCK

Error in executing function, GlobalUnlock.

816	5560	MCIERR_TAM_GLOBALFREE

Error in executing function, GlobalFree.

817	5561	MCIERR_TAM_LCLOSE

Error in executing function, _lclose.

818	5562	MCIERR_TAM_LLSEEK

Error in executing function, _llseek.

819	5563	MCIERR_TAM_LREAD

Error in executing function, _lread.

820	5564	MCIERR_TAM_LSTRCPY

Error in executing function, lstrcpy.

821	5565	MCIERR_TAM_OPENFILE

Error in executing function, OpenFile.

822	5566	MCIERR_TAM_POSTMESSAGE

Error in executing function, PostMessage.

823	5567	MCIERR_TAM_NO_ELEMENT_ALLOWED

MCI_OPEN was called with a device element specified. No device element is allowed for simple devices.  This return code is not returned after driver Version 2.2.

824	5568	MCIERR_TAM_FLAGS_NOT_COMPATIBLE

Flags cannot be set together.

825	5569	MCIERR_TAM_UNRECOGNIZED_KEYWORD:

Invalid or unknown keyword used in request.

826	5570	MCIERR_TAM_CANNOT_SET_HOOK

Phone hook cannot be set.

827	5571	MCIERR_TAM_UNRECOGNIZED_COMMAND

Invalid or unknown command requested.

828	5572	MCIERR_TAM_UNRECOGNIZED_FLAG

Invalid or unknown flag used in request.

829	5573	MCIERR_TAM_INVALID_DIAL_DIGIT

Invalid dial digit found in dial string.

830	5574	MCIERR_TAM_NULL_DIAL_STRING

Empty dial string.

831	5575	MCIERR_TAM_FILENAME_REQUIRED

Filename is required for the execution of command.

832	5576	MCIERR_TAM_UNSUPPORTED_FUNCTION

Requested function is not supported.

833	5577	MCIERR_TAM_MISSING_FLAG

Required flag not set.

834	5578	MCIERR_TAM_GLOBALREALLOC

Error in executing function, GlobalReAlloc.

835	5579	MCIERR_TAM_LSTRCAT

Error in executing function, lstrcat.

836	5580	MCIERR_TAM_WRONG_COMMAND

Command complete status received is not for outstanding command.

837	5581	MCIERR_TAM_COMMAND_REJECT

Undefined command was received or the command is disallowed in the particular country.  This is also received if the application indicates dial and wait for dial tone, but no dial tone is heard.

838	5582	MCIERR_TAM_NO_FREE_STATUS_BLOCK

Insufficient buffers for status blocks.

839	5583	MCIERR_TAM_UNRECOGNIZED_STATUS

Invalid or unknown TAM status received.

840	5584	MCIERR_TAM_GLOBALPAGEUNLOCK

Error in executing GlobalPageUnlock.

841	5585	MCIERR_TAM_GLOBALPAGELOCK

Error in executing function, GlobalPageLock.

842	5586	MCIERR_TAM_LWRITE

Error in executing function, _lwrite.

843	5587	MCIERR_TAM_UNRECOGNIZED_STREAM_ID

Invalid or unknown stream identifier.

844	5588	MCIERR_TAM_UNHOOKWINDOWSHOOKEX

Error in executing UnhookWindowsHookEx.

845	5589	MCIERR_TAM_INVALID_MEDIA_HANDLE

Invalid handle for media.

846	5590	MCIERR_TAM_INVALID_MEDIA_LENGTH

Media length less than 1 quality word.

847	5591	MCIERR_TAM_INVALID_MEDIA_HEADER

Invalid quality word in media header.

848	5592	MCIERR_TAM_INVALID_MEDIA_FRAME

Invalid length for SSTM frame.

849	5593	MCIERR_TAM_INVALID_MEDIA_FORMAT

Invalid format for SSTM frame.

850	5594	MCIERR_TAM_INVALID_MEDIA_DATA

Invalid data for SSTM frame.

851	5595	MCIERR_TAM_DWFROM_OUTOFRANGE

The dwFrom parameter greater than dwTo position or greater than the length of the media.

852	5596	MCIERR_TAM_DWTO_OUTOFRANGE

The dwTo parameter greater than length of the media.

853	5597	MCIERR_TAM_ACCESS_ZERO_LENGTH

The interval between dwFrom and dwTo is 0 or too small (i.e. it is within the same frame) to be executed.

854	5598	MCIERR_TAM_EMPTY_MEDIA

Media (file) is empty.

855	5599	MCIERR_TAM_NO_STREAM_EXIST

Attempt to operate a non-existing stream.

856	5600	MCIERR_TAM_ANOTHER_STREAM_RUNNING

Attempt to operate a stream while another stream is running.

857	5601	MCIERR_TAM_MODULE_NOT_LOADED

Requested module not loaded.

858	5602	MCIERR_TAM_DEVICE_NOT_USED

An unused device is selected in keyword of MCI command.

859	5603	MCIERR_TAM_DATA_OUTOFRANGE

The value of keyword of MCI command is out of range.

860	5604	MCIERR_TAM_WRONG_CONNECT

Invalid connect to devices.

861	5605	MCIERR_TAM_INVALID_FILE_HANDLE

Invalid file handle.

862	5606	MCIERR_TAM_INVALID_POSITION

Request to move to an invalid position in the media.

863	5607	MCIERR_TAM_INVALID_CONFIG

Configuration requested is invalid.

864	5608	MCIERR_TAM_INVALID_STATUS_BLOCK

Empty status block or its handle is null.

865	5609	MCIERR_TAM_UNSUITABLE_CONDITION

Operating conditions for a command are wrong or not ready.

866	5610	MCIERR_TAM_UNSUITABLE_OBJECT

Device exists but should not be operated for current command.

867	5611	MCIERR_TAM_DATA_INCORRECT

Parameter is within range, but is incorrect.

868	5612	MCIERR_TAM_MULTIPLE_OPEN

Open attempted for already open device.

869	5613	MCIERR_TAM_INVALID_MODE

Invalid mode.

870	5614	MCIERR_TAM_TPL_MEIO_ALREADY_OPENED

TAM TPL error.  MEIO already opened.

871	5615	MCIERR_TAM_TPS_MEIO_ALREADY_OPENED

TAM TPS error.  MEIO already opened.

872	5616	MCIERR_TAM_SWITCH_TO_TPS_AUDIO

TAM error switching to TPS audio.

873	5617	MCIERR_TAM_SWITCH_TO_TPS_AUDIO_REC

TAM error switching to TPS audio record.

874	5618	MCIERR_TAM_SWITCH_TO_TPS_HANDSET

TAM error switching to TPS handset.

875	5619	MCIERR_TAM_SWITCH_TO_TPL_PHONELINE

TAM error switching to TPL phoneline.

876	5620	MCIERR_TAM_SWITCH_TO_TPL_SPEAKERPHONE

TAM error switching to TPL speakerphone.

877	5621	MCIERR_TAM_SWITCH_TO_TPL_NORMALPHONE

TAM error switching to TPL normal phone.

878	5622	MCIERR_TAM_SWITCH_TO_TPL_CALL_SCREEN

TAM error switching to TPL call screening.

879	5623	MCIERR_TAM_NO_DIAL_TONE

No dial tone received.

880	5624	MCIERR_TAM_UNSUPPORTED_FLAG

Flag combination is not allowed.

881	5625	MCIERR_TAM_INVALID_HANDLE

The dwCallback was specified with an invalid HWND.

882	5626	MCIERR_FT_malloc

Error allocating storage.

883	5627	MCIERR_FT_dspLockMem

Error locking memory down for DMA transfer.

884	5628	MCIERR_FT_dspUnlockMem

Error unlocking memory after a DMA transfer.

885	5629	MCIERR_TAM_OPEN_WAVE_DRIVER

The TAM driver was unable to open the supporting wave driver.

886	5630	MCIERR_TAM_LOAD_WAVE_DRIVER

The TAM driver was unable to load a wave file.

887	5631	MCIERR_TAM_PLAY_WAVE_DRIVER

The TAM driver was unable to play a wave file.

888	5632	MCIERR_TAM_RECORD_WAVE_DRIVER

The TAM driver was unable to record a wave file.

889	5633	MCIERR_TAM_CLOSE_WAVE_DRIVER

The TAM driver was unable to close the supporting wave driver.

890	5634	MCIERR_TAM_PAUSE_WAVE_DRIVER

The TAM driver was unable to pause a wave file.

891	5635	MCIERR_TAM_SAVE_WAVE_DRIVER

The TAM driver was unable to save a wave file.

892	5636	MCIERR_TAM_RESUME_WAVE_DRIVER

The TAM driver was unable to resume playing or recording a wave file.

893	5637	MCIERR_TAM_STOP_WAVE_DRIVER

The TAM driver was unable to stop a wave file.

894	5638	MCIERR_TAM_SEEK_WAVE_DRIVER

The TAM driver was unable to seek in a wave file.

895	5639	MCIERR_TAM_STATUS_WAVE_DRIVER

The TAM driver was unable to determine the status of a wave file.

896	5640	MCIERR_TAM_SET_WAVE_DRIVER

The TAM driver was unable to set an item for a wave file.

897	5641	MCIERR_TAM_CONVERT_WAVE_DRIVER

The TAM driver was unable to convert a wave file.

908	5652	MCIERR_TAM_GAIN_CHANGE_NOT_ALLOWED

The application tried to change the microphone sensitivity in a country where that is not permitted at this time.

## Discriminator Error Codes

The error codes in this section are specific to the Call Discriminator.

Win 	OS/2

913	5657	MCIERR_DIS_TYPE_ALREADY_REGISTERED

Discriminator type already registered.

914	5658	MCIERR_DIS_TYPE_NOT_REGISTERED

Discriminator type not registered.

915	5659	MCIERR_DIS_NOT_LOADED

Discriminator not loaded.

916	5660	MCIERR_DIS_INVALID_TYPE

Discriminator invalid type.

917	5661	MCIERR_DIS_LOAD_FAIL

Discriminator load failed.

918	5662	MCIERR_DIS_APPLICATION_NOT_REGISTERED

Application of the specified type is not registered for autoanswer.

919	5663	DISCR_ALREADY_REGISTERED

An application of  the specified type is already registered for autoanswer.

920	5664	DISCR_TYPE_INVALID

The discriminator parameter usType is not valid.

921	5665	DISCR_FAX_HAS_LINE

The line is already 'owned' by the fax application.

922	5666	DISCR_TAM_HAS_LINE

The line is already 'owned' by the TAM application.

923	5667	DISCR_MODEM_HAS_LINE

The line is already 'owned' by the modem application.

924	5668	DISCR_REQUESTOR_NOT_REGISTERED

Not currently used.

925	5669	DISCR_DISCRIM_ID_INVALID

The LineID is greater than the maximum number of phone lines supported.

926	5670	DISCR_PASS_PARM_INVALID

Parameter to set pass call is invalid.

927	5671	DISCR_OPERATING_SYSTEM_ERROR

Some operating system function failed.

928	5672	DISCR_HWND_INVALID

The specified window handle is not valid.

929	5673	DISCR_INVALID_STATE

Discriminator invalid state for requested action.

930	5674	MCIERR_DIS_LOADLIBRARY_ERROR

Unable to load the discriminator library (DLL).

931	5675	MCIERR_DIS_GETPROCADDR_ERROR

Unable to get the discriminator procedure address.

932	5676	DISCR_AT_FAX_HAS_LINE

The line is already ‘owned’ by a fax modem that uses the AT command set.

## TIFF Error Codes

The error codes in this section are specific to I/O problems with TIFF files. The Mwave FAX driver uses the TIFF file format to store fax documents.

Note: The following descriptions use the abbreviation MH to refer to Modified-Huffman.

Win 	OS/2

1025	5769	MCIERR_FAX_TIF_MHTIF_CANNOTCREATETIFF

Cannot create a TIFF file for storing the MH images, because an invalid TIFF file name was supplied.  Use a TIFF file name that conforms to DOS convention.

1026	5770	MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE

Cannot allocate sufficient global memory to store byte aligned MH data.  Increase available RAM.

1027	5771	MCIERR_FAX_TIF_MHTIF_GLOCKHWRITE

Cannot lock memory for storing byte aligned MH data because either the memory block or handle is invalid, or the memory block is 0 byte.  Check the file handle.

1028	5772	MCIERR_FAX_TIF_MHTIF_GALLOCHTGT

Cannot allocate global memory to store MH filenames.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE

1029	5773	MCIERR_FAX_TIF_MHTIF_GLOCKHTGT

Cannot lock global memory for storing MH filenames. See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1030	5774	MCIERR_FAX_TIF_MHTIF_GALLOCHMEM

Cannot allocate global memory to read MH data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1031	5775	MCIERR_FAX_TIF_MHTIF_GLOCKHMEM

Cannot lock global memory designated for reading MH data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1032	5776	MCIERR_FAX_TIF_MHTIF_WRITETIF

Unused.

1033	5777	MCIERR_FAX_TIF_MHTIF_IMAGEMH

Cannot open a MH file because the file is either invalid or does not exist in the current directory.  Check the MH filename for validity.

1063	5778	MCIERR_FAX_TIF_TIFMH_GALLOCHR

Cannot allocate global memory for reading image data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1064	5779	MCIERR_FAX_TIF_TIFMH_GLOCKHR

Cannot lock memory for reading image data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1065	5780	MCIERR_FAX_TIF_TIFMH_GALLOCHW

Cannot allocate global memory for swapping every two bytes of Modified Huffman data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1066	5781	MCIERR_FAX_TIF_TIFMH_GLOCKHW

Cannot lock memory for swapping every two bytes of MH data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1067	5782	MCIERR_FAX_TIF_TIFMH_GALLOCHMHLIST

Cannot allocate global memory for storing MH filenames to be returned to calling function (FAX driver).  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1068	5783	MCIERR_FAX_TIF_TIFMH_GLOCKHMHLIST

Cannot lock memory for storing MH filenames.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1069	5784	MCIERR_FAX_TIF_TIFMH_GALLOCTIFTOMHBUF

Cannot allocate global memory for storing MH filenames.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1070	5785	MCIERR_FAX_TIF_TIFMH_GLOCKTIFTOMHBUF

Cannot lock memory for storing MH filenames.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1071	5786	MCIERR_FAX_TIF_TIFMH_GALLOCHPART

Cannot allocate memory for storing MH filenames for sorting.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1072	5787	MCIERR_FAX_TIF_TIFMH_GLOCKHPART

Cannot lock memory for storing MH filenames.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1073	5788	MCIERR_FAX_TIF_TIFMH_GALLOCHTEMP

Cannot allocate memory for an intermediate buffer during sorting of MH filenames.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1074	5789	MCIERR_FAX_TIF_TIFMH_GLOCKHTEMP

Cannot lock memory for sorting of MH filenames.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1075	5790	MCIERR_FAX_TIF_TIFMH_NOTINTELFORMAT

The TIFF file format is either invalid or not Intel. Use Intel format TIFF file.

1076	5791	MCIERR_FAX_TIF_TIFMH_CANNOTCREATEMH

Cannot create the MH file because the specified filename is invalid.  Use proper MH filename.

1077	5792	MCIERR_FAX_TIF_TIFMH_CANNOTOPENTIFF

Cannot open the TIFF file because the specified filename is either invalid or non-existent.  Ensure TIFF file is present in the current directory and specify its name correctly.

1103	5793	MCIERR_FAX_TIF_TIFBMP_CANNOTOPENTIFF

Cannot open the TIFF file because the TIFF file does not exist or is invalid.  Ensure TIFF file is present in current directory or valid.

1104	5794	MCIERR_FAX_TIF_TIFBMP_NOSUCHPAGEINTIFF

Cannot find the specified page number in TIFF file because it does not exit in the TIFF file.  Specify a valid page number.

1105	5795	MCIERR_FAX_TIF_TIFBMP_CANNOTCREATETEMPMH

Cannot create temp.mh, which is the intermediate image file extracted from the TIFF file for converting to BMP format because there is insufficient disk space. Free up disk space.

1106	5796	MCIERR_FAX_TIF_TIFBMP_GALLOCHTIFF

Cannot allocate global memory for reading image data from TIFF file.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1107	5797	MCIERR_FAX_TIF_TIFBMP_GLOCKHTIFF

Cannot lock memory for reading image data from TIFF file.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1108	5798	MCIERR_FAX_TIF_TIFBMP_LOADCCITTIMAGE

Cannot decode MH data into BMP format. Invalid MH image file, device context, bitmap handle.  Check image filename, device context, bitmap handle.

1109	5799	MCIERR_FAX_TIF_TIFBMP_SAVEBITMAP

Cannot save the bitmap into a file because there is insufficient memory for buffers to store bitmap. Ensure sufficient RAM and hard disk space is available.

1110	5800	MCIERR_FAX_TIF_TIFBMP_NOTTIFFFILE

Source file is not a TIFF file.  Ensure source file is a valid TIFF file.

1133	5801	MCIERR_FAX_TIF_INSERT_LOADBITMAPFROMFILE

Cannot load bitmap into memory from the BMP file, because either the BMP file is invalid or there is insufficient memory to load the bitmap.  Ensure sufficient RAM space and valid BMP file.

1134	5802	MCIERR_FAX_TIF_INSERT_SAVEBITMAPINCCITTFORMAT

Cannot save the memory bitmap into a MH file. The encoding of the bitmap into MH format has failed; this indicates a device context problem. Check device context and ensure the bitmap is valid.

1135	5803	MCIERR_FAX_TIF_INSERT_OPENTEMPMH

Cannot open the temp.mh file created by SaveBitmapInCcittFormat function. Too many files are open. Check the FILE parameter in config.sys to ensure that it is sufficiently large and close all unnecessary files.

1136	5804	MCIERR_FAX_TIF_INSERT_OPENTIFF

Cannot open the TIFF file because the specified TIFF file is either invalid or non-existent.  Check that TIFF file exists in current directory and is a valid one.

1137	5805	MCIERR_FAX_TIF_INSERT_GALLOCHTIFF

Cannot allocate global memory for storing MH data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1138	5806	MCIERR_FAX_TIF_INSERT_GLOCKHTIFF

Cannot lock memory for storing MH data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1163	5807	MCIERR_FAX_TIF_REPLACE_LOADBITMAPFROMFILE

Cannot load bitmap into memory from the BMP file. Invalid BMP file; insufficient memory to load the bitmap. Ensure sufficient RAM space and valid BMP file.

1164	5808	MCIERR_FAX_TIF_REPLACE_SAVEBITMAPINCCITTFORMAT

Cannot save the memory bitmap into a MH file. Encoding of bitmap into MH format failed; device context problem. Check device context; ensure valid bitmap.

1165	5809	MCIERR_FAX_TIF_REPLACE_CANNOTOPENTEMPMH

Cannot open the temp.mh file created by SaveBitmapInCcittFormat function because too many files are open. Verify that the FILES parameter in config.sys is sufficiently large and close all unnecessary files.

1166	5810	MCIERR_FAX_TIF_REPLACE_OPENTIFF

Cannot open the specified TIFF file. Check that TIFF file exists in current directory and is valid.

1167	5811	MCIERR_FAX_TIF_REPLACE_OPENTEMPTIFF

Cannot create temp.tif for duplicating current TIFF file because there is a DOS or Windows problem. Check system configuration.

1168	5812	MCIERR_FAX_TIF_REPLACE_GALLOCHTIFF

Cannot allocate global memory for storing MH data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1169	5813	MCIERR_FAX_TIF_REPLACE_GLOCKHTIFF

Cannot lock memory for storing MH data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1170	5814	MCIERR_FAX_TIF_REPLACE_CANNOTCREATETIFF

Cannot create the new TIFF file because there is a DOS or Windows problem. Check system configuration.

1193	5815	MCIERR_FAX_TIF_BMPTIF_LOADBITMAPFROMFILE

Cannot load bitmap into memory from the BMP file. If there is insufficient memory to load the bitmap, increase the available RAM space.  Verify that the BMP file is valid.

1194	5816	MCIERR_FAX_TIF_BMPTIF_SAVEBITMAPINCCITTFORMAT

Cannot save the memory bitmap into a MH file. The encoding of bitmap into MH format has failed, indicating a device context problem. Check device context and ensure the bitmap is valid.

1195	5817	MCIERR_FAX_TIF_BMPTIF_CANNOTOPENTEMPMH

Cannot open the temp.mh file created by SaveBitmapInCcittFormat function because too many files are open. Check the FILES parameter in config.sys to ensure that it is sufficiently large and close all unnecessary files.

1196	5818	MCIERR_FAX_TIF_BMPTIF_CANNOTCREATETIFF

Cannot create the TIFF file because the specified TIFF filename is invalid. Check that the TIFF filename supplied is valid.

1197	5819	MCIERR_FAX_TIF_BMPTIF_GALLOCHTIFF

Cannot allocate global memory for storing MH data.  See MCIERR_FAX_MHTIF_GALLOCHWRITE.

1198	5820	MCIERR_FAX_TIF_BMPTIF_GLOCKHTIFF

Cannot lock memory for storing MH data.  See MCIERR_FAX_TIF_MHTIF_GALLOCHWRITE.

1199	5821	MCIERR_FAX_TIF_BMPTIF_NOTBMPFILE

The specified source file is not a valid .bmp file.  Ensure source file is a valid BMP file.

1213	5957	MCIERR_FAX_TIF_NUMBER_CANNOTOPENTIFF

Unable to open the specified TIFF file.  The TIFF file is either invalid or non-existent.  Check for TIFF file validity or that it exists in the current directory.

## MEIO error codes

One MEIO-specific error code exists:

1313	6057	MCIERR_MEIO_DSPMEIOCONNECT

Mwave MEIO disconnect error.