GETTING STARTED WITH SHUTTLE DEVICE DRIVERS FOR OS/2

________________________________________________________

DOCUMENTATION

______________

o   README.DOC

This text file describes features of the drivers.

There are also instructions for installing the drivers onto

your harddisk.

o   EPIACD.REL

This file contains the release notes of the driver EPIACD.ADD

INSTALLING THE SHUTTLE DRIVERS

______________________________

This section contains information that you will need to:

o   Install the Shuttle drivers from a diskette drive

o   Update your CONFIG.SYS file

INSTALLING SHUTTLE'S DRIVERS (FROM A DISKETTE DRIVE)

__________________________________________________

STEP BY STEP PROCEDURE FOR INSTALLATION OF THE DRIVERS

_______________________________________________________

To install the drivers, follow these steps:

1. Insert the distribution disk into a diskette drive (in this

example the A drive is used).

2. Type the following on the OS/2 command line:

A:\INSTALL

3. Press Enter.

4. Follow the on-screen prompts

o To begin with the installer displays a series of bitmaps

o Next an information window pops up:

At this point you can choose either to CONTINUE with the installation

or to CANCEL the installation, by selecting the appropriate buttons.

o Choosing CONTINUE in the previous step leads to one of the following:

(a) A confirmation menu for the updation of the CONFIG.SYS (See Note1)

(This happens when you are installing the drivers for the first time)

Check the box near the Update CONFIG.SYS menu (default) to

automatically update your CONFIG.SYS.

Remove the check from the box near the Update CONFIG.SYS menu to

manually update your CONFIG.SYS. This can be done using the Tab and

Space keys or by simply clicking your mouse in the box.

or (b) A Delete/Update query menu

(This happens if you are reinstalling)

Choose the components (if any) to be deleted.

Choose the Delete button to delete the selected components.

Note: Deletion does not remove the entries made in the CONFIG.SYS file.

o Choosing CONTINUE in the previous step (a) leads to an Install Query menu.

Choose the components to be installed.

To edit the default destination path, use the Tab keys or simply click in the area.

After editing the default path, Press Enter.

Note: We recommend to specify the OS/2 system path.

OS/2 takes the drivers from \OS2 directroy or from the root directory (where OS/2 is installed).

Choose the Install button to Install the selected components.

o A status box then pops up to indicate the status of the installation.

o Finally a message box appears indicating the status of the installation.

Simply press Enter to clear the message.

5. Close the application (Alt+F4) to take you back to the OS/2 desktop.

Note1:	UPDATING CONFIG.SYS FILE

__________________________________

The installer by default does the updation of CONFIG.SYS,

however, the default can be overruled, enabling manual

updation of CONFIG.SYS file.

On the first installation the required lines are appended to the

end of the CONFIG.SYS file. In case of re-installing or if

the CONFIG.SYS already had the required entries, the installer,

however, retains those lines and no new entries are made.

6. From within the OS/2 desktop:

o Open the 'OS/2 System' folder.

o Open the 'System Setup' folder.

(in the OS/2 System folder)

o Open the 'Selective Install' application.

(in the System Setup folder)

o Enable the 'CD-ROM Support' option and choose 'OK'.

o Look for the CD-ROM device connected to your system

in the given list of CD-ROM devices.

If the device is present in the list choose it, if not

choose the 'OTHERS' option and choose 'OK'.

o The Installer queries for the Source and Destination

paths of the required files.

o The Installer then prompts for the Insertion of

Installation Diskettes in the source drive.

The Installer updates the system's CONFIG.SYS and

now the CD-ROM device can be accessed from OS/2 Window,

OS/2 Full Screen etc., just like any other drive.

COMMAND-LINE SWITCHES

______________________

1.  /Rx  -  x=0  Nibble Fast

x=1  Nibble Normal

x=2  Nibble Slow

x=3  Uni Fast

x=4  Uni Normal

x=5  Uni Slow

x=6  Toshiba Fast

x=7  Toshiba Slow

x=8  PS/2  Fast.

x=9  PS/2  Slow

x=10 EPP Normal

x=11 EPP Fast

2.  /Wx  -  x=0  Write Fast

x=1  Write Normal

x=2  Write Slow

x=3  EPP Normal

x=4  EPP Fast

3.  '/Zx'

The default EPP wait state programmed by the ADD Driver is 2.

To change the wait state, give the appropriate value in '/Zx'.

For example, to program the EPP Shuttle to zero wait, use '/Z0'.

Valid range of x : 0 to 9.

4.   '/de'

Disables the EPP automatic detection during the initialization

of the Driver.

5.  '/da'

Disables auto-detection. With no command line options, the

driver defaults to 'Nibble Slow' and 'Write Slow' modes.

6.  '/dp' - Disables PS/2 DMA arbitration.

7.  '/fe' - Force EPP mode

8.  '/fp' - Force PS/2

9.  '/fb' - Enables Bi-directional capability in some EISA chipset machines.

SHUTTLE'S PARALLEL PORT DEVICES AND DRIVERS

___________________________________________

Overview

--------

Shuttle's Parallel-port adapters provide interface between the

parallel port of the PC to either the SCSI or the IDE devices,

apart from providing a pass-thru parallel-port on the adapter.

The available adapters are:

o  EPSA (Enhanced Parallel-Port to SCSI Adapter)

o  EPIA (Enhanced Parallel-Port to IDE Adapter)

These adapters can be connected to machines based on different

Operating Systems, provided the appropriate drivers are installed

on the system.

More on the Adapters

-----------------

The EPSA-II Parallel-Port-to-SCSI protocol chip provides a

means of generating a full-blown SCSI port, as well as

"regenerating" the printer port, from the Parallel-Port signals of

a PC, allowing the connection of upto 8 SCSI devices to any

Parallel-Port (ECP/EPP/SPP) in addition to a printer.

The EPIA Parallel-Port-to-IDE protocol chip provides a means of

generating a full-blown IDE port, as well as "regenerating" the

printer port, from the Parallel-Port signals of a PC, allowing the

connection of upto 2 IDE devices to any Parallel-Port (EPP/SPP)

in addition to a printer.

WHERE TO GET MORE INFORMATION

______________________________

More information about the Shuttle-EPIA will be available in the

User Manual of the EPIA adapter.

GLOSSARY

_________

ADD    	- Adapter Device Driver

ATAPI    	- AT Attachment Packet Interface

CD-ROM  	- Compact Disk- Read Only Memories

EISA    	- Extended Industry Standard Architecture Bus

EPIA    	- Enhanced Parallel-Port to IDE Adapter

EPIACD  	- EPIA- CD-ROM driver

EPP    	- Enhanced Parallel-Port

EPSP CARD  - Enhanced and Standard Parallel-Port Card

IDE    	- Integrated Drive Electronics

MASTER DRIVE	- One of the two possible IDE drives which can

control another drive (slave drive).

NIBBLE-MODE  	- The slower modes of data transfer across the

Parallel-Port.

PS/2-MODE  	- The Bi-Directional mode  of data transfer

across the Parallel-Port.

SHUTTLE  	- Generally refers to Shuttle Technology

Parallel-Port Adapters and here to EPIA.

SLAVE DRIVE  	- One of the two possible IDE drives which can

be controlled by a master drive.

UNI-MODE  	- The Unidirectional mode of data transfer across

the Parallel-Port.