How to Install NDIS 2.0.1 Drivers

===================================

NDIS (Network Driver Interface Specification) 2.0.1 is a driver

specification developed by Microsoft Corporation to offer a standard

driver interface for all adapter cards.  A standard driver interface

provides flexibility to support not only multiple network adapter

cards but also multiple protocol stacks.  As a result, many operating

systems have incorporated the NDIS interface to make configuration

and installation of network adapter cards easy and independent of

network adapter card.

Included in this manual are instructions to install the ETHER-32V/P family

NDIS 2.0.1 driver for the following environments:

Microsoft LAN Manager 2.x

Microsoft Windows for Workgroups 3.1

Microsoft Windows for Workgroups 3.11

Banyan VINES

IBM LAN Server 2.0, 3.0

Sun Microsystems PC-NFS 5.0

How to install the Microsoft LAN Manager NDIS 2.0.1 Driver

=============================================================

The hardware for the Microsoft LAN Manager environment may be configured

to run as an OS/2 server, OS/2 workstation, enhanced MS-DOS workstation,

or a basic MS-DOS workstation. Please refer to the Microsoft documentation

for hardware and software requirements.

The directory structure provided on the ETHER-32V/P driver diskette has been

setup for automatic OEM installation.  The OEM Installation section in this

manual provides a brief explanation regarding OEM driver installation.

Follow the guidelines in the Microsoft LAN Manager Installation Guide for

more details to install the family driver from the ETHER-32V/P driver

diskettes.

Note: Some of the custom installation procedures provided here are either

an abbreviated or a duplication of the OEM installation guidelines.

LAN Manager has a SETUP program that functions to import and configure

drivers. The SETUP utility for LAN Manager will install the NDIS

driver that is provided in the ETHER-32V/P Driver Diskette. Setup requires

that two information files be present in order to install the drivers:NIF,

and PROTOCOL.INI.  These files are located in the MSLANMAN.DOS and

MSLANMAN.OS2 directories of the Driver Diskette.

The PCNTND.NIF file provides the information that the setup utility need to

coordinate and install the various drivers that are available. Information

that is contained in the two files include driver names, paths on the

diskette where the driver is stored, and the type of driver.

The PROTOCOL.INI file is used by the LANMAN Protocol manager and netbind

program at startup time. This file contains the NDIS and Transport specific

parameters for the network. Both the NDIS (ETHER-32V/P driver) and Transport

(NETBEUI) will have their own PROTOCOL.INI, and when setup installs the

NDIS and Transport, the two PROTOCOL.INI files will be merged into one

file.

To install the family NDIS driver, follow the instructions given below:

1. Insert the ETHER-32V/P driver diskette into the floppy drive.

2. From the LAN Manager subdirectory on your hard disk, run SETUP

to begin installation.

3. Use the ALT key on your keyboard and the highlighted letter to

pull down the Configuration menu (ALT + C).

4. From the Network Adapter Drivers dialog box, select the ETHER-32V/P

PCnet family driver from the menu, if shown.  Otherwise, select Other

Driver to import the driver from the ETHER-32V/P disk.

5. Then, enter the disk drive letter for the ETHER-32V/P diskette. key in

path MSLANMAN.DOS subdirectory and choose OK.

6. Select the ETHER-32V/P PCnet family Ethernet adapter and choose OK.

7. Select the desired protocol from the Network Protocol dialog box

and choose OK.

8. In the Workstation Configuration dialog box, select the ETHER-32V/P

PCnet family Ethernet adapter and choose OK.

9. Follow the remaining instructions from LAN Manager to complete

installation.