How to Installing the NDIS 3.0 Driver

========================================

Installing the Windows NT Driver

The ETHER-32V/P driver diskette also includes the NDIS 3.0 driver for the

Windows NT environment.

The directory structure provided on the ETHER-32V/P driver diskette has been

setup for automatic OEM installation.  The OEM Installation section

in this manual provides a brief explanation regarding OEM driver

installation. Follow the guidelines in the Microsoft Windows NT User's

Guide fro more details to install the family driver from the ETHER-32V/P

driver diskettes.

Note: Some of the custom installation procedures provided here are

either an abbreviated or a duplication of the OEM installation

guidelines.

To install the ETHER-32V/P NDIS 3.0 driver, simply follow the instructions

given below to navigate through the menu driven functions in Windows NT.

1. Insert the ETHER-32V/P Driver diskette into the system's floppy drive.

2. From the Control Panel in the Windows NT Main window, double

click on the Network icon to install the ETHER-32V/P PCnet family

Adapter card.

3. In the Network Settings dialog box, choose "Add Adapter."

4. In the Add Network Adapter dialog box, pull the adapter card

list menu and select "<Other> Requires disk from manufacturer" from

the list.

5. In the next window, specify the drive and path for the ETHER-32V/P files.

The Windows NT files for ETHER-32V/P reside in the WINNT subdirectory.

So, for example, enter A:\WINNT.

6. Then, select the ETHER-32V/P PCnet family Ethernet Adapter from the

dialog box to install the ETHER-32V/P adapter card.

7. Now, while in the ETHER-32V/P PCnet family Ethernet Adapter Card Setup

dialog box, select the bus type to "All" (defaults to All if nothing

is selected).

For the ETHER-32V, and ETHER-32P  adapter cards, no

IRQ or DMA channel values need to be specified.

8. Finally close the Setup dialog box and exit from the Add Network

adapter dialog box to complete the installation.  Follow any

remaining instructions from Windows NT to reboot your system,

if required.

For additional guidelines, please refer to the Microsoft Windows

NT documentation.