How to Install NetWare Server 3.11 Driver

==========================================

For a NetWare 3.11 server configuration, the ETHER-32V/P Server 3.11 driver is

included on the ETHER-32V/P driver diskette.The driver is located under the 3.11

subdirectory in the \NETWARE\SRV directory of the ETHER-32V/P disk.

Use the LOAD and BIND commands to load the NetWare 3.11 open datalink

interface (ODI) server driver as described below.  For more information

on these commands, refer to the NetWare 3.11 LAN driver installation

documentation.  Located in the Driver Disk is the version 3.11 server driver

labeled PCNTNW.LAN. Two methods for loading this driver are to use the

server command line prompted by ":" or to edit AUTOEXEC.NCF.

If you use the server command line, you will need to load the driver each time

the systemis booted. If you edit AUTOEXEC.NCF, it will automatically load the

driver each time your system is booted.

To install the NetWare Server 3.11, follow the instructions given below:

1. Install NetWare 3.11 on your system.

2. Copy all the files from 3.11 server directory of the ETHER-32V/P

diskette to the hard disk directory where NetWare 3.11 system files

are located. The following path should be used to access the files

from the ETHER-32V/P diskette:

A:\NETWARE\SRV\3.11\LSLENH.NLM

A:\NETWARE\SRV\3.11\MSM31X.NLM

A:\NETWARE\SRV\3.11\PATCHMAN.NLM

A:\NETWARE\SRV\3.11\MONITOR.NLM

A:\NETWARE\SRV\ETHERTSM.NLM

A:\NETWARE\SRV\PCNTNW.LAN

3. Boot NetWare by executing SERVER.EXE. Refer to NetWare documentation

on loading disk drivers and mounting volumes.

4. To load the driver manually, simply follow the instructions given

below.

For ETHER-32V, and ETHER-32P adapter boards, type the

following command at the file server console:

( All files below needed for NetWare 3.11 and must use versions

included on ETHER-32V/P disks)

LOAD LSLENH.NLM

LOAD PATCHMAN.NLM

LOAD MSM31X.NLM

LOAD ETHERTSM.NLM

LOAD MONITOR.NLM

<enter>

( You will need to specify DOS path if the  driver is located in

a DOS partition).

Do not use Port, INT, and DMA keywords.It is recommended that

the user use IOP, IRQ, and DRQ keywords instead.However, for the

ETHER-32V, and ETHER-32P adapter cards, no keywords need

to be specified for inter

Make sure that LSLENH, MSM31X, PATCHMAN, and MONITOR files are

present in the  same directory as PCNTNW.These files should be copied

from the ETHER-32V disk from the 3.11 subdirectory under the Netware

directory.

Then, bind the driver to the IPX protocol stack by typing:

BIND IPX TO PCNTNW NET=xxxx  <enter>

(Bind to ipx or any other protocol stack).

NOTE: Please make sure that you use the monitor program which

is distributed  on the ETHER-32V driver diskette.The original

monitor shipped with NetWare  3.11 has a bug and will

crash the system.

If two or more adapters to install:

For ETHER32-V card:

LOAD  PCNTNW IOP=xxx IRQ=xxx FRAME=Ethernet_xx

BIND IPX TO PCNTNW NET=xxxx

(Bind to ipx or any other protocol stack).

LOAD  PCNTNW IOP=xxx IRQ=xxx FRAME=Ethernet_xx

BIND IPX TO PCNTNW NET=xxxx

(Bind to ipx or any other protocol stack).

.

.

.

For ETHER32-P card:

LOAD  PCNTNW BOARD=0 BUSTYPE=PCI1 FRAME=Ethernet_xx

BIND IPX TO PCNTNW NET=xxxx

(Bind to ipx or any other protocol stack).

LOAD  PCNTNW BOARD=1 BUSTYPE=PCI1 FRAME=Ethernet_xx

BIND IPX TO PCNTNW NET=xxxx

(Bind to ipx or any other protocol stack).

.

.

.

5. To load the driver using AUTOEXEC, you must edit AUTOEXEC.NCF. First,

type "LOAD INSTALL" at the server prompt ":"  to access the install

utility.  Open  the "SYSTEM OPTIONS" menu  to access "EDIT AUTOEXEC".

Add the following command lines in AUTOEXEC.NCF file.

For example on a ETHER-32V card:

.One adapter and more frame types

LOAD  PCNTNW IRQ=3 IOP=320 FRAME=Ethernet_802.2 name=xxx

BIND IPX TO xxx NET=xxxx

LOAD  PCNTNW IRQ=3 IOP=320 FRAME=Ethernet_802.3 name=xxx

BIND IPX TO xxx NET=xxxx

.

.

.

For example on a ETHER-32P card

.One adapter and more frame types

LOAD  PCNTNW BOARD=0 BUSTYPE=PCI1 FRAME=Ethernet_802.2 name=xxx

BIND IPX TO xxx NET=xxxx

LOAD  PCNTNW BOARD=0 BUSTYPE=PCI1 FRAME=Ethernet_802.3 name=xxx

BIND IPX TO xxx NET=xxxx

.

.

.

Note:

1. You will need to specify DOS path if the  driver is located in

a DOS partition.

2. You can modify BUSTYPE=PCI2 for PCI mech#2 machines.