Trident OS/2 2.1 Display Driver (version 2.1)

January 1994

Thank you for using Trident display adapter. Currently, Trident

display drivers support the following resolutions with OS/2 2.1:

640x480x16    (standard VGA)

640x480x256

800x600x16

800x600x256

1024x768x16

1024x768x256

Disk Contents:

README.DOC      --      This File

INSTALL.BAT     --      Installation

TRIDENT.BAT

INSTALL.CMD

TR480A.os2      --      DSP File for OS/2 2.1

TR480AC.os2

TR480B.os2

TR480BC.os2

TR600A.os2

TR600AC.os2

TR600B.os2

TR600BC.os2

TR768A.os2

TR768AC.os2

TR768B.os2

TR768BC.os2

PSVGA32.os2

PSSVGA32.os2

TR480A.osw      -- DSP File for OS/2 2.1 for window

TR480AC.osw

TR480B.osw

TR480BC.osw

TR600A.osw

TR600AC.osw

TR600B.osw

TR600BC.osw

TR768A.osw

TR768AC.osw

TR768B.osw

TR768BC.osw

PSVGA32.osw

PSSVGA32.osw

PSTRID.DSC

SVGA.EXE        --      Generate PMI File

VGA             --      BVHSVGA

VSVGA.SYS       --      VDD

SV60016.DL_     --      800x600x16  Display

SV76816.DL_     --      1024x768x16 Display

VGA60016.DR_    --      800x600x16  Seamless

SW60016.DR_

VGA76816.DR_    --      1024x768x16 Seamless

SW76816.DR_

Warning: This driver is only for Final Official Release of OS/2 2.1

or OS/2 2.1 for window.

It will not work with any BETA version of OS/2 2.1

WHO NEEDS THESE DRIVERS

IBM has done a great job  to support all major super VGA

cards in OS/2 2.1. But when they wrote the code, some of Trident

chips were still under development. OS/2 2.1 is guarantteed to

work with TVGA8900B, TVGA8900C and TVGA9000, but might have trouble

to work with newer chips, such as TVGA8900CL/CXR/CXi, TGUI9420, etc.

If you happen to use the newer Trident chip, you need drivers

in this diskette.

If you use TVGA8900B/C or TVGA9000 and meet trouble, you may try

these drivers also, because they are newer and fixes some potential

problems.

PREREQUIRMENT:

We strongly recommend you installing boot manager onto the

hard disk so that you can boot OS/2 and DOS. In case you make

a big mistake in OS/2, you will have good chance to correct it by

booting from DOS and manually modify configuration files.

Before you install Trident OS/2 display drivers, make sure that

the OS/2 is installed properly for generic VGA.

To make OS/2 work in standard VGA mode, you need to tell

OS/2 that the adapter is a VGA (Video Graphics Array) rather

than Trident super VGA. The reason is: OS/2 2.1 knows how to

recognize Trident chip. When it finds out a Trident card is

installed, it will assume the board is a TVGA8900B/C or TVGA9000

board, and use the corresponding special parameters to drive the

card. Sometimes, that will set the new chip into an incorrect status.

Use the following procedure to tell OS/2 to use generic VGA:

1.  After most of OS/2 is installed, you are asked to select

video card. It will display the "Primary Display" is

"Trident super VGA"

Now, use mouse to click the square beside "Primary Dispaly"

and it becomes "checked", and click "OK".

2.  In the list of display cards, select

"Video Graphics Array (VGA)"

NOTE: Do NOT choose "IBM VGA 256c", which is not standard

VGA.

3.  Continue the installation.

When you install the driver first time, be sure to make

ORIGINAL OS/2 2.1 display driver diskettes handy, since

most of the files needed to install Trident super VGA

drivers are there.

INSTALLATION

Since the drivers are too big to fit into one floppy, we

need to copy all necessary files into a hard disk. Using hard

disk will let you change resolutions easily.

If you do not want to change resolution after the installation.

you can simple delete the whole directory after you selected

the right driver. You can save about 1.5M bytes hard disk

space by deleting the whole directory.

The installation procedure only works under DOS. It can be a full

screen DOS session under OS/2 or a true DOS by booting from DOS

partition. If you install OS/2 under HPFS, you must run this

installation under full screen DOS session of OS/2.

To copy drivers into hard disk, at "A:\>" prompt, run

the following command:

INSTALL boot_drv: [dsp_drv:\dsp_path] [dst_drv:\dst_path]

Where:

boot_drv            OS/2 boot drive. Must be given.

dsp_drv,dsp_path    Drive and path where the original OS/2

display drivers is located. It is an

optional parametr. The default is A:.

If dst_drv is A, B, or CD ROM  don't

give dsp_path.

dst_drv,dst_path    The destination drive and path where

Trident display driver is copied to.

An optional paramrter. The default is

OS2_drive:\TROS2DRV. If this parameter

is given, dsp_drv:\dsp_path must be

specified.

Examples:

INSTALL C:

The OS/2 is installed on C: drive.

The original OS/2 display driver will be in A:.

Trident OS/2 drivers will be copied into C:\TROS2DRV.

INSTALL D: D:\DISP

The OS/2 is installed on D: drive.

The original OS/2 display driver is in D:\DISP.

Trident OS/2 drivers will be copied into D:\TROS2DRV.

INSTALL C: A: C:\MYDRV

The OS/2 is installed on C: drive.

The original OS/2 display driver will be in A:.

Trident OS/2 drivers will be copied into C:\MYDRV.

INSTALL D: G: D:\MYDRV

The OS/2 is installed on D: drive.

The original OS/2 display driver will be in G:

(CD ROM).

Trident OS/2 drivers will be copied into D:\MYDRV.

After installation, the following files will be in \TROS2DRV:

DSPRES.DL_              TR480A.DSP

IBMVGA32.DL_            TR480AC.DSP

VGA                     TR480B.DSP

WINVGA                  TR480BC.DSP

WINXGA                  TR600A.DSP

SV60016.DL_             TR600AC.DSP

SV480256.DL_            TR600B.DSP

SV600256.DL_            TR600BC.DSP

SV768256.DL_            TR768A.DSP

WSPDBF.DR_              TR768AC.DSP

WSPDSBF.DR_             TR768B.DSP

WSPDSF.DR_              TR768BC.DSP

WSPDSSF.DR_             PSVGA32.DSP

DSP256.DL_              PSSVGA32.DSP

DSP76816.DL_

SW256.DR_

SVGA.EXE   is copied to \OS2;

VSVGA.SYS  is copied to \OS2\MDOS;

PSTRID.DSC is copied to \OS2\INSTALL

A new SVGADATA.PMI file will be generated in \OS2.

SET RESOLUTION

You need to run DSPINSTL (display installation) to select

color and resolution you want.

During DSPINSTL, choose "Trident" as primary display.

When asked to assign where the driver is, set "Source Directory"

to D:\TROS2DRV or where you copy the drivers to.

Copyright Trident Microsystem, 1993, all rights reserved.