Beta v7 CENTRE.EXE Utility Documentation File

The program CENTRE.EXE will permit you to edit the video mode setup

tables for all the standard and ergonomic modes supported.  After

changing the timings for a mode the modified table can be stored in the

TSR "PGRAPH.COM" (or the TSR will be created, if it does not exist in

the current directory).

This program supports those STB video cards that have the S3 86C911 GUI

Accelerator chip or the Tseng Labs ET4000 Super VGA chip.

To run CENTRE, copy the CENTRE.EXE file to the directory of the

application you need to run CENTRE for.  For instance, if you need to

use CENTRE for Windows 3.0, you would type "copy a:\centre.exe

c:\windows", assuming CENTRE.EXE is in the root directory of your A:

drive, and Windows 3.0 is under C:\WINDOWS.  CENTRE.EXE must be run from

the directory it is in because it creates another program (PGRAPH.COM),

which is the utility that actually modifies the configuration.  The

program is invoked with one of the following command lines:

CENTRE                    Normal

WARNING: Usage of these two modes can cause damage to some monitors. Use

them only if you are knowledgable about advanced video functions.

CENTRE EXPERT             Enable all functions

CENTRE EXPERT CLOCKS      For non-standard STB ET4000 based cards

When you run CENTRE.EXE, a box will be drawn around the border of the

screen.  Within this box the current video parameters and a help window

showing available functions are displayed.  This screen is normally

visible in video modes with 80 or more text columns and it becomes

visible if F1 or F9 are pressed.

The arrow keys can be used to shift the screen image in the direction

selected (that is, the LEFT-ARROW key will cause the image to move to

the left, UP-ARROW will cause the image to move up, etc.).

F1 shows or redraws a brief help screen.  It is most useful when working

in a 40 column mode where it is normally not visible.  If the screen is

in a 40 column mode, you can revert to the normal screen by pressing any

key.

The F2 function saves the current table over the one in memory if

PGRAPH.COM has been run.  It always saves it over the table for the

current mode in PGRAPH.COM in the current directory, creating it if it

does not exist.  THIS IS THE ONLY OPERATION THAT RECORDS (SAVES) YOUR

CHANGES TO A TABLE.  Because CENTRE saves only to PGRAPH.COM, if you

wish to set up multiple configurations you will need to exit CENTRE,

rename PGRAPH.COM to something else, and re-enter CENTRE.

F3 allows you to select another video mode.  The video modes and their

resolutions are listed in the documentation for your video card.

F4 discards any changes you have made to the current video mode.

The F5 and F6 functions are similar to the F2 function, but in addition

to updating the current mode these functions copy the horizontal or

vertical changes, respectively, to all tables that exactly match the old

horizontal or vertical timings of the mode just modified.  The table is

then updated in PGRAPH.COM and, if present, in the RAM resident tables.

These functions should make the time spent changing video timings much

more efficiently used.

**************************************************************************

*   Functions F7, F8, and F9, as well as the HOME/END and PGUP/PGDN key  *

*   operation are disabled unless the program is entered with the EXPERT *

*   command line parameter.                                              *

*                                                                        *

*   WARNING: THESE FUNCTIONS CAN CAUSE DAMAGE TO SOME MONITORS!          *

**************************************************************************

The F7 key permits changing non-variable parameters for a video mode.

Modes supported in this beta version are the video dot clock frequency,

the interlaced/non-interlaced characteristic, and the polarities of the

vertical and horizontal sync signals.  To change the state of the

"DotClock/2" bit in sequencer register 1, enter the clock desired

followed by a "/1" for undivided clocks or by "/2" for a clock divided

by 2 (as in most 320x200 resolution modes).

The F8 key invokes a non-functional mode setup facility that will

eventually permit use of this utility to create non-standard text and

graphics modes. Do not expect this function to do anything useful.

The F9 key selects one of the 6 possible parameters of the horizontal

and vertical timings to be manipulated with the HOME/END or

PAGE-UP/PAGE-DOWN keys, respectively.  The parameters are the number of

displayed bytes or lines, the start of the blanking period, the start of

the sync pulse, the end of the sync pulse, the end of the blanking

period, and the total number of bytes or lines in the display period

(horizontal or vertical, respectively). Once a parameter is selected,

its value can be reduced by pressing the HOME key (horizontal) or

PAGE-UP key (vertical).  The value can be increased by pressing the END

or PAGE-DOWN key.  Since the current state of the parameter selected is

visible only on the help screen, the help screen will be made active in

40 column modes whenever the F9 key is pressed.  Shift-F9 will rotate

through the parameters in the reverse order.

The F10 key is used to exit the program.

Once you have exited CENTRE and saved any modifications to PGRAPH.COM,

you may run PGRAPH and then your application. For example, assuming you

are in the C:\WINDOWS directory and want to run Windows, you would

typically enter

PGRAPH

WIN

It is recommended that you create a batch file to do this automatically.

A typical batch file for Windows would be:

CD\WINDOWS

PGRAPH

WIN

*************************** END OF DOCUMENTATION ***********************