VideoLogic GrafixStar 300 for Windows 95

23th May 1997.

Contents

1. Introduction

2. Installing GrafixStar 300

3. Note for users with 2MB GrafixStar 300 cards

4. Troubleshooting

5. Field notes

1. Introduction

This file contains installation instructions, late-breaking information, troubleshooting information, and notes for GrafixStar 300.

Please note: This version of the GrafixStar 300 driver is DirectX 2 compatible, but not SmartTools or VMC compatible.

2. Installing GrafixStar 300

Introduction

The easiest way to install your GrafixStar 300 is to install the hardware first, referring to the User Guide, and then install the software using the Interactive Installation Guide which runs automatically when you insert the CD into your PC. However, if you don’t have a User Guide, or you want to install the software manually, you should refer to the following instructions.

Handling GrafixStar 300

Follow these guidelines to minimize the risk of electrostatic charges damaging the card:

	Keep the card in its protective packaging until you are ready to install it.

	Just before handling the card, touch the metal frame of your computer with the back of your hand to discharge any static electricity which may have built up on you or your clothes.

	Hold the card by the adapter bracket or by its edges; do not touch the printed circuit board.

	Do not place the card on top of your computer or on any other metal surface.

	Make sure that the card is not accidentally touched by anyone else.

	Handle the card with care to avoid damage to the fragile electronic components mounted on the circuit board.

Installing the hardware

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

1.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

2.	Disconnect your monitor cable from your computer.

3.	Remove the computer’s cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

4.	Remove your existing display card or disable your built-in graphics controller. Refer to the documentation supplied with your computer for instructions on how to disable a built-in graphics controller.

5.	Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

6.	Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

7.	Align GrafixStar 300 with the slot you have chosen. Firmly press the card into the slot. You will need to use some pressure to push the card into position, but do not use excessive force.

8.	Secure GrafixStar 300 using the screw that you removed previously.

9.	Replace the cover of your computer.

10.	Connect the monitor cable to GrafixStar 300. All cables connected to GrafixStar 300 should be properly shielded and comply with radio frequency interference standards.

11.	Switch on your computer and your computer monitor. You are now ready to install the software.

Installing the software

The easiest way to install the software is to use the Display Properties window. Installation will vary slightly depending upon which build of Windows 95 you are using. Use the following steps to find out which build of Windows 95 you are using, then follow the instructions for that version.

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the System icon to open the System Properties Window.

3.	Ensure that the General tab is selected. This page has details about the System, Registration information, and Computer details. The build number is located in the System section, under the ‘Microsoft Windows 95’ line.

Software installation for Windows 95 build 4.00.950 or 4.00.950a

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Change Display Type button.

5.	Make sure that the Monitor Type line contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

6.	Click the Change button for the Adapter Type.

7.	Insert the GrafixStar CD into your computer (close the Interactive Installation Guide if it autoruns at this point) and click Have Disk from the Select Device menu. Browse to the folder [disk]:\gs300\DX2\disk1 where [disk] is your CD drive.

8.	Click GrafixStar 300 from the list. Click OK.

9.	When the GrafixStar files have been copied to your PC, the Change Display Type window is displayed again. Click Close.

10.	The Display Properties windows is displayed again. Click Apply.

11.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

Software installation for Windows 95 build 4.00.950 B (otherwise known as OSR2)

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Advanced Properties button.

5.	Click the Monitor tab. The Monitor page is displayed.

6.	Make sure that the Monitor page contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

7.	Click the Adapter tab. The Adapter page is displayed.

8.	Click the Change button to change display driver.

9.	Insert the GrafixStar CD into your computer (close the Interactive Installation Guide if it autoruns at this point) and click Have Disk from the Select Device menu. Browse to the folder [disk]:\gs300\dx2\disk1 where [disk] is your CD drive.

10.	Click GrafixStar 300 from the list. Click OK.

11.	When the GrafixStar files have been copied to your PC, the Advanced Display Properties window is displayed again. Click Close.

12.	The Display Properties windows is displayed again. Click Apply.

13.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

3. Note for users with 2MB GrafixStar 300 cards

After you have installed this version of    GrafixStar 300 drivers you will only be able to select 1MB display modes. To select 2MB display mode, restart your PC in MS-DOS ode and add the following line to your AUTOEXEC.BAT:

c:\windows\system\gx30012m.com

This is assuming that your Windows folder is called c:\windows.

Reboot your system.

When you restart Windows 95 you might notice graphics corruption. This will occur when using gx30012m.com and restarting Windows 95 in 1MB display modes. Change to a 2MB display mode to work-around the graphics corruption. Colour switching on the fly to 1MB display modes works with this driver. If you don’t have the 4.00.950b release of Windows 95, you can use VLQWKRES.EXE to switch colour depth on the fly. You can find a copy of VLQWKRES.EXE on the VideoLogic Software Library CD v3 or later under \UTILS\ or download it from our online sites (under GrafixStar 450 - Patch 248).

If you don’t want to use 2MB display modes, you don’t have to install gx30012m.com.

4. Troubleshooting

Introduction

This section describes problems you may encounter when using GrafixStar 300.

Connectivity - Feature connector

Drivers

Vertical bars and square mouse cursor when starting Windows 95

You have a 2MB GrafixStar 300 card and the TSR gx30012m.com is loaded (see chapter 3). When 	starting Windows 95 in a 1MB display mode, you will suffer from the above mentioned corruption. 	Change to a 2MB display mode. You can then colour switch on the fly back to a 1MB display 	mode.

QuickTime for Windows Version 2.X

If you experience problems when viewing QuickTime movies, run the QuickTime Control Panel from the Windows 95 Control Panel, and follow the steps below:

1.	Run the QuickTime Control Panel, click the More button and click the Video tab.

2.	From within the Optimization section, the Draw Method setting will be set to Video Hardware. Change this to Video Driver.

3.	Close the QuickTime Control Panel.

Playing AVI with MS PLUS! pack

If you are using a display mode which is capable of video acceleration and you have the Show Window Contents While Dragging option enabled, you will see the accelerated video displaced from the video window while dragging the video window around the desktop. Disable the Show Window Contents While Dragging on the PLUS page in Display Properties.

5. Field notes

Introduction

This section gives a summary of VideoLogic field notes. Field notes contain frequently asked questions, troubleshooting information, late-breaking information or general comments about GrafixStar 300. Where possible software patches have been provided.

Field notes file format

All of the field notes are in Adobe Acrobat format, and have the extension .PDF. To read these files you must first install the Adobe Acrobat reader which is supplied on the CD. It is located in the directory [drive]:\reader\win95 where [drive] is your CD drive. When you have installed the reader, double-click the .PDF file to read the field note, or open it from within the reader.

All of the patch descriptions are in DOS .TXT format.

Field notes folders

All the notes are located in their own folder within the \UPDATES folder on the CD, so that, for example, field note 192 is in the folder GS300\UPDATES\FN0192. Patches are in their own folders within the field note folders. For example, patch 106 is in the folder GS300\UPDATES\FN0192\PATCH106.

Field Note	Summary	Patch

FN0156 	VLMEMCHK.SYS (Updated to include fix for NEXGEN PCs)