VideoLogic GrafixStar 400 for Windows NT

27th February 1997

Contents

1. Introduction

2. Installing GrafixStar 400

3. Removing GrafixStar 400

4. GrafixStar 400 display modes

5. Troubleshooting

1. Introduction

This file contains installation and removal instructions, late-breaking information, troubleshooting information, and notes on the GrafixStar 400.

2. Installing GrafixStar 400

Introduction

The easiest way to install your GrafixStar 400 is to install the hardware first, referring to the User Guide, and then install the software using the Interactive Installation Guide which runs automatically when you insert the CD into your PC. However, if you don’t have a User Guide, or you want to install the software manually, you should refer to the following instructions.

Handling GrafixStar 400

Follow these guidelines to minimize the risk of electrostatic charges damaging the card:

Keep the card in its protective packaging until you are ready to install it.

Just before handling the card, touch the metal frame of your computer with the back of your hand to discharge any static electricity which may have built up on you or your clothes.

Hold the card by the adapter bracket or by its edges; do not touch the printed circuit board.

Do not place the card on top of your computer or on any other metal surface.

Make sure that the card is not accidentally touched by anyone else.

Handle the card with care to avoid damage to the fragile electronic components mounted on the circuit board.

Installing the hardware

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

1.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

2.	Disconnect your monitor cable from your computer.

3.	Remove the computer’s cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

4.	Remove your existing display card or disable your built-in graphics controller. Refer to the documentation supplied with your computer for instructions on how to disable a built-in graphics controller.

5.	Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

6.	Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

7.	Align GrafixStar 400 with the slot you have chosen. Firmly press the card into the slot. You will need to use some pressure to push the card into position, but do not use excessive force.

8.	Secure GrafixStar 400 using the screw that you removed previously.

9.	Replace the cover of your computer.

10.	Connect the monitor cable to GrafixStar 400. All cables connected to GrafixStar 400 should be properly shielded and comply with radio frequency interference standards.

11.	Switch on your computer and your computer monitor. You are now ready to install the software.

Installing the Windows NT software

To install GrafixStar 400 on your Windows NT system:

1.	When your computer has restarted, the Display Properties page is displayed. Insert the VideoLogic Software Library CD-ROM into your CD-ROM drive.

2.	At the Display Properties page, click Display Type.

3.	At the Display Type dialog box, under Adapter Type, click Change.

4.	At the Change Display dialog box, click Have Disk.

5.	At the Install from Disk dialog box, browse to the folder \gs400\others\NT40\ on your 
CD-ROM drive. After browsing, click OK.

6.	At the Change Display dialog box, select GrafixStar 400 from the list, and click OK.

7.	At the Third Party Driver dialog box click Yes to proceed. The drivers are copied across.

8.	When the drivers have been copied, at the Installing Driver dialog box, click OK.

9.	Close the Display Type dialog box, and close the Display Properties page.

10.	The System Settings Change dialog box is displayed. Click Yes to restart your computer.

When your computer has restarted, GrafixStar 400 is ready to use.

3. Removing GrafixStar 400 from Windows NT

You remove the card from Windows NT in two stages: first the hardware, then the software.

To remove GrafixStar 400 from your Windows NT system:

1.	Switch off your computer, monitor, and any hardware devices such as printers 
connected to your computer. Leave the power cable connected to the power outlet 
so that your computer is grounded.

2.	Disconnect the monitor cable from your computer.

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

3.	Remove the computer’s cover. You may need to refer to the documentation supplied 
with your computer for instructions on how to do this.

4.	Remove the GrafixStar card from its PCI slot.

5.	Replace the cover of your computer.

6.	Switch on your computer and your computer monitor.

7.	When your computer has restarted, open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

8.	Double-click the Devices icon to open the Devices dialog box.

9.	From the device list, click GrafixStar 400.

10.	Click Startup.

11.	Select the Disabled option, and click OK.

12.	Click Close.

13.	Delete the following files from the disk on which you are running Windows NT:

\winnt\system32\vlgs400d.dll

\winnt\system32\drivers\vlgs400d.sys

4. GrafixStar 400 Display Modes

GrafixStar 400 supports the following display modes under Windows NT 4.0:

Display Modes for all cards:

Resolution                   Colours              Refresh rates

640x480	  256	60, 72, 75, 85

800x600	  256	56, 60, 72, 75, 85

1024x768	  256	43i, 60, 70, 75, 85

640x480	65,536	60, 72, 75, 85

800x600	65,536	56, 60, 72, 75, 85

Additional Modes available with 2MB cards:

Resolution	Colours	Refresh rates

1152x864	  256	60

1280x1024	  256	43i, 60, 75

1600x1200	  256	48i

1024x768	65,536	43i, 60, 70, 75, 85

640x480	16.7mil	60, 72, 75, 85

800x600	16.7mil	56, 60, 72, 75, 85

5. Troubleshooting

Introduction

This section describes common problems you may encounter when running GrafixStar 400.

Corrupt or blank display

Windows NT does not restrict the available refresh rates depending on the monitor used.  There is therefore a possibility that you could select a refresh rate that your monitor does not support. This usually results in a corrupted or black display in Windows NT. To rectify this problem, restart Windows NT, and at the boot menu select “Windows Workstation NT v4.00 (VGA)”. Windows NT will boot into 640x480, 60Hz. In the Display Properties you can then select a more suitable refresh rate.