VideoLogic GrafixStar 600 for Windows NT 4.0

8th May 1997

Contents

1. Introduction

2. Installing GrafixStar 600

3. Removing GrafixStar 600

4. Troubleshooting

1. Introduction

This file contains installation and removal instructions, late-breaking information, troubleshooting information, and notes on the GrafixStar 600.

2. Installing GrafixStar 600

Introduction

The easiest way to install your GrafixStar 600 is to install the hardware first, referring to the User Guide, and then install the software referring to the following instructions or the instructions given in the user guide.

Handling GrafixStar 600

Follow these guidelines to minimize the risk of electrostatic charges damaging the card:

Keep the card in its protective packaging until you are ready to install it.

Just before handling the card, touch the metal frame of your computer with the back of your hand to discharge any static electricity which may have built up on you or your clothes.

Hold the card by the adapter bracket or by its edges; do not touch the printed circuit board.

Do not place the card on top of your computer or on any other metal surface.

Make sure that the card is not accidentally touched by anyone else.

Handle the card with care to avoid damage to the fragile electronic components mounted on the circuit board.

Installing the hardware

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

1.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

2.	Disconnect your monitor cable from your computer.

3.	Remove the computer’s cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

4.	Remove your existing display card or disable your built-in graphics controller. Refer to the documentation supplied with your computer for instructions on how to disable a built-in graphics controller.

5.	Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

6.	Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

7.	Align GrafixStar 600 with the slot you have chosen. Firmly press the card into the slot. You will need to use some pressure to push the card into position, but do not use excessive force.

8.	Secure GrafixStar 600 using the screw that you removed previously.

9.	Replace the cover of your computer.

10.	Connect the monitor cable to GrafixStar 600. All cables connected to GrafixStar 600 should be properly shielded and comply with radio frequency interference standards.

11.	Switch on your computer and your computer monitor. You are now ready to install the software.

Installing the Windows NT software

Please note: Only the Administrator can install new graphics card drivers under Windows NT 4.0

To install GrafixStar 600 on your Windows NT system:

1.	When your computer has restarted, the Display Properties page is displayed. Insert the VideoLogic Software Library CD-ROM into your CD-ROM drive.

2.	At the Display Properties page, click Display Type.

3.	At the Display Type dialog box, under Adapter Type, click Change.

4.	At the Change Display dialog box, click Have Disk.

5.	At the Install from Disk dialog box, browse to the folder \gs600\winNT\disk1 on your 
CD-ROM drive. After browsing, click OK.

6.	At the Change Display dialog box, select GrafixStar 600 from the list, and click OK.

7.	At the Third Party Driver dialog box click Yes to proceed. The drivers are copied across.

8.	When the drivers have been copied, at the Installing Driver dialog box, click OK.

9.	Close the Display Type dialog box, and close the Display Properties page.

10.	The System Settings Change dialog box is displayed. Click Yes to restart your computer.

When your computer has restarted, GrafixStar 600 is ready to use.

The Display Properties Advanced page

After installing the Windows NT drivers, an Advanced tab appears on your Display Properties Control Panel dialog box, which enables you to center the screen.

3. Removing GrafixStar 600 from Windows NT

You remove the card from Windows NT in two stages: first software, then the hardware. The GrafixStar 600 NT drivers come with an uninstall program, making software removal very easy.

To remove GrafixStar 600 from your Windows NT system:

1.	In Windows NT 4.0 click the Start button and point to Run. Browse to the file \gs600\winNT\disk1\vluninst.exe on the VideoLogic Software Library CD-ROM and click OK.

2.	A dialog box asks which drivers you want to remove. Click GrafixStar 600, and click OK. When the drivers have been uninstalled a message is displayed confirming this. Shut down Windows NT.

3.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

4.	Disconnect the monitor cable from your computer.

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

5.	Remove the computer’s cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

6.	Remove the GrafixStar card from its PCI slot.

7.	Replace the cover of your computer.

4. Troubleshooting

Introduction

This section describes common problems you may encounter when running GrafixStar 600.

Monitors

Corrupt or blank display

Windows NT does not restrict the display settings to a given set of refresh rates according to the monitor used, so the driver allows any refresh rate to be selected. It is therefore possible that you may select a refresh rate that your monitor does not support. This usually results in a corrupted or black display in Windows NT. To rectify this problem, restart Windows NT, selecting the “Windows NT 4.0 (VGA mode)” option on boot-up. This starts Windows NT in standard VGA mode, allowing you to select a more suitable refresh rate.

Upgrading from a 2MB GrafixStar 600 to a 4MB GrafixStar 600

Intermittent flickering horizontal line

When upgrading from a 2MB GrafixStar 600 to a 4MB GrafixStar 600 without reinstalling the drivers, a a flickering horizontal line can be noticed intermittently at the left side of the desktop. To fix this problem, remove the GrafixStar 600 drivers (as explained in chapter 3 in this readme) and reinstall the GrafixStar 600 NT 4.0 drivers (as explained in chapter 2 in this readme).