*********************************************************

*                                                       *

*                    MIC System I v4.0.5                *

*                                                       *

*********************************************************

*************************************************************************

*****************************  News Flash  ******************************

*************************************************************************

For information on the new features of this release, please refer to the

end of this file.

*************************************************************************

***************************  Hints and Tips  ****************************

*************************************************************************

The following points have been included as an addition to the MIC System I

Reference Manual and the MIC System I User Guide.  Should you have any

problems with your system, please refer to these publications, and especially

to the trouble-shooting appendix in the Reference Manual.

MIC System I v4.0 and Interrupt Vectors

=======================================

MIC uses a resident patch processor for implementing patches.  The resident

patch processor uses a system interrupt vector in order to communicate with

the rest of MIC.  As a default, the system uses interrupt vector 64 (int 64).

It is possible that this may clash with some software packages.

If this is the case, the system interrupt vector used may be changed by

editing the test file M30PATCH.DAT.

S-Video and the NTSC Comb filter

================================

Some DVA-4000 boards have been fitted with a comb filter.  If the comb filter

is present (and the DVA-4000 has been configured for NTSC video input), it

may be switched ON or OFF using DVA-AVS.EXE.  If the comb filter is switched

ON, and the video input is S-VIDEO, all the picture colour information will

be lost, and the displayed video will be in black and white only.

To rectify this situation, turn the comb filter to the OFF position if you

are going to use a NTSC S-VIDEO source.

AMS and MIC Checkout Program

============================

If you are installing this release of MIC System I over a previous release,

you will find two copies of the MIC Checkout program in the AMS menu,

one in the normal list and one in the reserve list.  You should delete the

entry "M.I.C Checkout Program" by using the "Remove An Application" option

from the AMS main menu.

Using Super VGAs

================

If you have a MIC supported Super VGA card installed in your system, then it

is essential that you have correctly configured MIC to use this card.  You

can configure MIC by using the MICSETUP.EXE program.

If you select the wrong Super VGA, the horizontal and vertical registration

may be affected leaving a "gap" between the edge of the graphics and the edge

of the video, when running in the Super VGA supported by MIC

*************************************************************************

********************** Developer Interface Files ************************

*************************************************************************

When MIC has been installed, the following undocumented files are placed

in the MICFILES directory.

TEN-MIC.EXE  - Allows you to use MIC commands from the

TenCORE authoring system (versions 2.0-2.03).

TEN-MIC is used in conjunction with the TenCORE

source unit MICD.SRC.  This routine executes

an MIC command given within a TenCORE 'PACK'

command.

ex:  PACK        MICCON;MICLEN;STILL 1025

DO MICD

PACK        MICCON;MICLEN;AUDIO ON/A1

DO MICD

PACK        MICCOM;MICLEN;TRANSC ALL

DO MICD

etc.

Variables within the TenCORE Defines Unit

must contain the following:

MICCOM,80

MICRET,80

.    keytype,5

.    keycode,5

.    filler,70

MICLEN,2

Include TEN-MIC within your AUTOEXEC.BAT and

reset the TenCORE default of 240 lines to 200

lines.

eg: AUTHOR/D=EGA 200

Please Note:  These files have been made redundant

by subsequent releases of TenCORE, and have been

provided to maintain compatibility with past

applications.

MICD.SRC     - A TenCORE source module providing the

functionality embodied within the "DO MICD"

statements in the TenCORE/MIC interface example

described in the section above (on TEN-MIC.EXE).

PCSB-MIC.EXE - Allows you to use MIC from IBM's PC Storyboard.

PCSB-MIC is a memory resident program. It must

be loaded before you enter Storyboard.

The program works by copying a file (MIC-SB.nnn)

containing MIC commands to the MIC Device Driver.

You edit sequences of MIC commands into

individual files named MIC-SB.nnn where the

extension (ie, the nnn part) is a three digit

number between 001 and 999. These sequences are

then incorporated into your storyboard program

and are invoked by using the /INT 66h command.

Example:

To issue the commands held in MIC-SB.014

insert /INT 66h,14 into your story. The

"14" indicates file extension .014.

Although the interface is essentially output only

(it is not possible to get response back from the

Virtual Device Interface), you can still use READKEY

to wait for keyboard input or return at the end of

the current video segment.

*************************************************************************

*************************** MICFILES Contents ****************************

*************************************************************************

The following is a list of files you should expect to see in the /MICFILES

directory after installing MIC System I v4.0.4.  A brief explanation of the

purpose of each is also given.  It is very strongly recommended that you do

not amend the contents of any of the files except as instructed in the User

Guide and Reference Manual, as this can lead to unpredictable results.

AUDENAB.EXE     MediaSpace audio passthrough enable

AUTOEXEC.SAV    Copy of old AUTOEXEC.BAT before changes made due to MIC

installation.

CHK3C3.COM      Test program to be used as directed by VideoLogic

technical support if required

meanies" problem is likely to occur with early DVA-4000's.

COMPORT.HLP     Helpfile for MICSETUP.EXE

CONFIG.HLP      Helpfile for MICSETUP.EXE

CONFLICT.HLP    Helpfile for MICSETUP.EXE

CONFIG.SAV      Copy of old CONFIG.SYS before changes made due to MIC

installation.

DRIVER.VLV      DVA driver version number.

DVA-4000.SYS    Main MIC command kernel

DVA-AVS.DAT     Configuration file created by DVA-AVS

DVA-AVS.EXE     Audio-visual setup program, enabling brightness, contrast,

volume etc. to be set by the user.

DVA-AVS.HLP     Help file for DVA-AVS.

DVA-HPOS.DVA    Alignment bar used by DVA-HPOS

DVA-HPOS.DAT    Data file created by DVA-HPOS

DVA-HPOS.EXE    Horizontal registration program

DVA-HPOS.HPG    Horizontal and Vertical registration figures

DVA-MAIN.CLP    On-board code for the DVA-4000 local processor

DVA-MON.DAT     Data tables for VGA modes

DVA-SET1.EXE    Program to load tables and upload the local processor

DVA-SET2.EXE    Program to load and run the DVA-AVS and DVA-HPOS programs

when required.

DVA-SHAR.DAT    Sharpness tables for DVA-4000 video filters

DVA-SMOD.EXE    Program providing support for Super VGA modes

DVA-SRV.EXE     Program to enable the DVA-4000 to be set up.  Invoked by

DVA-SET1

DVATOTGA.EXE    Program to convert .IM4 image files to .TGA (TAGA) format files

DVA-UPLD.EXE    Program to upload the DVA-4000 local processor with its

executable code

DVA-VGA.HLP     Help file for MICSETUP

DVAVALID.DAT    Video input data for the DVA-4000

DVA-VIP1.DAT    File containing video input 1 data

DVA-VIP2.DAT    File containing video input 2 (auxiliary) data

EMSUSED.EXE     Program giving the amount of EMS memory used by MIC

ETSS-SUP.EXE    MIC's expanded memory supervisor.  resides in low memory

accessing modules in expanded memory.

GRAFADAP.HLP    A MICSETUP help file

IWSTCK10.EXE    Fix for MIC InfoWindow stack overflow bug

LOAD.DCF        Data file created by MIC-SSL

LOAD.LOG        Text file created when MIC is loaded, detailing all the

modules loaded

LOAD.PAF        Data file created by MIC-SSL

LOAD.SCF        Data file created by MIC-SSL

M30DYN.CNF      Dynamic configuration file for IVA-3000 compatibility mode.

This determines the modules loaded and commands available

when "MIC-LOAD" or "MIC-LOAD M30DYN.CNF" is issued.

M40DYN.CNF      Configuration file for full DVA functionality mode.  This

determines the modules loaded and commands available when

"MIC-LOAD M40DYN.CNF" is issued.

M40DYN.EMS      Configuration file for expanded memory support, detailing

which modules should be loaded into expanded memory rather

than system memory (if possible).

MIC-2000.CNF    Data file relating to MIC-2000.EXE

MIC-2000.EXE    The MIC system kernel providing IVA functionality.

MIC-232.SYS     The RS-232 comms driver.

MIC-EMS.EXE     Program providing support for using MIC in expanded memory.

MIC-HPOS.EXE    Provides horizontal registration compatibility for IVA-3000

and IVA-3000 based programs

MIC-LOAD.EXE    MIC first stage loader.

MIC-MARK.EXE    MIC memory marker program

MIC-REL.EXE     MIC memory release program

MIC-SERV.EXE    The FSTORE command handler

MIC-SSL.CNF     MIC loading indicator graphic

MIC-SSL.EXE     MIC second stage loader

MIC-UNL.EXE     MIC unloader program

MICCMD.EXE      MIC command interface program.

MICD.SRC        A TenCORE source module for use when interfacing between

TenCORE and MIC.  Used in conjunction with TEN-MIC.EXE

MICWIN.COM      A Microsoft Windows 3 launch program to allow MIC System I

to work correctly with MicroSoft Windows 3.x

M30BBASE.DAT    An MICSETUP help file

M30CMMND.DAT    An MICSETUP help file

M30SETUP.CNF    Configuration file created by MICSETUP

M30IVECT.DAT    Interrupt vector used in MIC

M30PATCH.DAT    File containing patch information

M30STAT.CNF     Static configuration file for MIC (functionality always

loaded)

M30VALID.DAT    Configuration data file containing information on touch

screens, players and monitors available under MICSETUP

M40IVECT.DAT    A data file for MICSETUP

MIC-4000.EXE    The MIC system program providing DVA functionality support

MIC-BM.EXE      MIC buffer manager used to manage buffers for uploading /

downloading still images to / from disk

MIC-CP.SYS      The colour palette manager

MIC-CSM.EXE     The context switching manager for Comsell courses (see

InfoWindow Emulator User Guide for more information)

MIC-DD.SYS      The MIC device driver

MIC-DD1.SYS     The transient part of the MIC device driver

MIC-DIO.SYS     Direct I/O support to players

MIC-EMT.EXE     Error message translation module

MIC-ERR.SYS     Error message driver

MIC-FB.SYS      Frame base module used to support frame numbers > 65,000

MIC-GRAF.EXE    IVA functionality covering the SETDISPLAY command

MIC-HI95.SYS    Hitachi 9500 player driver

MIC-HSET.DAT    Data file for MIC-HSET.EXE

MIC-HSET.EXE    Application specific horizontal setup program

MIC-HSET.PCX    Alignment image for MIC-HSET.EXE

MIC-LM.EXE      The MIC library manager

MIC-MAC.SYS     The MIC macro manager

MIC-MSG.SYS     The MIC error message text module

MIC-NEC.SYS     The NEC players driver

MIC-PANA.SYS    The Panasonic 2023 and 2027 players driver

MIC-PAT.EXE     The MIC patch applicator

MIC-PHIL.SYS    The Philips player driver

MIC-PI42.SYS    The Pioneer 2100, 2200,4100, 4200, 4300, 4400, and 8000

players driver.  Newer Pioneer players may well be supported

by this driver

MIC-PI60.SYS    The Pioneer 6000 player driver

MIC-PI61.SYS    The Pioneer 6100 player driver

MIC-RPP.EXE     The MIC resident patch processor

MIC-SO11.SYS    The Sony 9011 player driver

MIC-SO90.SYS    The Sony 9000 player driver

MIC-SONY.SYS    The Sony videodisc player driver (all Sony players)

MIC-THOR.SYS    The Thorn-EMI player driver

MIC-TTX.EXE     MIC Teletext extension module

MICDLL.DLL      Windows 3 DLL allowing MIC command string to be written and

responses read by Windows applications to MIC I

MICDLL.H        Header file for use by programmers using MICDLL.DLL

MICDLL.HLP      Help file for MICDLL.DLL

MICDLL.LIB      The Library for programmers wishing to use MICDLL.DLL

MICDLL.UCD      Windows DLL allowing Authorware Professional for Windows to

interface with MIC I

MICSETUP.EXE    The MIC setup program

MICSETUP.HLP    Help file for MICSETUP.EXE

MICVER.VLV      Version information for MIC modules

MICWINGO.EXE    Used by MICWIN.EXE to launch Microsoft Windows 3.x

MON3000.HLP     A MICSETUP help file

MONITOR.HLP     A MICSETUP help file

PATCH.VLV       Version no. of MIC-RPP

PCSB-MIC.EXE    Interface for IBM Storyboard

PLAYER.HLP      A MICSETUP help file

POINTDEV.HLP    A MICSETUP help file

PRODDATE.DAT    Date of production of this software

ROMDATE.COM     Prints date of VGA ROM BIOS

SCREEN.SYS      The XY device interface module

SCRN-CAL.EXE    Screen calibration program for touch screen support.

SCRN-CMM.SYS    The Cameron Interact M touch screen driver

SCRN-CRL.SYS    The Carroll 1200 touch screen driver

SCRN-ELG.SYS    The Elographics touch screen driver

SCRN-ELL.SYS    The Ellinor touch screen driver

SCRN-MCT.SYS    The Cameron Microtouch touch screen driver

SCRN-MVT.SYS    The Microvitek touch screen driver

SCRN-NUL.SYS    The null touch screen driver

SCRN-TKT.SYS    The Tecknotronics touch screen driver

SYSTEM.HLP      A MICSETUP help file

SYSTEM.VLV      System version no.

TEN-MIC.EXE     The TenCORE - MIC interface program

TXTFNT.C08      Teletext font file - NTSC sized

TXTFNT.C10      Teletext font file - PAL English

TXTFNTG.C10     Teletext font file - PAL German

TXTFNTI.C10     Teletext font file - PAL Italian

TXTFNTS.C10     Teletext font file - PAL Spanish

VIDINP.HLP      A MICSETUP help file

*************************************************************************

***************************  Known Problems  ****************************

*************************************************************************

1) Initial fade after INIT

The first FADE for the audio channel(s) after an INIT command will be

carried out instantaneously, rather than over any period specified.

Subsequent FADEs will be carried out over the period specified (if any)

in the command.  The same is true for fading graphics when in a 256

colour mode (modes 13h, s1 and s4).  Once again the initial fade is

carried out instantaneously with subsequent graphics fades working as

expected.

*************************************************************************

*************************** Revision history ****************************

*************************************************************************

Version 4.0.1:

1) A problem existed with MIC System I v4.0 when using an NTSC video

source.  If the DVA-4000 was configured for NTSC video and a

"FST INPUT 0 n ENABLE" command was issued, the video picture height

would be reduced.  MIC System I v4.0.1 solves this problem.

Version 4.0.2:

1) If an "INIT" command was followed a "GENLOCK OFF" command in Compatibility

mode, the graphics plane would roll vertically while the video plane would

remain steady.  Some IIS courses exhibit this problem.  This has been

fixed.

2) MICCMD when invoked, would set the MIC response terminator to a

carriage return ($0D).  Previous versions did not.  MICCMD now no longer

sets the MIC response terminator.

3) When using the Philips player driver, commands such as fast-forwarding

to a frame number less than 50 frames from the current frame, could result

in the system "hanging" until it eventually timed out.  The Philips player

driver has been modified to correctly handle such short events.

Version 4.0.3:

1) Inclusion of support for ICL Super VGA card

Version 4.0.4:

1) Compatibility mode now supports the ALTERNATE display mode.  This was

unavailable on previous versions, Thus courses written for the IVA range

now have access to even higher quality still images from videodisc.

2) Fading of all 256 colours is now supported.

3) Developing TenCORE applications with MIC in Super VGA modes will sometimes

cause some register contents to be changed and the development

environment to fail.  This has been rectified in this release

4) Support for the Pro Designer II Super VGA card has been included in this

release.  Please note that you will require a VGI adapter for this Super

VGA to work correctly with DVA-4000.  This is available from your

VideoLogic stockist.

5) Support for the Diamond Speedstar Super VGA card has been included.

6) Pioneer player driver (MIC-PI42.SYS) has been improved.  It has been made

more generic so that players released from now onwards may well run without

the need for customisation of this driver.

Version 4.0.5:

1) Support for the IBM 8516 touchscreen.

This driver cannot as yet be selected in the usual way from the MICSETUP

application. Instead use MICSETUP and choose a touch screen (say Ellinor).

Edit \MICFILES\M30STAT.CNF and replace the SCRN-ELL.SYS line with

SCRN-IBM.SYS

The 8516 touchscreen has its own IBM supplied DOS device driver. To work

with MIC, the IBM 8516 driver must be configured. The IBM install program

asks you to choose between mouse emulation and no mouse emulation. the MIC

8516 touchscreen drivers turns mouse emulation off regardless of your

choice here.

There is no need to run the IBM calibration program when using with MIC,

although MIC screen calibration (SCRN-CAL) must be run.

The IBM  installation program also inserts two lines in the AUTOEXEC.BAT...

C:\TOUCH\MOUSE

C:\TOUCH\CALIBRAT -C C:\TOUCH\CALIBRAT.

These are required, so do not remove them.

2) Fading of all 256 colours is now supported correctly.

To fade all 256 colours, you must group them together using the

GRA G1=ALL

command, and make all colours solid using

GRA SOL=ALL

then fade the G1 group using

FADE <level> <time> /G1

3) Support for the ProDesigner IIs Super VGA in selected super VGA modes.

4) Usercode support for relevant Pioneer videodisc players.

5) Fix for MIC-LOAD on PCs with a fast (50 or 66MHz processor) machines.

6) Audio pass-through for MediaSpace users.

If you run MIC System I, but have a MediaSpace installed and route your

audio through the MediaSpace in the recommended (versatile) configuration,

then you will not hear any audio from the DVA-4000 unless MIC System II is

initialised. This can be fixed by running the application

AUDENAB.EXE

which is supplied in your MICFILES directory. This simply forces the

MediaSpace to pass all audio through by default. It may be easiest to run

the program from your AUTOEXEC.BAT file.

7) The TenCORE fix introduced in v4.0.4 solved most problems, but was

incomplete. This release completes that fix.

Version 4.0.6:

1) This enhancement is supplied to allow the DVA-4000 to operate in

supported SuperVGA modes with selected ATI graphics cards, the DELL

466ME on-board S3-based VGA and the Diamond SpeedStarPro.

2) The enhancement has been tested on an ATI Integra with 512Kb RAM

(supporting 640 x 480 256 colours and 1024 x 768 16 colours), and

with an ATI VGAWONDER XL24 with 1Mb RAM (supporting 640 x 480 256

colours, 1024 x 768 16 colours and 1024 x 768 256 colours).

- For the ATI graphics cards to correctly sense that the monitor

connected is capable of 1024 x 768 interlaced modes, you must

have a VGA terminator attached to the ATI graphics output connector.

If the terminator is not present, the boards assume that only

640 x 480 modes are supported, and this will cause confusion if

any attempt is made to enter modes S3 or S4 (1024 x 768).

- Certain ATI graphics cards are not supported by MIC. The ATI

Graphics Ultra has no feature connector, and the ATI Graphics

Ultra+ displays palette corruption.

3) The S3 enhancement is listed as VESA compatible VGAs within

MICSETUP.EXE. However there is no guarantee that any particular

VESA compatible VGA will work as the standard does not appear to be

strictly adhered to.

4) The Diamond SpeedStar Pro enhancement supports all VGA modes and the following

SVGA modes:

RESOLUTION   COLOURS

640 x 480     256

1024 x 768    16

1024 x 768    256

5) To ensure compatiblity with DVA-4000 the SVGA card's Monitor sync

rates must be set as follows:

RESOLUTION  COLOURS    HSYNC   VSYNC   INTERLACED

640x480      16      31.5KHz   60Hz      No

640x480      256     31.5KHz   60Hz      No

1024x768     16      35.4KHz   43Hz      Yes

1024x768     256     35.4KHz   43Hz      Yes