Advanced Logic Research, Inc.

ALR OPTIMA DT

Jerry E. Uy

Version 1.0 (June 1995)

60550050

© 1995 Advanced Logic Research, Inc.  All rights reserved.  No part of this publication may be reproduced or distributed in any form or by any means without prior permission in writing from ALR.

ALR is a registered trademark of Advanced Logic Research, Inc. ALR OPTIMA DT is a trademark of Advanced Logic Research, Inc.  Other brand and product names mentioned herein are trademarks and/or registered trademarks of their respective owners.

This publication could include technical inaccuracies or typographical errors.  Changes are periodically made to the information herein; these changes will be incorporated in new editions of the publication.  Also, ALR may make improvements and/or changes in the product(s) described in this publication at any time.

Advanced Logic Research, Inc.

9401 Jeronimo Road

Irvine, CA 92718

Version 1.0

Contents

ALR OnLine Help

Why Have OnLine Help?

How to Use OnLine Help

Printing OnLine Help

Changing the Hypertext Color

Getting Started

Important Safety Instructions

Unpacking

Inspecting the Contents

Connecting the Keyboard, Monitor and I/O Ports

Video Port

Serial Port 1

Serial Port 2

Parallel Port

Keyboard Port

Mouse Port

AC In

AC Out

Powering Up the System

On/Off Switch

Reset Switch

Floppy Disk Drive Slot

LED Indicators

Drive Bay

Setting Up the System Via the BIOS

Preparing the Hard Disk for DOS

Partitioning the Hard Drive

Standard System Specifications

ALR OPTIMA DT Models

ALR OPTIMA DT

Components and Performance Features

ALR OPTIMA DT System Board

CPU

Memory

Floppy Drive and Controller

Hard Drive and Controller

Expansion Slots

Storage Bays

BIOS

System Board Jumpers

System Video Unit

Keyboard

Standard Software

Options

Available Options

Installing Optional Equipment

ESD Precautions

Opening the System Unit

Closing the System

Additional RAM

Installing a 5.25-inch Device

Installing a Hard Drive

Installing Adapter Cards

Upgrading the CPU

BIOS Setup

The BIOS Setup Program

Preparing to Run Setup

Setup Entering BIOS

Basic System Configuration

Date and Time

Floppy Disk Drives

Hard Disk Drives

Communication Parameter Settings

Enhanced IDE Features

Large Memory Support Mode

Num Lock After Boot

Memory Test

Auto Configuration Mode

Fast Boot Mode

Advanced System Configuration

Shadow RAM

Internal Cache

System Memory Parity

Memory at 15MB - 16MB Reserved

Power Saving Configuration

Power Management Mode

System Security Setup

Disk Drive Control

Onboard Communication Ports

Setup Password

Power On Password

PCI System Configuration

Load Setup Default Settings

Leaving BIOS Setup

Flash BIOS EPROM

Updating the Flash BIOS

Troubleshooting

Looking Things Over

Monitor Problems

Verifying Your Configuration

Authorized ALR Reseller

System Error Messages

Hard Drive Characteristics

ALR Technical Support

Glossary of Terms

Index

# ALR OnLine Help

## Why Have OnLine Help?

Why has ALR chosen to put the bulk of its reference information online?  The advantages for doing so far outweigh the disadvantages.  In short, the primary reasons are as follows:

·	Information you need the most may be accessed more readily.

·	Trees do not have to be cut down, and old disks can be reused.

·	Lower printing costs result in lower prices for consumers.

Simply stated, electronic distribution is the wave of the future.  There may come a time when you receive a complete telephone directory for the entire country on a single CD-ROM or interactive multimedia magazines in place of your usual monthly periodicals.

## How to Use OnLine Help

ALR's OnLine Help system contains all the information included with your User's Guide---and a whole lot more!  By putting this guide online, we are able to include information that could not be provided in the past.

The OnLine Help is especially organized to appeal to both novices and experts.  Basic information is provided up front, and more in-depth information and configuration instructions can be accessed through the subsequent levels.

ALR's OnLine Help uses the same conventions as the help systems of other popular Windows programs.  If you have come to rely on the online help in such programs as Word for Windows and Excel, then you will feel right at home with ALR's OnLine Help system.

### Printing OnLine Help

If you are in the midst of performing an upgrade on your system, you will not be able to refer to the ALR OPTIMA DT OnLine Help.  Therefore, we recommend you review a procedure before performing it.  You can print any part of this guide on any printer supporting Windows.  The print command can be accessed through the File menu.

### Changing the Hypertext Color

It is possible that the green hypertext words in the OnLine guide may clash with the colors you have selected for your Windows and make them difficult to read.  If this is the case, you can change any color by altering your WIN.INI file which can be found in the C:\WINDOWS directory and edited with the Notepad utility.

WARNING!  Incorrectly altering your WIN.INI file can prevent your Windows from operating.  It is a good idea to back up configuration files before editing them, so that they can be easily restored in the event of an error.

Once your WIN.INI file has been loaded into a text editor, search for the section listed below:

[Windows Help]

M_WindowPosition=[-4,-4,808,608,0]

H_WindowPosition=[266,200,266,200,0]

C_WindowPosition=[14,61,470,225,0]

A_WindowPosition=[43,51,307,237,0]

jumpColor=0 255 0

Popupcolor=0 255 0

·	JumpColor refers to the color used for cross-references or text that lets you “jump” to other parts of this guide when you click on them.

·	Popupcolor is for the words that display definitions when you click on them.

The numbers represent red, green, and blue, respectively.  0 is for no color, and 255 is for maximum color.  You can create custom colors by mixing different amounts of red, green, and blue.  You can even have different colors to differentiate between cross-references and definitions.

The table below lists some of the possible colors:

| SETTING 1 | SETTING 2 | SETTING 3 | COLOR |
| --- | --- | --- | --- |
| 0 | 255 | 0 | Green (default) |
| 255 | 0 | 0 | Red |
| 0 | 0 | 255 | Blue |
| 255 | 0 | 255 | Purple |
| 0 | 255 | 255 | Cyan |
| 255 | 127 | 0 | Orange |
| 255 | 127 | 127 | Pink |

These are but a few of many possible colors available to you.  You might wish to first experiment with different colors in the Windows Paint program to eliminate some of the guesswork.

Once you have made the changes, you must save the WIN.INI file and restart Windows for the new settings to take effect.

NOTE:  Making the changes described above will affect all your other Windows applications as well.

# Getting Started

## Important Safety Instructions

Observe the following guidelines when performing any work on your system:

1.	Follow all instructions marked on the product and in the documentation.

2.	Unplug this product from the wall outlet before cleaning.  Do not use liquid cleaners or aerosol cleaners.  Use a damp cloth for cleaning.

3.	Do not use this product near water.  Never spill liquid on the product.

4.	Do not place this product on an unstable surface.

5.	Openings in the cabinet are provided for ventilation.  Do not block or cover  these openings.  Never place this product near or over a radiator or heat register.

6.	Use only the type of power source indicated on the marking label.  If you are not sure of the type of power available, consult your reseller or local power company.

7.	This product is equipped with a 3-wire grounding type plug (a plug having a third/grounding pin).  This plug will only fit into a grounding-type power outlet.  This is a safety feature.  If your outlet does not accommodate a 3-wire plug, contact your electrician to replace your outlet.

8.	Do not walk on the power cord or allow anything to rest on it.

9.	If you use an extension cord with this product, make sure that the total ampere ratings on the products plugged into the extension cord do not exceed the extension cord ampere rating.  Also, the total of all products plugged into the wall outlet must not exceed 15 amperes.

10.	Never push objects of any kind into this product through cabinet slots.

11.	Do not attempt to service this product yourself, except as explained elsewhere in the OnLine Help.  Adjust only those controls covered in the instructions.  Opening or removing covers that are marked “Do Not Remove” may expose you to dangerous voltage points or other risks. Refer all servicing in those compartments to qualified service personnel.

12.	Unplug this product from the wall outlet and refer servicing to qualified service personnel under the following conditions:

a.	The power cord or plug is damaged.

b.	Liquid has been spilled into the product.

c.	The product does not operate normally when the operating instructions are followed.

d.	The product has been dropped or the cabinet has been damaged.

e.	The product exhibits a distinct change in performance.

## Unpacking

Follow the guidelines listed below when unpacking your system:

·	Prepare a clean, flat, hard surface for your computer.  Allow at least three inches at the rear of the chassis for cabling and free air circulation.

·	Keep the computer shielded from extremes in temperature and humidity.  Avoid direct sunlight, heater ducts, and other hot areas.

·	Invest in a 3-prong, 115-volt or 230-volt (depending on the voltage supplied in your locality) AC surge control outlet station.  The system requires only one outlet; however, future expansion will be easier if other outlets are readily available.  A surge control station is recommended for protection against AC line spikes.

·	Keep your system away from equipment that generates magnetic fields.  Even a telephone placed too close to the computer can cause interference.

·	Keep the carton and foam packing (used to protect your computer during shipment) for future transportation purposes.

## Inspecting the Contents

Unpack the carton and inspect the contents.  The standard ALR OPTIMA DT (Slimline chassis) system includes the following:

·	packing list

·	computer chassis

·	101-key enhanced keyboard

·	AC power cord

·	ALR OPTIMA DT User's Guide

·	ALR OPTIMA DT OnLine Help (3.5-inch diskette)

·	manuals for optional equipment and software

Check the packing list to verify that all equipment and associated manuals are included in your shipment.

Inspect all equipment for any damage that may have occurred during shipment.  If any damage occurred, contact your authorized ALR reseller.

You must obtain a Return Material Authorization (RMA) number from the reseller or ALR Technical Support before returning the system to the factory.

## Connecting the Keyboard, Monitor and I/O Ports

The figure below illustrates the rear of the ALR OPTIMA DT.  Refer to it as you connect your equipment:

The keyboard and optional monitor must be connected to their respective I/O ports with properly shielded cables.  (Shielded cables are required by the FCC.)  The above illustration identifies the keyboard and video port locations:

1.	Plug the keyboard into the keyboard port.  The connector is specially keyed to prevent improper connection.

2.	Connect the monitor signal cable to the VGA PORT.  The connector is keyed to prevent improper connection.

NOTE: If you do not have to use the system’s AC POWER OUT source.  If you do, your monitor will be powered ON and OFF with the system’s power.

3.	Connect the monitor power cable to the AC POWER OUT socket or to an external AC power receptacle.  Refer to the figure below for details.

4.	Connect the system power cable to the AC POWER IN socket.  Make certain that the AC Voltage Switch (located in the rear of the chassis) is set for the AC line voltage used in your locality (115VAC or 230VAC).

5.	Connect the other end of the system AC power cable to an AC power receptacle that is within an easy reach of the system operator.  The system AC power cable acts as the primary disconnect device.

### Video Port

This 15-pin VGA out connector is where you plug your monitor.

### Serial Port 1

This port is also known as COM 1.  If you have a serial mouse, connect it into this port.  The figure below illustrates the pinouts of serial port 1.

### Serial Port 2

This port is also known as COM 2.  The figure below illustrates the pinouts of serial port 2.

### Parallel Port

This port is addressed by your computer as LPT1.  Connect your printer into this port.  The figure below illustrates the pinouts of the parallel port.

### Keyboard Port

Connect the keyboard into this socket.  The figure below illustrates the pinouts of the keyboard port.

### Mouse Port

This port supports a PS/2 style mouse.  The figure below illustrates the pinouts for the mouse port.

### AC In

Connect the computer power cable between this socket and the AC surge control station.

### AC Out

This is an AC socket for optional equipment.  Generally, a monitor is plugged into this socket.  Any piece of equipment plugged into this socket is automatically turned off when the system is switched off.

## Powering Up the System

The power to the system can be turned on by pressing the power on/off switch, as shown in the figure below:

NOTE:  In Some ALR OPTIMA DT systems, the 5.25-inch drive bay may be located below the 3.5-inch disk drive.

Once power is applied, the ALR OPTIMA DT checks its system memory, even if the monitor is not connected or turned on.

If your system fails its memory test, the monitor displays an error message preceded by two beeps.

If your video adapter fails to initialize and your system is unable to display an error message, a beep code signals the error message.  For a listing of the error messages and their meanings see "System Error Messages."

If you encounter an error, it will most likely be a nonfatal one.  However, in the event of a fatal error, contact your ALR reseller for field service support.  Do not return any equipment to the factory without first obtaining a Return Material Authorization (RMA) number from your ALR reseller or ALR Technical Support.

For your system to boot at power up, you must have either a formatted hard drive installed or a DOS disk in drive A:

Once your system boots, you will receive the following message:

If the time or date are incorrect, or if the system gives the message INVALID CONFIGURATION INFORMATION, you can run the BIOS SETUP program to make corrections.  Refer to "BIOS Setup" for a complete description of how to use the BIOS Setup to configure your system and enable password support.

### On/Off Switch

Power to the system is applied by pressing the ON/OFF switch.  If the switch is pressed again, the system will power OFF.  If you turn off your system, you must wait at least ten seconds before you turn it back on.  Powering your system off and then on again will generate a reset.

### Reset Switch

The Reset switch allows you to reset your system without having to power it off and then on again.

### Floppy Disk Drive Slot

This is where you insert a floppy disk into the 3.5-inch floppy disk drive included with your system.  The floppy disk drive has an eject button and a Floppy Disk Activity LED indicating when the drive is in use.   NEVER eject a floppy disk when this light is ON.

### LED Indicators

The green Power LED lights up when power is applied to the system.  This light should not be flickering while the system is running.

The amber Hard Disk Activity LED lights up only when the hard disk is in use by your system.  NEVER reset or power down your system while this light is on (unless your hard drive had been improperly configured).

### Drive Bay

The rectangular panel on the right side of  the 3.5-inch floppy disk drive is a 5.25-inch drive bay which can be used for other optional devices such as hard disk drive, floppy disk drive, or magnetic tape device.  You can even convert your system into a multimedia system by installing a CD-ROM drive into this bay.

NOTE:  In Some ALR OPTIMA DT systems, the 5.25-inch drive bay may be located below the 3.5-inch disk drive.

## Setting Up the System Via the BIOS

You can invoke the BIOS SETUP program by simultaneously pressing <Ctrl>-<Alt>-<Esc>.  The following menu screen will appear:

NOTE:   Close all open files and leave your application before invoking the BIOS Setup.  You cannot exit back into an application.  The system will automatically reboot when you leave Setup.

Verify that the configuration information displayed on the Basic System Configuration screen reflects your current configuration.  Press the up or down arrow key to highlight any incorrect information.  Press the left or right arrow key to change the highlighted parameters.  Pressing the <PgUp> and <PgDn> keys will take you to other setup screens.

Once you have made the appropriate changes, press <Esc> to leave the system configuration setup.  If you have made any changes in the configuration values and security setup, the following screen will appear:

Press the left or right arrow key to select your response.  Select [Yes] to store the new data in the CMOS RAM.  Selecting [No] will retain the old configuration values and discard any changes that you may have made.  Press <Enter>.  Your system will automatically reboot and reconfigure according to the changes that you have made and that have been stored in the CMOS RAM.

For more comprehensive information on the BIOS Setup, refer to "BIOS Setup."

## Preparing the Hard Disk for DOS

If a hard drive came with your system, it should have been formatted at the factory.  If this is not the case, or if your system does not boot from the hard disk when you apply power, follow the procedures outlined below.

IMPORTANT!  These procedures should only be performed if your system does not boot from the hard drive.  Partitioning and formatting the hard drive will erase any data and programs stored on it!

To initialize the hard disk, use the DOS commands FDISK and FORMAT. The following procedures should only be performed the first time you use your system or after a low-level format.

### Partitioning the Hard Drive

If you have not already done so, insert your DOS diskette in Drive A: and either turn on your system or boot it by pressing <F1>.

To partition the drive, enter the following command at the A: prompt:

FDISK <Enter>

The FDISK program will display a series of menus to help you partition the hard disk for MS-DOS.  For a more comprehensive explanation of FDISK, refer to your DOS manual.

After performing the above procedure, you can format your hard drive with the following command:

FORMAT C:/S <Enter>

This will format your drive as well as transfer the DOS system information.  After the format is complete, your system should boot from the hard drive during power up.

# Standard System Specifications

## ALR OPTIMA DT Models

### ALR OPTIMA DT

The ALR OPTIMA DT comes with the following standard features:

| STANDARD FEATURES | DT 5100 Model 1 | DT5133 Model 1 | DT 5150 Model 1 |
| --- | --- | --- | --- |
| Intel Pentium® CPU | 100 MHz | 133 MHz | 150 MHz |
| System RAM | 8-MB | 8-MB | 8-MB |
| 1-MB integrated video DRAM | Yes | Yes | Yes |
| 1.44-MB 3.5" floppy drive | Yes | Yes | Yes |
| Storage bays: external half-height 5.25-inch external 3.5-inch internal 3.5-inch | 7 3 1 3 | 7 3 1 3 | 7 3 1 3 |
| Embedded floppy drive controller with support for up to two floppy disk drives | Yes | Yes | Yes |
| Upgradeable "Flash" EPROM BIOS | Yes | Yes | Yes |
| Total expansion slots: PCI ISA PCI/ISA | 4 2 1 1 | 4 2 1 1 | 4 2 1 1 |
| I/O ports: serial port parallel port mouse port keyboard port VGA port | 2 1 1 1 1 | 2 1 1 1 1 | 2 1 1 1 1 |
| Two-button mouse | -- | -- | -- |
| MS-DOS | -- | Optional | -- |
| Microsoft Windows for Workgroups 3.11 | -- | Optional | -- |
| Windows®95 | -- | Optional | -- |
| 200-Watt power supply | Yes | Yes | Yes |
| Limited 5/36 factory warranty | Yes | Yes | Yes |

## Components and Performance Features

The ALR OPTIMA DT comes with a wide range of standard features as illustrated in the figure below:

Refer to the following related topics for information on the above features.

### ALR OPTIMA DT System Board

The ALR OPTIMA DT system board is the heart of your computer.  Many of the major components in your system are located on this board. The ALR OPTIMA DT system.

#### More about the System Board

Located on the system board are the CPU, system RAM, I/O ports, expansion slots, hard and floppy drive controller circuits, CMOS, and battery.

#### Configuring the System Board

If you wish to alter the configuration of the embedded components on the system board, you must do so through the BIOS Setup as described in "BIOS Setup."

### CPU

The CPU in your system performs all calculations required to operate your software applications.  All models of the ALR OPTIMA DT are equipped with a Pentium processor.

#### More about the CPU

The CPU in your system operates at either 100-MHz or 133-MHz, depending on which model you have purchased.  Refer to "Standard System Specifications" if you need to know your processor speed.

Intel's Pentium processor is the most powerful CPU available for the PC.  It contains a 64-bit data path and has internal cache and an integrated math coprocessor.

#### Configuring the CPU

The CPU type and speed are automatically detected by the system.

### Memory

The ALR OPTIMA DT comes standard with 8-MB or 16-MB of RAM depending on the system you purchased.  Refer to “Standard System Specifications” to find out how much memory your system came with.  Memory can be expanded to a maximum of 128-MB on the system board.  For an illustration of the system board and the location of the RAM sockets, refer to "ALR OPTIMA DT System Board."

#### Configuring RAM

The ALR OPTIMA DT automatically configures the new memory, provided the SIMM units were installed properly.  You may enter the BIOS Setup if you wish to see the new memory configured.  You could also type [MEM] at the DOS prompt, and the size of the system memory will be displayed.

### Floppy Drive and Controller

The ALR OPTIMA DT comes standard with a 3.5-inch 1.44-MB floppy disk drive.  The controller for this drive is embedded directly on the system board and can support up to two floppy drives.

#### More about the Floppy Drive and Controller

Floppy Disk Drive

The ALR OPTIMA DT floppy disk drive supports all previous 3.5-inch formats.  This includes double-density 720-KB diskettes.  If you wish to format a double-density (720-KB) diskette in your high-density drive, you must use the format command with the following parameters:

FORMAT A:/F:720

For a more comprehensive explanation of the FORMAT command, refer to your DOS manual.

Floppy Drive Controller

The floppy drive controller on the system board can support up to two floppy drives of the following sizes:

·	5.25-inch double-density, 360-KB

·	5.25-inch high-density, 1.2-MB

·	3.5-inch double-density, 720-KB

·	3.5-inch high density, 1.44-MB (standard)

·	3.5-inch high density, 2.88-MB (special configuration, available upon request)

#### Configuring the Floppy Drive and Controller

If you remove, install, or replace a floppy drive in your system, you must modify your system configuration through the BIOS Setup.  This configuration program allows you to specify the type, capacity, and location (A: or B:) of the floppy drives in your system.  Refer to "BIOS Setup" for instructions on using the BIOS Setup.

### Hard Drive and Controller

Depending on the model that you purchased, your ALR OPTIMA DT system may or may not be equipped with a hard drive.  Refer to “ALR OPTIMA DT Models” for more information on the different models and types of hard drive available.

#### More about the Hard Drive and Controller

Hard Disk Drive

ALR hard drives are all high-performance IDE drives designed to offer maximum performance.

Hard Disk Drive Controller

The circuitry for the hard disk drive controller is located on the system board.  The ALR OPTIMA DT BIOS supports 102 drive types of various sizes, allowing you to easily expand and upgrade your computer.  A drive type not stored in the BIOS can be added through a user-definable option.  Refer to "Hard Drive Characteristics" for a list of the ALR OPTIMA DT drive types.

#### Configuring the Hard Drive and Controller

If you remove, install, or replace a hard disk drive in your system, you must modify your system configuration through the BIOS Setup.  This configuration program allows you to specify the type, capacity, and parameters of the hard drives in your system.  The ALR OPTIMA DT also offers an Autosense feature that can automatically sense the parameters of your hard drive.  For more information, refer to "BIOS Setup."

### Expansion Slots

The ALR OPTIMA DT has a total of four expansion slots:  two 32-bit PCI slots, one PCI/ISA shared slot, and one ISA slot.

#### Configuring the Expansion Slots

Standard ISA cards require no special system configuration after installation.  If you wish to install any additional expansion cards into your system, refer to "Installing Adapter Cards."

### Storage Bays

The ALR OPTIMA DT has up to seven storage bays available for your expansion needs:

- one external half-height 5.25-inch

- one external 3.5-inch

- one internal 3.5-inch

You can use these bays to install additional hard and floppy drives, a CD-ROM, or tape backup.

### BIOS

Basic Input/Output System.  This is a chip on the ALR OPTIMA DT system board that stores a set of programs in ROM.

#### More about the BIOS

The BIOS is responsible for the transfer of data between the ALR OPTIMA DT and its peripherals, such as hard drives, floppy drives, and a printer.  Although the ALR OPTIMA DT BIOS contains its own unique code, it operates in a manner that is completely IBM-compatible.  The information in your BIOS remains intact even when you turn system power OFF.

The ALR OPTIMA DT also contains what is known as a "Flash" BIOS.  Unlike most systems where the BIOS chip must be physically replaced in order to be upgraded, ALR's "Flash" BIOS can be upgraded through software alone.  This allows you to easily update your system with the latest BIOS information.  "Flash" BIOS upgrades can be downloaded from ALR's BBS.  For information on calling the BBS, refer to "ALR BBS Service."

#### Configuring the BIOS

You can alter the configuration of the BIOS through the BIOS Setup Program by simultaneously pressing <Ctrl>-<Alt>-<Esc> during POST (Power On Self Test).  For detailed information on the BIOS Setup, refer to "BIOS Setup."

### System Board Jumpers

Jumpers on the system board allow you to configure your system to suit your specific needs.  The figure below shows the location of all the jumpers on the ALR OPTIMA DT system board.

#### System Board Jumper Settings

Use the table below as a guide for changing the jumper settings on your system.

WARNING!  DO NOT change a default jumper setting unless you know what you are doing!
System Board Jumpers

| System Board Jumpers |  |  |  |  |
| --- | --- | --- | --- | --- |
| JUMPER | SETTING |  | FUNCTION |  |
| JP2 | 1-2 2-3* |  | Check Password Bypass Password |  |
| JP6 - JP7 | 1-2 - 1-2 1-2 - 2-3 2-3 - 2-3 |  | 256K 512K 1 MB |  |
| CPU Speed | JP4 | JP16 | JP19 | JP21 |
| 100 MHz | 1-4 | 1-2 | 2-3 | 1-2 |
| 133 MHz | 3-6 | 1-2 | 2-3 | 2-3 |
| 150 MHz | 2-5 | 2-3 | 2-3 | 2-3 |

NOTE:  This jumper is reserved for manufacturer’s use only.  DO NOT change it.

### System Video Unit

The ALR OPTIMA DT system incorporates local-bus video technology to provide state-of-the-art VGA and SVGA display capabilities.  The system board comes with 1 MB of video memory which is expandable to 2 MB.  This is done by installing two 256 Kilobit x 16, 70 NS, SOJ package style, memory units into the two VGA memory expansion sockets.

Refer to the table below for the ALR OPTIMA DT graphics modes:

| Pixel Resolution | Maximum Memory | Number of Colors | Vertical Refresh (Hz) |
| --- | --- | --- | --- |
| 1280 x 1024 | 1-MB | 16 | 43.5, 60 |
| 1024 x 768 | 1-MB 1-MB | 16 256 | 43.5, 60, 70, 72, 75 43.5, 60, 70, 72, 75 |
| 800 x 600 | 1-MB 1-MB 1-MB | 16 256 65,536 | 60, 72, 75 60, 72, 75 60, 72, 75 |
| 640 x 480 | 1-MB 1-MB 1-MB | 16 256 65,536 | 60, 75 60, 75 60, 75 |
| 1280 x 1024 | 2-MB | 256 | 43.5, 60 |
| 1024 x 768 | 2-MB | 65,536 | 43.5, 60, 70, 72, 75 |
| 800 x 600 | 2-MB | 16.8 million | 56, 60 |
| 640 x 480 | 2-MB | 16.8 million | 60, 75 |

#### Video Drivers

Video drivers are included as required with the system, so you can take advantage of the full potential of your system VGA/SVGA graphic capabilities.  Video drivers are not always necessary, but they are required if you wish to use higher resolutions and a greater number of simultaneous colors.

The system video drivers diskette contains a README file that explains how to install the drivers on your system.  The README.TXT file can be viewed with any text viewer or word processor.

### Keyboard

The ALR OPTIMA DT system includes an enhanced 101-key IBM AT-compatible keyboard.  The long interface cable allows you to place the keyboard at a convenient distance from the system.

#### Configuring the Keyboard

The ALR OPTIMA DT keyboard requires no special configuration to operate and is automatically detected by your system when you apply power.  Some keyboards have a switch underneath labeled AT/PC XT.  This switch should ALWAYS be in the AT position.

### Standard Software

The software included with your system depends on the model you purchased and your choice of either:  (a) MS-DOS and Windows for Workgroups 3.11 or (b) Windows® 95.  Refer to “Standard System Specifications” to find out about the software included with the different ALR OPTIMA DT models.

#### MS-DOS and Windows for Workgroups 3.11

Some ALR OPTIMA DT systems shipped with a hard drive come standard with the latest version of MS-DOS and Windows for Workgroups 3.11.

MS-DOS is a character-based operating system that provides a basic platform through which Windows operates; Microsoft Windows is a multi-tasking, Graphical User Interface (GUI) that allows you to get the most out of your system.

#### Windows® 95

Some ALR OPTIMA DT systems shipped with a hard drive come standard with Windows® 95.

# Options

## Available Options

To ensure optimum system performance, quality, and reliability, use only ALR optional equipment with your ALR OPTIMA DT.  Optional equipment for all ALR OPTIMA DT models includes the following:

ALR OPTIMA DT OPTIONS

- Additional RAM (up to 128-MB)

- Additional 1-MB integrated video DRAM (up to 2-MB total)

- ClearView 14-inch SVGA multi-scan, non-interlaced, non-glare monitor

- ClearView 15-inch SVGA multi-scan, non-interlaced, non-glare monitor

- ClearView 17-inch SVGA multi-scan, non-interlaced, non-glare monitor

- IDE hard disk drive

- 5.25-inch floppy disk drive

- Second 3.5-inch floppy disk drive

- Tape backup drive

- CD-ROM drive

- Multimedia kit

- Audio card

- Fax modem

- Network adapter board

- Two-button mouse

NOTE:  For a complete listing of the available options for the ALR OPTIMA DT, contact ALR Sales at (800) 444-4257 or your ALR reseller.

## Installing Optional Equipment

### ESD Precautions

Electro Static Discharge (ESD) can damage or destroy your Central Processing Unit (CPU); system, video, and cache memory SIMM units and integrated circuits; disk drives; adapter cards; and other components.  Always observe the following precautions before you install a system component.

- Be sure to unplug the AC Power Cable from the AC Power Receptacle before you open the system.

- DO NOT remove a component from its protective packaging until you are ready to install it.

Wear a wrist grounding strap and connect it to the system metal frame before handling any components.  If you do not have a wrist grounding strap, maintain contact with the system metal frame throughout any procedure requiring ESD protection.  The ideal arrangement is to work on a metal table top which has been grounded to earth (with both the wrist grounding strap and the system metal frame connected to the table top).  Be especially careful when the relative humidity is low (<60% RH).

### Opening the System Unit

WARNING!  Power the system OFF and disconnect the power cord from the system unit before proceeding.  Turn off any other equipment connected to the computer.   Installing any component while the power is on is not only dangerous but can permanently damage your computer and its parts.

To open the chassis to re-configure the boards and/or add optional accessories, follow the procedure below:

- Turn OFF power and unplug the AC power cord from the AC power receptacle.

- Unlock the Keylock on the rear of the chassis.

2.	Remove the five screws which secure the cover to the chassis.

4.	Slide the chassis cover back.  Then, lift the cover off as shown in the figure below.

### Closing the System

Before closing the system, make certain that all connectors and cards are properly installed and firmly connected.

- Hook the locking tabs on the bottom of the cover over the edge of the chassis.

- Slide the cover toward the front of the unit.

- Secure the cover with the two screws that you removed when opening the system.

- Lock the chassis keylock on the front right side of the system.

- Connect the AC power cord into the AC power receptacle.

### Additional RAM

The ALR OPTIMA DT comes standard with 8-MB or 16-MB of memory, depending on the system that you purchased.  Refer to “Standard System Specifications” to find out how much memory your system came with.  Up to 128-MB can be installed on the system board.  When adding RAM to your system, use only ALR approved SIMMs.  Standard SIMMs cannot guarantee the reliability and compatibility of tested and approved ALR OPTIMA DT RAM.  Refer to the following section for a list of the RAM configurations available for your system.

#### Memory Configurations

The following table lists the possible memory configurations of the ALR OPTIMA DT:

| Bank A (2) SIMMs/Bank | Bank B (2) SIMMs/Bank | Total Memory |
| --- | --- | --- |
| 1M x 32 (4-MB) | -- | 8-MB |
| 1M x 32 (4-MB) | 1M x 32 (4-MB) | 16-MB |
| 1M x 32 (4-MB) | 2 M x 32 (8-MB) | 24-MB |
| 1M x 32 (4-MB) | 4M x 32 (16-MB) | 40-MB |
| 1M x 32 (4-MB) | 8M x 32 (32-MB) | 72-MB |
| 2M x 32 (8-MB) | -- | 16-MB |
| 2M x 32 (8-MB) | 1M x 32 (4-MB) | 24-MB |
| 2M x 32 (8-MB) | 2M x 32 (8-MB) | 32-MB |
| 2M x 32 (8-MB) | 4M x 32 (16-MB) | 48-MB |
| 2M x 32 (8-MB) | 8M x 32 (32-MB) | 80-MB |
| 4M x 32 (16MB) | -- | 32-MB |
| 4M x 32 (16MB) | 4M x 32 (16-MB) | 64-MB |
| 4M x 32 (16MB) | 8M x 32 (32-MB) | 96-MB |
| 8M x 32 (32MB) | -- | 64-MB |
| 8M x 32 (32MB) | 8M x 32 (32-MB) | 128-MB |

#### RAMM Installation

NOTE:  When increasing the RAM in your system, be sure to use only ALR OPTIMA DT RAM.

The following procedure describes how to install additional RAM into your system:

1.	Open the system as described under "Opening the System Unit."

2.	Follow the procedures detailed in “ESD Precautions.”

3.	Check the RAM Expansion Chart before proceeding.

4.	Carefully slip a SIMM at a 45 degree angle into a socket with the component side facing down as shown in the figure below.

5.	Gently push the SIMM up until the pegs of the socket slip into the holes on the SIMM and the holding clips lock the SIMM into a vertical position.

6.	Repeat steps 2 through 5 to install the remaining SIMM units.

7.	Close the system following the procedure described in “Closing the System.”

8.	Power up your system.

9.	You may enter the BIOS Setup program if you with to see the new memory configured.  You could also type [MEM] at the DOS prompt, and the size of the system memory will be displayed.

NOTE:  Inspect the memory module to make sure it is properly installed within the socket.  This can be verified by examining the top of the modules to ensure that they are all the same depth within the socket.  If the memory modules are not installed properly, memory errors will occur when you power up your system.

### Installing a 5.25-inch Device

You can increase the functionality and storage capability of your system by installing any of the following optional 5.25-inch device in the system’s 5.25-inch drive bay:

- floppy disk drive

- hard drive

- magnetic tape drive (for backing up files)

- CD-ROM drive

To install a 5.25-inch device, follow the steps below:

1.	Configure the device according to the instructions provided with it.

2.	Remove the cover, following the instructions in Opening the System.

3.	Detach the Front Bezel by removing the screw from behind the front panel of the chassis, underneath the support bar.  Pull the Front Bezel away from the chassis, from beneath the bezel, until it is released from the four snap-on clips on each corner.

4.	Remove the mounting rails.  Install the device as shown on the figure below.

5.	Connect the power and the inteface cables.

6.	Configure the device through the BIOS Setup program and/or the accompanying configuration software.

### Installing a Hard Drive

If your system did not come with a hard drive installed in the internal drive bay, you may install a hard drive yourself.  You may also replace the existing hard drive with a larger one if you need more hard disk space.

Follow the steps below to install or replace the internal hard drive in your system.

1.	Turn OFF the system power.  Unplug the AC power cable from the AC power receptacle and disconnect all peripheral signal and power cables.

2.	Unlock the keylock on the rear panel of the system chassis.

3.	Slide out the hard drive drawer.

4.	If you already have a drive in the bay, remove it.  Disconnect the power and the hard drive interface cables from the rear of the drive.

5.	Install your new hard drive in the drawer.  Connect the hard drive power and interface cables as shown in the figure below.

6.	Close the drawer and secure it with the keylock.

7.	Run the BIOS Setup program and enter the parameters of the new drive.  Refer to “BIOS Setup” for details.

### Installing Adapter Cards

ALR offers a wide variety of adapter cards.  Since these options change on a regular basis, ALR recommends that you call the ALR TechFAX for the most current available options for your system.

Any standard ISA card can be installed into your system.

IMPORTANT!   Observe the following procedures to prevent damage to expansion cards:

·	Review the “ESD Precautions” before performing any of the following procedures.

·	Always hold the adapter cards by the edges.

·	NEVER lay expansion cards on any static-producing surface such as carpeting.

·	Discharge static electricity from your body by touching the metal cabinet of any electrical equipment that is plugged into a three-prong grounded outlet.

1.	Open the system following the procedure described under "Opening the System Unit."

2.	Using a cross-tip (Phillips) screwdriver, remove the screw that secures the cover plate from an available adapter card slot (ISA or PCI).  Save the screw.

3.	Read the adapter card installation instructions before proceeding.  Remove the adapter card from the protective anti-static bag and install any jumpers that are required.

4.	Install the adapter card firmly and completely into the slot as shown in the figure below.  If you are installing a full-size card, make certain it aligns with the card guide at the front of the system chassis.

5.	Secure the card with the screw you removed in step 2.  Double-check the board, making certain the card is intstalled correctly.

6.	Connect any cables required according to the adapter card instructions.

7.	Replace the system cover following the procedure in "Closing the System."

8.	Power up your system and configure the new card according to the instructions provided with the adapter card and as described in the “BIOS Setup.”

### Upgrading the CPU

The ALR OPTIMA DT features an easily upgradeable central processing unit (CPU) with a system board that accepts a variety of CPU upgrade units.

- Open the system as described in "Opening the System Unit.”

- Pull up the CPU socket lever to release the old CPU as shown below.

- Remove the old CPU from the CPU socket.

- Insert the upgrade CPU into the CPU socket.  Align pin 1 of the socket and pin 1 of the CPU (notched corner) as shown below.  Press the CPU gently but firmly into place.  Be careful not to bend any pins.

- Push down the CPU socket lever to lock the CPU into the socket.

- Change the “CPU Type Selection” according to the speed of the CPU upgrade you just installed.

- Close the system as described in “Closing the System.”

# BIOS Setup

## The BIOS Setup Program

The BIOS Setup program comes factory installed in a chip on your system board.  It sets the contents of the CMOS RAM which is a “library of information” that tells your system, for example, how it has been configured and what kinds of devices are connected to it.  The BIOS Setup program also allows you to customize your system configuration.

You must run the BIOS Setup program whenever you want to perform any of the following:

- Install or remove a peripheral device

- Change the display type (for example, from VGA to SVGA)

- Change the password or security features

- Change the CPU speed (for example, from Low to High)

- Change the memory size or configuration

- Change or set the date and time

- Change the power management features

- Configure the PCI adapter card features

## Preparing to Run Setup

Before running the BIOS Setup program, have the following information ready:

- Floppy disk drive type(s).  You can install one or two floppy drives of the following types:

| Drive Size | Capacity |
| --- | --- |
| 3.5-inch | - 720 KB (obsolete but can be installed) - 1.44 MB (standard with the ALR OPTIMA DT) - 2.88 MB (special, available upon request) |
| 5.25-inch | - 360 KB (obsolete but can be installed) - 1.2 MB (optional) |

- Hard disk drive type(s).  You can install up to four IDE Devices.  To determine your drive type number and characteristics, compare the information on the label on your hard disk (or supplied in vendor documentation) with the hard disk types (numbers 1 through 100) which are listed in Appendix A.  The specific information you need to know for each hard drive you are going to install is the Number of Cylinders, the Number of Heads, and the Number of Sectors.  When you have installed your hard drive(s) successfully, RECORD the hard drive information in the Appendix of the User’s Guide.  Space is provided for all four hard drives (0-3) which is the maximum you can install as an option.  When your CMOS RAM battery has expired, the hard drive information will be LOST, and you will need to install a new battery and rerun the BIOS Setup program.

The BIOS can automatically configure the Hard Disk Drive for you.  When you run the BIOS Setup program, select Auto for Hard Disk 0 when you are in the Basic Configuration Menu Screen.  When you power up your system, the BIOS will automatically determine the Hard Disk Drive parameters.

## Setup Entering BIOS

The BIOS Setup Program can be activated by pressing <Ctrl>-<Alt>-<Esc> at anytime.  This will display the first BIOS Setup screen as illustrated below:

You can scroll through the other Setup screens by pressing the up or down arrow key and then <Enter>.

IMPORTANT!  Close all open files and leave your application program before starting the BIOS Setup.  You cannot exit back into an application.  The system automatically reboots when you leave Setup.

## Basic System Configuration

Select the Basic System Configuration to input configuration values such as date, time, and disk types.  The screen below shows page 1 of the Basic System Configuration menu:

The command line at the bottom of the display has three functions.  It tells you how to view the configuration options for each parameter, how to move the highlight bar from one parameter to the next, and how to change the value for each parameter.

- Press the up or down arrow key to move the highlight bar to the desired parameter.

- Press the <PgDn> key to turn to the next page, or press the <PgUp> key to return to the previous page.

- Press the left or right arrow key to select the desired option for a parameter.  This option is now set as the configuration value.

NOTE:  Settings that appear in gray have been pre-set and cannot be changed.

The following sections explain the different parameter settings.

### Date and Time

#### Date

To set the date, move the highlight bar to the Date parameter.  Press the left or right arrow key to set the current date, following the month/day/year format.  If you want to change the date, highlight the Date parameter and set the new date.

Valid values for the month, day, and year are as follows:

Month:	1 to 12

Day:	1 to 31

Year: 	00 to 99

#### Time

To set the time, move the highlight bar to the Time parameter.  Press the left or right arrow key to set the current time in the hour/minute/second format.  If you want to change the time, highlight the Time parameter and set the new time.

Valid values for hour, minute, and second are as follows:

Hour:	1 to 12

Minute:	1 to 31

Second: 	00 to 99

After setting the date and time, you need not set them again.  As long as the CMOS RAM battery is “alive”, the clock continues to keep the date and time accurately, whether your system is on or off.  When the operating system asks for the date and time, press <Enter> twice if you do not want to change the values of these parameters.  The operating system will not ask for the date and time if you have created an AUTOEXEC.BAT file.  Refer to your operating system manual for more information.

### Floppy Disk Drives

#### First Diskette Drive

To enter the configuration value for the first floppy disk, move the highlight bar to the Diskette Drive A parameter.  If you replace the installed drive with a 2.88 MB 3.5” diskette drive, you will need to change this parameter.

Press the left or right arrow key to view the options for this parameter and select the drive type you have installed in your system if you have replaced the original drive.  The parameter options are listed below:

- [None]

- [360 KB,	5.25-inch]

- [1.2 MB,	5.25-inch]

- [720 KB,	3.5-inch]

- [1.44 MB,	3.5-inch ] standard

- [2.88 MB, 3.5-inch] special setting, available upon request

#### Second Diskette Drive

If you have installed a second diskette (or floppy disk) drive, highlight the Diskette Drive B parameter and follow the same procedure as First Diskette Drive to set the drive type.  If there is no second diskette drive, select [None].

### Hard Disk Drives

Your system does not come with a standard hard disk drive.  If your reseller installed a hard disk drive for you, the settings are probably correct.  Write down these settings in the Appendix so that you can set them again if the CMOS RAM “forgets” them.  Otherwise, proceed with the configuration of the first hard disk drive as follows.

#### Hard Disk Drive 0

To configure the “0” or first hard disk drive (C: Drive), move the highlight bar to the Hard Disk 0 parameter.

Press the left or right arrow key to view the options and select the value that corresponds to your hard disk drive type.  Read the list of hard drives in the Appendix (drive numbers 1 through 100).  If you find your drive listed there, write down the number.  Press the left or right arrow key until that number is displayed.  Check that the number of cylinders, number of heads, and number of sectors per track match the drive number in the Appendix.  Push the down arrow key to exit.  If you do not find your drive listed in the Appendix list, you may try selecting [Auto] and the BIOS utility should automatically determine the type of drive during the POST routine when you turn ON your system.  If this does not work, you will have to choose [User] and manually enter the number of cylinders, the number of heads, and the number of sectors per track.  When finished, press the down arrow key to exit.

If you have installed a second hard disk drive or a CD-ROM drive, highlight the Hard Disk 1 parameter and follow the same procedure to set a value for the second hard disk drive.

#### Hard Disk Drive 1

If your system does not have a second hard disk drive, set the value to [None].  If you have installed a second hard disk drive or a CD-ROM drive, highlight the Hard Disk 1 parameter and follow the same procedure as Hard Disk Drive 0 to set a value for the second hard disk drive.

#### Hard Disk Drives 2 and 3 or CD-ROM Drives

These parameters are optional.  If the Enhanced Local Bus IDE has been installed in your system, it can support up to four IDE drives.  The onboard IDE Drive Connector 2 is reserved for the third and fourth IDE drives (or Drives 2 and 3).  If the Enhanced Local Bus IDE has not been installed, leave these values set to [None].

If you have installed a third or fourth drive, you may choose [Auto] for a hard disk drive.  The last drive should be used as the CD-ROM drive.  It is best to use the IDE Drive Connector only for CD-ROM drives and magnetic tape drives.  If you are installing a magnetic tape drive, follow the manufacturer’s instructions carefully.

The BIOS utility will automatically configure the additional hard disk drive(s).  If not, you will have to configure the hard disk drive(s) as Hard Disk 0 or 1 on IDE Drive Connector 1.

#### Sytem Memory

The BIOS utility automatically detects the total amount of onboard system memory and displays the system memory size in the BIOS Setup program.  You cannot alter this setting.  If you install additional system memory, the BIOS Utility will detect and automatically display it when you turn ON your system and when you run the BIOS setup.

#### Math Coprocessor

Because your system comes with a built-in Math Coprocessor, this parameter cannot be changed.

NOTE:  You cannot install a separate Math Coprocessor into your ALR OPTIMA DT system.

#### Video Display

Your system contains a built-in VGA/SVGA video port.  This parameter cannot be changed.

### Communication Parameter Settings

The Communication Settings parameters let you set the baud rate, parity, stop bit, and data length for the first 9-pin serial port (COM 1).

- Baud rate: 110 to 9600 bps

- Parity: odd, even, or none

- Stop bit: 1 or 2 stop bits

- Data length: 7 or 8 bit data length

The default values are 9600 bps, no parity, 1 stop bit, and 8 bit data length.  You should not bother to change these values because the application software that handles the installed serial device (mouse, modem, printer, & etc.) will almost always override these values.  If you install a fast external modem on one of the two serial ports, your communications software can usually “lock” the data rate between the serial port and the modem at speeds higher than 9600 bps (typically 19,200 or 38,400 bps).  The two serial ports can be operated as fast as 56,000 bps if necessary.

### Enhanced IDE Features

#### Hard Disk Block Mode

This function enhances disk performance, depending on the hard disk in use.  If [Enabled], it allows data transfer in blocks (multiple sectors) by increasing the data transfer rate to 256 bytes/cycle.  If your system does not boot after enabling this parameter, change the setting to [Disabled].  Disable the parameter if your hard disk does not support this function.

#### Advanced PIO Mode IDE

Enabling this parameter improves system performance by allowing your system to support the Advanced PIO Mode transfer.  If your hard disk does not support this function or if you want your system to run in normal mode, set this parameter to [Disabled].

#### Hard Disk Size > 504 MB

This enhanced IDE feature is normally set to Disabled and only works under the DOS and Windows 3.x environments.   If [Enabled], it will allow you to use a hard disk with a capacity of more than 504 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  Other operating systems such as Novell require that this parameter be set to [Disabled].

#### Hard Disk 32-bit Access

Enabling this parameter improves system performance by allowing the use of the 32-bit hard disk access.  This enhanced IDE feature only works under the DOS, Windows 3.x, and Windows 95 environments.  Novell NetWare and other operating systems require that this parameter be set to [Disabled].

### Large Memory Support Mode

When set on [Advanced], this parameter enables operating systems such as Windows NT or OS/2 to recognize memory upgrades beyond 64MB.  The default setting is [Normal].

### Num Lock After Boot

This parameter enables the system to set the “Num Lock” function on your keyboard when you turn your system ON.  After the system boot is finished, the Num Lock LED will be lit.  The default setting is [Enabled].

### Memory Test

The BIOS utility will perform a system memory test during the POST routine if you enable this parameter.  Set this parameter to [Disabled] to bypass the system memory test during the POST routine.  The default setting is [Enabled].

### Auto Configuration Mode

When enabled, this parameter sets the hard disk drive type to [Auto] (this is overridden by the settings in page 1 of the Basic System Configuration screen).  Since this parameter also enables the system memory, the video memory, and the cache memory, it is necessary to leave this parameter [Enabled] which is the default setting.

### Fast Boot Mode

This parameter enables the system to boot faster by skipping some POST routines.  If enabled, it sets the system speed to [High].  Since this parameter also enables the Shadow RAM mode and all of the cache memory, it is necessary to leave this parameter [Enabled] which is the default setting.

## Advanced System Configuration

Select the Advanced System Configuration from the main menu to configure your system memory.

### Shadow RAM

The Shadow RAM parameter allows you to change the system BIOS and video BIOS locations from ROM to RAM.  When the system boots, the BIOS routines are copied from ROM into RAM.  This mode of operation is called Shadow RAM.  Relocating the BIOS information to RAM enhances system performance because information access is faster from RAM.

To change the BIOS location from ROM to RAM, select [Enabled].  If you select [Disabled], the BIOS information will be accessed from ROM.  The default setting for the system BIOS and video BIOS is [Enabled].  The remaining Shadow RAM is [Disabled].

### Internal Cache

This parameter is set at [Enabled] and cannot be changed.

### System Memory Parity

This parameter lets you enable or disable the optional parity DRAM.  If you install system memory SIMM units with parity, it will be necessary to set this parameter to [Enabled].  The default is [Disabled].

### Memory at 15MB - 16MB Reserved

To prevent memory address conflicts between the system and expansion boards, this memory range can be reserved for the use of either the system or an expansion board.  The default setting is [System] Use.

## Power Saving Configuration

Selecting the Power Saving Configuration option from the main menu will display the following screens:

### Power Management Mode

The Power Management Mode, when [Enabled], allows you to change the power consumption settings of your ALR OPTIMA DT.

#### Monitor Power Saving Timer

This parameter enables the VESA DPMS compatible display monitor to go to Suspend Mode if no keyboard or mouse activity has occurred during the set time period.  Any keyboard or mouse action will return the monitor to full power.  Use the arrow keys to set the inactivity time (1 ~ 15 minutes).

#### IDE Fixed Disk Standby Timer

This parameter allows you to set the hard disk drive to activate Standby Mode after inactivity of 1 ~ 15 minutes.  When the hard drive is accessed again, allow 3 ~ 5 seconds (depending on the hard drive in use) for the hard drive to return to normal speed.  Set this function to [Off] if your hard drive does not support this function.  Use the arrow keys to set the inactivity time (1 ~ 15 minutes).

#### System Standby Timer

This parameter provides a fast-on power-saving mode by setting the monitor to Suspend Mode and the hard disk to Standby Mode.  If the system is idle for a specified period of time, it will automatically enter the System Standby Mode.  Any keyboard or mouse action returns the system to the Normal Operation Mode.  Use the arrow keys to set the inactivity time (1 ~ 150 minutes).

#### System Suspend Time

This parameter supports the lowest power-saving mode.  The CPU clock will stop when the system enters the System Suspend Mode.  To resume normal operation, you can either press a key or move the mouse.  Use the arrow keys to set the inactivity time (10 ~ 150 minutes).

#### Monitored Activities

This parameter allows you to configure the power management mode by checking for activity on the IRQ and DRQ (same as DMA) ports.  If your system is connected to a LAN, determine the IRQ number used by the network interface card and disable the IRQ number.  Also, disable the DRQ (DMA) Request (DRQ0 - DRQ7).

## System Security Setup

The BIOS Setup program has a number of security features to prevent unauthorized access to the system and data.

Enter the BIOS Setup program and select [System Security] from the main menu to access the screen below.

### Disk Drive Control

The disk drive control features allow you to enable or disable the read/write functions of a disk drive.  These features can also control the diskette drive or hard disk drive boot function to prevent loading operating systems or other programs from a certain drive while the other drives remain operational.

NOTE:  The driver control settings that you specify will be activated only if the diskette and hard disk controllers are not disabled through jumper settings.

The tables below summarize the functions of the drive control parameters:

| Diskette Drive Parameter | Function |
| --- | --- |
| Normal | Diskette drive functions normally. |
| Write Protect All Sectors | Disables the write function on all sectors |
| Write Protect Boot Sector | Disables the write functions only on boot sector |
| Disabled | Disables all diskette functions |

| Hard Disk Drive Parameter | Function |
| --- | --- |
| Normal | Hard disk drive functions normally. |
| Write Protect All Sectors | Disables the write function on all sectors |
| Write Protect Boot Sector | Disables the write functions only on boot sector |
| Disabled | Disables all hard disk functions |

| System Boot Drive Parameter | Function |
| --- | --- |
| Drive A then C | System boots from drive A to C. |
| Drive C then A | System boots from drive C to A. |
| C | System always boots from drive C. |
| A | System always boots from drive A. |

### Onboard Communication Ports

#### Serial Port 1

This parameter enables the onboard serial port 1 and sets one of five possible addresses as shown below:

| Parameter | Function |
| --- | --- |
| Serial 1 (3F8h) | COM1 with address 3F8h |
| Serial 2 (2F8h) | COM2 with address 2F8h |
| Serial 3 (3E8h) | COM3 with address 3E8h |
| Serial 4 (2E8h) | COM4 with address 2E8h |
| Disabled | Disables serial port 1 |

#### Serial Port 2

This parameter enables the onboard serial port 2 and sets one of five possible addresses as shown below:

| Parameter | Function |
| --- | --- |
| Serial 1 (3F8h) | COM1 with address 3F8h |
| Serial 2 (2F8h) | COM2 with address 2F8h |
| Serial 3 (3E8h) | COM3 with address 3E8h |
| Serial 4 (2E8h) | COM4 with address 2E8h |
| Disabled | Disables serial port 1 |

NOTE:  Some serial port adapter cards or internal modems may require you to disable the onboard serial ports.  Refer to your adapter card or internal modem manual for information on how to configure the system.

#### Parallel Port

The ALR OPTIMA DT has one parallel port.  The table below lists the options for selecting the parallel port address:

| Parameter | Function |
| --- | --- |
| 3BCh | Corresponds to the parallel port with address 3BCh |
| 378h | Corresponds to the parallel port with address 378h |
| 278h | Corresponds to the parallel port with address 278h |
| Disabled | Disables the parallel port |

If you install an adapter card that has a parallel port whose address conflicts with the onboard parallel port, the system will automatically disable the onboard parallel port.

Check the parallel port address on the adapter card and change the address to one that does not conflict with the onboard parallel port address.

#### Operation Mode

This parameter allows you to set the operation mode of the parallel port as shown in the table below:

| Setting | Function |
| --- | --- |
| Enhanced Parallel Port (EPP) | Allows bidirectional parallel port operation at maximum speed |
| Extended Capabilities Port (ECP) | Allows parallel port to operate in bidirectional mode and at a speed higher than the maximum data tranfer rate |
| Standard and Bidirectional | Allows normal speed operation in a two-way mode |

### Setup Password

This option lets you set a password for running the BIOS Setup program.  Move the highlight bar to the Setup Password parameter and press the left or right arrow key.  The screen will display a key pattern requesting you to enter the password as shown in the following sample screen:

As you type each character, it will be hidden with a +, so be careful while typing your password.  To ensure accuracy, the system will request you to retype the password before it stores the value.  If the second password does not match the first, the following message will appear:

If you have verified the Setup password, this parameter will change to [Present] automatically.  You can press <Esc> anytime you want to cancel.

### Power On Password

This option enables you to set a password at system power-on.  Select [None] to disable the password feature.  Press the left or right arrow key, and the screen will display a key pattern requesting you to enter the password as shown in the following screen:

Each character you type will be hidden with a +.  To ensure accuracy, the system will request you to retype the password before it stores the value.  If the second password does not match the first one, the following message will appear:

The password prompt will appear each time the system boots if the Password Type parameter is set as Power-on.  To disable this option enter the current system password at the password prompt followed by a slash (/).

NOTE:  After you set and save the password in BIOS, you must open up your system and set the System Board Jumper JP2 to pins 1-2 to enable “Check Password.”  Refer to “System Board Jumpers” for more information.

You can change your password at any time.

To change the existing password, enter the current password at the prompt when you boot the system, followed by a slash (/) and the new password you wish to specify.  Be careful when typing the password as each ASCII character is hidden by a +.

### PCI System Configuration

The PCI System Configuration menu is displayed below:

#### PCI IRQ Setting

This parameter allows for [Auto] or [Manual] configuration of PCI devices.  If you use plug-and-play (PnP) devices, you may keep the default setting on [Auto].  The system then automatically configures the PnP devices.  If your PCI device is not a PnP, you can manually assign the interrupt for each device installed.  Refer to your PCI device instruction manual for technical information about the PCI device parameters.

#### PCI Slots

These parameters allow you to specify the appropriate interrupt for all of the PCI devices.  You can assign IRQ5, IRQ9, IRQ10, IRQ11, or IRQ15 to each of the slots.  BIOS Setup will not permit you to assign conflicting interrupts to the slots.  The configurable PCI slot parameters depend on the number of functions that your device adapter card supports.

#### VGA Pallette Snoop

PCI devices support the VGA palette snooping technique that enables the device to control access to the device palette registers.  Set this parameter to [Enabled] to activate the palette snooping function for the PCI VGA devices installed in your system.  Check your PCI VGA device instruction manual for more information about this function.  The default setting is [Disabled].

### Load Setup Default Settings

This parameter setting loads default values to the system configuration parameters.  The default values are optimized configuration values for the system.  If you select this option, the following prompt appears on the main menu screen:

NOTE:  If you have replaced a “dead” CMOS memory battery, you can select the “Load Setup Default Settings” from the BIOS Utility menu which will automatically reload the default settings and configure the system.

### Leaving BIOS Setup

Verify all the system configuration values.  When you are satisfied that all the values are correct, write them down in the Appendix of this User’s Guide.  In the future, if and when the battery “dies” and can no longer be recharged, you will know what values to enter when you will have to run the BIOS Setup.

Press <Esc> to leave the system configuration setup.  If you have made any changes in the configuration values and security setup, the following screen will appear:

Use the arrow keys to select your response.  Select [Yes] to store the new data in the CMOS RAM.  Selecting [No] will retain the old configuration values and discard any changes that you may have made.  Press <Enter> to confirm your selection.   Your system will automatically reboot and reconfigure according to the changes that you have made and have been stored in the CMOS RAM.

## Flash BIOS EPROM

The ALR OPTIMA DT is one of several ALR systems that incorporates a "Flash" EPROM for the BIOS.  If you ever need to upgrade the BIOS in your system, you can download the latest revision from ALR's BBS.  For further information about the ALR BBS, refer to "ALR BBS Service."

### Updating the Flash BIOS

To update the code in the Flash BIOS, you will need the AFLASH.EXE program which you can obtain from either the ALR BBS or ALR Technical Support.  Refer to ALR Technical Support for more information on how to contact the ALR BBS and other ALR support services.

Before running the AFLASH.EXE program, clean your system memory of memory resident programs and device drivers by performing the following:

- Turn OFF the system.

- Insert the AFLASH.EXE program diskette into the floppy drive and turn ON the system.  The AFLASH program will automatically update the System BIOS and reboot itself.

- Remove the diskette when the system has finished rebooting.

# Troubleshooting

## Looking Things Over

If your system does not operate correctly, re-read the instructions for the procedure(s) you have performed.  If an error occurs within an application, consult the documentation supplied with the software.

Often times, the simplest things can cause the most trouble.  Before testing anything on the system, perform the following checks:

1.	Is the power cord connected to the PC and an AC outlet?

2.	Is the AC outlet supplying power?

3.	If the system is connected to a power strip, is it switched On?

4.	Does the voltage selection switch located in the rear of the chassis reflect the proper voltage (115V or 230V)?

### Monitor Problems

If you are experiencing difficulties with your monitor, perform the following checks:

1.	Is the monitor power cord connected to an active AC source (the wall or the rear of the system)?

2.	Is the monitor powered ON?

3.	Are the contrast and brightness knobs adjusted too low?

4.	Are there electronic devices nearby, causing interference?

NOTE:  Devices such as televisions, phones, radios, and fluorescent lights may cause interference if placed too close to your system.

5.	If the power LED is ON, try adjusting the display knobs at the rear of the monitor.

### Verifying Your Configuration

If your system is not operating correctly, the BIOS may contain an invalid configuration parameter.  Refer to “BIOS Setup” and check all the configurations.  If all the settings are correct, perhaps one of the system board jumpers is not correct or might have become disconnected.  Open the system and verify that all the jumpers are correct and firmly in place.

### Authorized ALR Reseller

If you have questions or problems, your authorized ALR reseller can be an invaluable source of information.

## System Error Messages

The following table provides a list of possible system errors.

| Message | Possible Cause | Solution |
| --- | --- | --- |
| CMOS Checksum Error | The data in the CMOS is invalid. The CMOS battery is bad (see below). | Run the BIOS Setup. Replace the battery. |
| Bad CMOS Batter | The Ni-Cad battery will fail to take a recharge after 2 or 3 years and will be “dead.” | Replace the battery. Contact your reseller. |
| Diskette Drive Controller Error | Bad or incorrect signal cable connection | Check the signal cable connection. Make sure that all connectors are firmly in place. |
| Diskette Drive Error | The floppy disk drive in your system does not match the one specified in the BIOS Setup. The drive signal cable is improperly connected or faulty. The drive power cable is not connected to the drive. The floppy disk drive is faulty. | Make certain the correct drive is selected in the BIOS Setup. Check the drive signal and power cables for proper connection. Replace any faulty cables. If the error continues, have the drive checked or replaced. |
| DRAM Configuration Error | The system memory may have been installed incorrectly. | Check that all SIMM units are properly installed in the correct sockets according to the “Memory Configuration Chart.” |
| Equipment Configuration Error | The equipment configuration does not match what is specified in the BIOS Setup. | Run the BIOS Setup and check the equipment configuration. |
| I/O Parity Error | Many possibilities | Contact your reseller. |
| Hard Disk Controller Error | The hard disk drive signal cable is faulty. | Check the hard disk drive signal cable for proper connection. |
| Hard Disk 0 Error | Bad signal or power cable connection Bad hard disk drive | Check the cables. Have the hard disk checked and replaced if necessary. |
| Hard Disk 1 Error | Bad signal or power cable connection Bad hard disk drive | Check the cables. Have the hard disk checked and replaced if necessary. |
| Keyboard Locked | The keyboard is locked. | Unlock the keyboard. |
| Keyboard Interface Error | Keyboard Interface may be inoperable. | Contact your reseller. |
| Memory Error | System memory SIMM not in socket. | Check all SIMM units for proper placement in sockets. Contact your reseller. |
| Serial 1 Conflict | Two devices assigned on COM1 | Run BIOS Setup. Disable Onboard Serial 1. |
| Serial 2 Conflict | Two devices assigned on COM2 | Run BIOS Setup Disable Onboard Serial 2. |
| Parallel 3 Conflict | Two devices assigned on the same parallel port | Run BIOS Setup Disable Onboard parallel port. |
| Pointing Device Error | Mouse not plugged into the Mouse Port on the system. Bad mouse | Check mouse connection. Replace bad mouse. |
| Pointing Device Interface Error | Mouse interface may have failed. | Contact your reseller. |
| Protected Mode Test Fail | CPU may be partially inoperable. | Contact your reseller. |
| RAM BIOS Error | May possibilities | Contact your reseller. |
| RAM Parity Error | System memory SIMM not in socket Bad SIMM | Check all SIMM units for proper placement in sockets. Contact your reseller. |
| Real Time Clock Error | Need to run BIOS Setup | Run BIOS Setup. |
| Shadow RAM Fail | Many possibilities | Contact your reseller. |
| System Memory Address Error | System memory SIMM not in socket | Check all SIMM units for proper placement in sockets. Contact your reseller. |
| Keyboard Error or No Keyboard Connected | Keyboard is not connected. Bad keyboard | Connect the keyboard to the system. Replace the keyboard. |

NOTE:  For recurring problems, perform the following:

·	Call your nearest authorized ALR reseller.

·	Call ALR Technical Support at (800) ALR-1230.

·	Remember, DO NOT send any equipment without a Return Material Authorization (RMA) and original packaging.

### Hard Drive Characteristics

The ALR OPTIMA DT supports a wide range of hard drive types including a user-definable drive.  Refer to the table below for these supported drive types.

NOTE:  Drive type 100 is autosensing and can automatically detect the drive parameters of the supported drive.  If you are unsure of the characteristics of your hard drive, try this drive type first.

| HARD DISK DRIVE TYPES AVAILABLE IN THE BIOS SETUP |  |  |  |
| --- | --- | --- | --- |
| TYPE NUMBER | CYLINDERS | HEADS | SECTORS/TRACK |
| 1 | 306 | 4 | 17 |
| 2 | 615 | 4 | 17 |
| 3 | 615 | 6 | 17 |
| 4 | 940 | 8 | 17 |
| 5 | 940 | 6 | 17 |
| 6 | 615 | 4 | 17 |
| 7 | 462 | 8 | 17 |
| 8 | 733 | 5 | 17 |
| 9 | 900 | 15 | 17 |
| 10 | 820 | 3 | 17 |
| 11 | 855 | 5 | 17 |
| 12 | 855 | 7 | 17 |
| 13 | 306 | 8 | 17 |
| 14 | 733 | 7 | 17 |
| 15 | (reserved) |  |  |
| 16 | 612 | 4 | 17 |
| 17 | 977 | 5 | 17 |
| 18 | 977 | 7 | 17 |
| 19 | 1024 | 7 | 17 |
| 20 | 733 | 5 | 17 |
| 21 | 733 | 7 | 17 |
| 22 | 733 | 5 | 17 |
| 23 | 306 | 4 | 17 |
| 24 | 612 | 4 | 17 |
| 25 | 306 | 4 | 17 |
| 26 | 612 | 4 | 17 |
| 27 | 698 | 7 | 17 |
| 28 | 976 | 5 | 17 |
| 29 | 306 | 4 | 17 |
| 30 | 611 | 4 | 17 |
| 31 | 732 | 7 | 17 |
| 32 | 1023 | 5 | 17 |
| 33 | 751 | 8 | 17 |
| 34 | 755 | 16 | 17 |
| 35 | 731 | 13 | 26 |
| 36 | 980 | 15 | 17 |
| 37 | 936 | 16 | 17 |
| 38 | 981 | 5 | 17 |
| 39 | 981 | 10 | 17 |
| 40 | 762 | 8 | 39 |
| 41 | 826 | 16 | 63 |
| 42 | 832 | 8 | 33 |
| 43 | 683 | 12 | 38 |
| 44 | 513 | 16 | 38 |
| 45 | 776 | 8 | 33 |
| 46 | 683 | 16 | 38 |
| 47 | 832 | 6 | 33 |
| 48 | 6152 | 34 |  |
| 49 | 98916 | 63 |  |
| 50 | 8234 | 38 |  |
| 51 | 100115 | 17 |  |
| 52 | 10247 | 17 |  |
| 53 | 72313 | 51 |  |
| 54 | 5488 | 38 |  |
| 55 | 10134 | 41 |  |
| 56 | 92915 | 17 |  |
| 57 | 81714 | 36 |  |
| 58 | 72313 | 51 |  |
| 59 | 8024 | 39 |  |
| 60 | 10249 | 17 |  |
| 61 | 8955 | 55 |  |
| 62 | (reserved) |  |  |
| 63 | 96610 | 34 |  |
| 64 | 10248 | 17 |  |
| 65 | 102411 | 17 |  |
| 66 | 91811 | 17 |  |
| 67 | 96914 | 49 |  |
| 68 | 102410 | 17 |  |
| 69 | 102412 | 17 |  |
| 70 | 102413 | 17 |  |
| 71 | 102414 | 17 |  |
| 72 | 10242 | 17 |  |
| 73 | 102416 | 17 |  |
| 74 | 91815 | 17 |  |
| 75 | 8206 | 17 |  |
| 76 | 10245 | 17 |  |
| 77 | 10248 | 17 |  |
| 78 | (reserved) |  |  |
| 79 | 100115 | 32 |  |
| 80 | 102416 | 63 |  |
| 81 | 102410 | 17 |  |
| 82 | 102411 | 17 |  |
| 83 | 102415 | 17 |  |
| 84 | 7768 | 33 |  |
| 85 | 92613 | 17 |  |
| 86 | 8054 | 26 |  |
| 87 | 9765 | 17 |  |
| 88 | 7454 | 28 |  |
| 89 | 7472 | 28 |  |
| 90 | 7822 | 27 |  |
| 91 | 98513 | 32 |  |
| 92 | 81615 | 32 |  |
| 93 | 9685 | 17 |  |
| 94 | 9038 | 46 |  |
| 95 | 9665 | 34 |  |
| 96 | 53510 | 50 |  |
| 97 | 71510 | 50 |  |
| 98 | 1016 | 16 | 63 |
| 99 | 996 | 16 | 63 |
| AUTO | (Automatic Configuration) |  |  |
| USER | Entered by User | Entered by User | Entered by User |
| CD | (reserved) |  |  |

To view these drive types, perform the following steps:

1.	Invoke the BIOS SETUP by simultaneously pressing <Ctrl>-<Alt>-<Esc>.

2.	Use the up or down arrow key to select the BIOS Utility menu and press <Enter>.

3.	Use the up or down arrow key to select the parameter “Fixed Disk 0” and press <F1>.

4.	Use the left or right arrow key to scroll through the different drive types supported.

CAUTION!  If you do not intend to change the Fixed Disk parameter settings, make sure you select [No] when you exit the BIOS Setup to discard any changes that you made.

Drive types and characteristics sometimes change with different BIOS versions, so it is always a good idea to first check the online information.

Drive type numbers not assigned are reserved for future use.  Drive type 100 is user-definable.  That is, you can change the parameters according to your drive specification.

### ALR Technical Support

The ALR OPTIMA DT offers a wide range of technical support and information services.  These services include standard technical support, a TechFAX service, and BBS.

#### TechFAX Service

If you require special information for your system or need the latest addendums and updates, you can usually find it by calling ALR's TechFAX service.  Using your touchtone phone, you may request a fax sheet listing all the currently available information or select specific information you require by entering the appropriate code.  ALR's TechFAX service can be reached at (714) 581-3332.

#### ALR BBS Service

ALR's electronic bulletin board service operates 24-hours a day to support the needs of the end-user.  The ALR BBS contains the latest information on ALR systems, software drivers, "Flash" BIOS updates, and even the most recent versions of the ALR OnLine Guides.

ALR currently operates the following BBS's:

Irvine---(714) 458-6834

Germany---49-69-304-961

Singapore---65-741-1805

United Kingdom---44-635-550-208

On Internet, “ftp.alr.com”

The ALR BBSs use 8 data bits, 1 stop bit and no parity.

The first time you call the ALR BBS, you will asked to enter a user-ID or "new" to start a new account.  If you type "new" you will be asked some questions that will give the ALR SYSOP a better knowledge of how to serve you.  After answering the questions, you will then have to select a user-ID as well as a password for yourself.   In subsequent loggings, you will only have to enter your user-ID and password to continue.

NOTE:  It is a good idea to record your User ID and Password, so you won’t forget them when you call again.

#### Logging Into the ALR BBS

If you are new to the ALR BBS:

- Type [NEW] at the User prompt.

- Answer all the questions on the screen.  This information helps us to improve our services.

- Select a User ID and Password.

The next time you call, enter your User ID and Password when asked to do so.  Take advantage of the extensive help feature.  The ALR BBS provides much useful information.  You can upload and download files.  You can read and write E-Mail type messages, including a note to the System Operator (SYSOP) if there is a service you would like to see or if you have a suggestion for improved services.  Read all the bulletins because ALR is constantly updating and improving the ALR BBS.  If you want to download or upload large files, you will need an external, fast modem (equal to Hayes or U.S. Robotics) and communications software capable of handling file transfer protocols such as Xmodem, Ymodem, and ZModem.

#### ALR Technical Support

If you need phone support, you may contact ALR Technical Support in Irvine, California at (800) ALR-1230 during our business hours:

- Monday through Friday from 6:00 A.M. to 6:00 P.M., Pacific Time

- Saturday, 7:00 A.M. to 1:00 P.M., Pacific Time

ALR Technical Support can also be reached via the following:

- Compuserve, “GO ALRINC”

- Internet World Wide Web (WWW), “http://www.alr.com” (Product Specification, Sales, Technical Support E-Mail)

- Internet, “tech@alr.com” (Technical Support E-Mail)

# Glossary of Terms

##### Autosense

Autosense automatically updates the BIOS Setup hard drive characteristics.  This feature works on supported drives only.

##### BIOS SETUP program

A program located within the system BIOS that allows you to alter and modify the configuration information in your computer. This is how you can configure your ALR OPTIMA DT system.

##### cache

A portion of high-speed RAM set aside to store frequently-accessed information. This can significantly increase the performance of a system by minimizing the need to return to the hard drive.

##### context-sensitive help

A help system used in the BIOS Setup program that provides help information only in the area that you require.  For example, if you highlight the section Hard Disk 1 on the first BIOS Setup screen and press <F1>, information pertaining only to that parameter will be displayed.

##### CD-ROM

Compact Disc Read-Only Memory.  A plastic disc (similar to those used in the music industry) capable of storing electronic data in digital format that can be read by a laser beam.  A standard disc can store up to 650-MB of data, roughly equivalent to 400 high-density floppy disks.

##### CMOS

Complementary Metal-Oxide Semiconductor.  A low-powered chip used to duplicate the function of other chips.

##### CPU

Central Processing Unit.  Much like the internal combustion engine in your car, this powerful chip performs the bulk of the calculations in your computer.

##### DRAM

Dynamic Random Access Memory. This is the type of memory your computer uses to run applications and perform calculations. This type of memory offers much high speed and reliability.

##### EPROM

Erasable Programmable Read-Only Memory. A chip that can be erased and reprogrammed repeatedly.

##### Excel

A spreadsheet program created by Microsoft Corporation.

##### fatal error

An serious system error that can prevent the computer from operating.  For example, a hard drive controller error that won't allow the system to boot.

##### FDISK

A DOS command that brings up a menu allowing you to partition your drive.  This allows you to divide your hard drive into several smaller "drives," or specify one large one. Through this command you can also indicate which partition is the active partition, or the one the system boots from when power is applied.

##### "Flash" EPROM

A special chip used by the ALR OPTIMA DT to store the BIOS code.  It differs from a standard EPROM in the fact that it can be reprogrammed through software alone--no longer requiring expensive lab equipment.

##### FORMAT

A DOS command that prepares new disks to accept files. This procedure creates the tracks, sectors and boot area necessary to use the diskette.

##### Graphical User Interface (GUI)

An operating environment that allows you to interact with it through graphics and icons as opposed to characters and cryptic commands. A graphical user interface can enable novices to work comfortably on today's computers, and can help more experienced users be even more productive.

##### Hardware Cursor

Controlling the cursor through the hardware instead of the software reduces tracking delays and lags which result in smoother cursor movement. System performance is increased since the need for software cursor checking is eliminated.

##### IDE

Integrated Device Electronics. The controller circuitry for hard disk drives that offer low cost and high performance. The ALR OPTIMA DT contains an internal controller capable of supporting up to two of these drives.

##### I/O

Input/Output

##### ISA

Industry Standard Architecture. Also known as AT architecture. This 16-bit design replaced the earlier 8-bit XT architecture of the first PC.

##### LED

Light Emitting Diode. These are ideal for indicator lights since they provide bright illumination and relatively low power drain. The ALR OPTIMA DT has two such indicators on its front panel--a green one for power on, and an amber one to hard disk usage.

##### local bus

This new technological breakthrough allows the CPU to communicate directly with peripherals, instead of having to journey through the PC bus.  This results in greater system performance, particularly when dealing with video and graphics.  The ALR OPTIMA DT's local bus conforms to the VESA VL-bus standard.

##### low-level format

Sometimes called a physical format, this operation lays out the physical pattern of magnetic tracks and sectors required before a high-level format can be done.  This procedure is usually performed at the factory and need only be done by the user in the event of a system crash.

##### multimedia

Information presented through more than one type of media.  On computer systems this media includes sound, graphics, animation and text.

##### multi-tasking

The ability to perform several tasks simultaneously.  Unlike the old days when you had to terminate a program before executing a new one, multitasking allows you to run multiple applications at the same time and exchange information among them. Operating environments such as Microsoft Windows and Microsoft Windows NT allow standard PCs to multi-task.

##### nonfatal failures

System failures that are relatively minor and do not prevent the system from operating. An example of this type of failure could be a memory error during boot up.

##### parallel port

A connector in the rear of your computer that transfers data along parallel lines, much like the lanes on a multi-lane highway.  This port connects devices such as a parallel printer to your system.

##### partitioning

Allows you to divide your hard drive into several smaller "drives" by marking off areas and establishing them as independent logical drives. This can come in handy if you  wish to have greater control over the organization of your hard drive, or you plan to use more than one operating system. For more information on this command, refer to your DOS manual.

##### PCI

A local bus that provides a high-speed data path between the CPU and the peripheral devices.

##### RAM

Random Access Memory. This is the portion of your computer where all the calculations and computations are performed. Any part of the data in random access memory can be accessed at any time, much like the tracks on an album. This differs from sequential memory, which must scan through a sequence of data before finding the desired information, much like a cassette tape. Unlike the hard disk, random access memory requires constant power to stay active, and all information stored in it is lost when the power is turned off.

##### README file

A file on a diskette that provides special instructions and/or updated information concerning the contents of the diskette. README files have names like README.DOC, README.TXT and READ.ME.

##### Return Material Authorization (RMA)

A number provided to you by either a reseller or ALR Technical Support that authorizes you to return your system to the factory for service. Once the RMA number is obtained, it must be written clearly on the outside of the box.

##### ROM

Read-Only Memory. A special type of system memory that contains codes and special instructions for your system. Read-only memory cannot be easily altered, and it remains completely intact even when power is removed.

##### serial port

A connector on the rear of your computer that transfers data one bit at a time, one after the other. This port can connect and communicate with serial devices such as serial printers, modems and a mouse.

##### SIMM

Single Inline Memory Module. The chips that comprise the RAM in your system. The ALR OPTIMA DT uses high-performance RAM that has been thoroughly tested for compatibility and reliability.

##### SYSOP

Acronym for System Operator. The term used to describe the person that operates a bulletin board.

##### User Definable

User Definable drive types are reserved for drives not supported by Autosense.  Use these drives types when entering drive characteristics manually.

##### VESA VL-bus

Local bus standard established to provide compatibility. The local bus slot in the ALR OPTIMA DT can accept any local bus adapter that conforms to this standard.

##### VRAM

Video RAM. This is the type of memory many high-performance video cards use to store the screen image. Video RAM offers higher performance than DRAM, but is much more expensive.

##### Word for Windows

A powerful word processing program created by Microsoft Corporation.

# Index

B

battery 37, 40, 51

C

CD-ROM 41

CMOS RAM 37, 40, 51

COM 1 10

COM 2 11

M

modem 42

P

password 49, 50

PnP

Plug and Play 50

POST 41, 43

R

Return Material Authorization (RMA) 9, 13, 55, 64

S

Shadow RAM 43, 44

V

VESA DPMS 45