COMPULAN

INSTRUCTION MANUAL

for

MODEL CPL-100E

32 BIT EISA

NETWORK ADAPTER

November 15, 1995

TABLE OF CONTENTS

General	3

Chapter 1  Overview	5

Overview

Topology

Adapters

Installation

Chapter 2  Loading LAN Drivers	11

General

Novell v3.xx Driver

Novell v4.xx Driver

Novell DOS ODI Workstation Driver

NetWare, Portable NetWare

NetWare-Lite v1.0, v1.1

Personal NetWare (DOS 7)

Novell DOS IPX Workstation Driver

Windows for Workgroups v3.11 Driver

Windows ‘95 Driver

NETBIOS (CBIS) Driver

NET/OS ODI Driver

Chapter 3  Diagnostics and Utilities	26

Setup Utility

Network  Simulator

Technical support

APPENDIX

A:  Definitions	31

B:  Cabling.................................................................34

PROPRIETARY NOTICE

All information contained herein relates to the CompuLAN Local Area Network hardware and software.  The content of this document is proprietary to CompuLAN.

DESIGN COPYRIGHTS

CompuLAN test software, network driver software, and hardware designs are protected by copyrights.

PATENTS

CompuLAN 100 FastNET adapters are protected by U.S. Patent number 4,860,309 and U.S. patent number 5,160,929.

COPYRIGHT

Copyright (c) 1995 by CompuLAN.  All rights reserved.  No part of this publication may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language or computer language, in any form or by any means by electronic, mechanical, magnetic, optical, chemical, manual, or otherwise, without prior written permission of CompuLAN.  Information presented in this manual is furnished for customer reference only and is subject to change without notice.

DISCLAIMERS

Although we do our best to accurately describe the hardware and software, this manual is not warranted to be technically correct.  CompuLAN does not accept responsibility for errors or omissions.  CompuLAN makes no representations or warranties with respect to the software and documentation herein described and especially disclaims any implied warranties of merchantability or fitness for any particular purpose.  Furthermore, CompuLAN reserves the right to revise this software and associated documentation without obligation of CompuLAN to notify any person of such revisions or changes.

LIMITATION OF LIABILITY

The product is sold as is.  The entire risk as to its quality and performance is with the buyer and not CompuLAN, its distributor, nor its retailer.  CompuLAN accepts no responsibility for loss or inaccuracy of data, nor does it accept any direct or consequential liability for damage or loss of any kind resulting from the use in any particular situation.  No party is authorized to assume any liability for CompuLAN without its expressed written permission.  Any liability of CompuLAN in relation to this  product shall be limited to moneys paid CompuLAN.

In the event that the product fails to perform as documented in the CompuLAN manual, the customer should report the discrepancy to CompuLAN Technical Support.  CompuLAN will make its best reasonable effort to effect a timely diagnosis and resolution of the problem.

TRADEMARKS

References are made in this manual to the IBM Personal Computers AT, PS/2 and to the IBM PC LAN Program, which are trademarks of International Business Machines Corporation, Armonk, New York.  References are made to NetWare, a trademark of Novell Incorporated, Provo, Utah.

FCC WARNING

This equipment generates, uses and can radiate radio frequency energy and if not installed and used in accordance with the instruction manual, may cause  interference to radio communications.  It has been tested and found to comply with the limits for a class "A" computing device pursuant to Subpart J of Part 15 of FCC Rules, which are designed to provide reasonable protection against such interference when operated in a commercial environment.  Operation of this equipment in a residential area is likely to cause interference, in which case the user at  his own expense will be required to take whatever measures may be required to correct the interference.

Chapter 1   OVERVIEW

This manual contains a brief introduction to the CPL-100E, 100 Megabits/sec. Network Interface Adapter.  It also provides installation, configuration, diagnostic procedures, network planning, cabling, and troubleshooting information.

This chapter includes a discussion of CompuLAN’s 100 FastNET Network topologies.  An understanding of this chapter will make installation and use of CompuLAN products easier.

The CPL-100E is a high performance, 100 Mbps, Network Interface Card (NIC) and is used to connect personal computers into a Local Area Network (LAN).  The CPL-100E is used in EISA compatible (32-bit I/O bus structure) personal computers.

To reach the full potential of a 100 Mbps network use 32-bit adapters.  A Network of all 32-bit adapters will operate 2 to 4 times faster than a Network of 16 bit adapters do to the much higher bus speeds.

The bus speed will control the 100 FastNET performance. Computer bus speed ranges from 1,400 K Bytes/sec to 2,200 KB/sec. for 16-bit ISA and from 5,000 K to 7,000 K for 32-bit PCI.  If the computer bus speed is low no high speed LAN adapter will perform well.

When a 32-bit server Adapter is used, server utilization will go down, benefiting networks with higher numbers of workstations.  Workstation performance will remain a function of workstations bus speed, however.  100 FastNET adapters perform best in networks that can effectively use CACHE memory on the server.

100 FastNET adapters use standard Ethernet Frame II, 802.2, 802.3, and 802.SNAP Frames. The drivers and hardware support an extended 4096 Byte frame length that improves performance by reducing software overhead.

100 FastNET adapters can be connected with lengths of Twisted-pair cable creating a trunk segment in the same fashion as Thin-Ethernet, or connected in a Star with cables connected to a 100 FastNET concentrator.

100 FastNET networks consist of several network stations connected together to allow electronic communications between stations.  Stations can be file servers, print servers, network workstations and bridges.

TOPOLOGY

A Network's topology refers to the type of cabling layout that the network supports.  The CompuLAN CPL-100E support a BUS  topology  as  well as a STAR topology.  The cables used to connect  the Network together run from computer to computer in a chain.  The whole "chain" or set of cables used to connect the Network together is referred to as the BUS or trunk segment.  Both ends of the trunk segment must be terminated with a special connector referred to as a "TERMINATOR".  The purpose of a TERMINATOR is to electrically balance the signals that travel through the wires. For every Network you need TERMINATORS at each end of the trunk segment.

A trunk segment is limited to a maximum length and a maximum number of stations that can be accommodated.  A network can be extended if cable runs exceed the cable limits or more stations are required by adding CompuLAN concentrators.  Concentrators form an amplifying pathway for network signals traveling from one trunk segment to another.

If LAN cables are connected to a central point or points, Concentrators should be included in the network. The Concentrator also provides an automatic port disconnection feature to keep the network operating if one cable is damaged.

Multiple Concentrators can be connected in several ways.  (1) The ports may be used as if they are repeater ports.  The Concentrator can use its standard ports for interconnection.  (2) Concentrators can be connected by putting one port of each Concentrator on a common trunk segment.  This segment can also have stations attached.  (3)  Each concentrator port can start a new trunk segment.

Total network size can also be increased by using more than one 100 FastNET adapter in a server. Maximum cable length and maximum number of Concentrator devices only apply to a continuous group of stations.

Networks may be built with a combination of hardware types, 100Mbps Ethernet, Token Ring, 100 FastNET, etc., as long as one Network adapter of each type is installed in a bridge server

Connectors

The CPL-100E adapters have two RJ45 connectors that are wired directly together.  No power is required to pass signals through an adapter.  Cables can be built with a RJ45 connector on each end.  This allows simple connections from station to station.  If a station must be removed, the ends of the two cables can be plugged together.

Network Adapter

Network adapters are inserted in each network station and cabled together to allow electronic communications between network stations.  The CPL-100E adapter must be installed in a EISA bus computer.

The CPL-100E operation and communication with your computer is controlled by two parameters:

×  INTERRUPT LEVEL (IRQ).  This parameter defines communication from the adapter to the main processor.

. I/O BASE PORT ADDRESS (I/O port). This parameter is required for communication between the main processor and the Adapter board. The CPL-100E uses slot specific I/O. The CPL-100E adapter does not contain jumpers.  The I/O Port is defined by the expansion slot the board is installed in.

Run the computers EISA configuration program with the EISA Network Adapter installed in the desired slot.  Use the !LPL0101.CFG file located in the \UTILITY subdirectory of the EISA distribution diskette to provide the configuration data for your computer.

Terminators

Devices used at each end of a cable trunk segment.  This allows peer to peer communication on a linear buss configuration.

Limitations

×  Maximum number of stations per server:  253.

×  Maximum trunk segment length:  328 feet (100 meters).

×  Maximum number of stations per trunk segment:  20.

×  Maximum total distance between any 2 stations:  1640 feet.

×  Maximum number of Concentrators:  32.

×  Maximum number of Concentrators between any two stations:  4.

Installation and Configuration Overview

.	Run the computers EISA configuration utility as described under NETWORK ADAPTER above.

× Install and configure one 100 FastNET adapter at a time using the INSTALL program located in the root directory of your distribution diskette.

× 	Connect the cables and terminators.

.	Initialize the CPL-100E	network adapter by running the FTXCFG.COM  program which uses the configuration data stored in the FTX.CFG file.

.	Load the network drivers.

FTXCFG.COM INITIALIZATION UTILITY

The FTXCFG.COM initialization utility, located in the \UTILITY subdirectory on the CPL-100E distribution diskette, is a program  that allows users to configure 100 FASTNET adapters by using an ASCII text file (FTX.CFG) to specify the board configuration, such as Node Number, Port Address, Interrupt Level, Hardware bit pattern, and Early Interrupt usage.

The FTXCFG.COM program uses data stored in the FTX.CFG file to initialize the EISA adapter card and must be run prior to loading any other network software.

INSTALLATION USING INSTALL.EXE

Run the INSTALL program located in the root directory of the distribution diskette.  Follow the instructions on the screen to configure the network adapter.  The FTX.CFG and NET.CFG files will be created and copied along with the designated drivers and utilities to a location of your choice.  The exact syntax to load the drivers for your configuration will be displayed at the end of the installation procedure and copied to your destination directory in a ???.TXT file for reference.  Specific details for each driver are discussed in chapter 2.

MANUAL INSTALLATION

STEP 1 :	POWER OFF THE COMPUTER, INSTALL THE COMPULAN CPL-100E  EISA adapter IN AN AVAILABLE SLOT AND POWER up THE COMPUTER.

STEP 2:	CONNECT THE TERMINATOR AND CABLES AS REQUIRED.

STEP 3 :	SELECT  THE  DESTINATION SUBDIRECTORY FOR THE NODE BEING CREATED.

STEP 4:		COPY FILES IN THE SUBDIRECTORY ON THE DISTRIBUTION DISKETTE CONTAINING THE REQUIRED DRIVERS TO YOUR DESTINATION SUBDIRECTORY.  COPY THE FTXCFG.COM,  E4PB0_6.BIT AND E4PB1_6.BIT FILES FROM THE \UTILITY SUBDIRECTORY ON THE DISTRIBUTION DISKETTE  TO YOUR DESTINATION SUBDIRECTORY.

STEP 5:		LOG TO THE DESTINATION DRIVE AND DIRECTORY.

STEP 6:		IF A NOVELL SERVER DRIVER WILL BE USED ON THIS STATION EDIT THE NOVELL AUTOEXEC.NCF AND STARTUP.NCF FILES TO INCLUDE CONFIGURATION INFORMATION.  (EXAMPLES OF THESE CHANGES ARE SHOWN UNDER SERVER INSTALLATION IN CHAPTER 2.)

STEP 7 :		CREATE THE FTX.CFG AND NET.CFG FILES TO REFLECT THE CONFIGURATION PARAMETERS AND BIND THE ADAPTER CARD.  (EXAMPLES OF THESE FILES ARE SHOWN LATER IN THIS CHAPTER AND CHAPTER 2.)

STEP 8:		EDIT THE AUTOEXEC.BAT FILE TO RUN THE FTXCFG.COM PROGRAM AND LOAD THE SELECTED DRIVERS.

STEP 9:		REBOOT THE COMPUTER TO RUN THE UPDATED AUTOEXEC.BAT FILE AND START THE NETWORK.

CREATING THE FTX.CFG FILE MANUALLY

There are two ways to create the FTX.CFG configuration file.  Use the configuration utility SETUP.EXE or use a text editor that can save files in an ASCII (text) format.

SETUP.EXE is a configuration and diagnostic program located in the \UTILITY subdirectory of the CPL-100E distribution diskette.  SETUP will configure the EISA adapter and create the FTX.CFG file to save the configuration for future use.

1.	Install the CPL-100E network adapter in your system with a terminator plugged into one of the RJ-45 connectors.  Do not attach any cables.

2.	Type \..path..\SETUP to start setup.  SETUP will perform a diagnostic check on the adapter card and prompt you for configuration information.  Follow the directions to configure the adapter.  KEY WORD definitions are located in Appendix A.

3.	Select the option to have SETUP create a file called FTX.CFG to store the configuration information.

4.	The EISA adapter card must be initialized on boot-up by running the FTXCFG.COM program to install the configuration stored in the FTX.CFG file prior to loading any network software.

Use a text editor that can save files in ASCII (text) format to create the FTX.CFG file containing the following KEY WORD statements.  KEY WORDS are defined in Appendix A.

BOARD [????]

NODE [???]

PORT [???]

IRQ [??]

BITPAT0 E4PB0_6.BIT

BITPAT1 E4PB1_6.BIT

EARLY_INT  [ON|OFF]

EXAMPLE:

BOARD EISA

NODE 12

PORT 300

IRQ 10

BITPAT0 E4PB0_6.BIT

BITPAT1 E4PB1_6.BIT

EARLY_INT ON

CHAPTER 2    LOADING LAN DRIVERS

This Chapter is a reference to LAN driver loading instructions according to the operating system you are using on your network.

Multiple frame types can be loaded with client or server LAN drivers.  Network stations including bridges, routers, print servers and workstations on the same network segment must use one of the same frame types as used on the server.  If the frame types do not match, communication will not take place.  One workstation on the segment can use a different frame type than another workstation as long as both frame types are loaded on the server.

Novell's new drivers default to the 802.2 frame type.  Workstation frame types for older drivers can be altered by using the NET.CFG file and frame types can be altered on servers when the LAN driver is loaded.  Older drivers for servers and workstations default to the 802.3 Ethernet frame type.

SERVERS

NetWare v3.xx

NetWare v4.xx

Workstations

DOS ODI

DOS IPX

NETBIOS

CBIS

To find the drivers  to  use  for the NOS (Network Operating System) being installed move to the correct subdirectory on the distribution disk.

\------------------------------------INSTALL program

\SERVER\NOV3X ------------NetWare 3.11, 3.12 SERVERS

\SERVER\NOV4X ------------NetWare 4.00, 4.01 SERVERS

\WS\NOVODI -----------------NOVELL ODI Workstation for NetWare 3.1x & 4.0x

\WS\NOVIPX ------------------NOVELL IPX Workstation for NetWare 3.1x & 4.0x

\WS\NOVDOS7 -------------- NOVELL DOS 7.x (Personal NetWare)

\WS\MSWFW -----------------MICROSOFT Windows for Workgroups V3.1

\WS\NOVODI------------------Microsoft Windows ‘95

\CBIS ----------------------------LANTASTIC, etc.

\UTILITY-------------------------Documentation and Diagnostics

NOTE: LAN Manager, LAN Server, Banyan and Windows NT use a 'NDIS'  driver which  is  not  supported  in  this  release.  Contact  CompuLAN (619) 541-0189 for current release information.

Novell v3.xx SERVER INSTALLATION:

Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to the server hard disk.

After copying the SYSTEM and PUBLIC files to the file server's hard disk, the next step is to edit the Novell files AUTOEXEC.NCF AND STARTUP.NCF.

The AUTOEXEC.NCF file must have the following key word statements added to the load line and BIND line:

KEY WORDS, FRAMES, and PROTOCOLS are defined in Appendix A.

:LOAD [<driver>] [FRAME=<ethernet frame>] [NODE ???] [PORT=???] [INT=?] [EARLY_INT=(ON|OFF)]

:BIND <protocol> TO <driver>  NET=<number>

Where <protocol> can be any Novell supported protocol stack e.g., IPX, IP, ARP, RARP.;  <number> can be any number (not previously used) you wish to assign to the network.  Consult your NetWare v3.xx manual for assistance.  The <driver> is the CompuLAN EISA Novell 3.xx server driver --  CD100EE3.

EXAMPLE:

:LOAD  CD100EE3  FRAME=ETHERNET_802.3  node=1  port=310  INT=B EARLY_INT=ON

:BIND  IPX TO CD100EE3  NET=32545

Edit the STARTUP.NCF file to add the following statement as the last line:

SET MAXIMUM PHYSICAL RECEIVE PACKET SIZE = 4202

This will enable the server to use its maximum data packet size and achieve optimum performance.

Note:  Make sure file server and workstation frame types, protocol stacks and protocol numbers match.

Note:	Make sure the configuration on the LOAD line matches the FTX.CFG file.

Run FTXCFG.COM prior to loading any network software.

NOVELL v4.xx SERVER INSTALLATION:

Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to the server hard disk.

After copying the SYSTEM and PUBLIC files to the file server's hard disk, the next step is to edit the Novell files AUTOEXEC.NCF AND STARTUP.NCF.

The AUTOEXEC.NCF file must have the following key word statements added to the load and BIND lines:

KEY WORDS, FRAMES, and PROTOCOLS are defined in Appendix A.

:LOAD MSM  (Novell's Media Support Module)

:LOAD FTXETSM  (CompuLAN's Ethernet Topology Support Module)

:LOAD [<driver>] [FRAME=<ethernet frame>] [NODE=???] [PORT=???] [INT=?] [EARLY_INT=(ON|OFF)]

:BIND <protocol> TO <driver>  NET=<number>

Where <protocol> can be any Novell supported protocol stack e.g., IPX, IP, ARP, RARP;  <number> can be any number (not previously used) you wish to assign to the network.  Consult your NetWare v3.xx manual for assistance.  The <driver> is the CompuLAN EISA Novell 4.xx server driver --  CD100EE4.

EXAMPLE:

:LOAD MSM

:LOAD FTXETSM

:LOAD  CD100EE4  FRAME=ETHERNET_802.3  NODE=125  PORT=300  INT=A EARLY_INT=ON

:BIND  IPX TO CD100EE4  NET=545

Edit the STARTUP.NCF file to add the following statement as the last line:

SET MAXIMUM PHYSICAL RECEIVE PACKET SIZE = 4202

This will enable the server to use its maximum data packet size and achieve optimum performance.

Note:  Make sure file server and workstation frame types, protocol stacks and protocol numbers match.

Note:	Make sure the configuration on the LOAD line matches the FTX.CFG file.  Run FTXCFG.COM prior to loading any network software.

Novell ODI DOS Workstation Drivers

I.  NOVELL NETWARE AND PORTABLE NETWARE

Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to the specified destination.

Load the following programs in this order:

>FTXCFG	(CompuLAN’s Initialization utility)

> LSL	(Novell's Link Support Layer)

> FTX100EE	(CompuLAN's CPL-100E driver)

> IPXODI	(Novell's IPX protocol stack)

> NETX	(Novell's redirector if using NETX)

MANUAL CREATION OF THE NET.CFG FILE FOR ODI WORKSTATIONS

You can create the NET.CFG file with a text editor that can save files in ASCII (text) format.  The NET.CFG file must contain the following KEY WORD lines:  KEY WORDS, FRAMES, and PROTOCOLS are defined in Appendix A.

Link support

BUFFERS  4  4202

Link driver FTX100EE

NODE [???]

PORT [???]

IRQ [??]

BITPAT0 E4PB0_6.BIT

BITPAT1 E4PB1_6.BIT

EARLY_INT [ON|OFF]

FRAME <Ethernet frame>

PROTOCOL <Ethernet protocol>

NETWARE DOS REQUESTER	(Only required for using VLM’s)

FIRST DRIVE = F		(Only required for using VLM’s)

Note:  Make sure file server and workstation frame types, protocol stacks and protocol numbers match.

Note:	Make sure the configuration in the NET.CFG file matches the FTX.CFG file.

II.  NOVELL NETWARE-LITE v1.0 AND v1.1

Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to the specified destination.

1.	Run INSTALL.EXE program from NETWARE-LITE as you would normally do.  When prompted to select "Network Interface Card Name", select the "OTHER CARDS" option.  Select setting for the I/O address (PORT) and interrupt level (INT) assigned during the CompuLAN installation.

2.	Create the NET.CFG file by selecting the options in the INSTALL program.

3.	Edit the NET.CFG file with a text editor that can save files in ASCII (text) format.  The NET.CFG file must contain the following KEY WORD lines:  KEY WORDS, FRAMES, and PROTOCOLS are defined in Appendix A.

Link support

BUFFERS  4  4202

Link driver FTX100EE

NODE [???]

PORT [???]

IRQ [??]

BITPAT0 E4PB0_6.BIT

BITPAT1 E4PB1_6.BIT

EARLY_INT [ON|OFF]

FRAME <Ethernet frame>

PROTOCOL <Ethernet protocol>

NETWARE DOS REQUESTER	(Only required for using VLM’s)

FIRST DRIVE = F		(Only required for using VLM’s)

Note: 	This driver will default to Ethernet 802.2 frame type, so the parameters for  <frame> and <protocol> are required.

4.	Load the following programs in this order:

>FTXCFG	(CompuLAN’s Initialization utility)

> LSL	(Novell's Link Support Layer)

> FTX100EE	(CompuLAN's CPL-100E driver)

> IPXODI	(Novell's IPX protocol stack)

> NETX	(Novell's redirector if using NETX)

Note: 	Make sure the file server and workstation frame types, protocol stacks and protocol numbers match.

Note:	Make sure the configuration in the NET.CFG file matches the FTX.CFG file.

III.  NOVELL PERSONAL NETWARE DOS v7.0

Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to the specified destination.

1.	Run the Novell SETUP utility selecting the CPL-100E as the primary network interface card.  Configure the primary interface card and save the settings as you exit.

2.	Edit the NET.CFG file with a text editor to make sure it is correct.  The NET.CFG file must contain the following KEY WORD lines:  KEY WORDS, FRAMES, and PROTOCOLS are defined in Appendix A.

Link support

BUFFERS  4  4202

Link driver FTX100EE

NODE [???]

PORT [???]

IRQ [??]

BITPAT0 E4PB0_6.BIT

BITPAT1 E4PB1_6.BIT

EARLY_INT [ON|OFF]

FRAME <Ethernet frame>

PROTOCOL <Ethernet protocol>

NETWARE DOS REQUESTER	(Only required for using VLM’s)

FIRST DRIVE = F		(Only required for using VLM’s)

3.	Load the following programs in this order:

>FTXCFG	(CompuLAN’s Initialization utility)

> LSL	(Novell's Link Support Layer)

> FTX100EE	(CompuLAN's CPL-100E driver)

> IPXODI	(Novell's IPX protocol stack)

> NETX	(Novell's redirector if using NETX)

Note:	Make sure the configuration in the NET.CFG file matches the FTX.CFG file.

Novell IPX DOS Workstation Drivers

This version of the IPX DOS Workstation driver is compatible with NetWare versions 3.xx and 4.xx file server software.  NetWare versions 3.xx and 4.xx must  be using IPX protocol bound to Ethernet 802.3 frame type.

FLOPPY DISKETTE INSTALLATION

Format a floppy diskette.  When format asks for a label, enter LAN_DRV_474. Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to this diskette.  Run WSGEN.  When the list of LAN drivers is displayed, press <INS>.  This will prompt you to insert a floppy diskette labeled LAN_DRV_474 in your drive.  Insert the disk labeled LAN_DRV_474 and the driver for the CPL-100E Network adapter will now be available.

HARD DISK OR NETWORK DRIVE INSTALLATION

After copying the files from the WSGEN diskette to the proper sub-directory on your hard disk or network drive, you must create a sub-directory named LAN_DRV_474.  Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to this subdirectory.  Continue with the workstation generation procedure as you would normally do, the driver for the CPL-100E Network adapter should appear as an available driver.  There is only one available configuration option which will be automatically selected by WSGEN:

0:  Use parameters in NET.CFG

MANUAL CREATION OF THE NET.CFG FILE FOR IPX WORKSTATIONS

You can create the NET.CFG file with a text editor that can save files in ASCII (text) format.  The NET.CFG file must contain the following KEY WORD lines:  KEY WORDS, FRAMES, and PROTOCOLS are defined in Appendix A.

Link support

BUFFERS  4  4202

Link driver IPX

NODE [???]

PORT [???]

IRQ [??]

BITPAT0 E4PB0_6.BIT

BITPAT1 E4PB1_6.BIT

EARLY_INT [ON|OFF]

FRAME EHHERNET_802.3

PROTOCOL IPX 0 ETHERNET_802.3

Note:	Run FTXCFG.COM prior to loading any network software.

Windows for Workgroups v3.11 Drivers

Following are the steps required to install the ODI DOS driver for use with Windows for workgroups v3.11.

Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to the specified destination.

1.	Install Microsoft Windows for Workgroups v3.11 without network support.

2.	Start Windows for Workgroups and select NETWORK SETUP from the NETWORK GROUP.

3.	Select  NETWORKS.

4.	Select INSTALL MICROSOFT NETWORK AND CLICK OK.

5.	Click on DRIVERS.

6.	Click on ADD ADAPTER.

7.	Highlight IPXODI  SUPPORT   DRIVER  (ETHERNET) and click OK.

8.	IPXODI  support driver (ETHERNET)  (ODI / NDIS3)  will appear under the network drivers box.

9.	Click on ADD PROTOCOL.

10.	Highlight IPX/SPX COMPATIBLE TRANSPORT WITH NETBIOS and click OK.

11.	Highlight IPX/SPX COMPATIBLE TRANSPORT WITH  NETBIOS and click on SET AS DEFAULT PROTOCOL.

12.	Click on CLOSE.

13.	Click on SHARING.

14.	Click on each option desired for your workstation then click OK.

15.	Click OK to exit NETWORK SETUP.

16.	Change the user name: WORKGROUP, and computer name:  as desired.

17.	Insert Windows for Workgroups diskettes as requested for file updates.

18.	SETUP will ask for the path to your NET.CFG file so it can be updated.

19.	Click on OK after Windows modifies the NET.CFG file.

20.	If NETWORK SETUP has modified your NET.CFG file the old file was saved as NET.001.

21.	Windows   notifies you that  "Your  AUTOEXEC.BAT and SYSTEM.INI  were modified, the old versions were renamed to AUTOEXEC.00x and SYSTEM.00x".  Click OK.

22.	SETUP gives you the option to reboot your computer or continue with Windows.  Click on CONTINUE.

23.	Edit the PROTOCOL.INI file to replace XXXX with CPL-100E.

24.	Edit the NET.CFG file to restore any KEY WORDS that may have been removed.  KEY WORDS and FRAMES are defined in Appendix A.

Link support

BUFFERS  4  4202

Link driver FTX100EE

NODE [???]

PORT [???]

IRQ [??]

BITPAT0 E4PB0_6.BIT

BITPAT1 E4PB1_6.BIT

EARLY_INT [ON|OFF]

FRAME ETHERNET_802.2

FRAME ETHERNET_802.3

FRAME ETHERNET_SNAP

FRAME ETHERNET_II

Note:	Make sure the configuration in the NET.CFG file matches the FTX.CFG file.

25.	Edit the AUTOEXEC.BAT file to add the following programs in this order:

>FTXCFG	(CompuLAN’s Initialization utility)

> LSL	(Novell's Link Support Layer)

> FTX100EE	(CompuLAN's CPL-100E driver)

>ODIHLP		(Microsoft’s ODI help utility)

26.	Reboot the computer and start Windows for Workgroups.

WINDOWS ‘95 ODI DRIVER

I.		PEER TO PEER

Following are the steps required to install the ODI network drivers for use with Windows ‘95 in a peer to peer application.

1.   	Install WINDOWS ‘95 per MICROSOFT’S instructions and do not enable networking.

2.	Install the CPL-100E Network Adapter as described in Chapter 1.

3.	Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to the specified destination.

4.   	From the directory where you installed the CompuLAN drivers run the following programs in this order:

>FTXCFG		(CompuLAN’s Initialization utility)

> LSL			(Novell's Link Support Layer)

> FTX100EE		(CompuLAN's CPL-100E driver)

5. 	Exit the DOS prompt window.

6.   	Click on the Start icon and proceed to the "Settings" and then "Control Panel".

7.   	In "Control Panel" double click on the "Network" icon , then click on "Add”.

8.	After clicking on "Add" double click on "Client" then choose "Microsoft" and select "Client for Microsoft Networks".

9.	From the network menu choose "Adapter" by double clicking on it, and select "Existing ODI Driver".

10.	You must assign a Workgroup name and Computer name by going back into the "Client" field and clicking on "Identification".

11. 	When done setting up the Workgroup/Computer information you will be prompted to insert diskettes.  when prompted for a "NetWare DOS Diskette" type in the path of the subdirectory you copied the network drivers to. When done Windows ‘95 will ask if you want to reboot answer NO.

12. 	Go to the DOS prompt window and edit AUTOEXEC.BAT to include the following commands in this order:

>FTXCFG	(CompuLAN’s Initialization utility)

> LSL	(Novell's Link Support Layer)

> FTX100EE	(CompuLAN's CPL-100E driver)

>ODIHLP		(Microsoft’s ODI help utility)

13.	Edit the NET.CFG file to restore any KEY WORDS that may have been removed.  KEY WORDS, FRAMES and PROTOCOLS are defined in Appendix A.

Link support

BUFFERS  4  4202

Link driver FTX100EE

NODE [???]

PORT [???]

IRQ [??]

BITPAT0 E4PB0_6.BIT

BITPAT1 E4PB1_6.BIT

EARLY_INT [ON|OFF]

FRAME <Ethernet frame>

PROTOCOL <Ethernet protocol>

Note:	Make sure the configuration in the NET.CFG file matches the FTX.CFG file.

14. 	When done editing exit the DOS prompt window and return to the "Network" icon to setup "File/Printer Sharing" this will enable your system to share files etc.  When done Windows ‘95 will ask if you want to reboot answer YES.

15. 	The frame type and maximum connections parameters need to be set.  Do this in the Network Setup by clicking on the "IPX/SPX Compatible" section in the network menu. and clicking on "Properties” and then selecting “Advanced”. change the "Maximum Connections” to a setting to enable other computers to log into yours.  Also change the frame type to match the one you are using. (Same as the one you selected in the INSTALL).

16. 	When done run the Explorer program and then click on “C:” .

17. 	Click on "File" and then choose "Properties" select "Sharing" to allow other systems to be able to access you system.

II.  ODI WORKSTATION

netbios (CBIS) DRIVERS

Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to the specified destination.

SYNTAX:

NXCD100E [/LP:???] [/LI:?] [/LE:{ON|OFF}]

/LP:???  	Specifies  the  I/O  port  used  by  the CPL-100E                         adapter.  Use 300, 310, 320 or 330 depending on jumper settings. Default is 300.

/LI:?  	Specifies the IRQ level used by the CPL-100E                         adapter (in HEX).  Use 9, A, B, C for interrupt levels 9, 10, 11, 12,  respectively.

/LE:{ON|OFF}  	Specifies the EARLY_INT operation mode the driver will be using.  Should be set to “ON” if no ISA Network Adapters will be installed in the network.

Note:	The parameters above must match those found in the FTX.CFG file.

Note:	CompuLAN Corporation netbios emulator supports up to 32 sessions with  the versions  included in this distribution floppy.

LANTASTIC INSTALLATION

1.	Install the Lantastic software and choose “Artisoft Compatible” for the adapter type.

2.	Edit STARTNET.BAT to include the NXCD100E.EXE driver with the proper settings in the location specified.

3.		Edit STARTNET.BAT to delete the line:  AILANBIO@STARTNET.CFG.

Note:	 Run the FTXCFG.COM program before running the network software.

NET/OS ODI Workstation Drivers

Run the CompuLAN INSTALL program to create and copy the required Network Adapter files to the specified destination.

Install the NET/OS software following the instructions provided.

Load the following programs in this order:

>FTXCFG	(CompuLAN’s Initialization utility)

> LSL	(Novell's Link Support Layer)

> FTX100EE	(CompuLAN's CPL-100E driver)

> IPXODI	(Novell's IPX protocol stack)

> NETX	(Novell's redirector if using NETX)

MANUAL CREATION OF THE NET.CFG FILE FOR ODI WORKSTATIONS

You can create the NET.CFG file with a text editor that can save files in ASCII (text) format.  The NET.CFG file must contain the following KEY WORD lines:  KEY WORDS, FRAMES, and PROTOCOLS are defined in Appendix A.

Link support

BUFFERS  4  4202

Link driver FTX100EE

NODE [???]

PORT [???]

IRQ [??]

BITPAT0 E4PB0_6.BIT

BITPAT1 E4PB1_6.BIT

EARLY_INT [ON|OFF]

FRAME <Ethernet frame>

PROTOCOL <Ethernet protocol>

NETWARE DOS REQUESTER	(Only required for using VLM’s)

FIRST DRIVE = F		(Only required for using VLM’s)

Note:  Make sure file server and workstation frame types, protocol stacks and protocol numbers match.

Note:	Make sure the configuration in the NET.CFG file matches the FTX.CFG file.

Chapter 3    DIAGNOSTICS AND UTILITIES

SETUP:  THE CONFIGURATION UTILITY

SETUP.EXE is a configuration and diagnostic program located in the \UTILITY subdirectory of the CPL-100E distribution diskette.  SETUP will configure the EISA adapter and create a file called FTX.CFG to save the configuration for future use.

operating procedure for the SETUP utility.

1.	Install the CPL-100E network adapter in your system with a terminator plugged into one of the RJ-45 connectors.  Do not attach any cables.

2.	Type \..path..\SETUP to start setup.  SETUP will perform a diagnostic check on the adapter card and prompt you for configuration information.  Follow the directions to configure the adapter.  KEY WORD definitions are located in Appendix A.

3.	Select the option to have SETUP create a file called FTX.CFG to store the configuration information.

4.	The EISA adapter card can be initialized on boot-up by running the FTXCFG.COM program located in the \UTILITY subdirectory of the CPL-100E distribution diskette to install the configuration stored in the FTX.CFG file.

NETSIM: THE NETWORK SIMULATOR

NETSIM is a complex Network simulator that allows simulation of real-world type Network traffic patterns and is located in the \UTILITY subdirectory of the CPL-100E distribution diskette..

NETSIM should be used when the network operating system keeps displaying communication errors or you are unable to establish a valid network link to a node.

NOTE:  NETSIM must be run without any Network software loaded.  Also avoid memory resident programs

operating procedure for the NETSIM utility.

1.	Initialize the CPL-100E network adapter by running either SETUP.EXE or FTXCFG.COM.  See above for details.

2.	Connect  cables to the other stations you wish to NETSIM with.

3.	NETSIM  must  be run without NETBIOS, Novell or any other driver software running.

4.	Type \..path..\NETSIM to start NETSIM and follow the directions to send packets of data to other stations.  (Both stations must be running NETSIM.)

5.	Multiple nodes can run NETSIM at the same time, creating any traffic pattern desired.

6.	Print out and use the included  NETSIM data sheet to record and calculate  errors and error percentages for each node in the network.

5.	Run the test  for at least 10 minutes to accumulate enough data about the simulated NETWORK.

NETSIM DISPLAY information

Test packets can be short (512 bytes) or long (4 KBytes) and separate counters and totals are kept for each statistic.

TRANSMITTED:

GOOD:  This is the number of packets sent without errors

RCVR NOT READY:  The packet  was rejected by the receiver because it wasn't ready to receive  it.   Slower receiving computers will cause a higher count here. This is not a problem.

NOISE:  The packet  was not transmitted successfully due to noise problems  or  real  collisions (two nodes sending at the same time).  If this count is higher  than  20% of the GOOD count then there are cable problems or a bad network board at this station.

ABORTED:  A  packet transmission was re-translated 40 times and every time a "RCVR NOT READY" was detected, or the packet was re-transmitted 10 times and each time there was a noise error.  This  could  be  caused  by  sending  to  a  station that is not running NETSIM and is not ready to receive messages or critical cable conditions are causing these errors.  After an ABORT is recorded a new packet is sent.

TXTOTAL:  This counter will accumulate all Messages sent GOOD and ABORTED:    Total  counters  are  also  kept for short and long packets.

RECEIVE:

GOOD:  Messages received without errors.

DETECTED ERRORS:  These errors are normal and unless excessive, will not cause network problems

COLLISION:  The  received  packet  was rejected due to a real collision resulting from two nodes sending at the same time.

NOISE <xK:  The  received  packet  was  rejected  due to noise on the cable, a bad cable or cable connection, or a bad driver on the sending computer's network  board.  xK represents the point of detection of the error (i.e. <1k, error detected before the 1st KBytes where received).

SOFT ERRORS:  These errors will cause performance loss because the packets rejected here will need to be re-transmitted after a time-out period from the sender.

TOO SHORT:  The packet length count exceeded the actual received byte count.  No error bit was detected on transmission.  If this error is presented  continuously, the station receiving  packets might have Ready Signal Bus timing problems.  Refer to WAIT TIMING FIX for further explanation.  If this condition is detected, the check sum test for HARD ERROR is not performed.

TOO LONG:  The  packet  length  count  was  less  than the actual received byte count.  No error bit was detected on transmission.  If this error is presented continuously, the station receiving packets might have Ready Signal Bus timing problems.  Refer to WAIT TIMING FIX for further explanation.  If this condition is detected, the check sum test for HARD ERROR is not performed.

LONGITUDINAL PARITY:  A longitudinal parity check is done for every message received.   Any error detected will be tallied here.  This error could be caused by faulty cable or cable too long.

BUS NOISE:  Erratic operation of FIFO chip was detected.  This can be due to noisy BUS signals, or faulty FIFO chips.  Check your power connections and board installation.

HARD ERROR:  No errors where detected by the network hardware or driver software but the data integrity errors where detected.

CHECKSUM:  Checksum calculated with packet contents does not match checksum calculated before transmission.  This may be due to computer  memory  problems, I/O incompatibility or failed network hardware.  This indicates a problem that MUST BE CORRECTED before the network is put to use !

DATA SHEET FOR NETSIM v1.20a

TRANSMITTED					5 nodes    10 nodes

Rcvr Not Ready                  		 > 10%     > 20%

Noise                            			 > 10%     > 15%

c        Aborted                          			    any          any

RECEIVED

Detected:  Collision                		 > 10%     > 15%

Noise < 1K                     		 > 10%     > 15%

Noise < 2K                     		 > 10%     > 15%

Noise < 3K                     		 > 10%     > 15%

Noise < 4K                     		 > 10%     > 15%

c        Soft Errors:  Bad Header            		 > 1%       > 1%

c           	Too Short                      		 > 5%       > 8%

c           	Too Long 		                       > 5%       > 8%

c           	Longitudinal Parity    	            > 1%       > 1%

c           	Bus Noise                      		    any          any

c         Hard Errors: Checksum              	 	    any          any

Node _________ Sending to Node _________

| Transmitted | Short | Long | Total |
| --- | --- | --- | --- |
| Good | _____________ | _____________ | _____________ |
| Rcvr Not Ready | (____________) | (____________) | (____________) |
| Noise | (____________) | (____________) | (____________) |
| Aborted | _____________ | _____________ | _____________ |
| TxTOTAL | _____________ | _____________ | _____________ |

| Received | Short | Long | Unknown | Total |
| --- | --- | --- | --- | --- |
| Good | _________ | _________ |  | _________ |
| Detected: Collision | (________) | (________) | (________) | (________) |
| Noise < 1K | (________) | (________) |  | (________) |
| Noise < 2K |  | (________) |  | (________) |
| Noise < 3K |  | (________) |  | (________) |
| Noise < 4K |  | (________) |  | (________) |
| Soft Errors: Bad Header |  |  | (________) | (________) |
| Too Short | _________ | _________ |  | _________ |
| Too Long | _________ | _________ |  | _________ |
| Longitudinal Parity | _________ | _________ |  | _________ |
| Bus Noise | _________ | _________ |  | _________ |
| Hard Errors: Checksum | _________ | _________ |  | _________ |
| RxTOTAL | _________ | _________ | (________) | _________ |

TECHNICAL SUPPORT

CompuLAN is responsible for the manufacturing, sales, and marketing of 100 FastNET connectivity products.  Technical support is provided through CompuLAN’s Technical Support Department.

Please have the following information ready before calling:

Adapter configuration

Other adapters installed in the system

Jumper settings and interrupt settings

Network information

Network operating system and version

Network drivers being used

Computer information

Manufacturer, model number, operating system

Bios and bios settings

History of the Problem

The symptoms of the problem.

Did the adapter work for a period of time or fail immediately after installation?

Was any workstation option changed (hardware or software) prior to the problem appearing?

RETURNING DEFECTIVE PRODUCTS

If you have a defective adapter, contact CompuLAN’s Technical Support Group.  They will verify if the adapter is defective and have the customer Service Department contact you to arrange an adapter replacement.

APPENDIX A:    DEFINITIONS

I.  KEY WORDS USED IN THE NOVELL AUTOEXEC.NCF FILE:

FRAME=<ethernet frame>	THIS IS THE FRAME TYPE YOU WISH TO USE.  FRAMES SUPPORTED ARE:

ETHERNET_802.2

ETHERNET_802.3

ETHERNET_SNAP

ETHERNET_II

PROTOCOL <ethernet	THIS IS THE COMMUNICATION PROTOCOL YOU

protocol>	WISH TO USE.   PROTOCOLS SUPPORTED ARE:

IPX

IP (TCP/IP)

ARP

RARP

NODE=[???]		DECIMAL  [3]

THIS IS THE NODE NUMBER FOR THE STATION.  EACH STATION MUST HAVE A UNIQUE NODE NUMBER.  VALID NUMBERS ARE:  1 THRU 254.

PORT=[???]		HEX  [4]

THIS IS THE I/O PORT ADDRESS SELECTED FOR THE ADAPTER CARD.  VALID ADDRESSES ARE:  1000, 2000, 3000, 4000, 5000, 6000, 7000, 8000.

INT=[?]				HEX  [1]

THIS IS THE INTERRUPT LEVEL SELECTED FOR THE ADAPTER CARD.  VALID INTERRUPTS ARE:  9, A, B, C.  THESE EQUATE TO DECIMAL VALUES:  9, 10, 11, 12  USED IN THE FTX.CFG AND NET.CFG FILES.

EARLY_INT=[ON|OFF]	THIS ALLOWS THE EISA NETWORK ADAPTER TO COMMUNICATE WITH 16 BIT ISA NETWORK ADAPTERS.  SET TO “ON” IF NO ISA NETWORK ADAPTERS ARE INSTALLED IN THE NETWORK.

II.  KEY WORDS USED IN THE FTX.CFG AND NET.CFG FILES:

BOARD [????]	THIS IS THE BOARD TYPE.  VALID BOARD TYPE IS:  EISA.

FRAME <ethernet frame>	THIS IS THE FRAME TYPE YOU WISH TO USE.  FRAMES SUPPORTED ARE:

ETHERNET_802.2

ETHERNET_802.3

ETHERNET_SNAP

ETHERNET_II

PROTOCOL <ethernet	THIS IS THE COMMUNICATION PROTOCOL YOU

protocol>	WISH TO USE.  PROTOCOLS SUPPORTED ARE:

IPX

IP (TCP/IP)

ARP

RARP

NODE [???]		DECIMAL  [3]

THIS IS THE NODE NUMBER FOR THE STATION.  EACH STATION MUST HAVE A UNIQUE NODE NUMBER.  VALID NUMBERS ARE:  1 THRU 254.

PORT [???]		HEX  [4]

THIS IS THE I/O PORT ADDRESS SELECTED FOR THE ADAPTER CARD.  VALID ADDRESSES ARE:  1000, 2000, 3000, 4000, 5000, 6000, 7000, 8000.

IRQ [??]				DECIMAL  [2]

THIS IS THE INTERRUPT LEVEL SELECTED FOR THE ADAPTER CARD.  VALID INTERRUPTS ARE:  9, 10, 11, 12.  THESE EQUATE TO THE HEX VALUES:  9, A, B, C USED IN THE NOVELL AUTOEXEC.NCF FILE.

BITPAT0  <bit pattern>	THIS  IS  THE  BIT  PATTERN USED TO CONTROL

BITPAT1    files>	COMMUNICATIONS WITH THE COMPUTERS EISA 	BUS.  THE FILES CONTAINING THE BIT PATTERNS ARE:  E4PB0_6.BIT AND E4PB1_6.BIT.

EARLY_INT [ON|OFF]	THIS ALLOWS THE EISA NETWORK ADAPTER TO COMMUNICATE WITH 16 BIT ISA NETWORK ADAPTERS.  SET TO “ON” IF NO ISA NETWORK ADAPTERS ARE INSTALLED IN THE NETWORK.

III.  SYNTAX FOR protocols and matching frames

| PROTOCOL | FRAME TYPE | HEX NUMBER |  |
| --- | --- | --- | --- |
| IPX | ETHERNET_802.2 |  | e0 |
|  | ETHERNET_802.3 |  | 0 |
|  | ETHERNET_SNAP |  | 8137 |
|  | ETHERNET_II |  | 8137 |
| IP (TCP/IP) | ETHERNET_SNAP |  | 800 |
|  | ETHERNET_II |  | 800 |
| ARP | ETHERNET_SNAP |  | 806 |
|  | ETHERNET_II |  | 806 |
| RARP | ETHERNET_SNAP |  | 8035 |
|  | ETHERNET_II |  | 8035 |

NOTE:  The FRAME statement must be prior to the PROTOCOL statement as indicated below.

FRAME <ethernet frame>

PROTOCOL  <protocol> <hex number> <frame>

EXAMPLES:

IPX		FRAME ETHERNET_802.2

PROTOCOL IPX e0 ETHERNET_802.2

IP		FRAME ETHERNET_II

PROTOCOL IP 800 ETHERNET_II

ARP		FRAME ETHERNET_SNAP

PROTOCOL ARP 806 ETHERNET_SNAP

RARP		FRAME ETHERNET_II

PROTOCOL RARP 8035 ETHERNET_II

APPENDIX B:    CABLING

CompuLAN’S 100 FastNET products are configured to use 4 twisted pairs with RJ45 modular connectors wired according to EPA T568A.  The cables use the following connections:

1         2          3           4        5                       6              7       8

1       2		         3        4             5        6              7        8

It is vital that TWISTED PAIRS be connected to matching pins on the modular plug (e.g.: 1&2,4&5,3&6, and 7&8).  If the wires are not paired correctly the networks with long overall cable length will have communication errors.

PAIR   WIRE COLOR 	 MARKER COLOR     PIN   T/R             12345678

-------  	-------------------- 	 -----------------------  	  -----    -----

1       	WHITE      	ORANGE     	    1      T3    	|    | | | | | | | |   	|

ORANGE     		WHITE        	    2      R3  		|    | | | | | | | |   	|

--------------------------------------------------------------------------		|                     	|

2        	WHITE       		GREEN       	    3      T2   			|              	|

GREEN       		WHITE         	    6      R2   		|  	           	|

---------------------------------------------------------------------------		| ---------------- 	|

3        	BLUE        		WHITE         	    4      T1   		|	           	|

WHITE      	 	BLUE         	    5      R1   		| RJ45  PLUG	|

---------------------------------------------------------------------------		|	              	|

4        	WHITE      	 	BROWN         	    7      T4     	     ------------------

BROWN      	 	WHITE         	    8      R4       		  | | | | | | | |

| | | | | | | |

| | | | | | | |

(Plug shown with pins facing out)                                          4 Pair Cable