KTI 100VG LAN Adapter NDIS3 Drivers

Installation for Windows 95

===================================

In this driver release, the NDIS3 driver is provided for the Windows 95

installation. The NDIS3 driver, KVG95ND3.VXD, is located in \WIN95

subdirectory.

The information covers the following topics:

* KTI DP16I/DP16I2 100VG NDIS Driver Installation Guide for Windows 95

* KTI DP32PCI/DP32PCI2 100VG NDIS Driver Installation Guide for Windows 95

* KTI DP32E/DP32E2 100VG NDIS Driver Installation Guide for Windows 95

-----------------------------------------------------------------------------

KTI DP16I 100VG NDIS

Driver Installation Guide

for Windows 95

==========================

Introduction

-------------

The following instruction shows how to install the KTI DP16I/DP16I2 100VG ISA

LAN Adapter NDIS driver for Microsoft's (R) Windows 95.

Installation procedure

----------------------

The following installation procedure is assumed that Windows 95 has been

installed in your computer without any network drivers installed.

1. Turn your computer off.

2. Install the KTI DP16I/DP16I2 100VG ISA LAN Adapter into your computer.

3. After the adapter installation, start your Windows 95 system.

4. Select <My Computer> icon on the [Main] window

5. Select <Control Panel> icon on [My Computer] window.

6. Select <Network> icon on [Control Panel] window.

7. Select <Add> button on [Network] window.

8. Highlight <Adapter> component on [Network Component Type] window, Select

<Add> button.

9. Select <Have Disk> button to load Other Manufactures Drivers. You will be

prompted for the path where the driver is located.

10. Insert the KTI 100VG LAN Adapter Driver Diskette in drive A:\.

11. Type A:\WIN95. Select <OK> button.

12. The [Select Network Adapters] window will appear.

13. Highlight the <KTI DP16I/DP16I2 100VG ISA LAN Adapter>. Select <OK> button.

14. Windows 95 system will install several different components:

Client for Microsoft Networks

Client for NetWare Networks

KTI DP16I/DP16I2 100VG ISA LAN Adapter

IPX/SPX-compatible Protocol

NetBEUI

14. Select <OK> on the [Network] window.

15. The [KTI DP16I/DP16I2 100VG ISA LAN Adapter Properties] window will appear.

Set the hardware settings for Interrupt Request and Input/Output Port.

NOTE: There may be a option for MEMORY ADDRESS, if you choose Shared Memory

Mode in the KVGSETUP.EXE (Software Setup Program for DP-16I), you

will also need to match this setting to that of your adapter.

You may use the mouse to click and display the available options of

Interrupt, Base I/O port, and Memory Address on screen. Select the correct

one for your setting of Interrupt, I/O base address, and Memory Address

on the adapter.

NOTE:  An asterisk (*) will appear if Windows 95 detects any conflicts

with any other hardware devices.

13. Select <OK> button.

14. System will begin copying files from the Windows 95 installation disks,

or CD-ROM.

15. A <System Settings Change> window will appear. Select <Yes> button to save

the settings and restart windows.

16. Your Windows 95 system is now configured to run the Networking functions.

NOTE: We recommend that you verify that your 100VG Adapter and Windows 95

Networking software have been properly configured.

If system hangs on the Windows 95 logo, you need to connect your

cable to the Adapter.

17. After restarting Windows 95, select <My Computer> icon on the [Main]

window.

18. Select <Control Panel> icon on the [My Computer] window.

19. Select <System> icon on the [Control Panel] window.

20. The [System Properties] window will appear, select <Device Manager> tab.

21. Select the <Network Adapters> device, and highlight the <KTI DP-16I 100VG

ISA LAN Adapter>.

NOTE: If there is a warning icon (Exclamation !, or a Not sign / ) in

front of the <KTI DP-16I 100VG ISA LAN Adapter> device, the adapter

has not been configured correctly or is not operating correctly.

We recommend that you remove the driver, and reboot the system.

Check to make sure that the settings you are using for the adapter match

the settings you are assigning in the Windows 95 system. Then follow the

above procedure to <Add> the 100VG LAN Adapter.

22. Select the <Properties> button, the [KTI DP-16I 100VG ISA LAN Adapter

Properties] window will appear.

23. Verify that <Device Type> is "Network Adapters", <Manufactures> is

"KATRON", and that "This device is working properly" under the

<Device Status> section.

24. Select the <Resources> tab on the [KTI DP16I/DP16I2 100VG ISA LAN Adapter

Properties] window. Verify that the "Interrupt Request", "Input/Output

Range", and "Memory Address" match the settings of you Adapter, and that

"There are no conflicts" under the <Conflicting Device List>.

25. Now, your Windows 95 system is ready to run with your adapter.

KTI DP32PCI 100VG NDIS

Driver Installation Guide

for Windows 95

==========================

Introduction

-------------

The following instruction shows how to install the KTI DP32PCI/DP32PCI2 100VG

ISA LAN Adapter NDIS driver for Microsoft's (R) Windows 95.

Installation procedure

----------------------

The following installation procedure is assumed that Windows 95 has been

installed in your computer without any network drivers installed.

1. Turn your computer off.

2. Install the KTI DP32PCI 100VG PCI LAN Adapter into your computer.

3. Turn your computer on.

4. A "New Hardware Found" dialog appears and prompts you for the selection of

PCI Ethernet Controller. You can select the "Dirver from disk provided by

hardware manufaturer" and press the OK button.

5. Enter "A:\WIN95" when prompts for the WIN95 drivers and inserts the KTI

100VG driver diskette in drive A.

6. Enter your Workstation and Workgroup names accordingly when prompts to

next. You'll have to have your WIN95 CD-ROM ready in this section for any

necessary networking files.

7. When it finishes copying, your Windows 95 system is ready to run with your

100VG adapter.

KTI DP32E 100VG NDIS

Driver Installation Guide

for Windows 95

==========================

Introduction

-------------

The following instruction shows how to install the KTI DP-32E 100VG NDIS

driver for Microsoft's (R) Windows 95.

Installation procedure

----------------------

The following installation procedure is assumed that Windows 95 has been

installed in your computer without any network drivers installed.

1.  Turn off your computer.

2.  Install the KTI DP32E 100VG EISA LAN Adapter into your computer.

3.  You may need to run the EISA configuration programs that comes with

your system and use the KVGSETUP program to setup the DP-32E to the same

settings as in the EISA configuration program.

After the adapter installation, start your Windows 95 system.

4.  Windows 95 will detect a EISA Adapter in slot #X, and the [New Hardware

Found] window will appear. Select the <Driver form disk provided by

hardware manufacturer> option. Select <OK> button.

5.  The [Install From Disk] window appears, Type A:\WIN95 as the path.

6.  Insert the KTI DP-32E 100VG EISA LAN Adapter Driver diskette in drive A:.

7.  Select <OK> button.

8.  A "Select Deivce" windows will appears, please select the "KTI DP32E EISA

LAN Adapter".

9.  System will begin copying files from the Driver diskette, and then ask for

Windows 95 installation disks, or CD-ROM.

10.  Now, your Windows 95 system is ready to run with your adapter.