*** ODI Driver Installation for Netware DOS Client ***

1.OVERVIEW

2.KVGODI.COM KEYWORD DEFINITION

3.KVGODIBM.COM KEYWORD DEFINITION

4.NETWARE DOS CLIENT INSTALLATION

OVERVIEW

========

This documentation describes the functions and the defined keywords used in the

ODI drivers (KVGODI.COM and KVGODIBM.COM) for DOS.

Both drivers support the adapter configurations respectively as follows:

-------------------------------------------------------------------------

ODI Driver       ISA adapter             EISA adapter	    PCI adapter

-------------------------------------------------------------------------

KVGODI.COM     I/O-, memory-mapped   I/O-, memory-mapped    I/O-mapped

KVGODIBM.COM      Not supported         Bus-Master mode     Not supported

-------------------------------------------------------------------------

To simplify the explanation, the following notations are defined for this

documentation:

ISA adapter    : The KTI 100VG ISA LAN adapter DP16I

EISA adapter   : The KTI 100VG EISA LAN adapter DP32E

PCI adapter    : The KTI 100VG PCI LAN adapter DP32PCI

Auto scan      : The driver identifies an object automatically.

Value          : The value specified for a specific parameter line in

NET.CFG file.

(none)         : The specific parameter line is not present in NET.CFG file.

EEPROM         : Adapter'S EEPROM in which the adapter configuration are stored

EEPROM setting : The configuration setting stored in EEPROM via KVGSETUP

setup program.

KVGODI.COM KEYWORD DEFINITION FOR NET.CFG

-----------------------------------------

Filename   :  KVGODI.COM

Description:  100VG DOS client ODI driver supports I/O-mapped and memory-mapped

operation for the ISA and EISA adapters. It also supports

I/O-mapped operation for PCI adapter.

Caution    :  This driver does not support the EISA adapter with Bus-Master

mode. Use KVGODIBM.COM instead.

DriverName :  KVGODI

Parameter   Usage

-------------------------------------------------------------------------------

PORT        Specify the Base I/O address for the ISA adapter

Value  : Use the value as the base I/O address for the adapter

(none) : Auto scan

The valid values are: 200,240,280,2C0,300,340,380, and 3C0.

-------------------------------------------------------------------------------

SLOT        Specify the EISA slot in which the EISA adapter is installed.

Value  : Use the value as the slot number where the adapter

is installed

(none) : Auto scan

The valid values are : 1 - 15.

-------------------------------------------------------------------------------

INT         Specify the interrupt (IRQ) used by the ISA or EISA adapter.

Value  : Use the value as the interrupt for the adapter

(none) : Use the interrupt setting in EEPROM

The valid values are : 3,4,5,6,7,9,10,11,12,15.

-------------------------------------------------------------------------------

PCIBOARD    Specify the board number of the PCI adapter if multiple PCI

adapters are installed in one system.

Value : 1 - 4

-------------------------------------------------------------------------------

IRQTYPE     Specify the interrupt trigger type for the EISA adapter

EDGE   : edge trigger type

LEVEL  : level trigger type

(none) : use factory default for the adapter

This parameter is ignored for the ISA adapter.

-------------------------------------------------------------------------------

OPMODE      Specify operating mode for the adapter

IO     : Use I/O-mapped mode for the adapter

MEMORY : Use memory-mapped mode for the adapter. (See MEM.)

(none) : See MEM description.

-------------------------------------------------------------------------------

MEM         Specify the memory-mapped address for the memory-mapped mode.

The valid addresses are : c0000 - ee000 ( 2000 increment)

The following table shows how the driver decides the memory-mapped

address according to the combination of OPMODE and MEM.

OPMODE      MEM             Driver uses

----------  --------------  ----------------------------------

MEMORY      valid address   memory-mapped mode at the address

(none)      valid address   memory-mapped mode at the address

MEMORY      (none)          memory-mapped mode at EEPROM setting

IO          valid address   I/O-mapped mode, ignore MEM

IO          (none)          I/O-mapped mode, ignore MEM

(none)      (none)          use EEPROM settings (mode and address)

If the memory-mapped address setting in EEPROM is over 1M bytes or

not in the valid range, the driver will use D0000 as the

memory-mapped address.

-------------------------------------------------------------------------------

PRIORITY    Specify the priority level for the transmission

NORMAL : Normal priority transmission

HIGH   : High priority transmission

(none) : Normal priority transmission

Before using HIGH option for this parameter, contact your

LAN administrator first to obtain authorization.

-------------------------------------------------------------------------------

FRAME       Specify the frame types. (multiple selections)

The valid values are : Ethernet_802.2

Ethernet_802.3

Ethernet_II

Ethernet_SNAP

-------------------------------------------------------------------------------

Note: All keywords and values are caps insensitive.

KVGODIBM.COM KEYWORD DEFINITION FOR NET.CFG

-------------------------------------------

Filename   :  KVGODIBM.COM

Description:  100VG DOS client ODI driver supports Bus-Master mode

operation for the EISA adapter.

DriverName :  KVGODIBM

KEYWORDS    Definition / Description

-------------------------------------------------------------------------------

SLOT        Specify the EISA slot where the EISA adapter is installed.

Value  : Use the value as the slot number where the adapter

is installed

(none) : Auto scan

The valid values are : 1 - 15.

-------------------------------------------------------------------------------

INT         Specify the interrupt (IRQ) used by the adapter.

Value  : Use the value as the interrupt for the adapter

(none) : Use the interrupt setting in EEPROM

The valid values are : 3,4,5,6,7,9,10,11,12,15.

-------------------------------------------------------------------------------

IRQTYPE     Specify the interrupt trigger type for the EISA adapter

EDGE   : edge-triggered type

LEVEL  : level-triggered type

(none) : use factory default for the adapter

This parameter is ignored for the ISA adapter.

-------------------------------------------------------------------------------

PRIORITY    Specify the priority level for the transmission

NORMAL : Normal priority transmission

HIGH   : High priority transmission

(none) : Normal priority transmission

Before using HIGH option for this parameter, contact your

LAN administrator first to get authorization.

-------------------------------------------------------------------------------

FRAME       Specify the frame types. (multiple selections)

The valid values are : Ethernet_802.2

Ethernet_802.3

Ethernet_II

Ethernet_SNAP

-------------------------------------------------------------------------------

Note: All keywords and values are caps insensitive.

Netware DOS Client Installation

-------------------------------

This subdirectory has been properly structured for Netware DOS Client software

installation. Just enter "A:\DOS" for driver disk drive prompt during client

software installation.

In the \DOS directory, there is one sample NET.CFG file for manually

installing your ODI driver. In the file, all parameter lines are marked with

"#" except the Link driver lines. That means the driver will configure the

adapter automatically using the sample NET.CFG file.

Link Driver KVGODI

#       PORT            300

#       SLOT            5

#       INT             3

#       IRQTYPE         EDGE

#       IRQTYPE         LEVEL

#       OPMODE          IO

#       OPMODE          MEMORY

#       MEM             D0000

#	PCIBOARD	1

#       PRIORITY        NORMAL

#       PRIORITY        HIGH

#       FRAME           Ethernet_802.3

#       FRAME           Ethernet_802.2

#       FRAME           Ethernet_SNAP

#       FRAME           Ethernet_II

NetWare DOS Requester

FIRST NETWORK DRIVE = F

Link Driver KVGODIBM

#       SLOT            5

#       IRQTYPE         EDGE

#       IRQTYPE         LEVEL

#       PRIORITY        NORMAL

#       PRIORITY        HIGH

#       FRAME           Ethernet_802.3

#       FRAME           Ethernet_802.2

#       FRAME           Ethernet_SNAP

#       FRAME           Ethernet_II

With no keyword specified in the NET.CFG file, the driver will search the

adapter automatically and use the EEPROM settings for the adapter.

Ethernet_802.2 frame type and normal priority are used.

You may remove the "#" mark in any specific parameter line and specify a valid

value for it. In principle, the driver gives precedence to the parameter value

and ignores the adapter EEPROM setting.

The batch file for client startup using ODI loading procedure (not VLM) and

the sample NET.CFG can be configured manually as below:

LSL

KVGODI                  ; or KVGODIBM

IPXODI

NETX

Netware 3.12/4.0x/4.1 Client

----------------------------

Netware 3.12/4.0x/4.1 use the DOS Requester as a replacement for the shell,

NETX.EXE. The DOS Requester is a set of files loaded into the client

station's memory. The Requester has two components:

. Manager (VLM.EXE)

. Virtual Loadable Modules (VLMs)

After client installation, DOS client configuration files are modified as

follows:

* CONFIG.SYS ---- The follwoing command line was included in CONFIG.SYS.

LASTDRIVE=Z

* AUTOEXEC.BAT -- The following batch command line was included in

AUTOEXEC.BAT to load the networking files from \NWCLIENT.

@CALL C:\NWCLIENT\STARTNET

* STARTNET.BAT -- It will load ODI environment and load VLM.EXE. The command

include the following:

@ECHO OFF

C:

CD \NWCLIENT

SET NWLANGUAGE=ENGLISH

LSL

KVGODI                  ; or KVGODIBM

IPXODI

VLM

* NET.CFG ------- The following example shows the NET.CFG file after the

client installation utility has been executed.

Link Driver KVGODI         ; or KVGODIBM

Netware DOS Requester

FIRST NETWORK DRIVE = F

The Netware DOS Requester is the preferred DOS Client connection softwares.

Please refer to Netware 3.12/4.0x/4.1 manuals for more information.