AllMouse Driver User Guide:

===============================================================================

<<<<<<<<<<<<< Install mouse driver for DOS system >>>>>>>>>>>>>>

Before you begin to use mouse, you must run a program

called 'INSTALL' to install Allspirit mouse.

INSTALL will ask you to specify whether you want to install

mouse driver on a hard disk.

To run INSTALL:

A:\>INSTALL <ENTER>

Use Mouse Driver Program:

ALLMOUSE.COM      -- Mouse Driver for DOS.

Type "ALLMOUSE/?" in the command line to demonstrate all the option and their

commands if you need help,then select the correct configuration by typing

it in the command line.The help menu will be shown as follows:

/1    - Use serial port COM1. port = 3F8, IRQ = 4

/2    - Use serial port COM2. port = 2F8, IRQ = 3

/3    - Use serial port COM3. port = 3E8, IRQ = 4

/4    - Use serial port COM4. port = 2E8, IRQ = 3

/Np,i - Use hexadecimal I/O port (p) and Decimal IRQ level (i).

/P    - Force Use Mouse Systems Mode.

/Dn   - Set times of dynamic resolution (n = 1 - 9).

/OUT  - Unload mouse driver and release memory.

/?,H  - For help display.

Mouse Test & Setting Program:

ALLTEST.EXE     -- Text Mode Test program.

Under Text mode,there is a status line on the bottom of screen to show user

the operation condition of Mouse button and Mouse Driving mode,beside,on

upper right side of screen will show out the position of current cursor.

ALLSET.EXE      -- Graphic Mode Test & Setting.

Under Graphic mode, You may change the resolution from this screen. The

default resolution setting is"5". You may toggle the mouse mode using the

slide switch in ms-mouse or pc-mouse.

===============================================================================

Install Allspirit mouse driver for Microsoft Windows 3.x

OEMSETUP.INF -- additional information about file SETUP.INF.

AMOUSE.DRV -- mouse driver for Windows 3.x.  With this driver you can use

a Microsoft mouse or a PS/2 mouse along with your computer.

Contents:

a. Files for Microsoft Windows 3.x in this diskette

b. Set Up This Mouse Under Microsoft Windows 3.x

Introduction:

This file is helping you to set up this mouse so you can use it

under Microsoft Windows 3.x environment.  Information in this file is the

latest, so if there is any conflicting to the Mouse User's Manual, you

should believe this file.

Files for Microsoft Windows 3.x in this diskette

You may use Microsoft Windows' mouse driver for your mouse

because they are compatible with each other.  However, we would still suggest

you to use our mouse driver.  It would work better that way.

Set Up This Mouse Under Microsoft Windows 3.x

a.  Go to your Microsoft Windows 3.x directory, then type "setup <enter>".

b.  Insert your mouse driver diskette into the right floppy drive and

choose 'Other mouse [Requires disk from OEM]...' for 'Mouse: '.

c.  Press <enter> and then choose your mouse driver, which is shown as

"Allspirit Mouse driver ".

d.  Follow instructions on the computer screen to finish the setup task.

e.  Restart the computer then you are all set!

Please go ahead to correct if any translation is not very suitable.