How to Install NDIS 2.0.1 Drivers

===================================

NDIS (Network Driver Interface Specification) 2.0.1 is a driver

specification developed by Microsoft Corporation to offer a standard

driver interface for all adapter cards.  A standard driver interface

provides flexibility to support not only multiple network adapter

cards but also multiple protocol stacks.  As a result, many operating

systems have incorporated the NDIS interface to make configuration

and installation of network adapter cards easy and independent of

network adapter card.

Included in this manual are instructions to install the ETHER-32V/P family

NDIS 2.0.1 driver for the following environments:

Microsoft LAN Manager 2.x

Microsoft Windows for Workgroups 3.1

Microsoft Windows for Workgroups 3.11

Banyan VINES

IBM LAN Server 2.0, 3.0

Sun Microsystems PC-NFS 5.0

How to Installing the Windows for Workgroups 3.11 Driver

============================================================

Please refer to the Installing the Windows for Workgroups 3.11 Driver under

the Installing NDIS 3.0 Driver section in this manual.The procedure to load

NDIS 2.0.1 is exactly the same as for NDIS 3.0.In fact, Windows for

Workgroups 3.11 will automatically load NDIS 2.01, NDIS 3.0, both drivers

together.

Microsoft's Windows for Workgroups 3.11 supports both NDIS 2.0.1 and

NDIS 3.0 drivers. ETHER-32V/P provides both NDIS drivers to run under

the Windows for Workgroups 3.11 environment.  Please note the

format and procedure to install the NDIS drivers for Windows for

Workgroups 3.11 is different from that of Windows for Workgroups 3.1.

The directory structure provided on the ETHER-32V/P driver diskette

has been setup for automatic OEM installation. The OEM

Installation section in this manual provides a brief explanation

regarding OEM driver installation.  Follow the guidelines in the

Microsoft Windows for Workgroups 3.11 User's Guide for more details

to install the family driver from the ETHER-32V/P driver diskettes.

Note:Some of the custom installation procedures provided here

are either an abbreviated or a duplication of the OEM installation

guidelines.

To install the NDIS 2.0.1 and/or NDIS 3.0 drivers, simply follow

the instructions given below:

1. Insert the ETHER-32V/P driver diskette into the system's floppy drive.

2. In the Windows for Workgroups 3.11 Program Manager, double click

on the Windows Setup icon from the MAIN window.

3. While in the Windows Setup dialog box, select the Change Network

Settings from the Options menu to install drivers from the

ETHER-32V/P driver diskette.

4. From the Network Setup dialog box, select the "Drivers...

" button to install the PCN NDIS 2.0.1 and NDIS 3.0 drivers.

5. Then, in the Network Drivers dialog box, select Add Adapter

button to install the device drivers.

6. Now, in the Add Network Adapter window, choose the "Unlisted

or Updated Network Adapter" from the menu and then select OK.

7. The Install driver dialog box will appear.  Enter the floppy

drive with the ETHER-32V/P driver diskette and specify the WFW311

path to install the drivers.  For example,

A:\WFW311

Then select OK.

8. The Advanced Micro Devices ETHER-32V/P family selection will appear

under the Network Adapters list. Simply select OK to continue.

9. Then, the user will be asked a family of questions for the correct

Interrupt channel, DMA channel, Twisted Pair keyword option, and the

four LED keyword options.  Refer to the Software Keywords section in

this manual for more details.

For the other family adapter boards (ETHER-32V,ETHER-32P),

no values need to be specified for the Interrupt channel and

DMA channel since these ETHER-32V/P devices are software configurable.

10.Then, select OK to exit.  Windows for Workgroups 3.11 will

ask the user to reboot the machine once installation is completed.

The installation for Windows for Workgroups is now completed.