ORPHEUS EASY PAINTER

FOR

EZPAD

MIT-100

FEATURES & FUNCTIONS

====================

Equipment Checklist

The following items are included in your EZPAD package :

o   EZPAD and Drawing Tool

o   Power Adaptor Cable

o   RS-232C Interface Cable

o   Easy Painter Diskette

o   User's Guide

INSTALLATION

============

Connect the Power Adaptor Cable

o   Turn off your computer.

o   Unplug the keyboard cable from your computer.

o   Plug your keyboard cable into the female DIN 5-pin connector

on the power adaptor cable.

o   Plug the male DIN 5-pin connector of power adaptor cable into

the keyboard connector of your computer.

o   Plug the power connector of the power adaptor cable into the

DC power port of the EZPAD.

Connect the RS-232C Interface Cable

o   Connect the female 9-pin connector of the RS-232C interface

cable to a serial port on the computer.

o   Plug the male DIN 5-pin connector of RS-232C interface cable

into the interface port of the EZPAD.

GETTING STARTED

===============

Starting Easy Painter on an One-drive System

o   Insert the DOS diskette in drive A and close the door.

o   Turn on the computer.

o   Enter the date and time if DOS prompts for them.

o   Remove DOS diskette from drive A.

o   Insert your Easy Painter Diskette in drive A and close the

door.

o   Turn on your EZPAD.

o   If your EZPAD is connected to serial port COM1:, type paint 1

and press ENTER.

o   If your EZPAD is connected to serial port COM2:, type paint 2

and press ENTER.

o   The drawing demonstration will show on the screen.

o   Touch the drawing area, color keys, or function keys of the

EZPAD with the drawing tool. The demonstration drawing disappear

immediately and the Easy Painter is ready for drawing.

Starting Easy Painter on a Two-drive System

o   Insert the DOS diskette in drive A and close the door.

o   Insert your Easy Painter Diskette in drive B and close the

door.

o   Turn on the computer.

o   Enter the date and time if DOS prompts for them.

o   Type B: and press ENTER.

o   Turn on your EZPAD.

o   If your EZPAD is connected to serial port COM1:, type paint 1

and press ENTER.

o   If your EZPAD is connected to serial port COM2:, type paint 2

and press ENTER.

o   The drawing demonstration will show on the screen.

o   Touch the drawing area, color keys, or function keys of the

EZPAD with the drawing tool. The demonstration drawing disappear

immediately and the Easy Painter is ready for drawing.

Starting Easy Painter on a Hard Disk

o   Create a directory for your Easy Painter first.

o   Copy all files in Easy Painter Diskette to your painter

directory.

o   Type cd followed by the name of the directory of your painter

and press ENTER.

o   Turn on your EZPAD.

o   If your EZPAD is connected to serial port COM1:, type paint 1

and press ENTER.

o   If your EZPAD is connected to serial port COM2:, type paint 2

and press ENTER.

o   The drawing demonstration will show on the screen.

o   Touch the drawing area, color keys, or function keys of the

EZPAD with the drawing tool. The demonstration drawing disappear

immediately and the Easy Painter is ready for drawing.

To Quit from Easy Painter

o   Type ESC.

o   When the quit menu is on the screen, type Y.

TO MAKE A SKETCH

================

Choose a Color

Decide the color and touch the color key with the drawing tool.

Choose a Function

Decide the function and touch the function key with the drawing

tool.

Make a Sketch

Before making a sketch,

o   Select Fine Pencil, Pencil, Bold Pencil, or Eraser with the

drawing tool.

o   Touch the drawing area of the EZPAD with the drawing tool,

the pencil (or eraser) cursor will appear on the CRT screen.

Once the drawing tool leave the drawing area, the cursor will

disappear.

o   When making a sketch, hold the drawing tool with one hand and

press the mark key with another hand. Be sure to keep pressing

the mark key and the the point of the drawing tool on the draw-

ing area of the EZPAD.

Mark Lock Mode

o   If the function you choose is Fine Pencil, Pencil, or Bold

Pencil, you can press two mark keys (both right and left) at

the same time to enter the Mark Lock Mode. At this time, the

Mark Lock On message will appear at the right of your CRT

screen, which means the Mark Lock Mode is set.

o   In Mark Lock Mode, you can make a sketch by moving the point

of the drawing tool on the drawing area without pressing the

mark key.

o   Mark Lock is cancelled when any other key is pressed.

PAINT BRUSH

===========

The Paint Brush is used to fill an enclosed area with appointed

color.

o   Choose the color that you need with the drawing tool.

o   Press the Paint Brush function key.

o   Touch the drawing area with the drawing tool, and the paint

brush cursor will appear on the CRT screen.

o   Move the drawing tool so the starting point of the cursor

will move to the enclosed area that you want to paint.

o   Press the mark key, the enclosed area will be filled with

the selected color.

o   During the process of painting, if the area is not closed,

the color will project out-side the area to be colored.

You may press the Undo key to cancel the painting action,

then make the break up and paint again.

TO ERASE DRAWING OR CLEAR SCREEN

================================

If you are not satisfied with the sketch that you made, you may

use the following ways to erase drawing.

Eraser

o   Press the Eraser key with drawing tool.

o   Move the erasing point of the eraser cursor to the position

you want to erase.

o   Press the mark key while pressing the point of the drawing tool

to erase.

Clear Button

The Clear button is located at the left side of the EZPAD.

To erase the entire screen, press the Clear button.

White Pencil and Paint Brush

White Pencil or Paint Brush are also effective on erasing.

o   Choose the white color.

o   Choose a Fine Pencil, Pencil, Bold Pencil, or Paint Brush.

o   Erase according to the drawing or painting procedure.

UNDO

====

When making a sketch, if you are not satisfied with the most

recent action, you can use the Undo key to cancel it. The

drawing before the last action will appear on the CRT screen.

STAMP PAD

=========

There are two groups of patterns in Stamp Pad function and each

group contains 28 kinds of patterns.

When using the stamp pad,

o   Press the color key with drawing tool to select color.

o   Touch the Stamp Pad function key with drawing tool, the

stamp pad menu will appear on the CRT screen.

o   Press the function key again, the printing menu will appear

appear on the CRT screen.

The 1st Group of Stamp Pad Patterns

o   When the stamp pad menu is on CRT screen, press the drawing

area with the drawing tool, and the arrow cursor will appear

on CRT screen.

o   Move the arrow cursor to the desired pattern and press the mark

key, the chosen pattern will appear on the screen and the stamp

pad menu will disappear.

o   Move the pattern with the drawing tool to the desired location

and press the mark key, the pattern will be printed at that

location.

If you press the mark key while moving the drawing tool, the pattern

will be printed one upon the other continuously following the trace

of the drawing tool.

If you want to change the color halfway, just press the color key.

The color of the stamp pad pattern will be changed at once, then

you can go on with new color.

The 2nd Group of Printing Patterns

The 2nd pattern group is for printing. You can fill an enclosed area

with the desired pattern.

o   When the printing menu is on CRT screen, press the drawing

area with the drawing tool, and the arrow cursor will appear

on CRT screen.

o   Move the arrow cursor to the desired pattern and press the mark

key, the pattern group will disappear, and the paint roller cursor

will appear on the CRT screen.

o   Move the contact point of the paint roller to the enclosed area

you want to print, and press the mark key. The enclosed area will

be printed with the desired pattern.

GRAPHIC LIBRARY

===============

In Graphic Library, there are two groups of pre-drawn pictures includ-

ing animals, flowers, fruit, and full screen pictures.

To enter the Graphic Library, touch the Graphic Library function

key with the drawing tool, and the menu for Graphic Library will

appear on the CRT screen.

In Graphic Library, all the colors of the patterns are set in advance.

Therefore, the color keys will not work.

The 1st Group of Full Screen Pictures

The 1st group drawings in Graphic Library contains 14 full screen

pictures.

To use these pictures,

o  When the menu of Graphic Library is on the CRT screen, move

the arrow cursor to the item you want in the 1st group and

press the mark key.

o  The graphic library menu will disappear and the selected full

screen picture will be drawn on the screen.

o  If you want to practice painting, you may press the Undo function

key with your drawing tool. All the colors on the drawing will

disappear and leave only the outlines of the drawing. Thus, you

can go on painting.

The 2nd Group of Pre-drawn Pictures

The 2st group drawings in Graphic Library contains 42 pre-drawn

pictures.

To use these pictures,

o   When the menu of Graphic Library is on the CRT screen, move

the arrow cursor to any item in the 2nd group and press the

draw key. The pre-drawn pictures of that item will appear on

the right window.

o   In this way, you can scan through all the pre-drawn pictures to

make your selection.

o   After deciding the picture, move the arrow cursor to the OK box

and press the mark key.

o   The graphic library menu will disappear and the selected pre-

drawn picture will appear on the screen.

o   Move the picture with the drawing tool to the desired location,

and press the mark key. The picture will be printed at that

location.

TEXT MODE

=========

The Text Mode allows you to add label on your drawing.

While using the Text Mode,

o   Press the color key with drawing tool to select color.

o   Touch the Text Mode function key with drawing tool, the

text mode menu will appear on the screen.

o   Press the drawing area with drawing tool, the arrow cursor

will appear on the screen.

o   Move the arrow cursor to the desired character and press the

mark key. At this time, the character will be printed at the

lower portion of the menu.

o   In this way, you can continue typing your label. Once the

label typing is completed, move the arrow cursor to OK box

and press the mark key.

o   The text menu will disappear and the label that you type

will appear on the screen.

o   Move the label to the desired location and press the mark

key, the label will be prinited at that location.

To change the color of the character and frame,

o   When the text menu is on CRT screen, press the color key with

the drawing tool. The color of the characters will be chagned.

o   If you want to change the color of character frame, move the

arrow cursor to the COLOR box and press the mark key.

o   The color of character frames will be changed as you press the

color keys.

o   If you want to change the color of character, move the arrow

cursor to the COLOR box and press the mark key.

o   The color of character will be changed as you press the color

keys.

In this way, you can change the color of character and color of

character frame while typing your label.

TOOL BOX

========

The Tool Box contains 14 tools including line-drawing, rectangle-

drawing, ellipse-drawing, spray can, flip-vertical, flip-horizontal,

move, copy, cut, color palette, load, save, zoom, and zoom grid.

To use the Tool Box,

o   Touch the Tool Box function key with the drawing tool and the

tool box selecting menu will appear on the screen.

o   Press the drawing area with the drawing tool and the arrow

cursor will appear on the screen.

o   Move the arrow cursor to the desired tool and press the mark

key. The tool will be avalibale.

Line Drawing

After choosing the tool for line drawing,

o   Press the drawing area with the drawing tool, the line drawing

cursor will appear on the screen.

o   Move the cursor to the desired location and press the mark key

to locate the starting point of the line segment.

o   Once the starting point of the line segment has been determined,

move the cursor again. A straight line will appear on the screen

and the ending point will be changed according to the location

of the cursor.

o   After moving the ending point to the desired location, press

the mark key again. The line will be drawn on the screen.

Thus, you can draw all the lines you want by following these steps.

There are three kinds of line width. If you choose the fine pencil

before entering tool box, the line width of the line drawn will be

fine. If you select pencil before pressing the tool box key, the

line drawn will be normal. If bold pencil was selected before

entering tool box, the line drawn will be bold.

When using the tool of line drawing, you can change the color

of the line by pressing the color key.

Before the ending point of the line segment is decided, you can

press the Undo key to cancel the starting point.

Rectangle Drawing

After choosing the tool for rectangle drawing,

o   Press the drawing area with the drawing tool, the rectangle drawing

cursor will appear on the screen.

o   Move the cursor to the desired location and press the mark key

to locate the 1st corner of the rectangle.

o   Once the 1st corner of the rectangle has been determined,

move the cursor again. A rectangle will appear on the screen

and the 2nd corner will be changed according to the location

of the cursor.

o   After moving the 2nd corner to the desired location, press

the mark key again. The rectangle will be drawn on the screen.

Thus, you can draw all the rectangle you want by following these steps.

The way to change the width of the rectangle outline, to change color,

and to cancel the 1st corner are same as line drawing.

Ellipse Drawing

After choosing the tool for ellipse drawing,

o   Press the drawing area with the drawing tool, the ellipse drawing

cursor will appear on the screen.

o   Move the cursor to the desired location and press the mark key

to locate the center of the ellipse.

o   Once the center of the ellipse has been determined, move the

cursor again. A ellipse will appear on the screen and the size

of the ellipse will be changed according to the location of the

cursor.

o   After moving cursor to the desired location, press the mark key

again. The ellipse will be drawn on the screen.

Thus, you can draw all the ellipses you want by following these steps.

The way to change the width of the ellipse outline, to change color,

and to cancel the center are same as line drawing.

Spray Can

After choosing the spray can,

o   Press the drawing area with the drawing tool, the spray can

cursor will appear on the screen.

o   Move the cursor to the desired location and press the mark key.

Dots with selected color will be painted on the screen.

Flip - Vertical

After choosing the tool for flip-vertical,

o   Press the drawing area with the drawing tool, the flip-vertical

cursor will appear on the screen.

o   Move the cursor to the 1st corner of the area to be flipped and

press the mark key to decide the 1st corner.

o   Once the 1st corner has been determined, move the cursor again.

A rectangular window will appear on the screen. The size of the

window will change as you move the cursor.

o   After moving the cursor to the 2nd corner of the area to be

flipped, press the mark key again. All the drawing inside the

window will be flipped.

After choosing the 1st corner and before choosing the 2nd corner,

you can press the Undo key to cancel the 1st corner.

Flip - Horizontal

After choosing the tool for flip-horizontal,

o   Press the drawing area with the drawing tool, the flip-horizontal

cursor will appear on the screen.

o   Move the cursor to the 1st corner of the area to be flipped and

press the mark key to decide the 1st corner.

o   Once the 1st corner has been determined, move the cursor again.

A rectangular window will appear on the screen. The size of the

window will change as you move the cursor.

o   After moving the cursor to the 2nd corner of the area to be

flipped, press the mark key again. All the drawing inside the

window will be flipped.

After choosing the 1st corner and before choosing the 2nd corner,

you can press the Undo key to cancel the 1st corner.

Move

After choosing the tool for move,

o   Press the drawing area with the drawing tool, the move cursor

will appear on the screen.

o   Move the cursor to the 1st corner of the area to be moved and

press the mark key to decide the 1st corner.

o   Once the 1st corner has been determined, move the cursor again.

A rectangular window will appear on the screen. The size of the

window will change as you move the cursor.

o   After moving the cursor to the 2nd corner of the area to be

moved, press the mark key again.

o   Move the drawing tool so that selected area will move to the

desired location.

o   Press the draw key. The selected area will be printed at that

location.

Before finishing the move, you can press the Undo function key to

cancel it.

Copy

After choosing the tool for copy,

o   Press the drawing area with the drawing tool, the copy cursor

will appear on the screen.

o   Move the cursor to the 1st corner of the area to be copied

and press the mark key to decide the 1st corner.

o   Once the 1st corner has been determined, move the cursor again.

A rectangular window will appear on the screen. The size of the

window will change as you move the cursor.

o   After moving the cursor to the 2nd corner of the area to be

copied, press the mark key again.

o   Move the drawing tool so that the area to be copied will be moved

to the desired area.

o   Press the mark key and the area to be copied will be printed

at the desired location.

Before finishing the copy, you can press the Undo key to cancel it.

Cut

After choosing the tool for cutting,

o   Press the drawing area with the drawing tool, the cut cursor

will appear on the screen.

o   Move the cursor to the 1st corner of the area to be cut

and press the mark key to decide the 1st corner.

o   Once the 1st corner has been determined, move the cursor again.

A rectangular window will appear on the screen. The size of the

window will change as you move the cursor.

o   After moving the cursor to the 2nd corner of the area to be cut,

press the mark key again. All the drawing inside the window will

be erased.

Before finishing the cutting, you can press the Undo key to cancel it.

Color Palette

The color palette allows user to select and set the desired color out

of 262,144 different colors. After choosing the color palette, the

palette menu will appear on the screen.

*  Color Keys Selecting Section

The EZPAD has a total of 14 color keys. The first and the second key

from the left can be used for mixing other colors. The default color

of these two keys are pink and light purple.

In color keys selecting section of the palette menu, the two color

suare are corresponding to color key 1 and color key 2 on the EZPAD.

*  RGB (Red, Green, and Blue) Level Adjustment Section

The RGB level adjustment section is used for changing the level of

RGB from degree 1 to 64. Therefore, there will be 262,144 colors

available.

*  Color Display Section

In the process of RGB level adjustment, the color of current RGB

setting will be shown on the color display square.

*  Color Palette Operating

After entering the palette,

o   Press the drawing area with the drawing tool. A arrow cursor will

appear on the screen.

o   Move the arrow to the color keys selecting section of the palette

menu. If you want to change the color of color key 1 on the EZPAD,

please select the left color square. If the color of color key 2

is to be changed, select the right color square.

o   Press the mark key while the arrow is on one of the color squares

to decide the color key your want to change.

o   Move the arrow to the RGB level adjustment section. If you wnat to

increase the level of green, just move the arrow to the green ">>"

and press the mark key. The green level indicator will increase by

1 degree and the color in color display square will change accord-

ingly. If you want to decrease the level of green, move the arrow

to the green "<<" and press the mark key. The green level indicator

will decrease by 1 degree and the color in color display square

will change accodingly.

Following these steps to adjust the Red, Green, and Blue levels, you

can select the color you want out of 262,144 different colors.

*  Quit From Color Palette

After finishing the color selection, just press any function key except

the sound switch and the Undo key, then you can quit from the palette.

The drawing before entering the palette will be restored.

If you want to use the color that you just create, you can press the

color key 1 or color key 2 with the drawing tool, then you will be

able to use that color and go on drawing.

LOAD

To load the drawing from diskette, you can use the load tool. After

choosing the load tool, a load menu will appear on the screen. Type

in the file name of the file to be loaded and press the ENTER. The

screen will be loaded with the picture in that file.

SAVE

The Easy Painter allows you to save your drawing on diskette. After

choosing the save tool, a save menu will appear on the screen. Type

in the file name and press the ENTER. The drawing on the screen will

be saved in the diskette.

ZOOM

To change the size of the screen drawing, you can select the tool

for zoom. After choosing the tool for zoom,

o   Press the drawing area with the drawing tool, the zoom cursor

will appear on the screen.

o   Move the cursor to the 1st corner of the area to be zoomed

and press the mark key to decide the 1st corner.

o   Once the 1st corner has been determined, move the cursor again.

A rectangular window will appear on the screen. The size of the

window will change as you move the cursor.

o   After moving the cursor to the 2nd corner of the area to be

zoomed, press the mark key again.

o   After the area to be zoomed has been determined, a destination

window is needed for the location and the size of the drawing

after zoom.

o   To determine the destination window, move the drawing tool to the

the desired location and press the mark key to determine the 1st

corner of the destination window.

o   Once the 1st destination corner has been determined, move the

cursor again. A rectangular window will appear on the screen.

The size of the window will change as you move the cursor.

o   After moving the 2nd corner to the desired location, press the

mark key and the zoom operation will start.

Before finishing the zoom operation, you can press the Undo key to

cancel it.

ZOOM GRID

To perform detailed editing on a part of your drawing, use the zoom

grid tool. After choosing the tool for zoom grid,

o   Press the drawing area with the drawing tool, a rectangular window

will appear on the screen. This window is to locate the area of

your drawing to be zoomed.

o   Move the window to the desired location and press the mark key.

A zoom grid menu will appear on the screen.

o   Press the drawing area with the drawing tool, a arrow cursor

will appear on the screen.

o   Select the color you want by pressing color key, move the arrow

cursor to the pixel you want yo edit and press the mark key.

A pixel with selected color will be drawn at that location.

o   After finishing the editing, move the arrow cursor to the

OK box and press the mark key.

o   The zoom grid menu disappear and the drawing after editing will

appear on the screen.

Before finishing the zoom operation, you can press the CANCEL box to

cancel it.

MUSIC BOX

=========

The Music Box contains a music keyboard and 13 backgournd songs for

selection. The music keyboard allows user to play on CRT with 8

different instruments.

To use the Music Box, press the Music Box function key with the

drawing tool. A music box menu will appear on CRT screen.

*   Music Keyboard

After entering the music box,

o   Move the arrow cursor to the keyboard item and press the mark key.

The music keyboard will appear on the scren.

o   Move the arrow to the desired instrument square and press the mark

key. The sound of the instrument will be played which menas the

instrument has been selected.

o   Move the arrow to the keyboard and press the mark key, then you can

play music on the CRT screen.

*   Background Music - (Available only when system has the AdLib melody

card)

There are 13 songs of backgournd music in the Music Box. When choosing

the desired background music,

o   Move the arrow to any song selecting item.

o   Press the mark key.

If you choose the 13rd song, the music will play from song 1 to song 12.

*   Quit From Music Box

After finishing the keyboard playing or the backgourng music selecting,

just press any function key except the Music Box, Sound Switch, and the

Undo, then you will leave the music box. At this time, the drawing

before entering the music box will be restored.

SOUND SWITCH

============

When drawing, there are two modes to control the backgroung music :

(1) Turn off the background music

(2) Turn on the backgroung music

*   Music Off

If you turn off the background music, the painter remains in silence

all the time.

*   Music On

If you turn on the background music, the selected song will start

playing.

*   Sound Switch Operating

The background music is controlled by the sound switch. The first

touch will turn off the background music, and the sencond touch will

turn on the background music.

PRINTING - (Under Microsoft Windows Paintbrush)

========

The picture data file created by Easy Painter is a PCX format.

Therefore, it can be accepted by Microsoft Windows Paintbrush.

To print a picture,

o   Enter Microsoft Windows Paintbrush.

o   Open the picure file created by Easy Painter with Paintbrush

OPEN command. Be sure to choose the PCX form before opening

the picture file.

o   Use the Paintbrush PRINT command to print the picture.

MOUSE MODE

==========

The EZPAD can also be used as a mouse. In Mouse Mode, it is fully

compatible with Microsoft Mouse.

*   TO ENTER MOUSE MODE

To enter Mouse Mode, press one of the mark keys while turning on the

power of EZPAD. At this time, the mouse mode indicator will be turned

on.

*   BUTTON ASSIGNMENTS

In Mouse Mode, the button assignments on the EZPAD are as follows:

o   Press the left mark key while turning on the power

EZPAD left mark key   =  Left mouse click

EZPAD right mark key  =  Right mouse click

o   Press the right mark key while turning on the power

EZPAD left mark key   =  Right mouse click

EZPAD right mark key  =  Left mouse click

APPENDIX A.  TROUBLESHOOTING

Generally the EZPAD is easy to setup and use. However, if the EZPAD does

not work properly with your software, the check-list and solution discussed

below will solve most problems.

Problems with Easy Painter

Problem 1:   Error Message "EZPAD POWER OFF OR NOT CONNECTED !" shown on

the screen.

Problem 2:   The cursor does not respond at all to drawing tool movements

or function keypad selections.

Solution:    Check each of the following items to be sure your system is

set up correctly

o   Is the EZPAD turned on ? (The power switch should be in

"on" position and the power indicator should be turned

on).

o   Is the EZPAD in Painter Mode ? (The mouse mode indicator

should not be turned on).

o   Are all of your plugs connected securely and correctly ?

o   Is the RS-232C interface cable inserted into the correct

communication port on your computer ? (The port should be

the one that you specified when starting the Easy Painter

software).

Problems with Mouse Mode

Problem:     The cursor does not respond at all to drawing tool movements.

Solution:    Check each of the following items to be sure your system is

set up correctly

o   Is the EZPAD turned on ? (The power switch should be in

"on" position and the power indicator should be turned

on).

o   Is the EZPAD in Mouse Mode ? (The mouse mode indicator

should be turned on).

o   Are all of your plugs connected securely and correctly ?

o   Is the RS-232C interface cable inserted into the correct

communication port on your computer ? (The port should be

the one that you specified in your mouse driver and appli-

cation software).

APPENDIX B:  SPECIFICATIONS

Physical:

Dimensions         321 x 266 x 70 (mm)

Weight             1.2 (kg)

Functional:

Function Key       12

Color Key          14

Mark Key            2

Clear botton        1

Resolution         256 x  192

Interface          RS-232C Via RS-232C Interface Cable

o   RS-232C Interface Cable

MINI DIN 5 PIN MALE   D-SUB 9 PIN FEMALE

RXD           1  ----------------  2

GND           2  ----------------  5

TXD           3  ----------------  3

RTS           5  ----------------  7

Electrical:

Power Source       Via Keyboard Adaptor Cable

Input Voltage      5V DC

Current Draw       300 mA

o   Keyboard Adaptor Cable

MINI DIN 5 PIN MALE   MINI DIN 5 PIN FEMALE   DC JACK

KBD_CLK         1  -----------------  3

KBD_DATA        2  -----------------  2

KBD_GND         4  -----------------  5  ----------  Shield

KBD_VCC         5  -----------------  4  ----------  Center

Environmental:

Operating Temperature      41  to  122  F (5  to  50  C)

Storage Temperature        14  to  122  F (-10  to  50  C)

Humidity                   20 % to 95 % (Non-Condension)