SEND.COM      Summagraphics Tablet Configuration Utility

Copyright Summagraphics Corporation 1990, 1991.      All rights reserved.

SEND.COM will transmit a string of ASCII characters to the tablet out the

selected serial port from the command line or from a data file.

Usage: SEND [ options ][/ Cxxx | / F [path] filename]

/1  (default) selects serial port 1 {PC, XT, AT, PS/2}

/2  selects serial port 2 {PC, XT, AT, PS/2}

/3  selects serial port 3 {PS/2 only}

/4  selects serial port 4 {PS/2 only}

/5  selects serial port 5 {PS/2 only}

/6  selects serial port 6 {PS/2 only}

/7  selects serial port 7 {PS/2 only}

/8  selects serial port 8 {PS/2 only}

/BAxxxx  selects serial port with base address of xxxx (in HEX)

/O  selects odd parity configuration

/E  selects even parity configuration

/N  selects no parity configuration

/M  selects MM format serial configuration      {9600,8,O,1} (default)

/U  selects UIOF format serial configuration    {9600,7,E,2}

/B  selects Bit Pad format serial configuration {9600,7,E,2}

/BRK  sets break state on transmit line for 100 milliseconds

/Cxxx...  transmit command characters "xxx...", until carriage return

/? or /H  displays help screen

/F [path] filename    Transmit command characters from specified file.

(example: /F c:\tablet.cmd)

/Sxx      select baud rate xx

arguments for xx are: 96 (for 9600 baud)  18 (for 1800 baud)

72 (for 7200 baud)  12 (for 1200 baud)

48 (for 4800 baud)  60 (for 600 baud)

36 (for 3600 baud)  30 (for 300 baud)

24 (for 2400 baud)  15 (for 150 baud)

20 (for 2000 baud)  11 (for 110 baud)

Notes: - When using the BA option (Base Address) all four digits must be

specified.  (example: for COM 1 - /BA03F8)

- Option /F or /C must be the LAST option on the command line.

- To transmit ASCII control characters precede each printable ASCII

character with " ^ ".  examples: ESC = ^[

NULL = ^@

BELL = ^G

- The /BA option should only be used when the serial port is located

at a non-standard address within the computer.

- A summary of command line switches are always available by starting

the program with the /? or /H options.

- When the command line switch /F is used there must be a space

between the switch and the filename.

- The break switch will always be executed first regardless of it's

position on the command line.

- The maximum file size allowed is 128 characters.

- There must not be any space between the /C switch and the commands

that follow.

- There must be a space between the /F switch and the path and filename

that follow.

Serial Port Address Table

Command Line	Serial	   Serial Port	 System

Switch		Port	   Address

------------------------------------------------------------------------------

/1		1	   03F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/2		2	   02F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/3		3	   3220 Hex	 Any PS/2

/4		4	   3228 Hex	 Any PS/2

/5		5	   4220 Hex	 Any PS/2

/6		6	   4228 Hex	 Any PS/2

/7		7	   5220 Hex	 Any PS/2

/8		8	   5228 Hex	 Any PS/2

------------------------------------------------------------------------------

*********************** 87-1015-001 ********************************