*******************************************************************

UNIXDRV.DOC

*******************************************************************

This file describes the features and use of the NCR SDMS device

drivers for the SCO UNIX operating system. It is divided into the

following sections:

Introduction

SCO UNIX 3.2

Features

Description

Installation

Troubleshooting

*******************************************************************

************************* Introduction ****************************

SCO UNIX is the first UNIX operating system licensed for IBM-compatible

microcomputers. The integrated communications, file system, international

application support, and documentation create an ideal platform for those

requiring a full-featured, UNIX-based operating system. SCO UNIX takes full

advantage of the capabilities of the 80386 and above microprocessors.

SCO UNIX 3.2 provides Basic Utilities, and Extended Utilities, as well as

their Development System packages. These items, plus the installation

software, are provided on diskettes (also available on tape or CD). SCO has

labeled these diskettes: N(1-2) and M1 for Installation, B(1-3) for Basic

Utilities, and X(1-7) for Extended Utilities.

The NCR SDMS 3.0 SCSI device drivers are dynamically link edited into the

UNIX kernel. The rebuilt kernel must reside on the boot drive: either an

internal, non-SCSI hard drive (IDE, ESDI, or ST506), or a SCSI hard drive.

SDMS facilitates this by providing a Boot Time Loadable Driver (BTLD), and

the CAM3 SCSI configuration tool. The BTLD permits an easy installation of

UNIX onto a SCSI hard disk with the software provided by SCO. The CAM3 SCSI

configuration tool simplifies loading the SCSI device drivers into the

operating system.

The following sections provide instruction on configuring the SCO UNIX kernel

with the SCSI device drivers, adding SCSI peripheral devices, and installing

SCO UNIX onto a SCSI hard drive. These instructions assume you are familiar

with UNIX system administration.

************************** SCO UNIX 3.2 ***************************

* Features *

Easy Unix installation on a SCSI hard disk with the NCR Boot Time Loadable

Driver (BTLD)

Easy SCSI device driver installation with the NCR CAM3 SCSI configuration

tool

Synchronous negotiation (including fast SCSI)

Supports multiple host adapters (SCO allows up to two)

Multiple logical unit number (LUN) support

Disconnect/Reselect

Supports scatter-gather

Tagged command queuing

Dynamic interrupt mapping

* Description *

UNIX installation on a SCSI system requires that a copy of SCO UNIX be created

that contains the device drivers to support SCSI devices. The installation of

UNIX depends on how the SCSI devices will be configured. There are two

possible installation procedures:

1. Install UNIX and the SCSI device drivers on a non-SCSI boot drive.

2. Install UNIX and the SCSI device drivers on a SCSI boot drive.

Procedure 1

If SCSI devices will be attached to a SCSI host bus adapter card while using

a non-SCSI hard drive (IDE, ESDI, ST506) as the boot hard drive, execution of

procedure 1 is all that is necessary. This will install the NCR SCSI UNIX

device drivers on the non-SCSI boot drive to access the SCSI devices. SCSI

devices supported include tape drives, CD-ROM drives, and SCSI hard disk

drives that will be used as secondary storage.

Procedure 2

If the user plans to install a SCSI hard disk drive to be used as the boot

hard drive, then procedure 2 will be executed. Procedure 2 uses a Boot Time

Loadable Driver (BTLD) to provide a user friendly method for installing UNIX

onto a SCSI hard disk drive.

Note:   To use the BTLD, the Basic and Extended Utilities packages provided

by SCO must be included during the installation.

Each of these procedures are described in detail below. NCR recommends that

the entire installation is accomplished in one session. Be sure that the

installer has sufficient time (possibly an hour) available to complete this

without interruption.

The NCR CAM3 SCSI Configuration Tool

Both installation procedures use the NCR CAM3 SCSI configuration tool to load

the SCSI device drivers into the operating system. This utility is provided

on the diskette with the NCR SDMS drivers for SCO UNIX. The CAM3 main menu

presents the following options:

1.      Configure the NCR SCSI Device Drivers

2.      Add or Remove Specific Devices

3.      Make a SCSI N1 Boot Floppy

4.      Relink Unix

5.      Remove All SCSI Devices

6.      Make Device Nodes for CAM Applications

7.      Show Current SCSI Configuration

Some of these options are referred to in the following procedures. Option

three is not required when the BTLD is used to install UNIX on a SCSI hard

drive. After completion of either installation procedure, CAM3 can be invoked

again from the UNIX prompt by typing:

mkdev cam3

In this way CAM3 can be used to make subsequent changes in the installation.

* Installation *

Procedure 1

This procedure assumes UNIX is already installed on an IDE, ESDI, or ST506

hard disk drive. It will show how to include the device drivers needed to

access SCSI devices installed after the non-SCSI drive. This installation is

necessary to build a new UNIX kernel which includes the NCR SCSI device

drivers. The basic steps for accomplishing this are outlined below. Further

detail on UNIX installation can be found in the SCO UNIX System

Administrator's Reference manual.

1. NCR recommends creating a second copy of UNIX. Log in as root and issue

the command:

cp /unix /unix.good

This creates a second copy of UNIX under the name /unix.good.

Note: If any problems arise with the Kernel being built in UNIX, this

procedure ensures that a valid copy of UNIX will be available.

To access this working UNIX copy, type at the colon after the boot message:

unix.good

To replace the system UNIX with the working UNIX copy, type at the system

prompt:

cp /unix.good /unix

2. Install the NCR SDMS SCSI UNIX device drivers.

a. Insert the SCO SDMS SCSI Drivers floppy diskette in the drive and type:

installpkg

Press the Enter key to continue.

b. A prompt will appear to insert the requested floppy diskette. Since the

NCR diskette has already been inserted, press the Enter key to continue.

c. A prompt will then ask for the name of the package. Type:

cam3

Press the Enter key to continue.

d. If the bhba driver is already present from a previous installation, a

prompt inquires about replacing it. Do not replace it unless there is

reason to believe it is corrupted, or a newer version is available.

e. To execute CAM3, remove the NCR floppy diskette from the drive (since

CAM3 has been installed on the hard drive), then type:

mkdev cam3

Press the Enter key to continue.

f. The CAM3 main menu is displayed. To check that there are no SCSI devices

installed, press 7 and Enter to show the current SCSI configuration. There

should be no SCSI devices listed. If there are, press 5 for the option to

remove all SCSI devices, then press y and Enter to accept the removal.

g. From the CAM3 main menu, select option 1 to configure the NCR SCSI device

drivers. Three options are displayed:

1. Configure NCR Host Adapter Software

2. Remove NCR Host Adapter Software from System Configuration

3. Return to Main Menu

h. Press 1 and Enter to configure the NCR UNIX drivers, then press 1 and

Enter to configure the NCR host bus adapter software.

i. Each device driver in UNIX is assigned a number known as the major device

number. The major device number assigned to the host bus adapter is

displayed. Write this number down, as it may be required later, and keep it

with the UNIX documentation.

j. A prompt is displayed asking what interrupt lines will be used by the

host bus adapter. Interrupt lines associated with particular device drivers

must be identified to the kernel at build time. A list of interrupt lines

available is displayed. The SDMS SCO UNIX driver performs dynamic interrupt

mapping. Therefore, press 0 (zero) and Enter to exit this process and

continue.

Note: If the system indicates that the proper interrupt lines for the SCSI

host bus adapter are not available, review the section under Troubleshooting

concerning possible kernel rebuild problems and edit the sdevice entries to

eliminate the conflict.

3. Adding a SCSI Hard Drive to UNIX.

Note: If a SCSI hard disk drive will not be added to this system, go to

step 4.

a. From the CAM3 main menu, select option 2 to add or remove specific

devices. A list will appear showing the current configuration followed by

a menu of choices. To add the SCSI hard disk drive, select option 1 to

reconfigure Sdsk, and press Enter.

b. Read the explanatory text that appears and follow the prompts. Press

Enter at the next two prompts. A prompt will appear to add another SCSI

disk. Answer y and press Enter. Select option 2 to add a drive to the SCSI

controller and press Enter.

c. The system will prompt for the prefix of the SCSI host adapter that

supports this device. If the default is bhba, press Enter. If the default

is not bhba, type:

bhba

Press Enter.

d. The system will prompt for which SCSI host adapter supports the SCSI

hard drive. Press 0 (zero) and press Enter.

e. The system will prompt for the Target ID of the SCSI hard drive. Type

the ID number and press Enter.

f. The system will prompt for the LUN (Logical Unit Number) of the SCSI

hard drive. Press 0 (zero) and press Enter.

g. The system will prompt to update the SCSI Configuration file. Answer y

and press Enter.

h. A prompt appears to create a new kernel to effect the driver change.

Answer n (as this will be done later) and press Enter.

4. Adding a SCSI Tape Drive to UNIX.

Note: If a SCSI tape drive will not be added to this system, go to step 5.

a. From the CAM3 main menu, select option 2 to add or remove specific

devices. A list will appear showing the current configuration followed by

a menu of choices. To add the SCSI tape drive, select option 2 and press

Enter.

b. Instructions on using a SCSI tape drive are displayed. Press Enter at

the next three prompts to start the mkdev tape utility for installing a

tape drive.

c. The Tape Drive Configuration Program menu appears. Select option 1 to

install a tape drive, and press Enter.

d. The Tape Drive Installation Menu appears. Select option 4 to install a

SCSI tape drive, and press Enter. From the next list select the type of

SCSI tape drive to install, and press Enter.

e. The system will prompt to configure the tape drive. Press y and Enter.

f. The system will prompt for the prefix of the SCSI host adapter that

supports this device. If the default is bhba, press Enter. If the default

is not bhba, type:

bhba

Press Enter.

g. The system will prompt for which bhba SCSI host adapter supports the

tape drive. Press 0 (zero) and Enter.

h. The system will prompt for the Target ID of the controller of this

device (ID number of the tape drive). Type the correct ID number (two is

recommended) of the tape drive and press Enter.

i. The system will prompt for the LUN (Logical Unit Number) of the tape

drive. Press 0 (zero) and Enter. The system will then prompt to update the

SCSI configuration file. Answer y and press Enter.

j. A list will be displayed showing the special devices that have been

created. Press Enter, and a list is displayed showing the links to the

installed tape drive. When prompted for a boot string, press q and Enter to

return to the Tape Drive Configuration Program menu. Press q and Enter to

continue.

k. A prompt appears to create a new kernel to effect the driver change.

Answer n (as this will be done later) and press Enter.

5. Adding a SCSI CD-ROM Drive to UNIX.

Note: If a SCSI CD-ROM drive will not be added to this system, go to Step 6.

a. From the CAM3 main menu, select option 2 to add or remove specific

devices. A list will appear showing the current configuration followed by a

menu of choices. To add the SCSI CD-ROM drive, select option 3 and press

Enter.

b. Instructions on using a CD-ROM drive are displayed. Press Enter two more

times at the prompts to start the mkdev cdrom utility for installing a

CD-ROM drive.

c. The CD-ROM Configuration Program menu appears. Select option 1 to

install a CD-ROM drive and press Enter.

d. The system will prompt to configure the CD-ROM Drive. Answer y and press

Enter.

e. The system will prompt for the prefix of the SCSI host adapter that

supports this device. If the default is bhba, press Enter. If the default

is not bhba, type:

bhba

Press Enter.

f. The system will prompt for which SCSI host adapter supports the CD-ROM

drive. Press 0 (zero) and press Enter.

g. The system will prompt for the Target ID of the CD-ROM Drive. Type the

ID number and press Enter.

h. The system will prompt for the LUN (Logical Unit Number) of the CD-ROM

drive. Press 0 (zero) and press Enter.

i. The system will prompt to update the SCSI Configuration file. Answer y

and press Enter. A prompt appears to configure a CD-ROM/TAPE installation

device. Unless this is desired, answer n and press Enter. A prompt appears

to add a high-sierra file system. Answer y and press Enter. The system

will display the High Sierra/ISO9600 filesystem Configuration Program menu.

Select option 1 to add high-sierra and press Enter. The system updates the

SCSI Configuration File and returns to the CD-ROM Configuration Program

menu. Select q and press Enter to continue.

j. A prompt appears to create a new kernel to effect the driver change.

Answer n (as this will be done later) and press Enter.

6. Rebuilding the Kernel.

a. The system now returns to the CAM3 main menu. Select option 4 to relink

UNIX and press Enter.

b. A prompt will appear asking to rebuild the kernel. Answer y and press

Enter. The kernel will now be linked, which takes approximately 3 minutes.

Messages will appear while the kernel is linking.

c. When the Kernel has been rebuilt, the system will prompt as to whether

this kernel is to boot by default. Press y and Enter.

d. The system will prompt whether to rebuild the kernel environment. Press

y and Enter. When the CAM3 main menu appears, press q and Enter. At the

custom Main Menu, highlight Quit and press Enter.

e. To activate the new kernel the system must be rebooted. Type the

command:

reboot

Press Enter.

f. Press Enter at the Boot: prompt and the new kernel will load. If there

is a host bus adapter with an NCR ROM installed, the NCR device driver

will display a load message when the kernel loads.

g. Log in as usual.

*****************************

Procedure 2

This procedure will install UNIX on a SCSI hard disk drive. It will include

the SCSI device drivers needed to access SCSI devices installed after the

SCSI drive. This installation is necessary to build a new UNIX kernel that

includes the NCR SCSI device drivers. With the BTLD, it is not necessary to

have UNIX already installed on a non-SCSI hard drive. Also, it is not

necessary to create a new SCSI N1 disk. The basic steps for accomplishing

this are outlined below. Further detail on UNIX installation can be found in

the SCO UNIX System Administrators Reference manual.

1. Link edit the BTLD into the UNIX kernel during installation.

a. Prepare the SCSI hard disk by deleting any existing partitions.

b. Insert the SCO UNIX N1 installation disk and reboot the system.

c. At the Boot: prompt type:

link

Press Enter.

d. At the next prompt type:

bscam

Do not remove the N1 diskette. Press Enter.

e. At the next prompt, replace the N1 diskette with the SCO SDMS Device

Drivers diskette (which contains the BTLD), and press Enter.

When using the BTLD to install UNIX, the CAM3 configuration tool is

automatically installed.

f. Continue from this point as a normal installation according to the SCO

UNIX documentation.

Note: Remember, the Basic and Extended Utilities packages must be loaded

when using the BTLD.

g. NCR recommends creating a second copy of UNIX.

Log in as root and issue the command:

cp /unix /unix.good

This creates a second copy of UNIX under the name /unix.good.

Note: If any problems arise with the Kernel being built in UNIX, this

procedure ensures that a valid copy of UNIX will be available.

To access this valid UNIX copy, type at the colon after the boot message:

unix.good

To replace the system UNIX with the valid UNIX, type at the system prompt:

cp /unix.good /unix

2. Install the NCR SDMS SCSI UNIX device drivers.

a. After rebooting the system, type:

mkdev cam3

Press Enter to continue.

b. The CAM3 main menu is displayed. To check that there are no SCSI

devices installed, press 7 and Enter. This will show the current SCSI

configuration. There should be no SCSI devices listed. If there are, press

5 for the option to remove all SCSI devices, then press y and Enter to

accept the removal.

c. From the CAM3 main menu, select option 1 to configure the NCR SCSI

device drivers. Three options are displayed:

1. Configure NCR Host Adapter Software

2. Remove NCR Host Adapter Software from System Configuration

3. Return to Main Menu

d. Press 1 and Enter to configure the NCR UNIX drivers, then press 1 and

Enter to configure the NCR host bus adapter software.

e. Each device driver in UNIX is assigned a number known as the major

device number. The major device number assigned to the host bus adapter is

displayed. Write this number down, as it may be required later, and keep

it with the UNIX documentation.

f. A prompt is displayed asking what interrupt lines will be used by the

host bus adapter. Interrupt lines associated with particular device

drivers must be identified to the kernel at build time. A list of

interrupt lines available is displayed. The SDMS SCO UNIX driver performs

dynamic interrupt mapping. Therefore, press 0 (zero) and Enter to exit

this process and continue.

Note: If the system indicates that the proper interrupt lines for the SCSI

host bus adapter are not available, review the section under Troubleshooting

concerning possible kernel rebuild problems and edit the sdevice entries to

eliminate the conflict.

3. Adding a SCSI Hard Drive to UNIX.

Note: If a SCSI hard disk drive will not be added to this system, go to

step 4.

a. From the CAM3 main menu, select option 2 to add or remove specific

devices. A list will appear showing the current configuration followed by

a menu of choices. To add the SCSI hard disk drive, select option 1 to

reconfigure Sdsk, and press Enter.

b. Read the explanatory text that appears and follow the prompts. Press

Enter at the next two prompts. A prompt will appear to add another SCSI

disk. Answer y and press Enter. Select option 2 to add a drive to the SCSI

controller and press Enter.

c. The system will prompt for the prefix of the SCSI host adapter that

supports this device. If the default is bhba, press Enter. If the default

is not bhba, type:

bhba

Press Enter.

d. The system will prompt for which SCSI host adapter supports the SCSI

hard drive. Press 0 (zero) and press Enter.

e. The system will prompt for the Target ID of the SCSI hard drive. Type

the ID number and press Enter.

f. The system will prompt for the LUN (Logical Unit Number) of the SCSI

hard drive. Press 0 (zero) and press Enter.

g. The system will prompt to update the SCSI Configuration file. Answer y

and press Enter.

h. A prompt appears to create a new kernel to effect the driver change.

Answer n (as this will be done later) and press Enter.

4. Adding a SCSI Tape Drive to UNIX.

Note: If a SCSI tape drive will not be added to this system, go to step 5.

a. From the CAM3 main menu, select option 2 to add or remove specific

devices. A list will appear showing the current configuration followed by

a menu of choices. To add the SCSI tape drive, select option 2 and press

Enter.

b. Instructions on using a SCSI tape drive are displayed. Press Enter at

the next three prompts to start the mkdev tape utility for installing a

tape drive.

c. The Tape Drive Configuration Program menu appears. Select option 1 to

install a tape drive, and press Enter.

d. The Tape Drive Installation Menu appears. Select option 4 to install a

SCSI tape drive, and press Enter. From the next list select the type of

SCSI tape drive to install, and press Enter.

e. The system will prompt to configure the tape drive. Press y and Enter.

f. The system will prompt for the prefix of the SCSI host adapter that

supports this device. If the default is bhba, press Enter. If the default

is not bhba, type:

bhba

Press Enter.

g. The system will prompt for which bhba SCSI host adapter supports the

tape drive. Press 0 (zero) and Enter.

h. The system will prompt for the Target ID of the controller of this

device (ID number of the tape drive). Type the correct ID number (two is

recommended) of the tape drive and press Enter.

i. The system will prompt for the LUN (Logical Unit Number) of the tape

drive. Press 0 (zero) and Enter. The system will then prompt to update the

SCSI configuration file. Answer y and press Enter.

j. A list will be displayed showing the special devices that have been

created. Press Enter, and a list is displayed showing the links to the

installed tape drive. When prompted for a boot string, press q and Enter

to return to the Tape Drive Configuration Program menu. Press q and Enter

to continue.

k. A prompt appears to create a new kernel to effect the driver change.

Answer n (as this will be done later) and press Enter.

5. Adding a SCSI CD-ROM Drive to UNIX.

Note: If a SCSI CD-ROM drive will not be added to this system, go to Step 6.

a. From the CAM3 main menu, select option 2 to add or remove specific

devices. A list will appear showing the current configuration followed by

a menu of choices. To add the SCSI CD-ROM drive, select option 3 and press

Enter.

b. Instructions on using a CD-ROM drive are displayed. Press Enter two

more times at the prompts to start the mkdev cdrom utility for installing

a CD-ROM drive.

c. The CD-ROM Configuration Program menu appears. Select option 1 to

install a CD-ROM drive and press Enter.

d. The system will prompt to configure the CD-ROM Drive. Answer y and

press Enter.

e. The system will prompt for the prefix of the SCSI host adapter that

supports this device. If the default is bhba, press Enter. If the default

is not bhba, type:

bhba

Press Enter.

f. The system will prompt for which SCSI host adapter supports the CD-ROM

drive. Press 0 (zero) and press Enter.

g. The system will prompt for the Target ID of the CD-ROM Drive. Type the

ID number and press Enter.

h. The system will prompt for the LUN (Logical Unit Number) of the CD-ROM

drive. Press 0 (zero) and press Enter.

i. The system will prompt to update the SCSI Configuration file. Answer y

and press Enter. A prompt appears to configure a CD-ROM/TAPE installation

device. Unless this is desired, answer n and press Enter. A prompt appears

to add a high-sierra file system. Answer y and press Enter. The system

will display the High Sierra/ISO9600 filesystem Configuration Program

menu. Select option 1 to add high-sierra and press Enter. The system

updates the SCSI Configuration File and returns to the CD-ROM

Configuration Program menu. Select q and press Enter to continue.

j. A prompt appears to create a new kernel to effect the driver change.

Answer n (as this will be done later) and press Enter.

6. Rebuilding the Kernel.

a. The system now returns to the CAM3 main menu. Select option 4 to relink

UNIX and press Enter.

b. A prompt will appear asking to rebuild the kernel. Answer y and press

Enter. The kernel will now be linked, which takes approximately 3 minutes.

Messages will appear while the kernel is linking.

c. When the Kernel has been rebuilt, the system will prompt as to whether

this kernel is to boot by default. Press y and Enter.

d. The system will prompt whether to rebuild the kernel environment. Press

y and Enter. When the CAM3 main menu appears, press q and Enter. At the

custom Main Menu, highlight Quit and press Enter.

e. To activate the new kernel the system must be rebooted. Type the

command:

reboot

Press Enter.

f. Press Enter at the Boot: prompt and the new kernel will load. If there

is a host bus adapter with an NCR ROM installed, the NCR device driver

will display a load message when the kernel loads.

g. Log in as usual.

* Trouble Shooting *

To solve some trouble shooting problems it is necessary to be aware of

certain files in the UNIX system. The directory etc/conf/cf.d contains the

files sdevice, mdevice, and mscsi. Two other files in this directory,

sdev.hdr and mdev.hdr, describe the values of the possible field entries in

sdevice and mdevice.

THE FOLLOWING CONDITIONS SHOULD EXIST AFTER A SUCCESSFUL INSTALLATION:

a. The directory etc/conf/sdevice.d should contain a file named bhba.

b. The bhba file should contain the bhba sdevice entry.

c. There should be a bhba entry for each SDMS SCSI controller configured in

the system in the sdevice file.

d. The second field for bhba in the sdevice file should be set to Y.

e. There should be a bhba entry in the mdevice file.

f. There should be a bhba entry for each SCSI device in the mscsi file.

g. The directory etc/conf/pack.d/bhba should exist and contain the files

Driver.o, RAMcore.h, and space.c.

AN INTERRUPT CONFLICT MESSAGE APPEARS.

a. In the sdevice file, check the fifth field of the bhba entry for a

conflict with that of any competing driver.

A SCSI DEVICE CANNOT BE FOUND.

a. Reboot to observe the system configuration table when it appears.

b. Look for bhba entries for each SCSI device connected to an NCR SCSI

controller.

c. Also, check to see that the desired value for each field exists.

UNIX SYSTEM BEHAVES UNRELIABLY AFTER A CONFIGURATION CHANGE.

a. When ever the hardware or software configuration is altered, the kernel

must be rebuilt. Use option four of the CAM3 configuration tool, or change to

the /etc/conf/cf.d directory and type:

./link_unix

to rebuild the kernel.

DURING KERNEL REBUILD, AN ERROR MESSAGE INDICATES THE CHARACTER DEVICE NUMBER

OF DEVICE cn CONFLICTS WITH THE CHARACTER DEVICE NUMBER OF bhba.

a. Reinstall the CAM3 software using the installpkg utility. Remember to

reload the bhba driver.

DURING INSTALLATION, THE ROOTDISK IS NOT FOUND, OR THE PARTITIONING OF THE

DISK FAILS.

a. If a SCSI tape drive is used in the initial installation:

The SCSI tape drive ID should be set to 2, and it should be configured to path 0, LUN 0.

The ID of the tape drive should not conflict with a SCSI hard disk ID.

b. Make sure no previous partitions exist on the root hard drive.