VideoLogic Apocalypse 3D for Windows 95

April 21st 1997

Contents

1. Introduction

2. Matrox update

3. Installing Apocalypse 3D

4. Removing Apocalypse 3D

5. Hints and tips

6. Troubleshooting

7. PowerVR D3D HAL Setup

1. Introduction

This file contains late-breaking information, detailed installation and removal instructions, hints and tips, system details and troubleshooting information for Apocalypse 3D.

You can print this file by clicking the File menu at the top of the window, then Print. The page

setup is for US Letter sized paper, but will also print to A4 size.

2. Matrox update

Introduction

At the time of the launch of the 1.0 PowerVR drivers there were two known problems with drivers running on the Matrox Millennium and Mystique graphics cards. These affected general full-screen Direct3D (D3D) operation and the running of certain applications which performed 2D/3D compositing, and were related to the operation of the D3D drivers only - PowerSGL applications were unaffected.

Full Screen Operation

In the initial release, there were problems with D3D titles hanging when they went into full screen. This was a synchronisation issue with the Matrox controller being unable to cope with PowerVR and host access at the same time. This has been fixed in the latest drivers.

2D/3D Compositing

2D/3D compositing affects D3D SDK samples like Tex4, Tex5, Oct1 and retail applications such as Hellbender and Monster Truck Madness. For these applications, the PowerVR drivers use the transparent Blit function of the    2D controller to render the images correctly. Unfortunately, Matrox cards behave differently from other graphics cards when perfoming transparent Blits . The side-effect in the initial release of the Apocalypse 3D driver is that images were rendered with magenta blocks.

This driver release provides an interim fix when running on Matrox cards, in which the PowerVR D3D HAL disables itself for those applications requiring 2D/3D compositing.

Work is still in progress to improve the operation of the PowerVR HAL with these Matrox graphics cards and VideoLogic are discussing a permanent technical solution with Matrox .

To overcome these problems, follow the steps below:

1.	To correct the hanging application problem, run the SGLHW.INI file by clicking Start, 	clicking Run, typing SGLHW.INI and clicking OK.

There is a section in the file that looks like this:

[default]

StrictLocks=0

Change the section to look like this:

[default]

StrictLocks=1

2.	To enable the 2D/3D compositing solution, add this line to the [PVRHAL] section:

MatroxFix=1

3.	Save SGLHW.INI file under the same name.

3. Installing Apocalypse 3D

Before Installing

Before starting the installation, work through the following checklist:

	Does your graphics card have 2 MB of RAM or more? If not, then Apocalypse 3D may run, but will not perform as expected, and you will not be able to run all Direct3D (D3D) applications.

	Does your motherboard have a free PCI slot which is bus-mastered (all PCI slots on modern PCI systems are bus-mastered)? If not, then Apocalypse 3D is not compatible with your system.

	Is your PCI bus running higher than 25 MHz (i.e. not running an Intel Pentium 75 or a 
Cyrix P120)? If not, then contact VideoLogic through one of the contact points listed at the end of the User Guide.

Manually installing Apocalypse 3D

1.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

2.	Remove the computer’s cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

3.	Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

4.	Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

5.	Align Apocalypse 3D with the slot you have chosen. Firmly press it into the slot. You will need to use some pressure to push the card into position, but do not use excessive force.

6.	Secure the card using the screw that you removed previously.

7.	Replace the cover of your computer and switch on your computer and your computer monitor.

8.	When your computer has restarted, a dialog box is displayed, telling you that Windows has found new hardware. Click Driver Provided by the Manufacturer, and click OK.

- Another dialog box is displayed, asking for the location of the software. Browse to the location of the drivers. After browsing, click OK.

10.	You are now ready to install the drivers. Click OK.

Two sets of drivers are installed, for PowerSGL and Direct3D.

PowerSGL is VideoLogic’s proprietary 3D language. Direct3D is Microsoft’s 3D graphics language. These drivers enable you to optimally run applications based on either language.

11.	When the drivers have been installed, a message is displayed asking whether you want to restart Windows. Click Yes.

12.	If your graphics card uses the S3 968 or 868 chipset, and your PC has the AWARD BIOS, see the AWARD BIOS section below before continuing (your PC tells you whether it has the AWARD BIOS when you switch it on, and your PC’s documentation should also tell you whether your PC uses it).

13.	Install MechWarrior 2. At the end of the installation, when you are asked whether or not to install DirectX 2, reply Yes.

14.	Install ULTIM@TE RACE.

15.	Install DirectX 3, by running DXSETUP.EXE from the \DXIII\REDIST\DIRECTX folder of the Apocalypse 3D CD-ROM. If you had DirectX installed on your machine already, click Yes when asked whether you want to replace the old files with the new, unless you are certain that your current drivers are DirectX 3 compliant. Don’t worry if your graphics drivers are not listed as ‘certified’.

16.	Set your graphics display up in 800 x 600 mode running 65 thousand colors (16-bit graphics). You can change this later, but it is the simplest mode for ensuring that everything is working.

17.	From the \DEMOS\POWERSGL folder of the Apocalypse 3D CD-ROM, run the Tower4, Tower5 and Ants demos. Note that the ship in Ants will spiral unless a joystick is attached. You can fly the ship by pressing button 1 on your joystick. Press F12 to exit.

18.	From the \DEMOS\D3D folder of the Apocalypse 3D CD-ROM, run TWIST.EXE. Verify that under the File menu, ‘Direct3D HAL’ is checked.

19.	Run MechWarrior 2.

20.	Run ULTIM@TE RACE.

Well done - you’ve successfully installed your Apocalypse 3D, and you are now ready for state-of-the-art gameplay.

Apocalypse 3D with the AWARD BIOS

If you have the AWARD BIOS and the S3 968 or 868 chipset, your PC may crash when using Apocalypse 3D. You can solve the problem by changing the memory allocation for the Apocalypse 3D card to a range outside the 64 MB used by your graphics card.

To change the memory allocation:

1.	Click the Start button, point to Settings and click Control Panel.

2.	Double-click the System icon.

3.	In the System Properties dialog box, click the Device Manager tab.

4.	Double-click Sound, video and game controllers.

5.	Click Apocalypse 3D.

6.	Click Properties.

7.	In the Apocalypse 3D dialog box, click the Resources tab.

8.	Clear the Use automatic settings check box.

9.	In the Resource settings list, double-click the first (top) Memory Range.

10.	In the Value box of the Edit Memory dialog box, type in the range FFBE0000 - FFBEFFFF, and click OK.

11.	In the Resource settings list, double-click the second (bottom) Memory Range.

12.	In the Value box of the Edit Memory dialog box, type in the range FF400000 - FF7FFFFF, and click OK.

13.	At the Apocalypse 3D dialog box, click OK.

14.	At the System Properties dialog box, click OK.

15.	Restart your PC.

You should not have to reconfigure the memory settings again as Windows 95 will remember the new settings every time it starts up and reallocate the memory automatically.

Memory Allocation Conflicts

The above steps assume that there is no memory allocation conflict due to devices present at the addresses you specified. To ensure there is no conflict, follow steps 1 to 7 above. If, in the Resources tab, the Conflicting devices list says ‘No conflicts.’, your Apocalypse 3D is ready to use. If a conflict is indicated, contact our technical support team via one of the contact points given in the User Guide.

Upgrading the drivers

From time to time, VideoLogic may issue upgrades to the drivers. If this happens, follow the instructions below to install them on your PC.

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	From the Control Panel, double-click System. The System Properties dialog box is displayed.

3.	Click the Device Manager tab. A list of devices available on your PC is displayed.

4.	From the list, double-click Sound, Video and game controllers. Under Sound, Video and game controllers, the driver for Apocalypse 3D is displayed.

5.	From the list, double-click Apocalypse 3D. The PCI Properties dialog box is displayed.

6.	Click the Driver tab. A list of driver files is displayed.

7.	Click Change driver. The Select Device dialog box is displayed.

8.	Click Have Disk. The Install from Disk dialog box is displayed.

9.	Browse to the root directory of the disk containing the Apocalypse 3D updates. When you have browsed, click OK.

10.	At the Install from Disk dialog box, click OK. At the Select Device dialog box, click OK. At the PCI Properties dialog box, click OK. The drivers are upgraded on your PC, which may take a short while.

11.	When the drivers have been upgraded, a message is displayed asking whether you want to restart Windows. Click Yes.

When your computer has restarted Apocalypse 3D is ready for use.

4. Removing Apocalypse 3D

You remove the card from Windows 95 in two stages: first the hardware, then the software.

To remove Apocalypse 3D from your system:

1. 	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2. 	Double-click the System icon to open the System Properties Window.

3.	Click the Device Manager tab.

4.	Double-click Sound, video and game controllers.

5.	Click Apocalypse 3D, and click Remove.

6.	A dialog box is displayed asking you if you want to restart Windows. Click No.

7.	Shut down Windows 95.

8.	Switch off your computer, monitor, and any hardware devices such as printers 
connected to your computer. Leave the power cable connected to the power outlet 
so that your computer is grounded.

9.	Disconnect the monitor cable from your computer.

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

10.	Remove the computer’s cover. You may need to refer to the documentation supplied 
with your computer for instructions on how to do this.

11.	Remove the Apocalypse 3D card from its PCI slot.

12.	Replace the cover of your computer.

13.	Switch on your computer and your computer monitor.

14.	When Windows 95 is running you can delete the following files from the disk on which you are running Windows 95:

PowerSGL files	Direct3D files

\windows\sgl.dll    		\windows\system\pvrhal32.dll

\windows\sglmid4.dll 		\windows\system\pvriload.exe

\windows\system\vsgl.vxd		\windows\system\pvrwin32.dll

\windows\inf\midas.inf		\windows\system\pvrthk16.dll

\windows\system\pvrhelp.hlp

5. Hints and Tips

Introduction

This section gives late-breaking information on how to get the most out of your Apocalypse 3D.

Installing games

The majority of DirectX games currently on the market are supplied with DirectX 2. If the game’s installation forces DirectX 2 to be installed, reinstall DirectX 3 immediately afterwards.

Microid Research BIOS

Some PCs with an MR BIOS can give a much lower frame rate with Apocalypse 3D. Switching to another BIOS can greatly enhance performance.

PCI Burst

Some PC BIOSes allow the user to alter the settings for ‘PCI Burst’. Enabling this feature enhances Apocalypse 3D's performance.

PCI Latency

Some PC BIOSes allow the user to alter the settings for ‘PCI Latency’. Other typical names for this feature are:

‘Latency Timer (PCI Clocks)’	A term used with AMI BIOS, usually found in the ‘Advanced chipset configuration’ menu.

‘PCI Latency Timer’ 		A term used with AWARD BIOS, usually found in the ‘PNP and PCI setup’ menu.

Setting this to 80 PCI clocks can enhance Apocalypse 3D's performance.

16-bit Color

To avoid problems with the Start menu and Windows menus containing corrupted text, or missing text, you should set your display mode to a high-color palette, of at least 65,536 colors (16-bit).

6. Troubleshooting

Introduction

This section describes common problems you may encounter when installing or running Apocalypse 3D.

System compatibility

Before going through the troubleshooting section, make sure that your system can run Apocalypse 3D. Verify the following:

- Does your graphics card have 2 MB of RAM or more? If not, Apocalypse 3D may run, but not optimally, and you will not be able to run all Direct3D (D3D) applications.

- Does your motherboard have a free PCI slot which is bus-mastered (all PCI slots on modern PCI systems are bus-mastered)? If not, then Apocalypse 3D is not compatible with your system.

- Is your PCI bus running higher than 25 MHz (i.e. not running an Intel Pentium 75 or a Cyrix P120)? If not, then contact VideoLogic through one of the contact points listed at the end of the User Guide.

Games graphics display a ‘halo’ around objects

The application may have problems with 2D/3D compositing. Run the PowerVR HAL setup utility, described in section 6 of this readme. For other graphics problems, access our web site for the latest version of our D3D drivers at http://www.videologic.com/VLSOFT/mmedia.htm.

I am not given the D3D Hal option with the D3D demos

Ensure that you have the DirectX III graphics drivers installed, and that your PC is in a display mode greater or equal to 16 bits per pixel.

The application display is corrupted

Your PC may not be fully DirectDraw compliant.

The application looks fuzzy, with lines appearing alongside the active window

The resolution of the window in which the application is running is too high, or the refresh rate of the screen is too high. Lower the resolution or the refresh rate in Display Properties.

The Start menu and Windows menus contain corrupted text, or text is missing

After playing a game in 8-bit display mode, the PC has changed from 8-bit to 16-bit display mode automatically, which the display driver may not be able to support. Restart the PC.

To avoid this problem in future, your PC should be in a 16-bit display mode before playing a game.

When running an application that uses PowerSGL or Direct3D, the system freezes or restarts

1.	Apocalypse 3D may be installed in a PCI bus slot that does not have bus-mastering capability.

2.	The PCI bus speed may be too slow. Apocalypse 3D cannot work in systems where the PCI bus speed is set to 25MHz (e.g. Intel Pentium 75 and Cyrix 6x86 P120)

When running an application that uses PowerSGL the system restarts

The display driver may not support buffer flipping. To support flipping the amount of free off-screen memory must be greater than the current on-screen memory. A lower screen resolution may free up sufficient memory, however, due to other graphics memory requirements (e.g. hardware cursors), 1 MB cards may not support flipping even in a resolution of 640x480 pixels with 65 thousand colors. This is why we recommend a minimum graphics memory of 2 MB.

The Use Hardware Accelerator option within an application is greyed out

1.	The application is currently set to run in a graphics resolution that cannot be accelerated by Apocalypse 3D. Setting the application’s graphics mode to a lower resolution mode makes the check-box available.

- Apocalypse 3D is not being recognised by the system. Check that it is in System Properties, check the BIOS, IRQ allocations etc.

Specific Applications

# Die-Hard Trilogy

The Die-Hard Trilogy title can use Apocalypse 3D for acceleration, but may suffer from random lockups during gameplay when using 512x384, 640x480 resolutions.

To avoid this, select 640x400 or 800x600 resolutions.

3DFX Wizmark lite 3D Benchmark

The Wizmark lite benchmark program fails to execute and reports the following error messages:

- CreateSurface for texture failed (loadtex). The pixel format was invalid as specified.

- Couldn’t download texture.

- Couldn’t download texture: “Wiz1\w1-urn-1.3df”

- DDHELP caused an invalid page fault in module <unknown> at 00de:01410000

Trying to execute subsequent D3D or SGL applications will either run with no D3D HAL option available, or report that there is a problem with the file SGL.DLL.

This happens because Wizmark lite uses a pixel format that is not supported by Apocalypse 3D. To allow the benchmark program to execute, the D3D part of the driver converts the pixel format from 565 to 555.

To enable this function, add the following lines to the SGLHW.INI file located in the \windows directory:

[PVRHAL]

Enable565=1

Remove these lines after running the benchmark.

7. PowerVR D3D HAL Setup

You can access the setup utility by clicking the PowerVR icon at the bottom right corner of your screen.

When you select PowerVR D3D HAL Setup, this window is displayed:

This utility enables the graphics of some D3D (PowerVR-enhanced) games to work optimally with PowerVR technology. You must be in 16 bits per pixel to run the utility.

Some games display a halo of solid color around objects, such as in the example below:

To correct this problem, follow the steps below.

If your game is listed in the dialog box

1.	Click the game's description to turn the 2D/3D Compositing setting on.

2.	Click Exit.

3.	Restart the game.

If your game is not listed in the dialog box

1.	Click Add. The add new game dialog box is displayed.

2.	In the Application column, type in the name of the executable file for the game, in the form app_name.exe

If you are not sure of this filename, the best way to find it is through the Find facility (click the Start button and point to Find). The filename usually has the extension .EXE or .COM. For example, if your game were called Chess, you would search for Chess.EXE on your hard drive. If this did not work, you would search for Chess.COM

3.	In the Description column, type in a brief description of the game, in capitals or lower-case including symbols and digits, up to a maximum of 35 characters.

4.	Click OK. The PowerVR D3D Setup dialog box is displayed.

5.	Click Exit.

6.	Restart the game.

If you want to modify an existing game's entry

1.	Highlight the game's application name, and click Edit. The edit game dialog box is displayed.

2.	In the Description column, type in a brief description of the game, in capitals or lower-case including symbols and digits, up to a maximum of 35 characters.

3.	Click the 2D/3D Compositing box to turn the utility on.

4.	Click OK. The PowerVR D3D Setup dialog box is displayed.

5.	Click Exit.

6.	Restart the game.