VideoLogic Captivator PCI/TV for Windows 95

23rd January 1997.

Contents

1. Introduction

2. Hints and tips on using Captivator PCI/TV

3. Installing Captivator PCI/TV

4. Removing the software

5. Setting IRQ values

6. Troubleshooting

7. Field Notes

1. Introduction

This file contains late-breaking information, advice on removing software, troubleshooting information, and general notes on the Captivator PCI/TV.

You can print this file by clicking the File menu at the top of the window, then Print. The page setup is for US Letter sized paper, but will also print to A4 size.

If you are reading this file outside of the VideoLogic Installer and want to install the Captivator PCI/TV manually, refer to section 3 below or the User Guide for complete instructions on how to do this.

2. Hints and tips on using Captivator PCI/TV

Introduction

This section gives general information on how to get the most out of your Captivator PCI/TV.

Vidcap32

Vidcap32 is a 32-bit version of Video for Windows’ Vidcap video capture application. You can find a copy of Vidcap32 on the VideoLogic Software Library CD-ROM, in the Utils\Vidcap32 folder.

Prolab multimedia

To capture still frames and video with Captivator PCI/TV, install Prolab Videowork software which you can find on the VideoLogic CD-ROM. Videowork also enables you to edit captured AVI clips. Prolab Imagefolio, also on the CD, enables you to edit the still frames you have captured with Videowork.

Vidcall conferencing

We have provided you with a free copy of Vidcall Video Conferencing software, also available on the CD-ROM, in the Utils\Vidcall folder. To use Vidcall with Captivator PCI/TV, Vidcall has to be configured for "AVI (Overlay)". See the Vidcall readme for further information.

Video display modes

You can display video both in overlay and in preview mode.

Overlay mode displays the live video with the best quality and in real-time. Preview mode displays captured uncontiguous frames, and its display speed is influenced by the performance of your PC. Capturing video in preview mode can pause the displaying of video. If you want to view video during the capturing process, set the display to overlay mode.

To get the best quality live video display and capture quality you should run in a 16-bit display mode.

DirectDraw compatibility

Captivator PCI/TV requires a Windows 95 DirectDraw compatible display driver. If Captivator PCI/TV cannot detect a DirectDraw-compatible display driver, launching a live video display will result in the message "BT848: No suitable DirectDraw provider - Overlay channel is disabled". In this case you will not be able to display live video in overlay mode.

If your display driver is DirectDraw compatible the display mode currently used probably does not support DirectDraw. Check in the graphics card documentation which display modes are supported, and change to an appropriate display mode. If your graphics card does not support DirectDraw, contact your graphics card manufacturer for updated drivers.

3. Installing Captivator PCI/TV

Introduction

This section explains how to install the Captivator PCI/TV hardware and Windows 95 software. Installing Captivator PCI/TV is a simple operation that should take no more than 15 minutes.

Handling Captivator PCI/TV

Follow these guidelines to minimize the risk of electrostatic charges damaging the card:

Keep the card in its protective packaging until you are ready to install it.

Just before handling the card, touch the metal frame of your computer with the back of your hand to discharge any static electricity which may have built up on you or your clothes.

Hold the card by the adapter bracket or by its edges; do not touch the printed circuit board.

Do not place the card on top of your computer or on any other metal surface.

Make sure that the card is not accidentally touched by anyone else.

Handle the card with care to avoid damage to the fragile electronic components mounted on the circuit board.

Conventions used in this readme

This readme describes installation of both Captivator PCI and Captivator PCI TV. When describing features common to both products, the readme refers to Captivator PCI/TV.

Installing the hardware

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

1.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

2.	Remove the cover from your computer. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

Note: If you are using Captivator PCI TV and you intend to connect a TV input to the tuner, install it as far away as possible from the other cards in your computer. All computers and cards generate radio frequency noise and this noise can affect the quality of your TV picture.

3.	Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

4.	Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

5.	Align Captivator PCI/TV with the slot you have chosen. Firmly press it into the slot, as shown in the diagram below. You will need to use some pressure to push the card into position, but do not use excessive force.

6.	Secure the card using the screw that you removed previously.

Note: If you are installing Captivator PCI TV, make sure that you fit all the adaptor card fixing screws and expansion slot covers to all unused slots to minimize radio frequency noise.

7.	Replace the cover of your computer.

8.	Switch on your computer and your computer monitor.

9.	When your computer has restarted, you are ready to install the software.

Installing the Captivator PCI/TV software

1.	When your computer has restarted, a dialog box is displayed saying that Windows has found new hardware. Click Driver from Disk provided by Hardware Manufacturer, 
and click OK.

2.	Insert the VideoLogic Software Library CD into your CD-ROM drive.

3. 	Browse to the folder [drive]:\captipci\disk1  where [drive] is your CD-ROM drive. When you have browsed, click OK.

4.	At the Install From Disk dialog box, click OK. The drivers are copied from the CD.

5.	A message is displayed notifying you of the change to your system’s settings, and asking whether you want to restart the computer. Click Yes.

6.	Follow the instructions that appear on the screen.

Multimedia and Videoconferencing software

The Prolab multimedia software consists of three applications: Videowork, for multimedia video editing; MediaScope, for playing virtually any kind of CD that your system supports; and ImageFolio, for processing scanned images. Each application has its own folder within the Prolab folder, and is installed by running the install.exe file in the application’s disk1 folder.

The Vidcall conferencing utility is located in the utils\vidcall folder, and is zipped using Winzip. Unzip the file, and read the file readme.dem for instructions on installation and usage.

Connecting the external cables

Warning: Permanent or temporary hearing loss or impairment may result from incorrectly running sound systems with headphones. Before using a set of headphones, you should set the volume while holding them away from your ears to avoid injury from unexpected noise.

Note: All cables connected to the card should be properly shielded and comply with radio frequency interference standards.

TV, satellite or cable in

Captivator PCI TV is available in the following variants using the Philips 1200 series TV tuner:

CountrY	Standard 	RF connector	Part No.

UK/Ireland/Hong Kong	PAL I	IEC	VL 60025

European except France	PAL B/G	IEC	VL 60026

North America	NTSC M	F-Type	VL 60024

Audio out

The 3.5 mm jack on Captivator PCI TV can be connected to an amplifier, powered speakers, or personal stereo headphones.

Composite and S-Video in

Captivator PCI and Captivator PCI TV accept video in Composite and S-Video format in the NTSC, PAL or SECAM video standard. Only one of the inputs can be used at one time.

The phono connector accepts video in Composite format only.

The 8 pin mini-DIN connector accepts S-Video or Composite format, and comes with an 8-pin to 4-pin adapter.

4. Removing the software

Introduction

This section explains how to remove the Captivator PCI/TV software.

Removing the drivers

To remove the drivers, follow the steps below:

1.	Click the Start button, and point to Settings.

2.	Click Control Panel.

3.	In Control Panel, double-click the System icon.

4.	In the System Properties window, click the Device Manager tab.

5.	In the list of devices displayed, double-click Sound, video and games controllers.

6.	Click VideoLogic Captivator PCI.

7.	Click Remove.

8.	A dialog box is displayed asking you to confirm that you want to remove this device from your System. Click OK.

Removing the driver files

To remove the driver files that have been installed by Captivator PCI/TV, delete the following files from the Windows\ System folder:

bt848.drv

bt848.vxd

bt848_16.dll

bt848_32.dll

bt848ddi.dll

bt848dlg.dll

btvvc.drv

capinter.dll

5. Setting IRQ values

Introduction

This section explains how to overcome potential IRQ conflicts.

Captivator PCI/TV is plug & play compatible. Some PC BIOSes might assign an interrupt to the PCI slot used by Captivator PCI/TV which is in fact already used by another device. The typical symptoms of an IRQ conflict are:

- the system locking up when trying to use Captivator PCI/TV

- an error message about missing drivers being displayed when trying to use Capativator PCI/TV

- Windows 95 Device Manager reporting a conflict

- another device not working properly after Captivator PCI/TV is installed

To check the interrupts allocated to Captivator PCI/TV and other devices, follow the steps below:

1.	Click the Start button, and point to Settings.

2.	Click Control Panel.

3.	In Control Panel, double-click the System icon.

4.	In the System Properties window, click the Device Manager tab.

5.	In the Device Manager page, click Properties.

6.	In the Computer Properties window, click the View Resources tab.

7.	In the View Resources page, click the IRQ option.

You can change the PCI-IRQ configuration of your system in the PCI BIOS setup. More information on how to change settings should be available from your PC documentation.

6. Troubleshooting

Introduction

This section describes common problems you may encounter when installing or running Captivator PCI/TV.

System crash

If your system locks up when trying to use Captivator PCI/TV, you may have an IRQ conflict. Refer to section 4 above.

Missing Drivers

If an error message is displayed about missing drivers or files when trying to use Captivator PCI/TV, then you may not have fully installed the drivers. Refer to the installation guide to reinstall the drivers.

If the problem continues, you may have an IRQ conflict, in which case refer to section 4 above.

Live video

Magenta vertical bar

A magenta vertical bar can appear at the left side of the desktop when displaying live video. This can happen with S3 based graphics cards, when the live video window is totally covered by the Start menu or program menu. Move the live video window to a different position, so that, when using the Start menu or program menu, at least part of the live video window is left unobscured. This problem does not occur when using 32-bit capturing applications so another solution would be to use Vidcap32, as described above.

No video displayed

No live or S-video is displayed if you have the wrong video source selected. Select the right video source, e.g. composite.

Bad colors

If your system is running in an 8-bit display mode then colors may be poorly defined. Change to a 16-bit or higher display mode.

Lines of corruption in lower part of live video

When displaying preview NTSC video at resolutions of 720x540 or 680x510, you might see some corrupted lines at the bottom of the live video window. Either display the video in overlay mode, or change to a smaller video window size.

Dropped frames

If live video is not displayed in real-time or frames are dropped, you are displaying the live video in preview mode Change to overlay mode.

Error message: "No suitable DirectDraw provider"

When displaying live video this message may appear, and the live video display will default to preview mode. You are running in a display mode which does not support DirectDraw. Change to a DirectDraw-supported display mode.

If the problem recurs, then your current graphics display driver does not support DirectDraw. Contact VideoLogic at one of the contact points described in the installation guide for an updated driver.

7. Field notes

Introduction

This section gives a summary of VideoLogic field notes. Field notes are frequently asked questions, troubleshooting information, late-breaking information or general comments about Captivator PCI/TV behavior under certain circumstances. Where possible a software patch has been provided.

Field notes file format

All of the field notes are in Adobe Acrobat format, and have the extension .PDF. To read these files you must first install the Adobe Acrobat reader which is supplied on the CD. It is located in the directory [drive]:\reader\win95 where [drive] is your CD drive. When you have installed the reader, double-click the .PDF file to read the field note, or open it from within the reader.

All of the patch descriptions are in DOS .TXT format.

Field notes folders

All the notes are located in their own folder within the UPDATES folder on the CD, so that, for example, field note 230 is in the folder Cappci\UPDATES\ FN0230. Patches are in their own folders within the field note folders.

Field Note	Summary	Patch

FN0230	Apocalypse 3D and/or Captivator PCI

- Award BIOS and S3 968

FN0233	Video capture and graphics systems

FN0234            Customising default video settings for installation