VideoLogic GrafixStar 450 for Windows 95

30th April 1997

Contents

1. Introduction

2. GrafixStar 450 display modes

3. The GrafixStar 450 drivers

4. Installing GrafixStar 450

5. Troubleshooting

6. Support Notes

1. Introduction

This file contains installation instructions, late-breaking information, troubleshooting information, and notes on the GrafixStar 450.

If you are reading this file outside of the VideoLogic Installer and want to install GrafixStar 450 manually, refer to section 3 below or the User Guide for complete instructions on how to do this.

2. GrafixStar 450 Windows 95 display modes

| Resolution | Colors | 2 MB Refresh Rates (Hz) | 4 MB Refresh Rates (Hz) |
| --- | --- | --- | --- |
| 640x480 | 256 | 60,72,75,85,90,100,120,150 | 60,72,75,85,90,100,120,150 |
|  | 65k | 60,72,75,85,90,100,120,150 | 60,72,75,85,90,100,120,150 |
|  | 16.7m | 60,72,75,85,90,100,120,150 | 60,72,75,85,90,100,120,150 |
| 800x600 | 256 | 56,60,72,75,85,90,100,120 | 56,60,72,75,85,90,100,120 |
|  | 65k | 56,60,72,75,85,90,100,120 | 56,60,72,75,85,90,100,120 |
|  | 16.7m | 56,60,72,75,85,90,100,120 | 56,60,72,75,85,90,100,120 |
| 1024x768 | 256 | 43,60,70,72,75,80,85,90,100 | 43,60,70,72,75,80,85,90,100 |
|  | 65k | 43,60,70,72,75,80,85,90,100 | 43,60,70,72,75,80,85,90,100 |
|  | 16.7m |  | 43,60,70,72,75,80,85,90,100 |
| 1152x864 | 256 | 60,70,72,75,80,85,90 | 60,70,72,75,80,85,90 |
|  | 65k | 60,70,72,75,80,85,90 | 60,70,72,75,80,85,90 |
|  | 16.7m |  | 60,70,72,75,80,85,90 |
| 1280x960 | 256 | 60,64,70,75 | 60,64,70,75 |
|  | 65k |  | 60,64,70,75 |
|  | 16.7m |  | 60,64,70,75 |
| 1280x1024 | 256 | 43,60,72,75 | 43,60,72,75 |
|  | 65k |  | 43,60,72,75 |
|  | 16.7m |  | 43,60,70,72,75 |
| 1600x1200 | 256 | 48,60 | 48,60 |
|  | 65k |  | 48,60 |

Refresh rates in bold are not available with the GrafixStar 450LC version

3. The GrafixStar 450 drivers

Introduction

This section describes the display modes available with overlay video acceleration, and how GrafixStar 450 interacts with DirectDraw.

Display modes with video acceleration

The GrafixStar 450 uses an intelligent graphics engine which provides true-color video playback even when the display is set to 256 colors. This means that you don’t have to sacrifice speed and functionality by being in a true-color display mode; instead, you can play video clips in true-color while using a 256 or 65k color mode. However this is NOT available in all display modes. The modes in which this is possible are listed below and are only achievable depending upon the video clip size and player component (DirectVideo, ActiveMovie, or Xing MPEG player) used.

Display size	Colors	Refresh rate (Hz)

640 x 480	256	60, 72, 75, 85

800 x 600	256	56, 60, 72, 75, 85

1024 x 768	256	43i, 60, 70, 75, 85

1152 x 864	256	60

640 x 480	65,536	60, 72, 75, 85

800 x 600	65,536	56, 60, 72, 75, 85

1024 x 768	65,536	43i, 60, 70, 75, 85

640 x 480	16.7m	60, 72, 75, 85

800 x 600	16.7m	56, 60, 72, 75, 85

1024 x 768	16.7m	43i, 60

All other modes display AVI and MPEG files using software only, which gives poorer quality playback.

Using games with DirectDraw

GrafixStar 450 display drivers for Windows 95 are compatible with DirectDraw 2.0.

4. Installing GrafixStar 450

Introduction

The easiest way to install your GrafixStar 450 is to install the hardware first, referring to the User Guide, and then install the software using the Interactive Installation Guide which runs automatically when you insert the CD into your PC. However, if you don’t have a User Guide, or you want to install the software manually, you should refer to the following instructions.

Handling GrafixStar 450

Follow these guidelines to minimize the risk of electrostatic charges damaging the card:

Keep the card in its protective packaging until you are ready to install it.

Just before handling the card, touch the metal frame of your computer with the back of your hand to discharge any static electricity which may have built up on you or your clothes.

Hold the card by the adapter bracket or by its edges; do not touch the printed circuit board.

Do not place the card on top of your computer or on any other metal surface.

Make sure that the card is not accidentally touched by anyone else.

Handle the card with care to avoid damage to the fragile electronic components mounted on the circuit board.

Installing the hardware

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

1.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

2.	Disconnect your monitor cable from your computer.

3.	Remove the computer’s cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

4.	Remove your existing display card or disable your built-in graphics controller. Refer to the documentation supplied with your computer for instructions on how to disable a built-in graphics controller.

5.	Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

6.	Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

7.	Align GrafixStar 450 with the slot you have chosen. Firmly press the card into the slot. You will need to use some pressure to push the card into position, but do not use excessive force.

8.	Secure GrafixStar 450 using the screw that you removed previously.

9.	Replace the cover of your computer.

10.	Connect the monitor cable to GrafixStar 450. All cables connected to GrafixStar 450 should be properly shielded and comply with radio frequency interference standards.

11.	Switch on your computer and your computer monitor. You are now ready to install the software.

Installing the software

The easiest way to install the software is to use the Display Properties window. Installation varies slightly depending upon which build of Windows 95 you are using. Use the following steps to find out which build of Windows 95 you are using, then follow the instructions for that version.

1. 	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2. 	Double-click the System icon to open the System Properties Window.

3. 	Ensure that the General tab is selected. This page has details about the System, Registration information, and Computer details. The build number is located in the System section, under the 'Microsoft Windows 95' line.

Software installation for Windows 95 build 4.00.950 or 4.00.950a

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Change Display Type button.

5.	Make sure that the Monitor Type line contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

6.	Click the Change button for the Adapter Type.

7.	Insert the GrafixStar 450 drivers disk into your computer and click Have Disk from the Select Device menu.

8.	Click GrafixStar 450 GrafixMAX from the list. Click OK.

9.	When the GrafixStar files have been copied to your PC, the Change Display Type window is displayed again. Click Close.

10.	The Display Properties windows is displayed again. Click Apply.

11.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

Software installation for Windows 95 build 4.00.950 B (otherwise known as OSR2)

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Advanced Properties button.

5.	Click the Monitor tab. The Monitor page is displayed.

6. 	Make sure that the Monitor page contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

7. 	Click the Adapter tab. The Adapter page is displayed.

8.	Click the Change button to change display driver.

9.	Insert the GrafixStar 450 drivers disk into your computer and click Have Disk from the Select Device menu.

10.	Click GrafixStar 450 GrafixMAX from the list. Click OK.

11.	When the GrafixStar files have been copied to your PC, the Advanced Display Properties window is displayed again. Click Close.

12.	The Display Properties windows is displayed again. Click Apply.

13.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

5. Troubleshooting

Introduction

This section describes common problems you may encounter when running GrafixStar 450.

Monitors

Display sizes

Windows 95 does not restrict the generic SVGA monitor settings to a given set of display sizes, so SmartTools allows any display size to be selected. If you have a standard monitor selected, there is therefore a possibility that you could zoom out to a display size that your monitor does not support.

Choose the correct monitor type, or contact your monitor supplier for more information.

Corrupt or blank display

Windows 95 does not restrict the generic SVGA monitor settings to a given set of refresh rates, so SmartTools allows any refresh rate to be selected. If you have a standard monitor selected, there is therefore a possibility that you could restart your system with a refresh rate that your monitor does not support. This usually results in a corrupted or black display in Windows 95. To rectify this problem, restart Windows 95, holding down the CTRL key on your keyboard. This starts Windows 95 in the lowest possible refresh rate for the resolution selected.

Connectivity - Feature connector

Feature connector

If you select Feature Connector from the Advanced page of Display Properties menu, your CONFIG.SYS file is modified to include the TSR VLENABLE.SYS. VLENABLE is a utility that enables DAC snooping when you turn on your PC.

If your system does not have a CONFIG.SYS file, or it has one that is zero bytes in size, you must create a file called CONFIG.SYS at the root of your hard disk and make sure that it contains some data (a rem line is sufficient), before you select Feature Connector.

When you have selected Feature Connector you must restart your system to run VLENABLE.

If you uncheck the Feature Connector option, the VLENABLE.SYS line in the CONFIG.SYS file will be disabled by changing it to a rem line.

6. Support Notes

Introduction

This section gives a summary of VideoLogic support notes. Support notes contain frequently asked questions, troubleshooting information, late-breaking information or general comments about GrafixStar 450. Where possible software patches have been provided.

Support notes file format

All of the support notes are in Adobe Acrobat format, and have the extension .PDF. To read these files you must first install the Adobe Acrobat reader which is supplied on the CD. It is located in the directory [drive]:\reader\win95 where [drive] is your CD drive. When you have installed the reader, double-click the .PDF file to read the support note, or open it from within the reader.

All of the patch descriptions are in DOS .TXT format.

Support notes folders

All the notes are located in their own folder within the UPDATES folder on the CD, so that, for example, support note 248 is in the folder GS450\UPDATES\SN0248. Patches are in their own folders within the support note folders.

Support Note	Summary	Patch

SN 248	        General protection fault with Quickres				    248