VideoLogic GrafixStar 600 for Windows 95

20th December 1996.

Contents

1. Introduction

2. The GrafixStar 600 drivers

3. Installing GrafixStar 600

4. Troubleshooting

1. Introduction

This file contains installation instructions, late-breaking information, troubleshooting information, and notes on the GrafixStar 600.

2. The GrafixStar 600 drivers

Introduction

This section describes the display modes available with overlay video acceleration, and how GrafixStar 600 interacts with DirectDraw.

Display modes with video acceleration

The GrafixStar 600 uses an intelligent graphics engine which provides true-color video playback even when the display is set to 256 colors. This means that you don’t have to sacrifice speed and functionality by being in a true-color display mode; instead, you can play video clips in true-color while using a 256 or 65k color mode.

Display size	Colors	Refresh rate (Hz)

640 x 480	256	60, 72, 75, 85, 90, 100, 120, 150

800 x 600	256	56, 60, 72, 75, 85, 90, 100, 120

1024 x 768	256	60, 70, 72, 75, 80

1152 x 864	256	60

640 x 480	65,536	60, 72, 75, 85, 90, 100, 120, 150

800 x 600	65,536	56, 60, 72, 75, 85, 90, 100

1024 x 768	65,536	60, 70

640 x 480	16.7m	60, 72, 75, 85, 90, 100, 120

800 x 600	16.7m	56, 60, 72, 75, 85

All other modes display AVI files using software only, which gives poorer quality playback. If you want to playback MPEG files without special hardware, you can obtain a FREE copy of Xing MPEG Player by returning your GrafixStar 600 registration card.

Using games with DirectDraw

GrafixStar 600 display drivers for Windows 95 are compatible with DirectDraw 2.0.

Refresh rate control for DOS-based applications

We have provided a refresh rate control utility called CONTROL.COM. This is located in the folder [drive]:\GS600\OTHERS\REFRESH where [drive] is your CD-ROM drive. For further information about this utility, read the file readme.txt in the REFRESH folder.

3. Installing GrafixStar 600

Introduction

The easiest way to install your GrafixStar 600 is to install the hardware first, referring to the User Guide, and then install the software. However, if you don’t have a User Guide you should refer to the following instructions.

Handling GrafixStar 600

Follow these guidelines to minimize the risk of electrostatic charges damaging the card:

Keep the card in its protective packaging until you are ready to install it.

Just before handling the card, touch the metal frame of your computer with the back of your hand to discharge any static electricity which may have built up on you or your clothes.

Hold the card by the adapter bracket or by its edges; do not touch the printed circuit board.

Do not place the card on top of your computer or on any other metal surface.

Make sure that the card is not accidentally touched by anyone else.

Handle the card with care to avoid damage to the fragile electronic components mounted on the circuit board.

Installing the hardware

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

1.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

2.	Disconnect your monitor cable from your computer.

3.	Remove the computer’s cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

4.	Remove your existing display card or disable your built-in graphics controller. Refer to the documentation supplied with your computer for instructions on how to disable a built-in graphics controller.

5.	Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

6.	Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

7.	Align GrafixStar 600 with the slot you have chosen. Firmly press the card into the slot. You will need to use some pressure to push the card into position, but do not use excessive force.

8.	Secure GrafixStar 600 using the screw that you removed previously.

9.	Replace the cover of your computer.

10.	Connect the monitor cable to GrafixStar 600. All cables connected to GrafixStar 600 should be properly shielded and comply with radio frequency interference standards.

11.	Switch on your computer and your computer monitor. You are now ready to install the software.

Installing the software

The easiest way to install the software is to use the Display Properties window. Installation varies slightly depending upon which build of Windows 95 you are using. Use the following steps to find out which build of Windows 95 you are using, then follow the instructions for that version.

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the System icon to open the System Properties Window.

3.	Ensure that the General tab is selected. This page has details about the System, Registration information, and Computer details. The build number is located in the System section, under the ‘Microsoft Windows 95’ line.

Software installation for Windows 95 build 4.00.950 or 4.00.950a

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Change Display Type button.

5.	Make sure that the Monitor Type line contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

6.	Click the Change button for the Adapter Type.

7.	Insert the GrafixStar 600 drivers disk into your computer and click Have Disk from the Select Device menu.

8.	Click GrafixStar 600 GrafixMAX from the list. Click OK.

9.	When the GrafixStar files have been copied to your PC, the Change Display Type window is displayed again. Click Close.

10.	The Display Properties windows is displayed again. Click Apply.

11.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

Software installation for Windows 95 build 4.00.950 B (otherwise known as OSR2)

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Advanced Properties button.

5.	Click the Monitor tab. The Monitor page is displayed.

6.	Make sure that the Monitor page contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

7.	Click the Adapter tab. The Adapter page is displayed.

8.	Click the Change button to change display driver.

9.	Insert the GrafixStar 600 drivers disk into your computer and click Have Disk from the Select Device menu.

10.	Click GrafixStar 600 GrafixMAX from the list. Click OK.

11.	When the GrafixStar files have been copied to your PC, the Advanced Display Properties window is displayed again. Click Close.

12.	The Display Properties windows is displayed again. Click Apply.

13.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

4. Troubleshooting

Introduction

This section describes problems you may encounter when running GrafixStar 600.

Monitors

Display sizes

Windows 95 does not restrict the generic SVGA monitor settings to a given set of display sizes, so  allows any display size to be selected. If you have a standard monitor selected, there is therefore a possibility that you could select a display size that your monitor does not support.

Choose the correct monitor type, or contact your monitor supplier for more information.

Corrupt or blank display

Windows 95 does not restrict the generic SVGA monitor settings to a given set of refresh rates, so allows any refresh rate to be selected. If you have a standard monitor selected, there is therefore a possibility that you could restart your system with a refresh rate that your monitor does not support. This usually results in a corrupted or black display in Windows 95. To rectify this problem, restart Windows 95, holding down the CTRL key on your keyboard. This starts Windows 95 in the lowest possible refresh rate for the resolution selected.

Connectivity - Feature connector

Feature connector

If you select Feature Connector from the Advanced page of Display Properties menu, your CONFIG.SYS file is modified to include the TSR VLENABLE.SYS. VLENABLE is a utility that enables DAC snooping when you turn on your PC.

If your system does not have a CONFIG.SYS file, or it has one that is zero bytes in size, you must create a file called CONFIG.SYS at the root of your hard disk and make sure that it contains some data (a rem line is sufficient), before you select Feature Connector.

When you have selected Feature Connector you must restart your system to run VLENABLE.

If you uncheck the Feature Connector option, the VLENABLE.SYS line in the CONFIG.SYS file will be disabled by changing it to a rem line.

Drivers

QuickTime for Windows Version 2.X

If you experience problems when viewing QuickTime movies, run the QuickTime Control Panel from the Windows 95 Control Panel, and follow the steps below:

1.	Run the QuickTime Control Panel, click the More button and click the Video tab.

2.	From within the Optimization section, the Draw Method setting will be set to Video Hardware. Change this to Video Driver.

3.	Close the QuickTime Control Panel.

Playing AVI with MS PLUS! pack

If you are using a display mode which is capable of video acceleration and you have the Show Window Contents While Dragging option enabled, you will see the accelerated video displaced from the video window while dragging the video window around the desktop. Disable the Show Window Contents While Dragging on the PLUS page in Display Properties.

Monitor screen centering

Some advanced monitors create a display setting by detecting the horizontal line frequency, and this may change the size and positioning of the Windows 95 screen when task switching from a DOS full-screen session to a Windows 95 session. This problem occurs when the monitor is at the borderline between two settings. To remedy this, use the controls on your monitor to adjust its screen settings.

Quickres

Quickres is part of a series of unsupported utilities provided by Microsoft. The mechanism used by Quickres to achieve color-depth switching on the fly can reduce GrafixStar 600 performance, and make Windows 95 unstable. We recommend that you do not use this utility.

Windows 95 OEM Service Release 2 also supports color-depth switching on the fly. GrafixStar 600 will support this feature but you might notice some color corruption. You can cure any color corruption by restarting your computer.

Xing 3.X

MPEG clips replayed in modes that are not video accelerated may cause Windows to become unstable. If this problem occurs, change the display mode to one that supports video acceleration.

Windows 95 Screensavers

If you are using the 'Mystify your Mind' Windows screensaver with the password option enabled, you may see additional lines appear when the password entry window is active. We recommend that you choose another screensaver if you are using password protection.

If you are using 'Curves and Colors' in 65k colors, you may see some color corruption. Select another color depth, or use another screensaver.

Installing Windows 95 with a GrafixStar 600 card

If you are installing Windows 95 for the first time from DOS, the Windows 95 auto-detection process will Plug'n'Play the GrafixStar 600 as an ET4000 Display adapter and an 'Other device: PCI VGA-Compatible display adapter'.

Although this does not affect GrafixStar 600 installation or performance, you may want to remove the section 'Other device'. To do this, delete both the 'Other devices' and the GrafixStar 600 entry in the 'Display adapter' section. After this, Windows 95 will Plug'n'Play the Grafixstar 600 on the next restart, producing a single entry for the display adapter.

Adobe Type Manager

If Adobe Type Manager is installed with Windows 95, GrafixStar 600 performance may be degraded.

To maximize performance, make the following changes to your SYSTEM.INI file:

[Boot]

;system.drv=atmsys.drv

;atm.system.drv=system.drv

system.drv=system.drv

Harvard Graphics 4.0

Running Harvard Graphics 4.0 for Windows in a 24-bit color mode (16.7m colors) causes a series of errors. Use a 16-bit (65k colors) mode instead.

Dorling Kindersley

On launching 'The Ultimate Human Body' or 'The Encyclopedia of Science' in 16-bit (65k colors) and 24-bit (16.7m colors) resolutions, you may find that the window background color is incorrect.

In addition, when scrolling the window over the body in the 'The Ultimate Human Body', the main window showing the zoomed portion of the selected window may not show the complete image.

Select an 8-bit mode (256 colors) for running these applications.

11th Hour

The 11th Hour game does not come with drivers that are capable of running with the GrafixStar 600. You can use drivers provide by SciTech called SciTech Display Doctor 5.3 which, when installed, allow the use of a later revision of UniVBE driver which is compatible with the chipset on the GrafixStar 600.

Duke Nukem 3D (shareware)

When starting the shareware version of Duke Nukem 3D, the first screen may show display corruption. This will not stop you playing the game but can be remedied by the use of UniVBE from the SciTech Display Doctor 5.3, or by obtaining a software BIOS TSR available from our online services.

cc:Mail v6.0

When using applications with cc:Mail v6 in the background, icons within Windows 95 and other active applications may be drawn incorrectly and, in extreme cases, system integrity problems may occur. This does not happen with earlier versions of cc:Mail.

ActiveMovie

When displaying accelerated overlay video with ActiveMovie in certain display modes, the video may become noisy when moving the video window around the desktop, or by resizing the video window.

Choose an alternative display mode.