*****************************************

*                                       *

*       MIC System - Release Note       *

*                                       *

*       Video Attributes Commands       *

*                                       *

*             Version 1.1               *

*                                       *

*****************************************

*************************  I M P O R T A N T  *************************

INSTALLATION

------------

This patch is only to be used with MIC System I v4.0 and v4.0.1.

It will not work with later versions of MIC.

Install the software using the standard AMS installation procedure :-

- Insert the distribution diskette in a floppy drive.

- from the AMS Main Menu, select the "Install an AMS Application"

option.

- Specify the source drive as the floppy drive containing the

diskette. Specify the destination as the drive on which the MIC

system is installed (should be C:)

USAGE

-----

This patch is provided to allow you to control the contrast, brightness

and saturation of your video image from within an MIC application. It

includes a new command module (MIC-ATT.EXE) which provides a new MIC

command group (ATTRIBUTE).

The installation procedure copies this module along with other necessary

files into your \MICFILES directory. It creates a file called

M30EXTRA.CNF (or adds a line to your existing M30EXTRA.CNF) which means

that the ATTRIBUTE command will be loaded into memory each time MIC is

loaded.

Should you wish to selectively load the module, delete the line

C:\MICFILES\MIC-ATT.EXE

from M30EXTRA.CNF, or delete the complete file if it is the only line.

Then to load the module, simply run the MIC-ATT.EXE program immediately

after loading MIC.

NEW COMMANDS

------------

The commands made available are...

ATTRIBUTE CONTRAST CLevel

ATTRIBUTE SATURATION SLevel

ATTRIBUTE BRIGHTNESS Input BLevel Hue

Where CLevel, SLevel and BLevel are values between 0 (minimum) and

255 (maximum), Input specifies the video source and must be 1 or 2,

and Hue is a value between -128 and +127.

All words can be abbrieviated to 3 characters.