�VIDEO CRUNCH IT 2000 





February 97


Non-linear Digital Video-Editing


�
Realtime Video Digitizer with TrueColor S-VHS / HI-8 Quality


Full NTSC or PAL / SECAM Resolutions (640 x 480 / 768 x 576)


Realtime M-JPEG Compression


Standard YUV 4:2:2 Format


Variable Compression-Rate


Y/C (S-Video) and Composite Video Inputs and Outputs


PCI Bus, jumperless Installation


�



Hardware





High performance PCI design for true S-Video quality


ZORAN ZR 36050 / 36055 Motion JPEG video processor


Video encoder & decoder (video-editing)


High performance 32-bit PCI 2.1 design using Philips SAA7145


Full NTSC / PAL / SECAM (640 x 480 / 768 x 576 pixels) resolution at full frame- / field-rate (30 / 25 frames or 60 / 50 fields), true-color


Cropping feature. The size of the frames to be captured can be adjusted freely.


Video-overlay using S3 Scenic Highway Bus (LPB)


Software-controlled video-filters for better quality at high compression-ratios


Bit-rate control for a constant data-stream without JPEG-peaks


Variable compression ratios from 1:4 (1:8) to 1:100


Video inputs:


1 composite video (FBAS) - Cinch / RCA connector


1 Y/C (S-VHS, Hi-8) - 4 Pin Mini-DIN connector


Video outputs:


1 composite Video (FBAS) - Cinch / RCA connector


1 Y/C (S-VHS, Hi-8) - 4 Pin Mini-DIN connector


S3 Scenic Highway (LPB) connector for DIAMOND graphics-boards with S3 Scenic Highway (LPB) connector


Adjustable Brightness, Contrast, Color and Hue















































Software





High performance video & audio buffering drivers for high data rates and lip-synch audio with standard sound cards. These drivers support Windows( 95 and Windows( 3.1x


Professional Video-editing software for capturing still images and live video, titling, editing, audio. Available in English, French, German (including. manual)


Supports all common PC-based video-editing software, like Adobe Premiere, Avid, Asymetrix DVP, Ulead MediaStudio, Software Dynamics Video Maestro etc.


Supports all software that can make use of Microsoft Video for Windows MCI-commands





System requirements





Pentium 90 processor equipped  system             (100 % compatible to PC standard)


16 MB of RAM


One free PCI 2.1 slot


Windows( 3.1x or Windows( 95 operating system


16 MB of free hard-drive space for applications


(Recommended) an extra (fast) hard-drive for recording and playback of video-sequences


(Recommended) A DIAMOND graphics-board (for example Stealth Video 2001) with S3 Scenic Highway (LPB) connector for video overlay.





Miscellaneous





Printed documentation in either English, French, or German. 


On-line documentation (SuperCD) in five languages (English, French, German, Italian, and Spanish)





�
Service & Support


5 years warranty 


Free Software updates via BBS, Internet (www.diamondmm.com), and CompuServe ( GO Diamond)


Fax-on-Demand System


Technical support through Hotline








Video Modes





Resolution*	Standards	Fields/s		Frames/s	Farben





768 x 576	PAL / SECAM**	max. 50		max. 25		TrueColour


768 x 288	PAL / SECAM**	max. 50		max. 25		TrueColour


384 x 576	PAL / SECAM**	max. 50		max. 25		TrueColour


384 x 288	PAL / SECAM**	max. 50		max. 25		TrueColour


640 x 480	NTSC		max. 60		max. 30		TrueColour


640 x 240	NTSC		max. 60		max. 30		TrueColour


320 x 480	NTSC		max. 60		max. 30		TrueColour


320 x 240	NTSC		max. 60		max. 30		TrueColour





*	Cropping possible


**	SECAM supported only on inputs








�
Germany: 


DIAMOND Multimedia Systems


Moosstraße 18 b, 


D-82319 Starnberg 








Fax: ++49 8151 21258�UK:


Diamond Multimedia Systems Ltd.


610 Wharfedale Road


Winnersh Triangle


Wokingham


Berkshire RG41 5TY  UK


Fax: ++44 1189 444401�France:


Diamond Multimedia Southern Europe


6, Blvd. Général Leclerc - Bat C - 15iéme Etage


Clichy 92115 - FRANCE


Fax: 33-1-47561139


�






Copyright


©  Diamond Multimedia Systems, Inc.  Subject to technical alterations





Trademarks


 (() The brands or product names mentioned are/may be trademarks or registered trademarks of their respective holders.





�



�SEITE  �








DB  � DATEINAME  \* FORMATVERBINDEN �Vci2000e.doc� � ZEIT \@ "tt.MM.jjjj" �12.02.1997�





DB  � DATEINAME  \* FORMATVERBINDEN �Vci2000e.doc� � ZEIT \@ "tt.MM.jjjj" �12.02.1997�








�














